3GPP TSG-RAN WG3 Meeting #125	R3-244575

Maastricht, Netherlands, Aug 19th – 23th, 2024

Agenda Item:	10.3.1

Source:	CMCC

Title:	(TP to 38.300) Discussion on SON/MDT enhancement for NTN

Document for:	Discussion and Decision

# 1 Introduction

In RAN3#124 meeting, there was discussion on the scenarios of SON/MDT enhancement for Intra-NTN mobility and the agreements were achieved as below:

Take the connection failure scenarios in §15.5.2.2.2 of TS 38.300 as starting point with necessary update, if needed, in MRO for intra-NTN mobility.

MRO for intra-NTN mobility shall consider the following trigger conditions:

-		Measurement-based trigger condition;

-		A time-based trigger condition;

-		A location-based trigger condition;

-		A time-based and Measurement-based combined trigger condition;

-		A location-based and Measurement-based combined trigger condition;

Do not consider MRO for feeder link switch.

MRO for intra-NTN mobility shall consider both conditional and non-conditional intra-NTN handover.

And the following issues were captured and need to be continued in next meeting:

FFS on whether there are any enhancements to RLF report due to support for multiple trigger conditions in intra-NTN mobility.

FFS on whether to consider any enhancement for MDT configuration e.g., area scope and trigger condition for NTN.

Discuss MRO for RACH-less HO of NTN, only for those different issues identified compared with MRO for LTM.

Time information when RLF occurs needs to be included in the RLF report, other information to be included can be checked as well in next meeting.

In this contribution, we would discuss on the issues of MRO mechanisms for intra-NTN and provide some proposals.

# 2 Discussion

FFS on whether there are any enhancements to RLF report due to support for multiple trigger conditions in intra-NTN mobility.

In case of failed mobility scenario of Intra-NTN mobility, we think RLF report should be enhanced to support information from UE to detect the problems in the related configuration in intra-NTN mobility, especially for multiple trigger conditions in intra-NTN mobility.

Based on the agreements achieved in RAN3#124 meeting, we can see that there are two cases for multiple trigger conditions configured:

-		A time-based and Measurement-based combined trigger condition;

-		A location-based and Measurement-based combined trigger condition;

Firstly, the trigger condition(s) fulfilled in NTN before the failure occurs should be considered in enhancements to RLF report. Let’s take the time-based and Measurement-based combined trigger condition as an example, there are several possible scenarios that cause the connection failure:

- Both the time-based trigger condition and the Measurement-based trigger condition are fulfilled;

- The time-based trigger condition is fulfilled but the Measurement-based trigger condition is not fulfilled;

- The Measurement-based trigger condition is fulfilled but the time-based trigger condition is not fulfilled;

- Both the time-based trigger condition and the Measurement-based trigger condition are not fulfilled.

We think with the information about the trigger condition(s) fulfilled before the failure occurs, NW can identify whether there are trigger condition(s) that cannot be fulfilled or fulfilled too late, or whether there are trigger condition(s) that are fulfilled too early.

This information could be the sequence of the fulfilled trigger condition, e.g., the trigger condition(s) fulfilled in chronological order; or the time information of the fulfilled trigger condition. And this information could be applicable for both a time-based and Measurement-based combined trigger condition and a location-based and Measurement-based combined trigger condition.

Observation 1: With the information about the trigger condition(s) fulfilled before the failure occurs, NW can identify whether there are trigger conditions that cannot be satisfied or satisfied too late, or whether there are CHO trigger conditions that are satisfied too early.

Secondly, we think the information about the distance between UE and the referenceLocation/moving referenceLocation in serving cell and in candidate cell respectively when the failure occurs should be considered in enhancements to RLF report. Since it is helpful to analysis whether the distance threshold configured in location-based trigger condition have the potential problem. And this information could be applicable for a location-based trigger condition or a location-based and Measurement-based combined trigger condition.

Observation 2: The information about the distance between UE and the referenceLocation/moving referenceLocation in serving cell and the distance between UE and the referenceLocation/moving referenceLocation in candidate cell when the failed CHO occurs is helpful to analysis whether the distance threshold configured in location-based trigger condition have the potential problem.

Besides that, from our point of view, the configured trigger condition(s) should be introduced into RLF report, since it could help network to identify the potential problem trigger condition(s). This information could be applicable for all scenarios including only one configured trigger condition and multiple trigger conditions.

Observation 3: The information of the configured trigger condition(s) could help network to identify the potential problem trigger condition(s).

Proposal 1: The following information reported by UE could be considered in enhancements to RLF report to optimize the configuration of trigger conditions in NTN:

- The sequence of the fulfilled trigger condition(s) for multiple trigger conditions configured in intra-NTN mobility, e.g.,

  - the trigger condition(s) fulfilled in chronological order; or

  - the time information of the fulfilled trigger condition(s);

- The distance between UE and the referenceLocation/moving referenceLocation in serving cell and the distance between UE and the referenceLocation/moving referenceLocation in candidate cell when the failure occurs for a location-based trigger condition or a location-based and Measurement-based combined trigger condition.

- The configured trigger condition(s), which is applicable for both only one configured trigger condition and multiple trigger conditions.

That means for the scenario where a time-based and Measurement-based combined trigger condition configured, the following information should be included in RLF report:

  - The sequence of the fulfilled trigger condition(s);

And for the scenario where a location-based and Measurement-based combined trigger condition configured, the following information should be included in RLF report:

  - The sequence of the fulfilled trigger condition(s);

  - The distance between UE and the referenceLocation/moving referenceLocation in serving cell and the distance between UE and the referenceLocation/moving referenceLocation in candidate cell when the failure occurs.

Proposal 2: For single trigger condition in intra-NTN mobility, RLF report could be enhanced to include the information from UE for the detection of the problems in the related configuration of NTN:

- for the scenario where Measurement-based trigger condition or a time-based trigger condition configured, the following information should be included in RLF report:

  - The configured trigger condition;

- for the scenario where a location-based configured, the following information should be included in RLF report:

  - The distance between UE and the referenceLocation/moving referenceLocation in serving cell and the distance between UE and the referenceLocation/moving referenceLocation in candidate cell when the failure occurs;

  - The configured trigger condition;

Proposal 3: For multiple trigger conditions in intra-NTN mobility, RLF report could be enhanced to include the information from UE for the detection of the problems in the related configuration of NTN:

- for the scenario where a time-based and Measurement-based combined trigger condition configured, the following information should be included in RLF report:

  - The sequence of the fulfilled trigger condition(s);

  - The configured trigger condition(s);

- for the scenario where a location-based and Measurement-based combined trigger condition configured, the following information should be included in RLF report:

  - The sequence of the fulfilled trigger condition(s);

  - The distance between UE and the referenceLocation/moving referenceLocation in serving cell and the distance between UE and the referenceLocation/moving referenceLocation in candidate cell when the failure occurs;

  - The configured trigger condition(s).

For connection failure due to intra-system mobility, there are three kinds of problems are defined in [2] as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

We think these problems are also applicable for CHO of Intra-NTN mobility. In case of CHO of Intra-NTN mobility, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell in Intra-NTN.

Proposal 4: The existing definition of the Too Late Handover, Too Early Handover and Handover to Wrong Cell could be reused for CHO of Intra-NTN mobility.

Proposal 5: The objective of failed mobility is to optimize configuration issues for Duration in the time-based trigger condition or distance thresholds/timeToTrigger in location-based trigger condition or the Thresholds in CHO events A3/A4/A5.

Based on above discussion, the corresponding TP for TS38.300 is attached in the Annex.

# 3 Conclusion

In this paper, we discuss on MRO mechanisms for intra-NTN mobility, we have the following observations and proposals:

Observation 1: With the information about the trigger condition(s) fulfilled before the failure occurs, NW can identify whether there are trigger conditions that cannot be satisfied or satisfied too late, or whether there are CHO trigger conditions that are satisfied too early.

Observation 2: The information about the distance between UE and the referenceLocation/moving referenceLocation in serving cell and the distance between UE and the referenceLocation/moving referenceLocation in candidate cell when the failed CHO occurs is helpful to analysis whether the distance threshold configured in location-based trigger condition have the potential problem.

Observation 3: The information of the configured trigger condition(s) could help network to identify the potential problem trigger condition(s).

Proposal 1: The following information reported by UE could be considered in enhancements to RLF report to optimize the configuration of trigger conditions in NTN:

- The sequence of the fulfilled trigger condition(s) for multiple trigger conditions configured in intra-NTN mobility, e.g.,

  - the trigger condition(s) fulfilled in chronological order; or

  - the time information of the fulfilled trigger condition(s);

- The distance between UE and the referenceLocation/moving referenceLocation in serving cell and the distance between UE and the referenceLocation/moving referenceLocation in candidate cell when the failure occurs for a location-based trigger condition or a location-based and Measurement-based combined trigger condition.

- The configured trigger condition(s), which is applicable for both only one configured trigger condition and multiple trigger conditions.

Proposal 2: For single trigger condition in intra-NTN mobility, RLF report could be enhanced to include the information from UE for the detection of the problems in the related configuration of NTN:

- for the scenario where Measurement-based trigger condition or a time-based trigger condition configured, the following information should be included in RLF report:

  - The configured trigger condition;

- for the scenario where a location-based configured, the following information should be included in RLF report:

  - The distance between UE and the referenceLocation/moving referenceLocation in serving cell and the distance between UE and the referenceLocation/moving referenceLocation in candidate cell when the failure occurs;

  - The configured trigger condition;

Proposal 3: For multiple trigger conditions in intra-NTN mobility, RLF report could be enhanced to include the information from UE for the detection of the problems in the related configuration of NTN:

- for the scenario where a time-based and Measurement-based combined trigger condition configured, the following information should be included in RLF report:

  - The sequence of the fulfilled trigger condition(s);

  - The configured trigger condition(s);

- for the scenario where a location-based and Measurement-based combined trigger condition configured, the following information should be included in RLF report:

  - The sequence of the fulfilled trigger condition(s);

  - The distance between UE and the referenceLocation/moving referenceLocation in serving cell and the distance between UE and the referenceLocation/moving referenceLocation in candidate cell when the failure occurs;

  - The configured trigger condition(s).

Proposal 4: The existing definition of the Too Late Handover, Too Early Handover and Handover to Wrong Cell could be reused for CHO of Intra-NTN mobility.

Proposal 5: The objective of failed mobility is to optimize configuration issues for Duration in the time-based trigger condition or distance thresholds/timeToTrigger in location-based trigger condition or the Thresholds in CHO events A3/A4/A5.

# Reference

- RAN3_124_agenda_20240524_eom, May 2024.

- 3GPP TS 38.300: " NR; NR and NG-RAN Overall Description".

# Annex: TP for TS38.300

15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

<<<<<< Start of CHANGES >>>>>>

In case of Intra-NTN mobility, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means the Too Late Handover, Too Early Handover and Handover to Wrong Cell for Intra-NTN mobility, and Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell for Intra-NTN mobility.

<<<<<< End of CHANGES >>>>>>