3GPP TSG-RAN WG3 Meeting #125	             R3-244577

Maastricht, NL, 19th – 23th Aug 2024

Agenda Item:	17.3

Source:	CMCC

Title:	Support On-Demand SIB1 for UEs

Document for:	Discussion

Introduction

In the RAN3 last # 124 meeting, it is agreed [1]:

Focus on case 2 in RAN3 first. The standard impact on Case 2: Xn interface: - Send UL WUS configuration from NES-cell to cell A - On-demand SIB1 operation activation/deactivation from NES-cell to cell A? F1 interface: - UL WUS configuration from NES gNB-DU to NES gNB-CU - UL WUS configuration from cell A gNB-CU to cell A gNB-DU - On-demand SIB1 operation activation/deactivation from CU to DU? Which node decides the WUS configuration and On-demand SIB1 mode? DU or CU or OAM? NES-cell gNB decides on-demand SIB1 operation activation and deactivation?

Discussions

Figure 1 depicts the Case 2: the NES cell sends UL WUS Configuration to the Cell A before switching to the on-demand SIB1 operation mode (step 0). The Cell A periodically broadcasts on-demand SIB1 request configuration of the NES cell to UEs in step 1. The UE triggers the on-demand SIB1 on the NES cell by sending the UL WUS signal directly to the NES cell in step 2. The NES cell sends the on-demand SIB1 information to the UE in step 3. This approach minimizes the delay for the UE to connect to the NES cell but may result in less energy savings for the NES cell.

Figure 1 On-Demand SIB1 of NES Cell, Case 2

It is required that the UL WUS configuration is sent from a NES Cell to a Cell A via Xn, e.g., before entering on-demand SIB1 mode in the NES Cell or when the UL WUS configuration is changed.

If the CU/DU split architecture is adopted in gNB, a NES cell and Cell A could be in different DU(s). In order to transit the UL WUS configuration, both F1 interface and Xn interface could be affected.

- The NES cell and Cell A are on the same gNB, but on different DU(s). gNB-CU is in the middle to forward the UL WUS configuration between two gNB-DUs on the same gNB.

- The NES cell and Cell A are on different gNB(s). Two gNB-CU(s) are in the middle to forward the UL WUS configuration between two gNB-DUs on different gNB(s).

On the other hand, as discussed in [2], if a NES cell returns to its active mode and broadcasts its SIB1 periodically, Cell A no longer needs to broadcast the OD-SIB1 request configuration for the NES cell. If the NES cell reverts from the active mode to the on-demand SIB1 operation mode again, the Cell A must broadcast the OD-SIB1 request configuration for the NES cell once more. Thus, UL WUS configuration could be started and stopped by the NES cell based on its mode change and the Cell A needs to be aware of the mode change of the NES Cell.

When the Cell A receives UL WUS configuration from the NES Cell, it might acknowledge the receipt of the message to the NES cell, indicating its acceptance to broadcast the OD-SIB1 request configuration for the NES cell. The NES cell then can safely and immediately enter the on-demand SIB1 operation mode.

Figure 2 shows the procedure in the CU/DU split architecture in details, in which the NES cell and Cell A are on different gNB(s). The NES Cell gNB is the gNB hosting the NES Cell and the Cell A gNB is the gNB hosting the Cell A.

Figure 2 On-Demand SIB1 Operation Procedure, Case 2

For both Xn and F1 interfaces, a new IE needs to be defined to carry UL WUS configuration between NES cell and Cell A.

The UL WUS configuration can be started and stopped by the NES cell based on its status change. The UL WUS configuration is associated with an “OD-SIB1 Request Configuration” broadcast START/STOP indicator.

The Cell A acknowledges the receipt of UL WUS configuration sent from the NES cell, which might include an acceptance or rejection response.

Which node decides the WUS configuration and On-demand SIB1 mode? DU or CU or OAM?

As agreed in RAN2, the UL WUS configuration includes at least RACH configuration. DU is the entity that obtains the RACH configuration. Thus, the UL WUS configuration should be determined/provided by the DU.  As shown in Figure 2, the CU of the NES Cell gNB could configure which DUs are included in the OD-SIB1 operation list. Before the OD-SIB1 operation mode is activated/started at an allowed DU of the NES Cell gNB or its UL WUS configuration changes, it initiates the UL WUS Configuration associated with OD-SIB1 request configuration broadcast start/stop indicator to the CU of the NES Cell gNB (it means that the UL WUS Configuration could also be initiated by the DU of the NES Cell gNB when it leaves the OD-SIB1 operation mode). The CU of the NES Cell gNB sends the UL WUS Configuration associated with OD-SIB1 request configuration broadcast start/stop indicator to the CU of the Cell A gNB. The CU of the Cell A gNB in turn sends the UL WUS Configuration associated with OD-SIB1 request configuration broadcast start/stop indicator to the DU of the Cell A gNB. After acceptance, the DU of the Cell A gNB starts to broadcast OD-SIB1 request configuration for the DU of the NES Cell gNB. After receiving the acknowledgement from the CU of the NES Cell gNB, the DU of the NES Cell gNB could safely and immediately enter OD-SIB1 Operation mode.

The UL WUS configuration is determined/provided by the NES Cell.

In the CU/DU split architecture, the UL WUS configuration is determined/provided by the NES gNB-DU.

The Cell A starts to broadcast the OD-SIB1 request configuration for the NES Cell after receiving the UL WUS configuration with OD-SIB1 request configuration START indicator from the NES Cell.

In the DU/CU split architecture, the DU of Cell A gNB starts to broadcast the OD-SIB1 request configuration for the DU of the NES Cell gNB after receiving the UL WUS configuration with OD-SIB1 request configuration START indicator from the CU of the Cell A gNB through the F1 interface, which gets this information from the CU of the NES Cell gNB through Xn interface, which gets this information from the DU of the NES Cell gNB through F1 interface.

Which node decides to enter the on-demand SIB1 mode?

The CU manages signalling between the network and the UE, including connection setup, release, mobility management, and security functions. And it has more global picture of the neighbouring cell deployment, inference level, signal strength and quality, energy consumption, power status, etc.  The CU can be helpful by providing information before the DU initiates the on-demand SIB1 operation for some NES cells. For instance, the CU could supply a list of cells where the on-demand SIB1 operation should be enabled.

The CU of the NES Cell could provide some assistance information (a list of cells where the on-demand SIB1 operation should be enabled) before a DU initiates the OD SIB1 operation.

On-demand SIB1 operation activation and deactivation

As discussed earlier, OD SIB1 operation of a NES cell could be stopped/deactivated. The NES cell enters the active mode by broadcasting its SIB1 by itself. The Cell A should stop broadcasting the OD-SIB1 request configuration for the NES Cell.

The NES cell could initiate the change between active mode and OD SIB1 operation mode (i.e., OD SIB1 operation activation and deactivation).

In the CU/DU split architecture, the NES gNB-DU could initiate the change between active mode and on-demand SIB1 operation mode mode (i.e., OD SIB1 operation activation and deactivation). The NES gNB-DU notifies the NES gNB-CU its mode change by UL WUS Configuration associated with OD-SIB1 request configuration broadcast start/stop indicator, which in turn notifies the Cell A gNB-CU, then Cell A gNB-DU.

The Cell A stops broadcasting the OD-SIB1 request configuration for the NES Cell A after receiving the UL WUS configuration with OD-SIB1 request configuration STOP indicator from the NES Cell.

In the DU/CU split architecture, the DU of Cell A gNB stops broadcasting the OD-SIB1 request configuration for the DU of the NES Cell gNB after receiving the UL WUS configuration with OD-SIB1 request configuration STOP indicator from the CU of the Cell A gNB through the F1 interface, which gets this information from the CU of the NES Cell gNB through Xn interface, which gets this information from the DU of the NES Cell gNB through F1 interface.

Cell A may request the NES cell to deactivate the on-demand SIB1 operation mode if it can no longer or chooses not to broadcast the OD-SIB1 request configuration for the NES cell. This may occur under conditions such as exceeding expected power consumption or being scheduled to enter energy-saving mode.

The Cell A could request the NES Cell to deactivate the on-demand SIB1 operation mode.

Conclusions

In this submission, we discussed the procedures of the OD-SIB1 Case 2 that is agreed to be focused on in RAN 3. The split CU-DU architecture is also being considered in the paper. Three major questions regarding Case 2 (might be applicable to other cases being discussed in RAN 1) are discussed in the paper: (1) Which node decides the WUS configuration and On-demand SIB1 mode? DU or CU or OAM? (2) Which node decides to enter the on-demand SIB1 mode? (3) On-demand SIB1 operation activation and deactivation. The following proposals are presented in the paper:

- For both Xn and F1 interfaces, a new IE needs to be defined to carry UL WUS configuration between NES cell and Cell A.

The UL WUS configuration can be started and stopped by the NES cell based on its status change. The UL WUS configuration is associated with an “OD-SIB1 Request Configuration” broadcast START/STOP indicator.

The Cell A acknowledges the receipt of UL WUS configuration sent from the NES cell, which might include an acceptance or rejection response.

The UL WUS configuration is determined/provided by the NES Cell.

In the CU/DU split architecture, the UL WUS configuration is determined/provided by the NES gNB-DU.

The Cell A starts to broadcast the OD-SIB1 request configuration for the NES Cell after receiving the UL WUS configuration with OD-SIB1 request configuration START indicator from the NES Cell.

In the DU/CU split architecture, the DU of Cell A gNB starts to broadcast the OD-SIB1 request configuration for the DU of the NES Cell gNB after receiving the UL WUS configuration with OD-SIB1 request configuration START indicator from the CU of the Cell A gNB through the F1 interface, which gets this information from the CU of the NES Cell gNB through Xn interface, which gets this information from the DU of the NES Cell gNB through F1 interface.

The CU of the NES Cell could provide some assistance information (a list of cells where the on-demand SIB1 operation should be enabled) before a DU initiates the OD SIB1 operation.

The NES cell could initiate the change between active mode and OD SIB1 operation mode (i.e., OD SIB1 operation activation and deactivation).

In the CU/DU split architecture, the NES gNB-DU could initiate the change between active mode and on-demand SIB1 operation mode mode (i.e., OD SIB1 operation activation and deactivation). The NES gNB-DU notifies the NES gNB-CU its mode change by UL WUS Configuration associated with OD-SIB1 request configuration broadcast start/stop indicator, which in turn notifies the Cell A gNB-CU, then Cell A gNB-DU.

The Cell A stops broadcasting the OD-SIB1 request configuration for the NES Cell A after receiving the UL WUS configuration with OD-SIB1 request configuration STOP indicator from the NES Cell.

In the DU/CU split architecture, the DU of Cell A gNB stops broadcasting the OD-SIB1 request configuration for the DU of the NES Cell gNB after receiving the UL WUS configuration with OD-SIB1 request configuration STOP indicator from the CU of the Cell A gNB through the F1 interface, which gets this information from the CU of the NES Cell gNB through Xn interface, which gets this information from the DU of the NES Cell gNB through F1 interface.

The Cell A could request the NES Cell to deactivate the on-demand SIB1 operation mode.

Reference

- draft_RAN3#124_MeetingReport_ver20240605

- R3-242057, CMCC, Discussion on on-demand SIB1 for idle inactive mode UEs