3GPP TSG-RAN WG3 Meeting #125	             R3-244579

Maastricht, NL, 19th – 23th Aug 2024

Agenda Item:	21.2

Source:	CMCC, Lenovo, Xiaomi

Title:	Further Discussion on XR Support in DC

Document for:	Discussion

1	Introduction

In the previous RAN3 # 124 meeting, the following has been agreed [1]:

[Agreement] For XR in NR-DC, non-homogenous deployment is possible similar like other features in DC. Turn following WA to agreement: WA: SN reports the PDU Set based Handling Indicator in S-NG-RAN node Addition Preparation procedure and M-NG-RAN node initiated S-NG-RAN node Modification Preparation procedure for the MN-terminated SCG bearer, SN-terminated MCG bearer and SN-terminated SCG bearer.

2	Discussions

2.1	PDU Set Based Handling

It agreed that For XR in NR-DC, non-homogenous deployment is possible similar like other features in DC.

Therefore, MN cannot assume MN has the capability of PDU set based handling as itself. In the MN initiated SN Addition procedure and SN node Modification procedure, if MN supports PDU Set based handling capability, MN shall notify this in SN Addition/Modification Request by including PDU Set QoS Parameters IE. On the other hand, SN shall notify its PDU set based handling capability in SN node Addition/Modification Acknowledge to MN through PDU Set Based Handling Indicator IE.

The MN and SN shall store this PDU Set Based Handling Indicator IE and use accordingly.

- Observation 1: Non-homogeneity of MN and SN in PDU Set Based Handling exists, MN and SN needs to notify each other whether PDU Set Based Handling is supported.

Turn the following WA to agreement:

- WA: SN reports the PDU Set based Handling Indicator in S-NG-RAN node Addition Preparation procedure and M-NG-RAN node initiated S-NG-RAN node Modification Preparation procedure for the MN-terminated SCG bearer, SN-terminated MCG bearer and SN-terminated SCG bearer.

PDU Set Based Handling Indicator IE is used to notify MN regarding SN’s PDU Set based handling capability in SN Addition/Modification Request Acknowledge.

For a QoS flow established with PDU Set QoS parameters, if the PDU Set based Handling Indicator IE set to "supported" is included in S-NODE ADDITION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, include the PDU Set information in the data to be forwarded.

2.2	PSI Discard Coordination

An LS (R3-243957) regarding PSI discard coordination issue in DC was sent to RAN2 at the end of the previous meeting. RAN3 may postpone the discussion until RAN2 provides a response.

RAN3 needs to resume the discussion on whether/how to support UL PSI discard coordination for the split bearer in DC after RAN2 returns the LS response.

3	Conclusion

This paper gives further discussions on the PDU set based handling, PSI discard coordination  in DC. The following observations and proposals are proposed:

- Observation 1: Non-homogeneity of MN and SN in PDU Set Based Handling exists, MN and SN needs to notify each other whether PDU Set Based Handling is supported.

- Turn the following WA to agreement:

- WA: SN reports the PDU Set based Handling Indicator in S-NG-RAN node Addition Preparation procedure and M-NG-RAN node initiated S-NG-RAN node Modification Preparation procedure for the MN-terminated SCG bearer, SN-terminated MCG bearer and SN-terminated SCG bearer.

PDU Set Based Handling Indicator IE is used to notify MN regarding SN’s PDU Set based handling capability in SN Addition/Modification Request Acknowledge.

For a QoS flow established with PDU Set QoS parameters, if the PDU Set based Handling Indicator IE set to "supported" is included in S-NODE ADDITION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, include the PDU Set information in the data to be forwarded.

RAN3 needs to resume the discussion on whether/how to support UL PSI discard coordination for the split bearer in DC after RAN2 returns the LS response.

4	References

[1] draft_RAN3#124_MeetingReport_ver20240605