3GPP TSG-RAN WG3 Meeting #125	             R3-244580

Maastricht, NL, 19th – 23th Aug 2024

Agenda Item:	21.1

Source:	CMCC

Title:	Discussion on SA2’s LS on FS_XRM Ph2

Document for:	Discussion

- Introduction

RAN3 received an LS (S2-2407351) [1] from SA2.  SA2 has started to evaluate solutions for key issues for the study of XR and Media Services Ph2 (TR 23.700-70). This paper replies the related questions contained in this LS for XRM Ph2 from RAN3 perspective:

- Question1 [for SA4, RAN2 and RAN3]:

  - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach.

  - To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension?

- Question2 [for SA4 and RAN2]: There is some discussion about time to next burst.

  - To SA4: is it possible that the application server provides the time to next burst (i.e. the time interval between the current burst and the next burst) in the 1st packet of the burst via N6?

  - Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server?

- Question3 [for RAN3]: SA2 discusses to support available data rate exposure for GBR QoS Flow via user plane, and kindly asks RAN3 to provide feedback if any.

- Discussions

2.1	Dynamic Changes of Periodicity

In this section, we discuss our reponse to the following question regarding the support of dynamic changes of the periodicity.

- Question1 [for SA4, RAN2 and RAN3]: - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach.

In the current specification, the Time Sensitive Communications (TSC) Assistance Information (TSCAI) describes traffic characteristics that may be provided for use by the gNB, to allow more efficiently scheduled radio resources for periodic traffic and applying to PDU session type Ethernet and IP. Thus, TSCAI is used to provide the periodicity of a QoS flow for semi-static resource scheduling and/or configuration. If the dynamic changes of the periodicity are slow, the existing TSCAI mechanism is sufficient to provide the periodicity information to RAN, and the value can be updated when the periodicity changes. If the dynamic changes of the periodicity are fast, updating TSCAI via CP signaling could be inefficient or not fast enough to provide the latest periodicity information to RAN. RAN3 would like SA2 provide more information and use cases on the fast dynamic changes of the periodicity.  And if the periodicity changes dynamically in a fast pace, whether the periodicity still exists. In-band signalling (i.e. in GTP-U) could ensure that periodicity information is delivered alongside user data, providing immediate updates when needed.

- Observation 1: The existing TSCAI mechanism is sufficient to provide the periodicity information to RAN if the periodicity change is slow. If the dynamic changes of the periodicity are fast, updating TSCAI via CP signaling could be inefficient or not fast enough to provide the latest periodicity information to RAN.

- Proposal 1: RAN3 would like SA2 to further clarify the use cases on the fast dynamic changes of the periodicity, whether the periodicity still exists.

- Proposal 2: RAN3 thinks using in-band signaling in GTP-U for updating dynamic and fast periodicity change information is a practical approach to ensure timely and efficient updates.

2.2	Data Rate Exposure for GBR QoS Flow

In this section, we discuss our reponse to the following question regarding the support of available data rate exposure for GBR QoS Flow via user plane.

- Question3 [for RAN3]: SA2 discusses to support available data rate exposure for GBR QoS Flow via user plane, and kindly asks RAN3 to provide feedback if any.

This issue has already been discussed in RAN3 at the last meeting. RAN3 has replied it is feasible to estimate available data rate for GBR QoS flows, which is up to RAN implementation [2]. The detailed reply is as below.

[RAN3 answer to Question3]: RAN3 thinks that it is feasible to estimate available data rate for GBR QoS flows, which is up to RAN implementation. Regarding the report of available data rate, companies have different views on how to provide it to core network (e.g., via CP or UP). There is no consensus on whether it is feasible for NG-RAN to estimate available data rate for non-GBR QoS flows. Some companies in RAN3 would like to clarify the usage of available data rate for GBR QoS flows.

Regarding the question of supporting available data rate exposure for GBR QoS Flow via user plane, from RAN3 perspective, reporting such information via use plane by for example including it in GTP-U is feasible. From our view, RAN3 will coordinate with SA2's efforts to support the exposure of available data rates for GBR QoS flows via the user plane.

- Proposal 3: RAN3 thinks reporting data rate for GBR QoS flows via use plane by for example including it in GTP-U is feasible.

- Conclusion

In this paper, we dicussed on our responses to the SA2 LS (S2-2407351). This paper also provides draft reply LS in annex.

The following observaitons and proprosals are presented:

- Observation 1: The existing TSCAI mechanism is sufficient to provide the periodicity information to RAN if the periodicity change is slow. If the dynamic changes of the periodicity are fast, updating TSCAI via CP signaling could be inefficient or not fast enough to provide the latest periodicity information to RAN.

- Proposal 1: RAN3 would like SA2 to further clarify the use cases on the fast dynamic changes of the periodicity, whether the periodicity still exists.

- Proposal 2: RAN3 thinks using in-band signaling in GTP-U for updating dynamic and fast periodicity change information is a practical approach to ensure timely and efficient updates.

- Proposal 3: RAN3 thinks reporting data rate for GBR QoS flows via use plane by for example including it in GTP-U is feasible.

- Reference

[1] S2-2407351, LS on FS_XRM Ph2.

[2] R3-243958, Lenovo, Response LS on FS_XRM Ph2.

# Annex:  draft reply LS to SA2

# 3GPP TSG-RAN WG3 Meeting #125	R3-24xxxx

# Maastricht, Netherland, 19 - 23 Aug 2024

Title:	[Draft] Response LS to SA2 on FS_XRM Ph2

Response to:	R3-244020/S2-2407351

Release:	Release 19

Work Item:	FS_XRM Ph2

Source:	CMCC [to be RAN3]

To:	SA2

Cc:	RAN2, SA4

Contact person:	Lijun Dong

lijundong@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

- Overall Description:

RAN3 would like to thank SA2 for the LS on FS_XRM Ph2(R3-244020/S2-2407351) and kindly ask SA2 to take the following response into consideration.

- Question1 [for SA4, RAN2 and RAN3]:

  - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach.

  - To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension?

- [RAN3 answer to Question1]: From RAN3’s perspective, the existing TSCAI mechanism is sufficient to provide the periodicity information to RAN if the periodicity change is slow. If the dynamic changes of the periodicity are fast, updating TSCAI via CP signaling could be inefficient or not fast enough to provide the latest periodicity information to RAN. RAN3 would like SA2 to further clarify the use cases on the fast dynamic changes of the periodicity, whether the periodicity still exists. RAN3 thinks using in-band signaling in GTP-U for updating dynamic and fast periodicity change information is a practical approach to ensure timely and efficient updates.

- Question3 [for RAN3]: SA2 discusses to support available data rate exposure for GBR QoS Flow via user plane, and kindly asks RAN3 to provide feedback if any.

- [RAN3 answer to Question3]: RAN3 has replied to SA2 in R3-243958 that it is feasible to estimate available data rate for GBR QoS flows, which is up to RAN implementation. RAN3 thinks reporting data rate for GBR QoS flows via use plane by for example including it in GTP-U is feasible.