3GPP TSG-RAN WG3 Meeting #125	R3-244581

Maastricht, NL, Aug 19th - Aug 23th, 2024

Agenda item:		11.2

Source:				CMCC

Title:	TP to 38.743 AI/ML based CCO

Document for:		Others

- Introduction

This TP is based on the discussion paper R3-244571.

- TP to 38.743

<<<<<<<<<<<<<<<<<<<< Start of the Change >>>>>>>>>>>>>>>>>>>>

- 4.1	AI/ML based Network Slicing

- 4.1.1	Use case description

Editor Note: Capture the description of use case

Support of network slicing in NG-RAN is defined in TS 38.300 [x].

The NG-RAN plays a key role in taking mobility, load balancing and Radio Resources Management decisions for the purpose of meeting target requirements derived from the SLA of each supported network slice.

AI/ML function can analyze metrics related to network and UE level performance related to perform optimal resource management and mobility decisions for network slicing to meet the requirements.

- 4.1.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based network slicing:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

For AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB:

Figure 1: Model training in OAM and inference in NG-RAN

Step 1: The NG-RAN node 1 configures the UE to provide measurements and/or location information (e.g., RRM measurements, MDT measurements, velocity, position).

Step 2: The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends the measurement report message(s) to NG-RAN node 1.

Step 4: NG-RAN node 1 further sends UE measurement reports together with other input data for Model Training to OAM. NG-RAN node 2 also sends input data for Model Training to OAM.

Step 5: Model Training at OAM. Required measurements and input data from other NG-RAN nodes are leveraged to train AI/ML models for network slicing.

Step 6: OAM deploys/updates AI/ML model into the NG-RAN node(s). The NG-RAN node can also continue model training based on the received AI/ML model from OAM.

Step 7: UE sends the UE measurement report(s) to NG-RAN node 1.

Step 8: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model inference of AI/ML-based network slicing.

Step 9: Based on local inputs of NG-RAN node 1 and received inputs from NG-RAN node 2 and UE, NG-RAN node 1 generates model inference output(s).

Step 10: NG-RAN node 1 executes AI-based network slicing actions according to the model inference output.

Step 11: NG-RAN node 1/2 provides feedback to OAM.

For AI/ML Model Training and AI/ML Model Inference are both located in the gNB:

Figure 2: Model training and inference in NG-RAN

Step 1: The NG-RAN node 1 configures the UE to provide measurements and/or location information (e.g., RRM measurements, MDT measurements, velocity, position).

Step 2: The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends the measurement report message(s) to NG-RAN node 1.

Step 4: NG-RAN node 2 sends input data for Model Training to NG-RAN node 1.

Step 5: An AI/ML Model Training is located at NG-RAN node 1. The required measurements and input data from other NG-RAN nodes are leveraged to train the AI/ML model.

Step 6: UE sends the UE measurement report(s) to NG-RAN node 1.

Step 7: NG-RAN node 2 sends the input information to NG-RAN node 1 for AI-based network slicing inference.

Step 8: NG-RAN node 1 performs model inference and generate network slicing resource allocation predictions or decisions.

Step 9: NG-RAN node 1 executes AI-based network slicing actions according to the model inference output.

Step 10: NG-RAN node 2 sends feedback information to NG-RAN node 1.

#### 4.1.2.4	Input data of AI/ML based Network Slicing:

To predict the optimized network slicing decisions, a gNB may need the following information as input data for AI/ML-based network slicing:

From local node:

-	Measured/Predicted radio resource status per slice

-	Measured/Predicted slice available capacity

-	Legacy predicted UE trajectory

-	Measured/predicted UE traffic

From neighbouring gNBs:

-	Measured/Predicted radio resource status per slice

-	Measured/Predicted slice available capacity

From the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

#### 4.1.2.5	Output data of AI/ML based Network Slicing:

AI/ML-based network slicing model in a gNB can generate the following information as output:

- Predicted radio resource status per slice

- Predicted slice available capacity

- Resource management decisions for resources within RRM policies (used by gNB internally)

- Slice aware mobility decisions (used by gNB internally)

#### 4.1.2.6	Feedback of AI/ML based Network Slicing:

To optimize the performance of AI/ML-based network slicing model, the following feedback can be considered to be collected from gNBs:

- Measured Radio resource status per slice

- Measured Slice available capacity

- Legacy UE performance feedback for those UEs handed over from the source gNB

- PDU session level or QoS flow level UE performance feedback for those UEs handed over from the source gNB.

#### 4.1.2.7	Potential standard impacts:

Following standard impacts is listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

Xn interface:

- Enhanced existing procedure to collect predicted information between gNBs:

  - Predicted radio resource status per slice

  - Predicted slice available capacity