3GPP TSG-RAN WG3 Meeting #125                	                                                 R3-244585

Maastricht, NL, Aug 19 - Aug 23, 2024

Agenda Item:	8.3

Source:	CMCC

Title:	Discussion on support barring exemption for emergency calls

Document for:	Discussion

Introduction

During RAN2#125bis meeting, RAN2 sent an LS associated with agreed CRs to RAN3 to support emergency call for (e)RedCap in barred cells, which has an influence on RAN3 specifications. The content of LS is excerpted as follows:

In RAN2#125bis RAN2 have endorsed the attached CRs for (e)RedCap UEs with the description of the CRs as below: If a (e)RedCap UE is barred in a cell where (e)RedCap is enabled since (e)RedCap UEs with 1Rx branch or 2Rx branches or both are barred in the cell, network may allow those (e)RedCap UEs to consider the cell as acceptable cell for emergency calls if cell selection criteria is fulfilled and, if the (e)RedCap UE supports only half duplex FDD operation, HD-FDD operation is allowed in the cell. The agreed RAN2 CRs also include the means for the cell to allow or not, the above operation via a SIB1 field. The CRs (R2-2402902, R2-2402903) when agreed, are considered to be implementable by Rel-17 UEs without any interoperability issue.

However, in RAN2#126 meeting, there are some updates on RAN2 previous CRs where one bit that enables EM calls for RedCap, eRedCap, and 2Rx XR in RRC message. The latest RAN2 agreements are reflected as follows:

Agreements 1 NES (i.e. ingoring MIB barring) will not be considered in our common solution discussion. FFS if anything specific for NES will need to be done. If anything needs to be done, it would not be part of the common solution. 2 For Rel-18, we introduce 1 bit that enables EM call for RedCap, eRedCap, and 2Rx XR. One RRC Rel-18 with magic sentence that it is early implementable in Rel-17. A CR for 38.304 doesn’t need to have the magic sentence. 3 This replaces the previous agreement and we will notify RAN3 verbally via delegates

During RAN3#124 meeting, RAN3 still wait for the progress in RAN2. Considering that RAN2 already made their progress on this issue, RAN3 will take into account their outcome and analyse impact on RAN3 specification. Thus, in this contribution, we will provide our understanding on supporting barring exemption for emergency calls.

Discussion

From the latest TS 38.331 CR shown below, it was agreed to introduce the barringExemptEmergencyCall IE in SIB1 message, allowing (e)RedCap UEs and 2Rx XR UEs to consider the cell as acceptable cell for emergency calls, where the cells are barred for those UE categories.

SIB1 message -- ASN1START -- TAG-SIB1-START *// skip unchanged part //* SIB1-v1800-IEs ::= SEQUENCE { ncr-Support-r18 ENUMERATED {true} OPTIONAL, -- Need S mt-SDT-ConfigCommonSIB-r18 MT-SDT-ConfigCommonSIB-r18 OPTIONAL, -- Need R musim-CapRestrictionAllowed-r18 ENUMERATED {true} OPTIONAL, -- Need R featurePriorities-v1800 SEQUENCE { msg1-Repetitions-Priority-r18 FeaturePriority-r17 OPTIONAL, -- Need R eRedCapPriority-r18 FeaturePriority-r17 OPTIONAL -- Need R } OPTIONAL, -- Need R si-SchedulingInfo-v1800 SI-SchedulingInfo-v1800 OPTIONAL, -- Need R cellBarredATG-r18 ENUMERATED {barred, notBarred} OPTIONAL, -- Need S cellBarredNES-r18 ENUMERATED {notBarred} OPTIONAL, -- Need R mobileIAB-Cell-r18 ENUMERATED {true} OPTIONAL, -- Need R eDRX-AllowedInactive-r18 ENUMERATED {true} OPTIONAL, -- Cond EDRX-RC intraFreqReselection-eRedCap-r18 ENUMERATED {allowed, notAllowed} OPTIONAL, -- Need S nonServingCellMII-r18 ENUMERATED {true} OPTIONAL, -- Need R sdt-BeamFailureRecoveryProhibitTimer-r18 ENUMERATED {ms50, ms100, ms200, ms500, ms1000, ms1500, ms2000, ms3000} OPTIONAL, -- Need R eRedCap-ConfigCommon-r18 ERedCap-ConfigCommonSIB-r18 OPTIONAL, -- Need R cellBarredFixedVSAT-r18 ENUMERATED {barred, notBarred} OPTIONAL, -- Cond NTN cellBarredMobileVSAT-r18 ENUMERATED {barred, notBarred} OPTIONAL, -- Cond NTN reselectionMeasurementsNR-r18 ENUMERATED{true} OPTIONAL, -- Need R cellBarred2RxXR-r18 ENUMERATED {true} OPTIONAL, -- Need R intraFreqReselection2RxXR-r18 ENUMERATED {allowed, notAllowed} OPTIONAL, -- Need R barringExemptEmergencyCall-r18 ENUMERATED {true} OPTIONAL, -- Cond EM-Barring nonCriticalExtension SEQUENCE {} OPTIONAL } *// skip unchanged part //* -- TAG-SIB1-STOP -- ASN1STOP

Correspondingly, during the handover, these kinds of cells can be considered as an acceptable candidate target cell when UE has an emergency call. Meanwhile, the disaggregated architecture also should be taken into account. Since SIB1 is encoded in gNB-DU, where gNB-DU should inform gNB-CU that the cell barring exempt information for emergency call over F1 interface before gNB-CU propagates the information to neighbour gNB. Since RAN2 already agreed that one bit indicator enables emergency call for RedCap, eRedCap, and 2Rx XR, RAN3 should follow a future proof solution provided by RAN2 to use a common new IE to support barring exemption for emergency call where separate bits are associated with different UE categories. Hence, it will have impact on XnAP and F1AP impact to support the cell barring exempt information for emergency call to align with the agreements of RAN2.

Observation 1: RAN3 should follow a future proof solution provided by RAN2 to use a common new IE to support barring exemption for emergency call where separate bits are associated with different UE categories.

Observation 2: It will have impact on XnAP and F1AP to support the cell barring exempt information for emergency call to align with the agreements of RAN2.

In our solution, XnAP CR and F1AP are provided to introduce a new BarringExemptEmergencyCall Information IE in served cell information, which enables UE to access the barred cell for emergency service. The design of IE is as follows:

| BarringExemptEmergencyCall Information | O |  | BIT STRING (SIZE(8)) | The presence of this IE indicates that the barringExemptEmergencyCall IE is broadcast in SIB1 of the corresponding cell, see TS 38.331 [8]. Each position in the bitmap indicates which UE categories support barring exempt for emergency calls. First bit = Redcap UEs, second bit = eRedcap UEs, third bit= 2Rx XR UEs, other bits reserved for future use. Value '1' indicates 'Barring Exemption is enabled'. Value '0' indicates 'Barring Exemption is disabled'. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

Proposal 1: Agree XnAP CR and F1AP CR to introduce a new BarringExemptEmergencyCall Information IE in served cell information over XnAP and F1AP, which enables UE to access the barred cell for emergency service.

Conclusions

In this contribution, we provide our views to support barring exemption enabling UEs have emergency calls access to the barred cells. The observations and proposals are captured as follows:

Observation 1: RAN3 should follow a future proof solution provided by RAN2 to use a common new IE to support barring exemption for emergency call where separate bits are associated with different UE categories.

Observation 2: It will have impact on XnAP and F1AP to support the cell barring exempt information for emergency call to align with the agreements of RAN2.

Proposal 1: Agree XnAP CR and F1AP CR to introduce a new barringExemptEmergencyCall Information IE in served cell information over XnAP and F1AP, which enables UE to access the barred cell for emergency service.

References

- RAN2#126 Chair Notes;

- R3-243012, LS on emergency call support for (e)RedCap in barred cells;

- R2-2405957, Introduction of barring exemption for RedCap UEs for emergency calls [RedCap_EM_Call];