3GPP TSG-RAN WG3 Meeting #125	             R3-244593

Maastricht, NL, 19th – 23th Aug 2024

Agenda Item:	16.2

Source:	CMCC

Title:	Discussion on RAN Architecture for Ambient IoT

Document for:	Discussion

1	Introduction

The TR capturing agreed TP associated with RAN architecture aspects for Ambient IoT is listed as follows:

This chapter attempts to identify and describe architectural elements necessary to define a RAN architecture for support of Ambient IoT embedded in the overall 5G system architecture in support of topology 1 and topology 2 (as defined in TR 38.848 [2]). Editor’s Note 2: What functionalities are hosted by the 5GS for AIoT is TBD. This chapter also attempts to identify a functional split between RAN and CN. Figure 6.3-1 depicts the logical system architecture for AIoT. It consists of the following architectural elements: AIoT device: equipment with characteristics outlined e.g. in TS 22.369 [x] and TR 38.848 [2]. Editor’s Note 3: Further details FFS, if any. AIoT RAS (AIoT Radio Access System): hosts certain functions for AIoT as part of the functional split between RAN and CN. Editor’s Note 4: Further details regarding AIoT functions hosted in the AIoT RAS and the respective functional split to be decided by RAN2, RAN3 and SA2. AIoT radio: radio interface between AIoT device and Common reader function. Editor’s Note 5: Further details on AIoT radio to be discussed by RAN1 and RAN2. AIoT CN: hosts certain functions for AIoT as of the functional split between RAN and CN Editor’s Note 6: Further details regarding AIoT functions hosted in the AIoT CN and the respective functional split to be decided by RAN2, RAN3 and SA2. XX interface: interface between the AIoT RAS and the AIoT CN on which certain AIoT specific functions are performed. Editor’s Note 7: The functions represented by the XX interfaces are FFS. It is also FFS whether this interface represents a new logical interface or is equal to NG. E.g. for topology 1 it may only represent a single interface instance, e.g. a new interface between AIoT RAS and AIoT CN, for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between AIoT CN and AIoT RAS, or one instance for NG alone. Common reader function: a function that communicates with the AIoT device by means of AIoT radio. Editor’s Note 8: Further details on Common reader function is to be discussed by RAN1 and RAN2. AIoT RAN node function: a function residing in AIoT RAS. It contains e.g. the control of the AIoT radio resources used towards the AIoT device. Editor’s Note 9: further details are FFS. Note that “control of AIoT radio resources” does not necessarily imply dynamic configuration of resources but could also rely on static assignment of resources by means of OAM. Aspects concerning coordination of the Upper Layer functions (e.g. Inventory, Command) e.g. in case these functions have to be performed over a multitude of instances of the Common Reader Function are FFS. Figure 6.3-1: Logical System Architecture for AIoT. Editor’s Note 9: whether the logical system architecture above is applied to topology 2 to be discussed in next meeting.

In this contribution, we will continue to provide our views on RAN architecture aspects for Ambient IoT.

2	Discussion

2.1	Logical System Architecture for AIoT

During last meeting, we already agreed the Logical System Architecture for AIoT shown in TR can be used for topology 1. As the illustration depicted, it is found that AIoT RAS included common reader function and AIoT RAN node function. The definition of AIoT RAS is a system hosts certain functions for AIoT as part of the functional split between RAN and CN. For the topology 1, it is obviously that AIoT RAS is referred to the AIoT RAN node. The AIoT RAN node should be an entity which interacts with AIoT device via AIoT radio and control of the AIoT radio resources, which reflects the two functions in AIoT RAS, i.e., common reader function and AIoT RAN node function. Therefore, we should clarify that the AIoT RAS is AIoT RAN node in topolgy1. It is suggested to capture the updated Logical System Architecture for AIoT for topology 1 in TR.

Figure 1: Logical System Architecture for AIoT for topology 1

Observation 1: The AIoT RAN node should be an entity which interacts with AIoT device via AIoT radio and control of the AIoT radio resources, which reflects the two functions in AIoT RAS, i.e., common reader function and AIoT RAN node function.

Proposal 1: The AIoT RAS is AIoT RAN node in topolgy1.

Proposal 2: It is suggested to capture the updated Logical System Architecture for AIoT for topology 1 in TR.

It is still FFS whether the logical system architecture above can be applied to topology 2. From our view, this existing logical system architecture for AIoT should be updated to fulfil the topology 2 deployment. The figure is illustrated as follows:

Figure 2: Logical System Architecture for AIoT for topology 2

For topology 2, we think it is unnecessary to depict the AIoT RAS in the figure. If the AIoT RAS is introduced, it will bring some controversial issue on which function of UE reader should be included in AIoT RAS. To make the deployment clearer without confusion, we think the figure should only emphasizes the functions in UE reader. It is suggested to adopt the above figure as logical system for topology 2 and capture in TR.

Proposal 3: It is suggested to capture the Logical System Architecture for AIoT for topology 2 in TR.

2.2	Protocol Stack

For topology 1, majority of companies share the common understanding that the data should be transmitted through control plane solution. At the end phase of 5G release, we do not recommend introducing a new service-based interface between AIoT RAN node and AIoT CN with new protocol stack for AIoT-RAN node, which is completely different from the protocol stack of legacy gNB. Building a new AIoT-RAN node with a new protocol stack is very costly. From our view, the protocol stack for topology 1 is shown below:

Figure 3: Protocol stack for topology 1

As the figure illustrated, for topology 1, the interface between AIoT RAN node and AIoT CN may only represent a single interface instance. Whether a new interface is introduced or NG interface is reused should be decide by SA2.

Proposal 4: It is suggested to capture the Protocol stack for topology 1 in TR.

For topology 2, taken the SA2 TR as reference and the discussion in last meeting, two typical solutions are summarized as follows:

- RRC-based solution

Figure 4: Protocol stack for topology 2 (RRC-based transmission solution)

In summary, for RRC-based solution, the gNB receives an inventory or command message transmitted from AIoT CN over NG/XX-AP interface. The gNB generates RRC message which may contain inventory or command and transmits to UE reader. Once the A-IoT device(s) performs the corresponding procedure and responses to the UE reader, The UE reader generates a RRC message with device response information to gNB. The gNB transmits a message with device response to AIoT CN in NG/XX-AP interface.

- NAS-based solution

Figure 5: Protocol stack for topology 2 (NAS-based transmission solution)

In summary, for NAS-based solution, UE reader receives a NAS message including AIoT related service content from AIoT CN. Then the UE needs to request the authorization of using A-IoT radio resource from gNB. The UE reader transmits the A-IoT paging to A-IoT device(s) and performs the corresponding procedure indicated outside the NAS message. After the UE gets the feedback from AIoT device(s), the UE reader transmits a NAS message containing the device response information to AIoT CN which is transparently to gNB.

From our point of view, both solutions are workable and feasible for us. It is suggested to capture the Protocol stack for topology 2 with RRC-based solution and NAS-based solution.

Proposal 5: It is suggested to capture the Protocol stack for topology 2 with RRC-based solution and NAS-based solution.

3	Conclusion

Based on the discussions mentioned above, in this contribution we provide following observations and proposals on RAN architecture aspects for Ambient IoT:

Observation 1: The AIoT RAN node should be an entity which interacts with AIoT device via AIoT radio and control of the AIoT radio resources, which reflects the two functions in AIoT RAS, i.e., common reader function and AIoT RAN node function.

Proposal 1: The AIoT RAS is AIoT RAN node in topolgy1.

Proposal 2: It is suggested to capture the updated Logical System Architecture for AIoT for topology 1 in TR.

Proposal 3: It is suggested to capture the Logical System Architecture for AIoT for topology 2 in TR.

Proposal 4: It is suggested to capture the Protocol stack for topology 1 in TR.

Proposal 5: It is suggested to capture the Protocol stack for topology 2 with RRC-based solution and NAS-based solution.

4	Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA;

[2] 3GPP TR 23.700-13, Study on Architecture support of Ambient power-enabled Internet of Things;

[3] 3GPP TR 38.848, Study on Ambient IoT (Internet of Things) in RAN;