3GPP TSG-RAN WG3 Meeting #125	              R3-244594

Maastricht, NL, 19th – 23th Aug 2024

Agenda Item:	16.3

Source:	CMCC

Title:	Discussion on paging for Ambient IoT

Document for:	Discussion

1	Introduction

In last meeting, the discussion on paging for Ambient IoT device reached following agreements:

RAN3#124: Inventory over CN-RAN interface can be used for a single device, or a group of devices, or all devices. Command over CN-RAN interface can be used for a single device. FFS for command on a group of devices, or all devices. What’s the device context? The content in Inventory and Command?

In this contribution, we will provide our views on paging for Ambient IoT.

2	Discussion

Regard to paging for AIoT, RAN2 reached some agreements in last meeting, which can be taken as reference:

1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion.

Although RAN2 focuses on the AIoT paging message in Uu interface, it can also be used as a reference for the AIoT paging message transferring in NG interface. It has been agreed that a message can contain a single device ID, a group ID and no device ID. The intention of AIoT paging is used for inventory and command, which is consistent over Uu interface and NG/XX interface. The type of device ID in AIoT paging message over NGAP/XXAP can follow the same design of AIoT paging over Uu interface.

Observation 1: The type of device ID in AIoT paging message over NGAP/XXAP can follow the same design of AIoT paging over Uu interface.

Regard to A-IoT paging for single device, it is agreed that the inventory or command can be used for a single device, so the device ID should be undoubtedly included in AIoT paging message over NGAP/XXAP.

Regard to AIoT paging for a group of devices, we already agreed the inventory over CN-RAN interface. For command over CN-RAN interface, we think the scenario of reading or writing a group of devices or all devices does exist, e.g., sellers want to retrieve the product information of a group of devices, or sellers may write some settings on the device IDs for a group of sold items. Therefore, it is proposed that command over CN-RAN interface can be used for a group of devices or all devices.

Observation 2: From the actual deployment, the scenario of reading or writing a group of devices or all devices does exist.

Proposal 1: It is proposed that command over CN-RAN interface can be used for a group of devices or all devices.

The group device ID contained in CN-RAN AIoT paging message can be expressed as mask ID or feature ID in A-IoT paging message over NGAP/XXAP. In addition, a message may contain multiple device IDs of AIoT devices. As the agreement in RAN2 stated, the need for this use case should be confirmed based on SA2 discussion. From our perspective, it is suggested to wait for the decision of SA2 to check whether including multiple IDs is necessary.

Proposal 2: It is suggested to wait for the decision of SA2 to check whether including multiple IDs is necessary.

Whatever the paging is for single device or a group of devices, it is noted that device ID included in the AIoT paging message can be stored in A-IoT devices’ memory or derived by A-IoT devices using specific algorithms. Followed by this way, each Ambient IoT device can determine whether the A-IoT paging is targeted to itself by comparing its own information and the identifier contained in A-IoT paging.

Observation 3: The device ID included in the AIoT paging message can be stored in A-IoT devices’ memory or derived by A-IoT devices using specific algorithms.

If A-IoT paging is targeted to all devices, no device ID can be included in the paging message. After reader receives the A-IoT paging message without device ID, it understands the AIoT paging is used for inventory or command all devices. The reader will correspondingly initiate AIoT paging to AIoT devices. Each A-IoT devices will perform the random access as the response of the A-IoT paging after finding that none of ID is included.

Observation 4: If A-IoT paging is targeted to all devices, no device ID can be included in the paging message.

In light of above analysis, the AIoT paging message over NGAP/XXAP can contain a single device ID, a group ID and no device ID.

Proposal 3: The AIoT paging message over NGAP/XXAP can contain a single device ID, a group ID and no device ID.

Besides the AIoT device ID, some other information should be taken into account to include in the AIoT paging message. Whatever for topology 1 or topology 2, in order for the AIoT RAN node or gNB to allocate resources efficiently for different kinds of services, it is beneficial for AIoT RAN node and gNB be aware of the service type of AIoT paging. Regard to different kinds of procedure, the resource allocation can be different. For example, if the AIoT paging is inventory, AIoT RAN node or gNB will allocate the D2R resource for AIoT devices up to 96bits according to the inventory type. If the AIoT paging is read, AIoT RAN node or gNB will reserve 1000bits at most to retrieve the content from AIoT device. If the AIoT paging is write, AIoT RAN node or gNB will only allocate limited resource to only receive the feedback on whether write is successful or not. Therefore, it is necessary to include service type (e.g., inventory, read, write, etc.) in the AIoT paging message.

Observation 5: In order for the AIoT RAN node or gNB to allocate resources efficiently for different kinds of services, it is beneficial for AIoT RAN node and gNB be aware of the service type of AIoT paging.

Proposal 4: Service type (e.g., inventory, read, write, etc.) should be included in the AIoT paging message.

With the consideration that the radio resources should be allocated by gNB, then the estimated number of AIoT devices should also be informed to gNB from CN. Otherwise, the gNB should always estimates the device number by itself, which may bring larger overhead and lose the efficiency. With the number of AIoT devices, gNB can allocated D2R resources to enable AIoT devices to perform CFRA efficiently. Therefore, the estimated number of AIoT devices should be included in AIoT paging message.

Proposal 5: The estimated number of AIoT devices should be included in AIoT paging message.

Furthermore, when reviewing the legacy paging procedure, the paging triggered by CN is broadcasted among all cells within the corresponding TAI list, while the paging triggered by RAN is broadcasted among all cells within the RAN Notification Area (RNA). For AIoT, it has been assumed that the device will not support tracking/RAN area update procedure. To improve the efficiency of AIoT paging, it is beneficial for CN to maintain an association relationship between A-IoT devices and associated area. With the consideration of supporting the association, something like AIoT paging area composed of AIoT readers can be included in AIoT paging message. The CN can inventory the AIoT device(s) within this specific area instead of always inventorying all A-IoT devices.

Observation 6: To improve the efficiency of AIoT paging, it is beneficial for CN to maintain an association relationship between A-IoT devices and associated area.

Proposal 6: AIoT paging area composed of AIoT readers should be included in AIoT paging message.

In addition, the task ID should be included in the AIoT paging message to support identifying separate task requested from different CNs or one CN. The reader may not support multiple tasks to run in parallel, AIoT RAN node or gNB/UE reader decide which one to execute first based on the request arrival time received.

Observation 7: The reader may not support multiple tasks to run in parallel, AIoT RAN node or gNB/UE reader decide which one to execute first based on the request arrival time received.

Proposal 7: The task ID should be included in AIoT paging message.

Moreover, sometimes the inventory requirements from the server are periodic. Naturally, the CN will also provide the RAN with inventory periodicity, indicating how often inventory should be taken.

Proposal 8: The inventory periodicity may be included in AIoT paging message.

If the request from application server is inventory, CN will trigger the inventory-only service to retrieve the AIoT device IDs by AIoT paging message. If the request from application server is command, CN may trigger the command-only service or the inventory and command service. If the command-only service is triggered, it implies that the CN has already triggered the inventory service for the same devices. If the inventory and command service is triggered by A-IoT paging from CN, it means the CN requests A-IoT RAN node to initiate the two services sequentially. Once receiving the A-IoT paging request from CN, A-IoT RAN node will initiate the inventory service first, and followed by command service.

For topology 1 and topology 2, the overall message flows for inventory/command procedure are depicted below:

Figure 1: Message Flow of AIoT Inventory/Command procedure (Topology 1)

For topology 1, the steps are described as follows:

- The AIoT CN initiates an inventory procedure by sending an inventory request message to the AIoT RAN node;

- The AIoT RAN node responses immediately with the feedback by sending an inventory response message to the AIoT CN;

- The AIoT RAN node triggers the AIoT paging for inventory to the device;

- The AIoT device(s) performs random access procedure to AIoT RAN node;

- The AIoT RAN node collects AIoT device(s) ID based on the service type;

- The AIoT RAN node initiates inventory report procedure including AIoT device(s) ID to the AIoT CN;

- The AIoT CN initiates a command procedure by sending a command request message to the AIoT RAN node;

- The AIoT RAN node triggers the AIoT paging for command to the device;

- The AIoT RAN node collects AIoT device(s) command information based on the command type;

- The AIoT RAN node sends command response message including AIoT device(s) command information to the AIoT CN;

Figure 2: Message Flow of AIoT Inventory/Command procedure (Topology 2)

For topology 2, the steps are described as follows:

- The AIoT CN initiates an inventory procedure by sending an inventory request message to the gNB;

- The gNB responses immediately with the feedback by sending an inventory response message to the AIoT CN;

- The gNB generates and transmits a RRC message which contains inventory to UE reader;

- The UE reader sends an AIoT paging to AIoT device(s);

- AIoT device(s) perform random access procedure to UE reader;

- The UE reader collects AIoT device(s) ID;

- The UE reader generates and transmits a RRC message with AIoT device(s) ID to gNB;

- The gNB initiates inventory report procedure including device IDs to the AIoT CN;

- The AIoT CN initiates a command procedure by sending a command request message to the gNB;

- The gNB generates and transmits a RRC message which contains command to UE reader;

- The UE reader sends an AIoT paging to AIoT device(s);

- The UE reader collects AIoT device(s) command information based on the command type;

- The UE reader generates and transmits a RRC message with command information to gNB;

- The gNB sends command response message including AIoT device(s) command information to the AIoT CN

From topology 1 and topology 2, the A-IoT CN can initiate a class1 inventory request procedure to the AIoT RAN node or gNB. When receiving the inventory request message, the AIoT RAN node or gNB can respond to or reject the inventory request from AIoT CN. The AIoT RAN node or gNB takes the responsibility of coordinating the frequency resource of A-IoT radio. Once the AIoT radio interface is congested, AIoT RAN node or gNB can reject the inventory procedure. If multiple procedures initiated by the different AIoT CNs are received, AIoT RAN node or gNB can determine which task to operate firstly. In order to avoid the AIoT CN waiting for a long time without response, feedback should be sent to AIoT CN immediately after the AIoT inventory request.

Observation 8: The AIoT RAN node or gNB can respond to or reject the inventory request from AIoT CN.

Proposal 9: It is proposed to introduce a class 1 inventory request procedure between the AIoT CN and the AIoT RAN node or gNB.

Taken the consideration to report the device ID, the AIoT RAN node or gNB can initiate a class 2 procedure to report one or multiple device ID(s) in the AIoT report message. The reporting behaviour of AIoT RAN node or gNB should be further discussed.

Observation 9: Taken the consideration to report the device ID, the AIoT RAN node or gNB can initiate a class 2 procedure to report one or multiple device ID(s) in the AIoT report message.

Proposal 10: It is proposed to introduce a class2 inventory report procedure from the AIoT RAN node or gNB to the AIoT CN.

3	Conclusion

Based on the discussions mentioned above, in this contribution we provide some observations and proposals on paging for Ambient IoT:

Observation 1: The type of device ID in AIoT paging message over NGAP/XXAP can follow the same design of AIoT paging over Uu interface.

Observation 2: From the actual deployment, the scenario of reading or writing a group of devices or all devices does exist.

Observation 3: The device ID included in the AIoT paging message can be stored in A-IoT devices’ memory or derived by A-IoT devices using specific algorithms.

Observation 4: If A-IoT paging is targeted to all devices, no device ID can be included in the paging message.

Observation 5: In order for the AIoT RAN node or gNB to allocate resources efficiently for different kinds of services, it is beneficial for AIoT RAN node and gNB be aware of the service type of AIoT paging.

Observation 6: To improve the efficiency of AIoT paging, it is beneficial for CN to maintain an association relationship between A-IoT devices and associated area.

Observation 7: The reader may not support multiple tasks to run in parallel, AIoT RAN node or gNB/UE reader decide which one to execute first based on the request arrival time received from AIoT RAN node or gNB/UE reader.

Observation 8: The AIoT RAN node or gNB can respond to or reject the inventory request from AIoT CN.

Observation 9: Taken the consideration to report the device ID, the AIoT RAN node or gNB can initiate a class2 procedure to report one or multiple device ID(s) in the AIoT report message.

Proposal 1: It is proposed that command over CN-RAN interface can be used for a group of devices or all devices.

Proposal 2: It is suggested to wait for the decision of SA2 to check whether including multiple IDs is necessary.

Proposal 3: The AIoT paging message over NGAP/XXAP can contain a single device ID, a group ID and no device ID.

Proposal 4: Service type (e.g., inventory, read, write, etc.) should be included in the AIoT paging message.

Proposal 5: The estimated number of AIoT devices should be included in AIoT paging message.

Proposal 6: AIoT paging area composed of AIoT readers should be included in AIoT paging message.

Proposal 7: The task ID should be included in AIoT paging message.

Proposal 8: The inventory periodicity may be included in AIoT paging message.

Proposal 9: It is proposed to introduce a class 1 inventory request procedure between the AIoT CN and the AIoT RAN node or gNB.

Proposal 10: It is proposed to introduce a class 2 inventory report procedure from the AIoT RAN node or gNB to the AIoT CN.

4	Reference

[1] 	RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA;

[2]	RP-234058, Time_budget_request_R19_Ambient_IoT_SID