3GPP TSG-RAN WG3 Meeting #124	           					R3-244595

Fukuoka, Japan, May 20- May 24, 2024

Agenda Item:	16.3

Source:	CMCC

Title:	Discussion on device context management and data transfer

Document for:	Discussion

1	Introduction

In RAN3#124 meeting, the discussion on device context management and data transfer reached following agreements:

RAN3#124: Inventory over CN-RAN interface can be used for a single device, or a group of devices, or all devices. Command over CN-RAN interface can be used for a single device. FFS for command on a group of devices, or all devices. What’s the device context? The content in Inventory and Command?

In this contribution, we would like to provide our views on device context management and data transfer for Ambient IoT.

2	Discussion

2.1 Data Transfer

Currently, there are two options of data transfer putting on the table. One is control plane solution; the other is user plane solution. If the user plane solution is adopted, the legacy data transfer between device and AIoT CN can be taken as reference. However, considered the motivation of designing the ultra-low complexity with ultra-low power consumption, a simple data transmission solution is expected for AIoT network. It is unwanted to introduce extra complexity of building the dedicated per device data path between UPF and device. It is unrealistic for device to manage PDU session function. Since the cost control of AIoT device would be ultra-low, so the device cannot store the PDU session context when out of power. From RAN3 point of view, it is unnecessary to follow legacy UP solution.

Proposal 1: The legacy user plane solution is not pursued.

Hence, the data should be transmitted through control plane solution, which means the AIoT data should be included in control plane signalling. From our view, AIoT data includes the device ID and the response information of the read/write. As our architecture contribution provided, the protocol stack for topology1 we supported is shown as follows:

For topology 1, we prefer AIoT NAS based transmission solution. The AIoT data is included in the AIoT NAS layer, and transparently sent to AIoT RAN node. The AIoT RAN node is not aware of AIoT data.

Figure 1: Protocol stack for topology 1

For topology 2, we think RRC-based transmission solution or NAS-based transmission solution are both feasible for us.

RRC-based transmission solution:

Figure 2: Protocol stack for topology 2 (RRC-based transmission solution)

For RRC-based solution, the gNB receives an inventory or command message transmitted from AIoT CN over NG/XX-AP interface. The gNB generates RRC message which may contain inventory or command and transmits to UE reader. Then, the UE reader transmits an A-IoT paging to A-IoT device(s) and perform the corresponding procedure indicated in the received RRC message. A-IoT device(s) responses with the device data to UE reader. The UE reader generates a RRC message with device response including device data to gNB. The gNB transmits a message with device response to CN over NG/XX-AP interface. In this solution, the AIoT device data is encoded in RRC message.

NAS-based transmission solution:

Figure 3: Protocol stack for topology 2 (NAS-based transmission solution)

For NAS-based solution, UE reader receives a NAS message which contains AIoT data from AIoT CN. The UE should request the authorization of using A-IoT radio resource from gNB. The UE reader transmits the A-IoT paging to A-IoT device(s) and performs the corresponding procedure indicated outside the NAS message. After that, A-IoT device(s) responses the AIoT data to UE reader. The UE reader transmits a NAS message which contains the device response to A-IoT CN. In this solution, the AIoT device data is encoded in NAS message and transparent to gNB. However, the service type of AIoT device should be visible to gNB indicated by RRC message, which should not be considered as AIoT data.

Proposal 2: For data transfer solution in topology 1, support AIoT-NAS based transmission solution.

Proposal 3: For data transfer solution in topology 2, support RRC-based transmission solution or NAS-based transmission solution.

Proposal 4: The service type of AIoT device should be visible to gNB indicated by RRC message, which should not be considered as AIoT data.

2.2 Device Context Management

Considering the device context management, there will be no registration procedure for AIoT devices. Before inventory all AIoT devices, the CN has no prior information about the distribution and location of the devices. Thus, AIoT CN will trigger an initial paging in a large area to inventory all devices. After all the devices reports its device ID to the CN via reader, the association between the devices and the reader should be stored in CN as the device context. For example, in topology 1, the association between the devices and the AIoT RAN node should be stored; in topology 2, the association between the devices and the UE reader should be stored. In addition, the association between the UE reader and gNB needs to be recorded by the CN accompanied by gNB reporting. The association information can be collected in the inventory procedure to improve the future inventory or command efficiency. The AIoT CN can use the association to easily find or command the device. The association is naturally updated in the inventory or command procedure, the AIoT CN will update its inside device context correspondingly.

Observation 1: The association information can be collected in the inventory procedure to improve the future inventory or command efficiency.

Proposal 5: RAN3 assumes that CN shall store the association between the devices and the AIoT RAN node as the device context.

To enable the AIoT RAN node or gNB to better identify or handle the AIoT device, RAN and CN interface will assign the ID pair to AIoT device. Followed the same rule with legacy RAN NGAP ID and CN NGAP ID, AIoT RAN node or gNB will allocate RAN device XXAP ID, and CN correspondingly will allocate CN device XXAP ID. The signalling transmitted over NGAP will use the {RAN device XXAP ID, CN device XXAP ID} pair to indicate the specific AIoT device.

Proposal 6: The signalling transmitted over NGAP will use the {RAN device XXAP ID, CN device XXAP ID} pair to indicate the specific AIoT device.

3	Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on device context management and data transfer for Ambient IoT:

Observation 1: The association information can be collected in the inventory procedure to improve the future inventory or command efficiency.

Proposal 1: The legacy user plane solution is not pursued.

Proposal 2: For data transfer solution in topology 1, support AIoT-NAS based transmission solution.

Proposal 3: For data transfer solution in topology 2, support RRC-based transmission solution or NAS-based transmission solution.

Proposal 4: The service type of AIoT device should be visible to gNB indicated by RRC message, which should not be considered as AIoT data.

Proposal 5: RAN3 assumes that CN shall store the association between the devices and the AIoT RAN node as the device context.

Proposal 6: The signalling transmitted over NGAP will use the {RAN device XXAP ID, CN device XXAP ID} pair to indicate the specific AIoT device.

4	Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA;

[2] 3GPP TR 23.700-13, Study on Architecture support of Ambient power-enabled Internet of Things;

[3] 3GPP TR 38.848, Study on Ambient IoT (Internet of Things) in RAN;