3GPP TSG-RAN WG3 Meeting #125	             R3-244596

Maastricht, NL, 19th – 23th Aug 2024

Agenda Item:	16.4

Source:	CMCC

Title:	Discussion on locating Ambient IoT device

Document for:	Discussion

1	Introduction

During last meeting, the discussion on locating an Ambient IoT device revolved around the purpose of locating AIoT device with following agreements:

AIoT device location information may be used for the following purposes: (a) improving the AIoT operation itself, e.g. by sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly. (b) providing location information to the consumer of the AIoT service. Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes.

In this contribution, we will continue to provide our views on locating AIoT devices.

2	Discussion

In last meeting, the purposes for locating an AIoT device are clearly defined, and the “reader ID” can be taken into account as the AIoT device location information. On behalf of the device location information, the reader ID needs to be perceived by the network.

In topology 1, the AIoT RAN node plays the role of the reader and the AIoT RAN node ID can be regarded as the reader ID. From our view, the existing global gNB ID can also be identified by AIoT RAN node. If the whole AIoT RAN node is served as a reader, AIoT RAN node does not need to report its reader ID as the AIoT CN already knows where the report message comes from.

Observation 1: If the whole AIoT RAN node is served as a reader, AIoT RAN node does not need to report its reader ID as the AIoT CN already knows where the report message comes from.

Since an AIoT RAN node may have multiple TRPs or multiple antennas, it is possible that an AIoT RAN node possesses multiple readers associated to multiple TRPs or antennas. Thus, each TRP ID or antenna ID may also be considered as a reader ID. If the whole AIoT RAN node is served as multiple readers, the TRP ID or antenna ID needs to be reported to AIoT CN node for identifying the AIoT device is located in which TRP or antenna.

Observation 2: If the whole AIoT RAN node is served as multiple readers, the TRP ID or antenna ID needs to be reported to AIoT CN node for identifying the AIoT device is located in which TRP or antenna.

Proposal 1: In topology 1, The AIoT RAN node ID, TRP ID or antenna ID can be regarded as the reader ID.

In topology 2, UE plays the role of the reader and the UE ID is considered as the reader ID. The reader is known by gNB when the reader needs to collect and report the device ID. The UE ID together with the collected device ID are reported from gNB to AIoT CN. The AIoT CN needs to be aware the mapping of device and UE reader, i.e., the device is under the control of which UE reader, and the mapping of UE reader and gNB i.e., the UE is under the control of which gNB. Based on the mapping, the AIoT CN can send the signalling to the specific UE which makes the AIoT operation more efficiently.

Observation 3: The AIoT CN needs to be aware the mapping of device and UE reader, i.e., the device is under the control of which UE reader, and the mapping of UE reader and NG-RAN node.

Proposal 2: In topology 2, UE plays the role of the reader and the UE ID is considered as the reader ID.

In the SID, there is a clear demand for locating an Ambient IoT device with no specification impact by reusing existing user location report, or minimal specification impact to convey location information to CN.

From SA2 TR 23.700-13, one solution has some related description of reporting location information of the AIoT device. From the yellow highlighted part, it also shows that AIoT RAN node may provide location information of the device reusing ULI (User Location Information).

Figure 6.8.2.1-1: Inventory Procedure 9. The A-RAN sends an NGAP message (Inventory Response or Inventory Notify) to the AIOTF, containing the device ID and the optional device capability information provided by the device. The A-RAN may further provide location information (ULI) of the device, if requested from the AIOTF and allowed by local policy. The A-RAN may further provide an end indicator to inform the AIOTF whether it is the last inventory response for the inventory round. Editor's note: Details of what location information A-RAN provides is FFS.

From current NGAP, the User Location Information IE is included in multiple procedures from gNB to AMF for location reporting. The previous agreement is to support locating the AIoT device at “reader” granularity, so reader granularity is enough for locating AIoT device. As the purpose mentioned, if a consumer wants to know the AIoT device location, the reader granularity is sufficient. Combined with the purposes for locating an AIoT device and the SID requirement, whatever topology 1 or topology 2, the location information can be included in Inventory/Command procedures from AIoT RAN node/gNB to CN. If the consumer only wants to know the AIoT device location, CN can initiate Inventory procedure to request the device location. In addition, in order to improve the AIoT operation or update the mapping relationship, the reader ID may be reported in Inventory/Command procedure every time from AIoT RAN node/gNB to AIoT CN associated with device ID.

Observation 4: If the consumer only wants to know the AIoT device location, CN can initiate Inventory procedure to request the device location.

Observation 5: In order to improve the AIoT operation or update the mapping relationship, the reader ID may be reported in Inventory/Command procedure every time from AIoT RAN node/gNB to AIoT CN associated with device ID.

When the signalling details on providing location information to AIoT CN node is determined, the next step is deciding the specific IE. Since we currently have not reached any agreements on procedure between NG-RAN node to CN, we would like to postpone the discussion until some new procedures are came out for AIoT device.

Proposal 3: Postpone the discussion on Stage3 details until some new procedures are came out for AIoT device.

3	Conclusion

Based on the discussions mentioned above, we provide some observations and proposals on locating Ambient IoT device:

Observation 1: If the whole AIoT RAN node is served as a reader, AIoT RAN node does not need to report its reader ID as the AIoT CN already knows where the report message comes from.

Observation 2: If the whole AIoT RAN node is served as multiple readers, the TRP ID or antenna ID needs to be reported to AIoT CN node for identifying the AIoT device is located in which TRP or antenna.

Observation 3: The AIoT CN needs to be aware the mapping of device and UE reader, i.e., the device is under the control of which UE reader, and the mapping of UE reader and NG-RAN node.

Observation 4: If the consumer only wants to know the AIoT device location, CN can initiate Inventory procedure to request the device location.

Observation 5: In order to improve the AIoT operation or update the mapping relationship, the reader ID may be reported in Inventory/Command procedure every time from AIoT RAN node/gNB to AIoT CN associated with device ID.

Proposal 1: In topology 1, The AIoT RAN node ID, TRP ID or antenna ID can be regarded as the reader ID.

Proposal 2: In topology 2, UE plays the role of the reader and the UE ID is considered as the reader ID.

Proposal 3: Postpone the discussion on Stage3 details until some new procedures are came out for AIoT device.

4	Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA;

[2] TR 38.848, Study on Ambient IoT (Internet of Things) in RAN (Release 18)