3GPP TSG-RAN WG3 Meeting #125	                                                                R3-244602

Maastricht, NL, Aug 19 - Aug 23, 2024

Agenda Item:	9.1

Source:	CMCC, China Telecom, China Unicom

Title:	Discussion on UE location information for NB-IoT NTN

Document for:	Discussion

Introduction

In last meeting, RAN3 received a reply LS R3-243009 from RAN2 on UE location for NB-IoT NTN. From the reply LS, RAN2 gave their feedback which can be seen as the response to the previous LS from SA2 on UE Location Information for NB-IoT NTN. The reply from RAN2 is shown as follows:

SA2 question: SA2 asks RAN2 and RAN3 whether they think it may be useful for MME to further signal the coarse location information received from the UE in NAS back to eNB e.g. using one of the existing Location Reporting Procedure in order for the eNB to send updated ULI to MME For the above question, from RAN2 perspective, even if not essential it may be useful for the eNB if the MME further signals the coarse location information received from the UE in NAS back to eNB.

Since no consensus is achieved in last meeting, we will continue to discuss this issue in this meeting and try to give a reply LS to SA2 in this meeting.

Discussion

For NB-IoT UEs, SA2 already agreed that UE can provide the coarse location information to MME via NAS in case LPP mechanism is not supported. Meanwhile, RAN2 confirms that the coarse location reporting via AS is not supported because there is no AS security for NB-IoT. Once receiving coarse location information from UE in NAS, the MME can further signal the location information back to eNB for generating the UE location derived TAC in LTE NTN IE. The UE location derived TAC in LTE NTN IE is sent in the ULI for MME to be aware of the UE location derived TAC. From the TS 36.300 shown as below:

For NTN, the following applies in addition to Network entity related Identities as described in clause 8.2: A Tracking Area corresponds to a fixed geographical area. Any respective mapping is configured in the RAN;

It is clearly stated that the mapping between a tracking area and a fixed geographical area is configured in the RAN, not in the CN. Therefore, CN currently cannot construct the mapping between TAC and the location information. While the CN desires to acquire the UE location derived TAC, it is necessary to send the coarse location information to eNB. Obviously, the UE location derived TAC cannot be constructed if no UE location information is there.

Observation 1: The mapping between a tracking area and a fixed geographical area is configured in the RAN, not in the CN.

Another benefit for MME signaling the coarse location information is to page the specific UE more efficiently. For paging optimization, the location information is useful for eNB to include the updated information on recommended cells in UE CONTEXT RELEASE COMPLETE message. Specifically, the eNB will update the visited cells in the Recommend Cell List IE based on implementation and send back to MME. Once MME decides to page a UE, MME will send to multiple eNBs associated with the cell lists in Paging message.

Observation 2: Another benefit for MME signaling the coarse location information is to page the specific UE more efficiently.

Based on above analysis, we think that coarse UE location for NB-IoT UEs is useful and should be signaled from MME to eNB in the S1AP message.

Proposal 1: Coarse UE location for NB-IoT UEs is useful and should be signaled from MME to eNB in the S1AP message.

Conclusions

In this contribution, we have the following observations and proposals on UE location information reporting for NB-IoT NTN.

Observation 1: The mapping between a tracking area and a fixed geographical area is configured in the RAN, not in the CN.

Observation 2: Another benefit for MME signaling the coarse location information is to page the specific UE more efficiently.

Proposal 1: Coarse UE location for NB-IoT UEs is useful and should be signaled from MME to eNB in the S1AP message.

References

- R3-243009, Reply LS on UE Location Information for NB-IoT NTN