3GPP TSG-RAN WG3 Meeting #125	R3-244607

Maastricht, Netherlands, 19th – 23rd August 2024

Agenda Item:	16.3

Source: 				LG Electronics Inc.

Title:			Discussion on NG impact for Ambient IoT

Document for:		Discussion and Decision

1.	Introduction

New Rel-19 SID on solutions for Ambient IoT (Internet of Things) in NR was agreed at RAN #102 [1]. One of the objectives in this SID is to identify necessary impacts on signaling and procedures for CN-RAN interface is necessary as follows:

… - RAN3-led: - Identify necessary impacts on signaling and procedures for CN-RAN interface, to enable: - Paging - Device context management - Data transport - Identify RAN architecture aspects, including whether support for split architecture is necessary. - Identify potential solutions for locating an Ambient IoT device with no specification impact, e.g. reusing existing user location report, or minimal specification impact to convey location information to core network. …

In this contribution, we will discuss the impacts on signaling and procedures for CN-RAN interface and provide our view on it.

2.	Discussion

In RAN3#123bis meeting, the RAN3 made the following agreements and captured the open issues in the Chair’s minute.

… RAN3 considers both Topologies at the same time looking whether commonalities are applicable. [Topo1] AIoT RAN node: Corresponds to the basestation in Figure 4.2.1.1-1 in TR 38.848; A RAN node providing AIoT radio, and connecting with an AIoT-aware CN node via the XX interface. Details of the AIoT-aware CN node is subject to SA2. [Topo2] UE Reader: A UE (corresponds to the intermediate node in Figure 4.2.1.2-1 in TR 38.848); Providing AIoT radio, and connecting with a gNB (may be an AIoT enhanced gNB, corresponds to the basestation in Figure 4.2.1.2-1 in TR 38.848) via NR Uu interface. For Topology 1, RAN3 starts with AIoT RAN node being aggregated. XX interface: FFS whether it is the NG interface or a new interface, details to be discussed by RAN3 with SA2 progress of AIoT-aware CN node. XXAP (XX Application Protocol): The radio network layer signalling protocol for the XX interface. It is FFS whether it is NGAP or a new XXAP. …

In Topology 1, the AIoT device directly connects to and communicates with the AIoT RAS which provides the AIoT radio to the AIoT device and is co-located with the Reader. Then, the AIoT RAS is connected to the AIoT CN via the XX interface.

Figure 1: Logical System Architecture for AIoT [2]

In [3], three alternatives for the CN node(s) connecting with the AIoT RAS are introduced based on SA2 TR. If the AIoT CN is the AMF supporting the AIoT function (i.e., Alt #1) or the new NF for the AIoT function which is connected to the AIoT RAN via the AMF (i.e., Alt #2), the existing NG interface can be used as baseline because of the existence of the connection between the RAN and AMF. If the AIoT RAS is directly connected to the AIoT CN without the help of the AMF (i.e., Alt #3), then a new XX (e.g., NG’) interface needs to be introduced. As a result, whether to enhance the NG interface or define a new NG’ interface is pending to the SA2 decision on how to define the AIoT CN.

However, from the RAN3 perspective, it is possible to discuss which functionality should be supported in the XX interface. Our understanding is that one or more Readers with its own service area can be co-located with the AIoT RAS. In last RAN3 meeting, it was already agreed to support locating the A-IoT device at “Reader” granularity. Therefore, if the AIoT signaling from the AIoT device is received via several Readers, then the AIoT RAS and the AIoT CN can deduce/derive the location of the AIoT device considering on the service area of each Reader. To do so, the AIoT CN needs to know the Reader related information such as the Reader ID, its corresponding service area, and the ID of the serving AIoT RAS. Also, the AIoT CN can use this information to select/determine an appropriate “Reader” close to the AIoT device. In the NG interface, the NG-RAN and AMF exchange the application level data to correctly interoperate on the NG interface. Similarly, the AIoT RAS needs to provide the Reader related information to the AIoT CN during the setup procedure for the XX interface.

Proposal 1: In Topology 1, the AIoT RAS provides the Reader related information (e.g., the Reader ID, its corresponding service area, and the RAN ID of the serving AIoT RAS) to the AIoT CN during the setup procedure for the XX interface.

In Topology 2, the AIoT device communicates bidirectionally with an intermediate node between the device and the NG-RAN (e.g., may be an AIoT enhanced gNB, or AIoT RAS as in Figure 1). The UE provides the AIoT radio and is co-located with the Reader.

Figure 2: Topology 2 [4]

In this case, from the network point of view, the intermediate node is the normal UE with the capability to act as the Reader. The UE is registered with the network before serving the AIoT device [5]. As in Topology 1, the AIoT-capable NF node (e.g., AMF, new NF, …) needs to know the Reader related information such as the Reader ID, its corresponding service area, the serving UE ID and the serving NG-RAN ID. One possible option is that during the UE’s Registration procedure, the information related to the Reader can be also provided to the AIoT-capable NF node. That is, the UE acting as the intermediate node sends to the AMF the Registration Request message containing the Reader related information, and then, the AMF forwards this information to the AIoT-capable NF node. However, the NG-RAN may need to know the Reader related information because it may be also able to determine/select the UE (acting as the Reader) close to the specific AIoT device(s). Therefore, the UE should provide the Reader related information to the AIoT-capable NF node via the NG-RAN.

Proposal 2: In Topology 2, the UE provides the Reader related information (e.g., the Reader ID, its corresponding service area, and the RAN ID of the serving AIoT RAS) to the AIoT-capable NF node via the NG-RAN during the UE’s registration procedure.

In TR 38.769, there are some Editor’s notes on information exchanged between AIoT CN and AIoT RAS. First FFS is whether AIoT RAS needs to interpret/store/process the AIoT device ID in Inventory. We think that at least, the AIoT RAS needs to store this ID in the Device context. For example, the AIoT RAS may store the association between the AIoT device ID and the Reader ID. Then, if the AIoT RAS receives the command message from the AIOT CN, it can use the association to find an appropriate “Reader” close to the AIoT device, and then forward the command message to the selected “Reader”. Also, if RAN3 agrees to support periodical inventory in AIoT RAS, the AIoT device ID needs to be stored.

Proposal 3: The AIoT RAN stores the AIoT device ID as the Device context.

Next open issue is the command message can be sent for a group of devices, or all devices. We think that there is no limitation on the number of devices the AIoT CN can request in the command. For example, in Topology 1, the AIoT CN may send the command message e.g., per Device ID level, per Reader ID level, or per AIoT RAS level. If the command message is sent per AIoT RAS level, the AIoT CN can include a group of devices related to the specific AIoT RAS into the command message. Based on the list of Device IDs, the AIoT RAS can tend the command message to different AIoT devices.

Proposal 4: The command message can be sent for a group of devices, or all devices.

It is also FFS whether AIoT RAS can remain agnostic of the type of request from the AIoT CN. From our perspective, the AIoT RAS needs to differentiate the command and inventory. As mentioned above, the AIoT RAS may perform the periodical inventory. Also, it is possible that the AIoT RAS may allocate the AIoT radio resources, taking into account the type of request from the AIoT CN (pending to RAN1/RAN2’s decision). To do so, therefore, the AIoT RAS needs to be aware of the type of request from the AIoT CN.

Proposal 5: The AIoT RAS should be aware of the type of request from the AIoT CN.

3.	Conclusion

In this contribution, we focused on the impacts on signaling and procedures for CN-RAN interface and provided our view on it. The following proposals are kindly suggested to RAN3:

Proposal 1: In Topology 1, the AIoT RAS provides the Reader related information (e.g., the Reader ID, its corresponding service area, and the RAN ID of the serving AIoT RAS) to the AIoT CN during the setup procedure for the XX interface.

Proposal 2: In Topology 2, the UE provides the Reader related information (e.g., the Reader ID, its corresponding service area, and the RAN ID of the serving AIoT RAS) to the AIoT-capable NF node via the NG-RAN during the UE’s registration procedure.

Proposal 3: The AIoT RAN stores the AIoT device ID as the Device context.

Proposal 4: The command message can be sent for a group of devices, or all devices.

Proposal 5: The AIoT RAS should be aware of the type of request from the AIoT CN.

4.	Reference

- RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR,” CMCC, Huawei, T-Mobile USA, March, 2024.

- R3-243962, “[TP for TR 38.769] CB:#AIoT1_Architecture,” Ericsson (Moderator), Huawei, May, 2024.

- R3-241680, “RAN architecture aspects for Ambient IoT,” Huawei, April, 2024.

- TR 38.848, “Study on Ambient IoT (Internet of Things) in RAN,” v18.0.0.

- TR 23.700-13, “Study on Architecture support of Ambient power-enabled Internet of Things,” v0.2.0.