3GPP TSG RAN WG3 Meeting #125	R3-244613

Maastricht, Netherlands, 19 - 23 August, 2024

Title:	Discuss LS on A Realization of Network Slices for 5G Networks Using Current IPMPLS Technologies

Agenda Item:	8.1

Source:	ZTE Corporation

Document for:	Discussion

1	Introduction

This contribution provides our view of LS on A Realization of Network Slices for 5G Networks Using Current IPMPLS Technologies.

2	Discussion

In the LS , The TEAS WG would like to request 3GPP to review the document and provide confirmation that the brief 5G overview included in the Appendix is accurate. RAN3 need to check RAN part overview as described in the section B.3. Radio Access Network (RAN) , the URL is https://datatracker.ietf.org/doc/html/draft-ietf-teas-5g-ns-ip-mpls#appendix-B.3-2.3.1.

The definition in the overview is misalignment with the definition in TS38.300 and TS38.401.

The concept of BBU and RU are defined by CPRI (Common Public Radio Interface), not defined in 3GPP. Regarding the figure 36, it shown that the DU in the figure does not contain RU. Based on the definition above the figure 36, the RU is part of PHY layer. Therefore, there is the misalignment between the definition gNB-DU in TS 38.401 and the IETF’s draft.

TS 38.401: gNB Distributed Unit (gNB-DU): a logical node hosting RLC, MAC and PHY layers of the gNB or en-gNB, and its operation is partly controlled by gNB-CU. One gNB-DU supports one or multiple cells. One cell is supported by only one gNB-DU. The gNB-DU terminates the F1 interface connected with the gNB-CU. For DC operation, the MgNB-DU designates the gNB-DU of an en-gNB or a gNB acting as master node, and the SgNB-DU designates the gNB-DU of an en-gNB or a gNB acting as secondary node.

Regarding the action part of this LS, we provide the following proposal:

Proposal : RAN3 to recommend IETF align the concepts in the draft of A Realization of Network Slices for 5G Networks Using Current IP/MPLS Technologies with 3GPP specification TS38.401 and TS38.300, including:

To replace BBU and RU with gNB-CU and gNB-DU.

To remove RU,BBU split and F2 from figure 36.

To replace N2 and N3 with NG-C and NG-U interface respectively.

4	Conclusion

This document proposes the following proposals:

Proposal : RAN3 to recommend IETF align the concepts in the draft of A Realization of Network Slices for 5G Networks Using Current IP/MPLS Technologies with 3GPP specification TS38.401 and TS38.300, including:

To replace BBU and RU with gNB-CU and gNB-DU.

To remove RU,BBU split and F2 from figure 36.

To replace N2 and N3 with NG-C and NG-U interface respectively.

The corresponding response LS can be found in [2].

5	References

- R3-244027 LS on "A Realization of Network Slices for 5G Networks Using Current IP/MPLS Technologies", IETF

- R3-244614 [Draft] Reply LS on "A Realization of Network Slices for 5G Networks Using Current IP/MPLS Technologies", ZTE Corporation