3GPP TSG RAN WG3 Meeting #125	R3-244615

Maastricht, Netherlands, 19 - 23 August, 2024

Agenda Item:       10.2

Source:	ZTE Corporation

Title: 		(TP for TS 38.300 TS37.340) Discussion on MRO

Document for:	Discussion and Approval

1	Introduction

This contribution provides discussion on MRO enhancement for LTM, CHO-CPAC, and Subsequent CPAC.

2	MRO for SCPAC

Conditional PSCell Addition/Change (CPA/CPC) was first introduced in Rel-17, The network informs the UE of the candidate PSCells and their respective execution conditions, which are configured by the initiating node.  CPA can only be initiated by MN; CPC can be initiated by MN or SN. Rel-18 SON MRO supports CPAC optimization, according to 37.340, the following MRO scenarios of CPAC failure are supported:

- Too Late CPC Execution: UE receives CPC configuration, while a SCG failure occurs before CPC execution condition is satisfied; a suitable PSCell different from source PSCell is found based on the measurements reported from the UE. - Too Early CPC/CPA Execution: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; in case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE. - CPC/CPA Execution to wrong PSCell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE. There are two sub-cases: - if the suitable PSCell is one of the candidate target PSCells provided by the node initiating the CPC or by the MN initiating the CPA, but not one of the candidate PSCells selected by the candidate or target SN, it is wrong target PSCell selection at the candidate or target SN; - else, it is wrong candidate PSCell list selection at the node initiating the CPC or at the MN initiating the CPA.

S-CPAC is introduced in Rel-18.  If the S-CPAC is configured for a UE, the UE keeps the configured subsequent CPAC configuration (unless the network indicates to release it) and evaluates the subsequent execution conditions of candidate PSCells after completion of a PSCell addition, a PSCell change, a PCell change or an SCG release.

At RAN3#125 meeting , RAN3 agreed that "CPAC failure scenarios and detection mechanism captured in stage2 used as baseline." Similar to MRO for CPAC, SCPAC failure also has MRO scenarios of being too early, too late, or targeting the wrong cell. From our point of view, the stage2 of MRO for CPAC in 37.340 can be reused to accommodate the SCPA failure cases. The following could be a example.

- Too Late CPC Execution: UE receives CPC configuration or SCPAC configuration, while a SCG failure occurs before CPC execution condition is satisfied; a suitable PSCell different from source PSCell is found based on the measurements reported from the UE. - Too Early CPC/CPA Execution: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; in case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE. - CPC/CPA Execution to wrong PSCell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE. There are two sub-cases: - if the suitable PSCell is one of the candidate target PSCells provided by the node initiating the CPC or by the MN initiating the CPA or by the node initiating the SCPAC, but not one of the candidate PSCells selected by the candidate or target SN, it is wrong target PSCell selection at the candidate or target SN; - else, it is wrong candidate PSCell list selection at the node initiating the CPC or at the MN initiating the CPA or at the node initiating the SCPAC. In case of SCPAC, the Too Late CPC Execution, Too Early CPC/CPA Execution, and CPC/CPA Execution to wrong PSCell in the definition above means Too Late initial or following subsequent CPC Execution, Too Early initial or following subsequent CPC/CPA Execution, and initial or following subsequent CPC/CPA Execution to wrong PSCell.

Proposal 1: RAN3 to take MRO for CPAC as the base line for SCPAC and agree on the following MRO cases for SCPAC failure: Too later SCPC, too early SCPC/SCPA execution, SCPC/SCPA execution to wrong PScell. Additionally, RAN3 to discuss whether and how the stage2 of MRO for CPAC in 37.340 can be revised to accommodate the SCPAC failure cases.

In Rel-18 MRO for CPAC, When CPAC failure occurs, UE reports SCGFailureInformation message to MN. And then the MN performs the initial analysis when SCGFailureInformation is received from the UE e.g. whether it is CPA or CPC, if CPC whether it is MN initiated or SN initiated . The XnAP SCG FAILURE INFORMATION REPORT message is used for the exchange of SCGFailureInformation between NG-RAN ndoes. Obviously, this message can be reused for report SCG failure between RAN nodes in case of SCPAC.

Proposal 2: Reuse SCG FAILURE INFORMATION REPORT over Xn for MN to report SCG failure of SCPAC to involved SN.

For MN initiated SCPAC, MN provides recommended candidate SN(s), and for each candidate SN, a list of PScells recommended; MN also needs to provide execution conditions for the initial CPAC execution. The candidate SN decides the PScells to prepare, and for each prepared PScell, the subsequent candidate PScells and the associated execution conditions for SCPAC; For SN initiated inter-SN SCPAC, The source SN provides recommended candidate SN(s), and for each candidate SN, a list of PScells recommended;  The source SN also needs to provide execution conditions for all proposed PScells. The candidate SN  decides the accepted PScells for SCPAC, and for each prepared PScell, the candidate PScells and the associated execution conditions for SCPAC. From our point of view, simlar with CPAC, MN performs the initial analysis when SCGFailureInformation is received from the UE, RAN3 shall discuss how SCPAC problem node to retrieve the SCG failure information.

Proposal 3: RAN3 to discuss how SCPAC problem node to retrieve the SCGFailureInformation.

In Rel-18 MRO, the MN may include the CPC candidate cell list and CPC execution conditions to the SN via the SCG FAILURE INFORMATION REPORT over Xn for failure analysis.To differentiate the CPC configuration over Xn, RAN3 discuss whether to introduce  SCPAC configuration into XnAP SCG FAILURE INFORMATION REPORT message.

Proposal 4:RAN3 to discuss whether to introduce the SCPAC configuration into XnAP SCG FAILURE INFORMATION REPORT message.

If subsequent CPAC is configured for UE, various reports might need to be generated more frequently. However, the sub-optimal success report, e.g, SPR might be over-written (this does not apply to the failure information as it is reported to network in real time).

If subsequent CPAC goes on and on, network fails to fetch the report in time, such info might never be able to be aware by network. Network therefore unfortunately is not able to spot the issues of the configurations for a subsequent CPAC.

Proposal 5: RAN3 to discuss the issue of  frequently over-written report for MRO in subsequent CPAC.

3	MRO for CHO-CPAC

Following was the progress during RAN3#124：

RAN3#124 progress: Do not distinguish between fast MCG recovery/no-fast MCG recovery for now (to simplify use cases) Concurrent error cases (MCG+SCG) is FFS. The definition of these error cases needs to be further clarified RAN3 will start with the failure scenarios with UEs configured with CHO with candidate SCGs Whether to include failure and near failure scenarios related with configuration of CHO with candidate SCG(s) and CHO only is FFS.

3.1 Failure case:

During past meeting, companies provides multiple user case for failure case for CHO-CPAC, the following cases captured in [R3-243279] can be taken as basis.

As agreed in last meeting, RAN3 will start with the failure scenarios with UEs configured with CHO with candidate SCGs, which means mixed CHO configurations (e.g R16 R17 CHO mix with CHO with candidate SCG configuation ) can be deprioritized in above cases. Therefore cases 4) 5) 6) can be deprioritized  for failure case.

| Original CHO-CPAC case | Polished CHO-CPAC case | New user case No. |
| --- | --- | --- |
| Case 1 | 1: fast recovery case can be removed. 2: SCG failure or MCG failure case can be separated as two independent cases. 3: Simultaneously MCG failure and SCG failure FFS. 4: In this case, neither execution condition for PCell nor for PScell are meet. | CHO-CPAC Case 1.1: The UE detects RLF in source cell after only receiving CHO-CPAC configurations (e.g without CHO only configuration) and neither execution condition for PCell nor for PScell is meet. CHO-CPAC Case 1.2: An SCG failure occurs after only receiving CHO-CPAC configurations (e.g without CHO only configuration) ,neither execution conditions for PCell nor for PScell is meet. |
| Case 2: | The only different with CHO-CPAC Case 1.1&1.2 is execution conditions is only meet for PScell. | CHO-CPAC Case 2.1: The UE detects RLF in source cell after only receiving CHO-CPAC configurations (e.g without CHO only configuration) and execution conditions only for PScell are meet. CHO-CPAC Case 2.2: An SCG failure occurs after only receiving CHO-CPAC configurations (e.g without CHO only configuration) , and execution conditions only for PScell are meet . |
| Case 3: | The The only different with CHO-CPAC Case 1.1&1.2 is execution conditions is only meet for PCell. | CHO-CPAC Case 3.1: The UE detects RLF in source cell after only receiving CHO-CPAC configurations (e.g without CHO only configuration) and execution conditions only for PCell are meet. CHO-CPAC Case 3.2: An SCG failure occurs after only receiving CHO-CPAC configurations (e.g without CHO only configuration) , and execution conditions only for PCell are meet . |
| Case 4,5,6: | This cases are refer to UE was configured with CHO only + CHO/CPAC and deprioritized based on last meeting ‘s agreement. RAN3 will start with the failure scenarios with UEs configured with CHO with candidate SCGs |  |
| Case 7: | 1: HOF for SCG or MCG case can be separated to two independent cases. 3: Simultaneously MCG and SCG handover failure FFS. 4: In this case, execution condition for PCell and for PScell are both meet. 5: It is not clear whether this case belong to MRO type of “too early” or ‘’wrong cell ‘’, because the following behavior e.g which Cell the UE select is not described in the cased. But the case can be taken into considered at first. | CHO-CPAC Case 4.1: The UE detects HOF in candidate MN cell after receiving CHO-CPAC configurations and execution conditions for PCell and PSCell are both meet. CHO-CPAC Case 4.2: CPC/CPA execution is not successful after receiving CHO-CPAC configurations, execution conditions for PCell and PSCell are both meet. |
| Case 8: | The only different with CHO-CPAC case 4.1 & 4.2 is the failure happen shortly after the handover. | CHO-CPAC Case 5.1: The UE detects RLF in candidate MN cell shortly after a successful CHO execution after receiving CHO-CPAC configurations and execution conditions for PCell and PSCell are both meet. CHO-CPAC Case 5.2: An SCG failure occurs shortly after a successful CPC/CPA execution after receiving CHO-CPAC configurations, execution conditions for PCell and PSCell are both meet. |
| Case 9: | The failure case refers to the execution conditions for PCell and PSCell are not meet simultaneously. Covered by CHO-CPAC case 1.1/1.2/2.1/2.2/3.1/3.2. |  |

Proposal 6: MRO for CHO with candidate SCGs can start to consider the following user cases:

CHO-CPAC Case 1.1: The UE detects RLF in source cell after only receiving CHO-CPAC configurations (e.g without CHO only configuration) and neither execution condition for PCell nor for PScell is meet.

CHO-CPAC Case 1.2: An SCG failure occurs after only receiving CHO-CPAC configurations (e.g without CHO only configuration) ,neither execution conditions for PCell nor for PScell is meet.

CHO-CPAC Case 2.1: The UE detects RLF in source cell after only receiving CHO-CPAC configurations (e.g without CHO only configuration) and execution conditions for PScell are meet.

CHO-CPAC Case 2.2: An SCG failure occurs after only receiving CHO-CPAC configurations (e.g without CHO only configuration) , execution conditions for PScell are meet.

CHO-CPAC Case 3.1: The UE detects RLF in source cell after only receiving CHO-CPAC configurations (e.g without CHO only configuration) and execution conditions for PCell are meet.

CHO-CPAC Case 3.2: An SCG failure occurs after only receiving CHO-CPAC configurations (e.g without CHO only configuration) , execution conditions for PCell are meet .

CHO-CPAC Case 4.1:The UE detects HOF in candidate MN cell after receiving CHO-CPAC configurations and execution conditions for PCell and PSCell are both meet.

CHO-CPAC Case 4.2:CPC/CPA execution is not successful after receiving CHO-CPAC configurations, execution conditions for PCell and PSCell are both meet.

CHO-CPAC Case 5.1: The UE detects RLF in candidate MN cell shortly after a successful CHO execution after receiving CHO-CPAC configurations and execution conditions for PCell and PSCell are both meet.

CHO-CPAC Case 5.2: An SCG failure occurs shortly after a successful CPC/CPA execution after receiving CHO-CPAC configurations, execution conditions for PCell and PSCell are both meet.

As elaborated in the above table, we filter out several new user case with definition for CHO with candidate SCGs. When consider the problem definition of CHO-CPAC configuration, not all above user cases need to be captured. It is because some of the user case can reused by the legacy MRO mechanism, e.g MRO for CHO.

The configurations for CHO with candidate SCGs can be split into two parts. One part is CHO, the other part is CPAC. The source MN decides the configuration and execution conditions of CHO. The target candidate MN and candidate SN involved in decides the configuration and execution conditions of CPAC. It is possible to reuse MRO for CHO to solve the issue raised due to CHO-CPAC configuration, it is because the problem definition and detection are same for the two cases.  While for MRO for CPAC(Conditional PSCell addition or change failure), the mechanism is not easy to be reused. The  MRO for CPAC algorithm includes two nodes which are MN node and SN node while the MRO for CHO with candidates includes at least three Nodes which are Source MN node ,Target MN node and candidate SNs.

Proposal 7: MRO for CHO can be reused for problem definition of CHO-CPAC configuration. RAN3 to focus on CPAC part when consider problem definition of CHO-CPAC.

If above proposal can be acceptable, the following step is to refine user case based on proposal X.

Since the problem definition of CHO-CPAC is only focus on CPAC, the CHO user case can be removed fro problem definition of CHO-CPAC.

Proposal 8: For problem definition of CHO-CPAC, the following user cases are not considered:

CHO-CPAC Case 1.1; CHO-CPAC Case 2.1; CHO-CPAC Case 3.1; CHO-CPAC Case 4.1; CHO-CPAC Case 5.1.

Based on above analysis, we provide the following CHO-CPAC problem definition and corresponding user cases.

| CHO-CPAC problem definition : | User case: |
| --- | --- |
| Too Late CHO-CPAC Execution: UE receives CPC configuration, while a SCG failure occurs before CPC execution condition is satisfied; a suitable PSCell different from source PSCell is found based on the measurements reported from the UE. | CHO-CPAC Case 1.2: An SCG failure occurs after only receiving CHO-CPAC configurations (e.g without CHO only configuration) ,neither execution conditions for PCell nor for PScell is meet. CHO-CPAC Case 2.2: An SCG failure occurs after only receiving CHO-CPAC configurations (e.g without CHO only configuration) , execution conditions for PScell are meet. CHO-CPAC Case 3.2: An SCG failure occurs after only receiving CHO-CPAC configurations (e.g without CHO only configuration) , execution conditions for PCell are meet . |
| Too Early CHO-CPAC Execution: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CHO-CPAC execution; in case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE. | CHO-CPAC Case 4.2:CPC/CPA execution is not successful after receiving CHO-CPAC configurations, execution conditions for PCell and PSCell are both meet. In case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE. CHO-CPAC Case 5.2: An SCG failure occurs shortly after a successful CPC/CPA execution after receiving CHO-CPAC configurations, execution conditions for PCell and PSCell are both meet. In case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE. |
| CHO-CPAC Execution to wrong Cell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE. | CHO-CPAC Case 4.X:CPC/CPA execution is not successful after receiving CHO-CPAC configurations, execution conditions for PCell and PSCell are both meet. A suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE. CHO-CPAC Case 5.X: An SCG failure occurs shortly after a successful CPC/CPA execution after receiving CHO-CPAC configurations, execution conditions for PCell and PSCell are both meet. A suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE. |

Proposal 9:  To agree the TP for stage 2 for TS37.340 in Annex A.

3.2 Near failure case:

Based on current specification, in addition to CHO-CPAC configuration, the network can provide a complementary CHO-only configuration /execution condition to the same UE.  And in case the CHO-CPAC execution conditions are not meet, instead the CHO-only configuration can be used when the execution condition is meet. In this case, no failure report will create while the  CHO-CPAC configuration seems useless. From RAN3 point view, the useless of CHO-CPAC with CHO-only can be taken as one type of near failure case.

Proposal 10: CHO only executed as  complementary for  configuration of CHO with candidate SCG(s) can be concluded as near failure case.

3.3 Other

Additionally, since a CHO-CPAC failure could potentially cause connection failed in both MCG and SCG simultaneously, there is a need to discuss how to correlate the failure reports related to MCG and SCG.

Proposal 11:  Since a CHO-CPAC failure could potentially cause connection failed in both MCG and SCG, RAN3 to discuss how to correlate the failure reports of MCG and SCG for the related CHO-CPAC.

Near failure cases are also needed to be considered for CHO-SCPAC, We suggest to reuse SHR/SPR for sub-optimization. Since nearby failure could be happened in  both MCG and SCG simultaneously, we believe network needs to consider the correlation of SHR/SPR for MRO optimization .There is a need to discuss how to correlate SHR/SPR reports related to a specific CHO-CPAC execution.

Proposal 12: Reuse SHR/SPR for sub-optimization for CHO-CPAC, and RAN3 to discuss how to correlate the SHR/SPR for the related CHO-CPAC.

Additionally, considering the possibility that one execution in the MCG or SCG might succeed while the other fails, in this case, there is a need to perform the correlation between the failure report and the success report.

Proposal 13： RAN3 to discuss whether there is a need to correlate the success/failure report for CHO-CPAC, in the case of  one execution in the MCG or SCG might succeed while the other fails.

Either CHO or CPAC, any execution condition not being met, could lead to a 'too late failure'. However, even when the execution of CHO-CPAC is successful, we need to minimize the time between the fulfilment of the CHO and CPAC execution conditions, ideally not exceeding a certain threshold. In other words, if this threshold is exceeded, we could classify this CHO-CPAC execution as near failure. We propose the following:

Proposal 14: Introduce a new trigger condition for SHR/SPR, specifically when the time between the fulfillment of CHO and CPAC execution conditions exceeds a certain threshold.

- 4 MRO for LTM

RAN3 to prioritize MCG LTM over SCG LTM. RAN3 takes RAN2#125-bis agreement on MRO for LTM scenarios as baseline for further study. For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization. For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) source DU performs optimization. RAN3 to discuss enhancements related to the information of RACH-less access and RACH-based access from UE related to MRO for LTM. For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) FFS if CU or DU performs root cause analysis. For stage-2, FFS if existing problem definitions (Connection failure due to intra-system mobility) can be reused or if a new chapter needs to be created.

4.1 Failure case

4.1.1 Scenario of Failure case

While last RAN3 meeting agree to use RAN2#125-bis agreement on MRO for LTM scenarios as baseline, some scenario still need further updates. For example, it should be clear in the definition that the the cell attempt to  recovery should be the cell that other than the source cell.

Too late LTM:

Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and performs reestablishment procedure in a cell other than the source cell.

Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell other than the source cell, detects HOF with the selected LTM cell.

Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM execution with the selected LTM candidate cell other than the source cell.

In addition one more user case from our point of view need also to be captured for Too late LTM. From RAN3 point of view, the Cell select during UE reestablishment procedure is a key indicator for MRO detection in the RAN node.

Case 1d:

the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell, detects HOF with the selected LTM cell, and the UE performs reestablishment procedure in a cell other than the source cell.

Proposal 15: Based on agreement on MRO for LTM scenarios, to update the definition of user cases for Too late LTM as following :

Too late LTM:

Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and performs reestablishment procedure in a cell other than the source cell.

Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell other than the source cell, detects HOF with the selected LTM cell.

Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM execution with the selected LTM candidate cell other than the source cell.

Case 1d:

the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell, detects HOF with the selected LTM cell, and the UE performs reestablishment procedure in a cell other than the source cell.

4.1.2 Reports from the UE

Beam related information

During last RAN2#126meeting, the following parameter were agreed to be captured in the

If available, log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure. Reuse the existing approach of using timeConnFailure and the reconnectCellId in RLF-report also for LTM failures, details TBD. Log the LTM cell ID upon performing recovery an LTM candidate cell, details TBD e.g. which field. Extend lastHO-Type in RLF-Report to indicate the LTM cell switch as last executed mobility procedure.

During the LTM procedure, the UE switches to the cell and beam indicated by MAC CE, but If the UE is switched to the right cell but wrong beam (or beam that could result in successful LTM cell switch but not stable connection at target beam), and the connection in the target cell would be failed within a short period if beam recovery is also failed. In this case, the UE needs to report the TCI state configuration of the current failed target PCell in the RLF, as well as the failed beam.

Based on above discussion, we provide proposal as follows:

Proposal 16：For LTM optimization, RAN3 to enhance RLF report with beam related information :TCI state configuration/failed beam.

RACH-Less /RACH-Based Information

At last meeting, the following agreement achieved for RACH less based LTM.

RAN3 to discuss enhancements related to the information of RACH-less access and RACH-based access from UE related to MRO for LTM.

TS 38.300 9.2.3.6 RACH-less handover During intra-gNB HO procedure, RACH-less handover can be configured for a UE. The RACH-less handover procedure applies the following functionality: - The UE uses the same timing advance value at the target cell as in the source cell or timing advance value of 0. - The handover command for the UE may contain a beam identifier for the beam to be used by the UE at the target cell. The beam may be determined based on a UE measurement report and/or left up to gNB implementation, e.g., using the target cell's knowledge about the beam(s) used by the UE at the co-located source cell. - The handover command may include a configured UL grant. UE can fallback to RACH when there is no valid configured uplink grant. Alternatively, an UL grant is dynamically signalled by the target cell. - The UE transmits the RRCReconfigurationComplete message using the configured or dynamically signalled UL grant. Successful UL data reception on the target cell terminates the RACH-less handover execution.

Based on Stage 2 description, it is possible that UE may be configured with RACH-Less handover configuration for intra-gNB LTM. After failure of RACH-Less handover, the UE is allowed to fallback to RACH based Handover. While the RACH based is also failure, the NG-RAN node may not aware which configuration cause the failure. It is beneficial for gNB to know the situation happen during LTM procedure, especially when failure happen.

Since the gNB aware whether RACH-Less and or RACH-Based configurations have been configured to the UE, the only missing information is whether UE fallback to RACH.

Proposal 17: For LTM optimization, RAN3 to enhance RLF report with the information whether UE fallback to RACH based handover.

4.1.3 Stage 2 description

One open issue of last meeting related to Stage 2 is as following:

For stage-2, FFS if existing problem definitions (Connection failure due to intra-system mobility) can be reused or if a new chapter needs to be created.

Based on current progress, it could be observed that the user cases taken into account  in MRO of LTM actually same as the user cases for Rel-17 MRO of CHO. It is easy to reuse the current MRO definition with limited extension.

Proposal18: RAN3 to agree reuse existing problem definitions (Connection failure due to intra-system mobility) for MRO of LTM.

Proposal19: RAN3 to agree the TP in Annex B for MRO of LTM problem definition.

4.1.4 F1 impact

4.1.4.1 Signalling impact

For failure case, in order for root analysis, the RLF report should be convey to CU and or DU. It is noted the RLF report already supported in ACCESS AND MOBILITY INDICATION message. It is propose to reused the Message for MRO of LTM.

9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |

Proposal 20: To reusing F1AP ACCESS AND MOBILITY INDICATION message to report LTM related information over F1 from CU to DU.

4.1.4.2 Root cause analysis

The progress and open issue of last meeting related to F1 is as following:

For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization.

For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) source DU performs optimization.

RAN3 to discuss enhancements related to the information of RACH-less access and RACH-based access from UE related to MRO for LTM.

For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) FFS if CU or DU performs root cause analysis.

The open issue relate to wrong cell selection, which means the cell may not in the candidate cell list.

Based on current LTM mechanism, both gNB-CU and candidate gNB-DUs involved in the selection of candidate cell list. It is possible the cell which selected by gNB-CU is not selected by gNB-DU. Therefore , the node which make the wrong decision should do the root cause analysis.

TS 38.401 for LTM The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU(s), containing one target candidate cell ID, the LTM configuration ID of the candidate cell, LTM configuration ID mapping list, and the CSI resource configuration. The gNB-CU indicates the source gNB-DU ID, and requests PRACH resources from the candidate gNB-DU. The gNB-CU may request the candidate gNB-DU to provide the lower layer configuration for the purpose of generating the reference configuration. 4. If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configurations (e.g., TCI state configuration, RACH configuration, and the CSI report configuration) for the accepted target candidate cell.

Proposal 21: For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing):

If the suitable Cell after failure is one of the candidate target LTM Cells provided by the gNB-CU, but not one of the candidate Cells accepted by the gNB-DU, it is wrong target Cell selection at the gNB-DU, gNB-DU do the final root cause analysis, else it is gNB-CU do the final root cause analysis.

4.2 Near failure case

If LTM is successful, meaning the UE successfully transmits and receives data based on the beam configured by the cell switch command. However, if the UE experiences beam failure in a short time, it indicates that the beam configured by LTM (the beam indicated in the cell switch command) is not good enough. The UE tries beam failure recovery, which is different from the beam configured by the cell switch command. This case is also a near failure case, and UE shall report failed and successfully recovery beam of target PCell, TCI state configuration of the target PCell, L1 measurement in SHR report, then RAN can use these information to analyse near failure.  Furthermore, beam recovery is need to be add as  new trigger for SHR.

Proposal 22: For MCG LTM,  Introduce a trigger cause for beam recovery into SHR report.

Proposal 23: For MCG LTM, Introduce failed and recovery beam of target PCell, TCI state configuration of the target PCell, L1 measurement into SHR report for analysis of near failure.

Similar as in FAILURE CASE, network does not aware whether UE suffer a recovery of RACH-Based handover.

Proposal 24: For LTM optimization, RAN3 to enhance SHR report with the information whether UE fallback to RACH based handover.

4.2 Others

subsequent LTM.

If subsequent LTM is configured for UE, various report might be generated, in a higher frequency. However, the failure information or the sub-optimal success report (if there are any), might be over-written very soon. If subsequent LTM goes on and on, network might fail to fetch the report in time, and won’t be able to spot the issues in a subsequent LTM (and therefore to optimize it).

Proposal 25: RAN3 to discuss the issue of frequently over-written report for MRO in subsequent LTM.

For early UL sync to get the TA value for the candidate cell, there are two ways: UE based or network based.

-For network based, it is triggered by network with a PDCCH order, and the final TA value is indicated to UE in the cell switch command.

-For the TA value obtained by UE implementation: the UE performs TA measurement for the candidate cells after being configured by RRC but the exact time the UE performs TA measurement is up to UE implementation. The UE applies the TA value measured by itself and performs RACH-less LTM upon receiving the cell switch command. The network may also(but not always) send a TA value in the LTM cell switch command MAC CE without early TA acquisition.

The possible issues for the TA obtaining process, especially UE-based, are that the TA might not be accurate enough, or not timely updated (UE does not check the validity of the TA value UE obtains the TA itself), which impact LTM failure or cause interruptions when using RACH-based LTM.

Proposal 26: RAN3 to discuss how issues in TA value (e.g., accuracy, validity) could impact LTM failure or cause interruptions when using RACH-based LTM.

- 4 Conclusion:

The following proposals are provided for MRO for SCPAC:

Proposal 1: RAN3 to take MRO for CPAC as the base line for SCPAC and agree on the following MRO cases for SCPAC failure: Too later SCPC, too early SCPC/SCPA execution, SCPC/SCPA execution to wrong PScell. Additionally, RAN3 to discuss whether and how the stage2 of MRO for CPAC in 37.340 can be revised to accommodate the SCPAC failure cases.

Proposal 2: Reuse SCG FAILURE INFORMATION REPORT over Xn for MN to report SCG failure of SCPAC to involved SN.

Proposal 3: RAN3 to discuss how SCPAC problem node to retrieve the SCGFailureInformation.

Proposal 4:RAN3 to discuss whether to introduce the SCPAC configuration into XnAP SCG FAILURE INFORMATION REPORT message.

Proposal 5: RAN3 to discuss the issue of  frequently over-written report for MRO in subsequent CPAC.

The following observations and proposals are provided for MRO for CHO-CPAC:

Proposal 6: MRO for CHO with candidate SCGs can start to consider the following user cases:

CHO-CPAC Case 1.1: The UE detects RLF in source cell after only receiving CHO-CPAC configurations (e.g without CHO only configuration) and neither execution condition for PCell nor for PScell is meet.

CHO-CPAC Case 1.2: An SCG failure occurs after only receiving CHO-CPAC configurations (e.g without CHO only configuration) ,neither execution conditions for PCell nor for PScell is meet.

CHO-CPAC Case 2.1: The UE detects RLF in source cell after only receiving CHO-CPAC configurations (e.g without CHO only configuration) and execution conditions for PScell are meet.

CHO-CPAC Case 2.2: An SCG failure occurs after only receiving CHO-CPAC configurations (e.g without CHO only configuration) , execution conditions for PScell are meet.

CHO-CPAC Case 3.1: The UE detects RLF in source cell after only receiving CHO-CPAC configurations (e.g without CHO only configuration) and execution conditions for PCell are meet.

CHO-CPAC Case 3.2: An SCG failure occurs after only receiving CHO-CPAC configurations (e.g without CHO only configuration) , execution conditions for PCell are meet .

CHO-CPAC Case 4.1:The UE detects HOF in candidate MN cell after receiving CHO-CPAC configurations and execution conditions for PCell and PSCell are both meet.

CHO-CPAC Case 4.2:CPC/CPA execution is not successful after receiving CHO-CPAC configurations, execution conditions for PCell and PSCell are both meet.

CHO-CPAC Case 5.1: The UE detects RLF in candidate MN cell shortly after a successful CHO execution after receiving CHO-CPAC configurations and execution conditions for PCell and PSCell are both meet.

CHO-CPAC Case 5.2: An SCG failure occurs shortly after a successful CPC/CPA execution after receiving CHO-CPAC configurations, execution conditions for PCell and PSCell are both meet.

Proposal 7: MRO for CHO can be reused for problem definition of CHO-CPAC configuration. RAN3 to focus on CPAC part when consider problem definition of CHO-CPAC.

Proposal 8: For problem definition of CHO-CPAC, the following user cases are not considered:

CHO-CPAC Case 1.1; CHO-CPAC Case 2.1; CHO-CPAC Case 3.1; CHO-CPAC Case 4.1; CHO-CPAC Case 5.1.

Proposal 9: To agree the TP for stage 2 for TS37.340 in Annex A.

Proposal 10: CHO only executed as  complementary for  configuration of CHO with candidate SCG(s) can be concluded as near failure case.

Proposal 11:  Since a CHO-CPAC failure could potentially cause connection failed in both MCG and SCG, RAN3 to discuss how to correlate the failure reports of MCG and SCG for the related CHO-CPAC.

Proposal 12: Reuse SHR/SPR for sub-optimization for CHO-CPAC, and RAN3 to discuss how to correlate the SHR/SPR for the related CHO-CPAC.

Proposal 13： RAN3 to discuss whether there is a need to correlate the success/failure report for CHO-CPAC, in the case of  one execution in the MCG or SCG might succeed while the other fails.

Proposal 14: Introduce a new trigger condition for SHR/SPR, specifically when the time between the fulfillment of CHO and CPAC execution conditions exceeds a certain threshold.

The following proposals are provided for MRO for LTM:

Proposal 15: Based on agreement on MRO for LTM scenarios, to update the definition of user cases for Too late LTM as following :

Too late LTM:

Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and performs reestablishment procedure in a cell other than the source cell.

Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell other than the source cell, detects HOF with the selected LTM cell.

Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM execution with the selected LTM candidate cell other than the source cell.

Case 1d:

the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell, detects HOF with the selected LTM cell, and the UE performs reestablishment procedure in a cell other than the source cell.

Proposal 16：For LTM optimization, RAN3 to enhance RLF report with beam related information :TCI state configuration/failed beam.

Proposal 17: For LTM optimization, RAN3 to enhance RLF report with the information whether UE fallback to RACH based handover.

Proposal 18: RAN3 to agree reuse existing problem definitions (Connection failure due to intra-system mobility) for MRO of LTM.

Proposal 19: RAN3 to agree the TP in Annex B for MRO of LTM problem definition.

Proposal 20: To reusing F1AP ACCESS AND MOBILITY INDICATION message to report LTM related information over F1 from CU to DU.

Proposal 21: For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing):

If the suitable Cell after failure is one of the candidate target LTM Cells provided by the gNB-CU, but not one of the candidate Cells accepted by the gNB-DU, it is wrong target Cell selection at the gNB-DU, gNB-DU do the final root cause analysis, else it is gNB-CU do the final root cause analysis.

Proposal 22: For MCG LTM,  Introduce a trigger cause for beam recovery into SHR report.

Proposal 23: For MCG LTM, Introduce failed and recovery beam of target PCell, TCI state configuration of the target PCell, L1 measurement into SHR report for analysis of near failure.

Proposal 24: For LTM optimization, RAN3 to enhance SHR report with the information whether UE fallback to RACH based handover.

Proposal 25: RAN3 to discuss the issue of frequently over-written report for MRO in subsequent LTM.

Proposal 26: RAN3 to discuss how issues in TA value (e.g., accuracy, validity) could impact LTM failure or cause interruptions when using RACH-based LTM.

Annex  A TP for TS 37.340

10.xx  CHO-CPAC Execution failure

Too Late CHO-CPAC Execution: UE receives CPC configuration, while a SCG failure occurs before CPC execution condition is satisfied; a suitable PSCell different from source PSCell is found based on the measurements reported from the UE.

Too late CHO-CPAC Case 1: An SCG failure occurs after only receiving CHO-CPAC configurations (e.g without CHO only configuration) ,neither execution conditions for PCell nor for PScell is meet.

Too late CHO-CPAC Case 2: An SCG failure occurs after only receiving CHO-CPAC configurations (e.g without CHO only configuration) , execution conditions for PScell are meet.

Too late CHO-CPAC Case 3: An SCG failure occurs after only receiving CHO-CPAC configurations (e.g without CHO only configuration) , execution conditions for PCell are meet.

Too Early CHO-CPAC Execution: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CHO-CPAC execution; in case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE.

Too Early CHO-CPAC Case 1:CPC/CPA execution is not successful after receiving CHO-CPAC configurations, execution conditions for PCell and PSCell are both meet. In case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE.

Too Early CHO-CPAC Case 2: An SCG failure occurs shortly after a successful CPC/CPA execution after receiving CHO-CPAC configurations, execution conditions for PCell and PSCell are both meet. In case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE.

CHO-CPAC Execution to wrong Cell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE.

CHO-CPAC  to wrong Cell Case 1: CPC/CPA execution is not successful after receiving CHO-CPAC configurations, execution conditions for PCell and PSCell are both meet. A suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE.

CHO-CPAC  to wrong Cell Case 2: An SCG failure occurs shortly after a successful CPC/CPA execution after receiving CHO-CPAC configurations, execution conditions for PCell and PSCell are both meet. A suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE.

Annex  B TP for TS 38.300

15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of LTM, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM Execution, Too Early LTM Execution and LTM Execution to Wrong Cell.

Detection mechanism

A failure indication may be initiated after a UE attempts to re-establish the radio link connection at NG-RAN node B after a failure at NG-RAN node A. NG-RAN node B may initiate the Failure Indication procedure towards multiple NG-RAN nodes if they control cells which use the PCI signalled by the UE during the re-establishment procedure. The NG-RAN node receiving this selects the UE context that matches the received Failure Cell ID and C-RNTI, and, if available, uses the shortMAC-I to confirm this identification, by calculating the shortMAC-I and comparing it to the received IE.

A failure indication may also be sent to the node last serving the UE when the NG-RAN node fetches the RLF REPORT from UE by triggering:

-	The Failure Indication procedure over Xn;

-	The Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

The detailed detection mechanisms for too late handover, too early handover and handover to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

-	Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO is configured but the CHO execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if DAPS HO is configured but an RLF is detected in the source cell with successful DAPS HO., or if LTM is configured but the LTM execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt).

-	Intra-system Too Early Handover: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the cell UE attempts to re-connect is the cell that served the UE at the last handover initialisation or fall back to the source cell configuration in case of DAPS HO.

-	Intra-system Handover to Wrong Cell: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the cell UE attempts to re-connect/the cell UE attempts CHO recovery /the LTM recovery is neither the cell that served the UE at the last handover initialisation nor the cell that served the UE where the RLF happened or the cell that the handover was initialized toward.

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO execution until connection failure or or the time elapsed since the LTM execution until connection failure.

In case of Too Early Handover or Handover to Wrong Cell, the NG-RAN node receiving the failure indication may inform the NG-RAN node controlling the cell where the mobility configuration caused the failure by means of the Handover Report procedure over Xn or the Uplink RAN Configuration Transfer procedure over NG. This may include the RLF report.