3GPP TSG RAN WG3 Meeting #125	R3-244618

# Maastricht, Netherlands, 19 - 23 August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1186 | rev |  | Current version: | 16.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Area of Interest RAN Node for location report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2024-08-7 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Global RAN Node ID IE actual used neither by AMF nor NG-RAN node/ This IE can be removed from the specification. The Global RAN Node ID IE is practically not useful. It is because AMF will anyway aware the RAN level location of the UE during RRC_CONNECTED states. Either utilizing NG based Handover or Xn Based Handover, the AMF will always aware the RAN node level information of the UE. If Core network want to collect RAN node level of the UE, the AMF just monitor the UE based on UE’s handover procedure, no extra Location report procedure needed. When track the history of introduce the Global RAN Node ID IE , it is noted the IE introduce based on SA2’s LS response[R3-184492][R3-185246]. - RAN node identifiers can be provided to the NG-RAN in the Location Reporting Control. It may be used in case of the source NG-RAN node transfers the requested location reporting information to target NG-RAN node. However the LS does not explain what is the benefit for RAN node transfers the requested location reporting information to target Node. If the source RAN node is not the in the list of requested location, the AMF does not necessary to send the information to the RAN. If the target RAN node is in the list of requested location, the AMF will anyway know whether the UE finally successful access to the target. No benefit can be found for source RAN node transfer such Global RAN Node ID IE information the the target. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Semantics description to declare the Global RAN Node ID IE is not used in the specification. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release) as it affects semantic part of Global RAN Node ID IE only. The CR is backwards compatible from a functional and protocol point of view. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The misused of the Global RAN Node ID IE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.66 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9.3.1.66	Area of Interest

This IE indicates the area of interest.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Area of Interest TAI List |  | 0..1 |  |  |
| >Area of Interest TAI Item |  | 1..<maxnoofTAIinAoI> |  |  |
| >>TAI | M |  | 9.3.3.11 |  |
| Area of Interest Cell List |  | 0..1 |  |  |
| >Area of Interest Cell Item |  | 1..<maxnoofCellinAoI> |  |  |
| >>NG-RAN CGI | M |  | 9.3.3.73 |  |
| Area of Interest RAN Node List |  | 0..1 |  | This IE is not used in the specification. If received, the IE is ignored. |
| >Area of Interest RAN Node Item |  | 1..<maxnoofRANNodeinAoI> |  |  |
| >>Global RAN Node ID | M |  | 9.3.1.5 |  |