3GPP TSG-RAN WG3 Meeting #R3-125	                       	R3-244633 Maastricht, Netherlands, Aug 19-23, 2024

Agenda Item:	17.3

Source: 	Rakuten

Title:  	Discussion on OD-SIB1

Document for:	Discussion and decision

# Introduction

The Rel-19 WI on enhancements of network energy savings for NR (RP-234065), with the objective to study on-demand SIB1 for idle/inactive UEs as highlighted below:

Study procedures and signalling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] Triggering method by uplink wake-up-signal using an existing signal/channel. Wake-up-signal configuration provisioning to UE Note: No modification of SSB will be discussed under this objective Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. Checkpoint for normative work in RAN#105

# Background

## Progress from RAN1

RAN1#116 (February 2024) made the following assumptions for the purpose of discussion:

For discussion purpose, the following assumption will be used in RAN1

  - Cell A: A cell that is periodically transmitting at least its own SIB1

  - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

RAN1#117 (May 2024) made the following agreement:

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 focuses its studies on the following cases:​

Case 1: Option 1+A+X ​

Case 2: Option 1+B+X​

Case 3: Option 2+B+Y

where

  - Option 1: UE transmits UL WUS to NES Cell

  - Option 2: UE transmits UL WUS to Cell A

  - Option A: UE obtains the UL WUS configuration from NES Cell

  - Option B: UE obtains the UL WUS configuration from Cell A

  - Option X: UE receives on-demand SIB1 from NES Cell

  - Option Y: UE receives on-demand SIB1 from Cell A

## Progress from RAN2

RAN2#126 (May 2024) made the following agreement:

Proposal 1: Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

  - More than one Cell A may provide configuration for the same NES cell.

  - The same Cell A may assist more than one NES Cells.

## Progress from RAN3

RAN3#123-bis (April 2024) made the following agreement:

  - Scenario1: UE obtains the UL WUS configuration from Cell A and transmits UL WUS to NES Cell.

  - UL WUS configuration from NES Cell to Cell A via Xn, e.g., before entering on-demand SIB1 mode in the NES Cell or when the UL WUS configuration is changed

# Activation of Energy Saving (ES) mode on NES Cell

As per TS38.473, gNB-DU is the network entity responsible for encoding and broadcasting SIB1 for a given cell. Hence, the information related to all the fields of SIB1 is available at the gNB-DU.

gNB-DU is the network entity responsible for broadcasting SIB1.

NOTE: In this document, we call the gNB-DU that is associated with the NES cell, a “NES cell gNB-DU” and the gNB-CU that is associated with the NES cell, a “NES cell gNB-CU”. Likewise for Cell A also.

- What are the various possibilities of SIB1 transmission in the NES cell?

The NES cell is a cell that may transmit On Demand SIB1 (OD-SIB1) transmission in response to UL WUS from a UE. It need not transmit periodic SIB1 all the time. Based on this understanding, a NES cell can have one of the following three “SIB1 modes”:

- NES cell is currently broadcasting periodic SIB1

- NES cell is currently broadcasting only OD-SIB1

- NES cell is currently not broadcasting either periodic SIB1 or OD-SIB1

NES cell gNB DU to store “SIB1 mode” which reflects whether the NES cell is transmitting periodic SIB1, OD-SIB1, or not transmitting any SIB1.

- Which entity in the NES cell makes the Energy Saving decision

There are several reasons which make the NES cell gNB-DU the preferred entity to decide whether the NES cell should transition to a energy saving mode.

Firstly, for any cell, the gNB-DU is aware of the current traffic, including the RRM resources in use, number of UEs connected or camped to the cell and the type of bearers established for the UEs.

Secondly, the gNB-DU receives the UL WUS configuration from the UEs when the UE intends to trigger OD-SIB1 from the NES cell.

And lastly, in any cell, the gNB-DU is the entity which broadcasts SIB1.

Therefore, on receiving the UL WUS configuration from a UE, the NES cell gNB-DU can begin OD-SIB1 transmission without any signaling overhead with any other entity e.g. NES cell gNB-CU. Moreover, on observing no/low traffic in the cell, the NES cell gNB-DU may decide to switch to an ES state, where it stops ongoing SIB1 transmissions.

Hence, we propose that the decision of which SIB1 mode be used should lie with the NES cell gNB-DU. This may be done in agreement with NES Cell gNB-CU.

NES cell gNB DU is the preferred network entity which is best equipped to decide SIB1 mode to be used in the NES cell.

What are the triggers for NES cell to transmit OD-SIB1 or periodic SIB1?

As proposed in the previous paragraph, the decision of which SIB1 mode to adopt lies with the NES cell gNB-DU. So irrespective of the trigger, the request for OD-SIB1 must come to the NES cell gNB-DU. Following are some of the triggers which can result in NES gNB-DU deciding on SIB1 transmission:

- If the gNB-DU receives UL WUS request from a UE, then it changes the SIB1 mode, and starts broadcasting OD-SIB1.

- If the gNB-DU receives a large number of UL WUS requests from multiple UEs, it may decide to change the mode to periodic SIB1 transmission and deactivate energy saving.

- If the gNB-CU receives a large number of incoming HO, or UE attach requests, then it can request the gNB-DU to deactivate the energy saving mode and begin periodic SIB1 transmission.

- Under some conditions in Cell A like an idle mode overload condition, Cell A may want to activate load balancing and offload some of its UEs to NES cell(s) if possible. In which case, Cell A may trigger the NES cell to broadcast OD-SIB1 or periodic SIB1 depending on particular use case. This request will be sent from Cell A gNB-CU to NES cell gNB-CU over Xn and then to NES cell gNB-DU over F1 interface.

There are various triggers from NES cell gNB-DU, NES cell gNB-CU and Cell A to transmit OD-SIB1 or periodic SIB1 by NES Cell

What are the triggers for NES cell to stop transmit OD-SIB1 or periodic SIB1?

Typically, when NES cell gNB-DU observes there are no UEs in RRC Connected, RRC Inactive or Idle mode, then it may decide to activate the energy saving mode. This may be done in agreement with NES Cell gNB-CU. There may be other conditions configured by the network operator to activate energy saving.

When the conditions are met, the NES cell gNB-DU stops broadcasting all SIB1 transmissions and sends SIB1 mode change update to NES cell gNB-CU over the Xn interface.

# NES Cell Information at Cell A

As proposed in the section 3, the NES gNB DU is aware of the latest “SIB1 mode” of an NES cell. This mode, and any change to this mode is notified by the NES gNB DU to the NES gNB CU. The current SIB1 transmission mode of NES cell(s) is required to be notified to Cell A to support Case 1 and Case 2 as highlighted in Section 2.1.

The NES cell gNB-CU may send the “SIB1 mode” and any modification to the SIB1 mode to Cell A gNB over Xn. In case the NES cell is under the coverage of multiple Cell As, then this information is transmitted to all the relevant Cell A gNB-CUs. Cell A gNB can use the NES cells’ SIB1 status reports appropriately to broadcast the UL WUS configurations in it’s SIB1 to the camped UEs.

NES cell gNB DU to transmit “SIB1 mode” and any change in the “SIB1 mode” of the NES cell to the NES cell gNB-CU over the F1 Interface.

NES cells gNB-CU to transmit the “SIB1 mode” to the Cell A (or multiple Cell As) gNB-CU over Xn.

- What information is required from the NES Cell at Cell A?

Cell A gNB gets information from NES cell gNB which includes the following:

- PCI of the NES cell

- UL WUS configuration of the NES cell

- “SIB1 mode” of the NES cell

The information from NES cell to Cell A includes PCI, UL WUS configuration and the current SIB1 mode of the NES cell.

Cell A gNB-CU transmits this information to the Cell A gNB-DU over the F1 Interface. The Cell A gNB-DU is responsible for broadcasting its SIB1. Cell A SIB1 is altered to include some of these fields to enable IDLE/INACTIVE UEs camped on Cell A to acquire UL WUS configuration and send UL WUS request to a NES cell to request OD-SIB1 transmission.

Cell A gNB-CU transmits the NES cell information to cell A gNB-DU which is responsible for broadcasting this to the IDLE/INACTIVE UEs in Cell A SIB1.

# Call flow to support the various SIB1 modes of NES Cell

- NES Cell transitioning to Energy Saving mode

Let’s assume the baseline scenario where NES cell is broadcasting periodic SIB1. If the gNB-DU observes there are no UEs in RRC Connected, RRC Inactive or Idle mode, then it may decide to activate the energy saving mode. This may be done in agreement with NES Cell gNB-CU. There may be other conditions configured by the network operator to activate energy saving. The NES cell gNB-DU then stops all SIB1 transmission, changes the SIB1 mode to reflect this and sends the SIB1 mode and the UL WUS Configuration to the NES gNB-CU over the F1 interface. The NES cell gNB-CU communicates this with Cell A gNB-CU over Xn, which in turn transmits it to Cell A gNB-DU over F1 for broadcasting in Cell A SIB1.

- NES Cell transitioning to OD-SIB1 mode

When conditions for a UE camped on Cell A are favorable, it sends a UL WUS request to the NES cell. The NES cell gNB-DU, on receiving such a request, changes the SIB1 mode to OD-SIB1, and begins to broadcast OD-SIB1. It also informs the NES cell gNB-CU with the SIB1 mode update and the UL WUS configuration over the F1 interface. The NES cell gNB-CU transmits this to Cell A gNB-CU over Xn, which in turn transmits it to Cell A gNB-DU over F1 for broadcasting in Cell A SIB1. The UE, on reading NES Cell SIB1, can perform cell reselection and camp on NES cell.

- NES Cell deactivating Energy Saving mode

When conditions are favorable at the NES cell to deactivate energy saving mode, the NES Cell gNB-DU changes the SIB1 mode to periodic SIB1. The change in SIB1 mode is conveyed from NES cell gNB-DU to NES cell gNB-CU over F1. NES cell transmits SIB1 mode along with the PCI of the NES Cell to Cell A gNB-CU over Xn, which communicates this information to Cell A gNB-DU over F1. The Cell A SIB1 can then be updated to not broadcast any NES cell information. If a UE camped on Cell A is in favorable condition for cell reselection, it reads the NES cell SIB1 and performs cell reselection to camp on the NES cell.

# Conclusion

We have the following observations:

Observation 1	gNB-DU is the network entity responsible for broadcasting SIB1.

We have the following proposals:

- NES cell gNB DU to store “SIB1 mode” which reflects whether the NES cell is transmitting periodic SIB1, OD-SIB1, or not transmitting any SIB1.

- NES cell gNB DU is the preferred network entity which is best equipped to decide SIB1 mode to be used in the NES cell.

- There are various triggers from NES cell gNB-DU, NES cell gNB-CU and Cell A to transmit OD-SIB1 or periodic SIB1 by NES Cell

- NES cell gNB DU to transmit “SIB1 mode” and any change in the “SIB1 mode” of the NES cell to the NES cell gNB-CU over the F1 Interface.

- NES cells gNB-CU to transmit the “SIB1 mode” to the Cell A (or multiple Cell As) gNB-CU over Xn.

- The information from NES cell to Cell A includes PCI, UL WUS configuration and the current SIB1 mode of the NES cell.

- Cell A gNB-CU transmits the NES cell information to cell A gNB-DU which is responsible for broadcasting this to the IDLE/INACTIVE UEs in Cell A SIB1.

# Reference

3GPP TR 38.864 V18.1.0, Study on network energy savings for NR.

Chairman’s Notes RAN3 #123-bis

Chairman’s Notes RAN3#124

R3-243868 Summary of offline disc