3GPP TSG RAN WG3 Meeting #125	R3-244634

Maastricht, Netherlands, 19 - 23 August, 2024
Agenda Item:	14.3

Source:    TCL.

Title:         Discussion on NTN regenerative architecture

Document for:	Discussion and Decision

# Introduction

As part of Release 19, a new work item is proposed to define further enhancements for NG-RAN based Non-Terrestrial Networks to support NTN architecture with 5G system functions on board of satellite (i.e., regenerative payloads) [1]. The support of NTN architecture with 5G system functions of regenerative payloads provides new architecture option(s) besides the transparent payload as specified in Rel-17 and Rel-18, which makes the deployment of non-terrestrial network more flexible. On this regard, during last RAN2 meeting, there was a discussion on UE Xn/NG related procedures impact due inter-satellite (ISL) link or feeder link disconnection, switching or failure and  the following agreements has been made.

Agreements:

1. Regarding potential issues with RRC re-establishment, RAN2 will wait for RAN3 input, if any 2. Regarding potential issues with support of Inactive state, RAN2 will wait for RAN3 input, if any 3. RAN2 aims at supporting RACH-less handover for regenerative architecture. Can further discuss whether any optimization is needed 4. RAN2 assumes that network verified UE positioning is supported for regenerative architecture 5. RAN2 assumes that the typical setting for common TA and kmac would be zero in case of regenerative mode with full gNB on board. FFS whether we capture in the spec that common TA and Kmac shall/should be set to zero.

This report discusses UE Xn and NG related procedures impact due to triggering of UE connected inactive RRC connection procedures during an Xn or NG disconnection, switching or failure.

# Discussion

3GPP is currently discussing the regenerative payload architecture of NTN satellite access in both SA and RAN WGs As part of Release 19, it is proposed to define further enhancements for NG-RAN based Non-Terrestrial Networks to support non-terrestrial network architecture with 5G system functions on board of satellite (i.e., regenerative payloads). In such an architecture the gNB can be deployed on-board of a satellite terminating the Uu radio interface toward the UE (via the service link) and terminating the NG network interface toward the 5GC (via the feeder link). Additionally, different gNBs hosted by two regenerative satellite payloads may often be interconnected to each other’s by the mean of an inter-satellite link (ISL) interface for exchanging of Xn protocol messages. Under such architecture, unlike terrestrial networks (TN) and transparent NTN access where NG and Xn interfaces are presumed to be always available, in regenerative NTN access, NG and/or Xn logical interfaces may often be disconnected, switched, suspended, or updated due to satellite movement or any other physical or logical factors that may impact the performance of the of feeder link between gNB and AMF or the inter-satellite links (ISLs) between the satellite gNBs [2, 3, 4]. For example, if a UE is accessing from an NTN access with gNB on-board, there will always be a single NG logical link for such a UE towards its selected AMF/UPF and one or more Xn interface toward its last serving gNB at a given time (Figure 1). If a XN and/or link failure or switching occurred (Figure 2), the following UE related RRC connection related procedures maybe impacted:

- UE NG link related procedures such as initial UE message, initial UE context setup message and/or the paging message between UE serving gNB and selected AMF.

- UE XN link related procedures such as UE context retrieval request and response messages between the satellite gNBs serving UE and last UE serving satellite gNB.

Figure 1: UE accessibly form NTN Regenerative Access

Figure 2: Example of Regenerative NTN access with ISL and Feeder Link Failure

An NG or XN failure or disconnection impacting one of the above mention procedures (Figure 2),  may lead to interruption of UE RRC resumption, RRC reestablishment , UE triggered state transition from RRC IDLE to RRC CONNECTED and/or network triggered RRC IDLE to RRC CONNECTED state transition via paging. One option to address this issue, allowing UE to maintain and timely initiate an RRC connection procedures during NG and/or XN failure, is to try to recover or reconnect the failed NG/XN link by AMF or satellite gNB if possible. Secondly, If the failed link is not recoverable (e.g., after time or a number of tries), the AMF or satellite gNB may perform exchanging information about other available XN and NG links, so as to find an alternative active neighbouring Xn or NG link or XN and NG links combination to reroute or carry out the UE NG/XN RRC connection associated procedures over (Figure 3). However, for the NG/XN recovery option to work, RAN3 may need to discuss the following Xn/NG enhancements.

- Exchanging between satellite gNB and AMF of UE NG related procedure such as initial UE message, initial UE context setup message and/or the paging message when trying to reconnect or resume a field NG interface in NTN regenerative architecture.

- Exchanging between satellite gNBs of UE Xn related procedure such as (UE context retrieval request and response messages) shall be exchanged between satellite gNBs when trying to reconnect or resume a field NG interface in NTN regenerative architecture.

Similarly, for the second option to work RAN3 may also need to discuss the following enhancements:

- Exchanging of UE Xn associated procedures such as initial UE message, initial UE context setup message and/or the paging message within NG protocol signalling.

- Exchanging of UE NG associated procedures such as retrieval context request and response messages within XN protocol signalling.

Procedures of connection procedures during NG or XN failure:

Based on the above discussion we kindly ask RAN3 to discuss the following proposals to

- Proposal 1: An NG or XN failure or disconnection may UE connected inactive RRC connection procedures, one way to address this is to reconnect or recover the failed NG/XN link by AMF or satellite gNB as per UE RRC connection trigger.

- Proposal 2: An alternative option to address impacting UE RRC connection during NG or XN failure,  is to try to find an alternative active neighbouring Xn or NG link or XN and NG links combination  and reroute or carry out the UE NG/XN RRC connection associated procedures over.

- Proposal 3: if proposal 1 is adopted, RAN3 is kindly asked to discuss the following Xn/NG enhancements:

- Allow of exchanging between satellite gNB and AMF of UE NG related procedure such as initial UE message, initial UE context setup message and/or the paging message when trying to reconnect or resume a field NG interface in NTN regenerative architecture.

- Allow of exchanging between satellite gNBs of UE Xn related procedure such as (UE context retrieval request and response messages) shall be exchanged between satellite gNBs when trying to reconnect or resume a field NG interface in NTN regenerative architecture.

- Proposal 4: if proposal 2 is adopted , RAN3 is kindly asked to discuss the following Xn/NG enhancements

- Allow exchanging of information about other available XN and NG links between satellite gNBs and AMF, so as to find an alternative active neighbouring Xn or NG link or XN and NG links combination to reroute or carry out the UE NG/XN RRC connection associated procedures over.

- Allow exchanging of UE Xn associated procedures such as initial UE message, initial UE context setup message and/or the paging message within NG protocol signalling.

- Allow of exchanging of UE NG associated procedures such as retrieval context request and response messages within XN protocol signalling.

# Conclusion

In this contribution, we have discussed UE Xn and NG related procedures impact due to triggering of UE connected inactive RRC connection procedures during an Xn or NG disconnection, switching or failure. Based on the discussion we proposed to discuss the following proposals:

- Proposal 1: An NG or XN failure or disconnection may UE connected inactive RRC connection procedures, one way to address this is to reconnect or recover the failed NG/XN link by AMF or satellite gNB as per UE RRC connection trigger.

- Proposal 2: An alternative option to address impacting UE RRC connection during NG or XN failure,  is to try to find an alternative active neighbouring Xn or NG link or XN and NG links combination  and reroute or carry out the UE NG/XN RRC connection associated procedures over.

- Proposal 3: if proposal 1 is adopted, RAN3 is kindly asked to discuss the following Xn/NG enhancements:

- Allow of exchanging between satellite gNB and AMF of UE NG related procedure such as initial UE message, initial UE context setup message and/or the paging message when trying to reconnect or resume a field NG interface in NTN regenerative architecture.

- Allow of exchanging between satellite gNBs of UE Xn related procedure such as (UE context retrieval request and response messages) shall be exchanged between satellite gNBs when trying to reconnect or resume a field NG interface in NTN regenerative architecture.

- Proposal 4: if proposal 2 is adopted , RAN3 is kindly asked to discuss the following Xn/NG enhancements

- Allow exchanging of information about other available XN and NG links between satellite gNBs and AMF, so as to find an alternative active neighbouring Xn or NG link or XN and NG links combination to reroute or carry out the UE NG/XN RRC connection associated procedures over.

- Allow exchanging of UE Xn associated procedures such as initial UE message, initial UE context setup message and/or the paging message within NG protocol signalling.

- Allow of exchanging of UE NG associated procedures such as retrieval context request and response messages within XN protocol signalling.

# References

- RP-240775, “Revised WID: Rel-19 WI Non-Terrestrial Networks (NTN) for NR Phase 3”, THALES, CATT, RAN#103, Maastricht, The Netherlands, March 18-22, 2024.

- R3-243017,” LS on Support of Regenerative-based Satellite Access” SA2(VIVO), Fukuoka, Japan, 20 - 24 May, 2024.

- R2-2405156” Regenerative payload for NR NTN” Samsung. Fukuoka, Japan, May 20th-24th, 2024

- R2-2404799 ” Considerations on regenerative payload in NTN” Fukuoka, Japan, May 20th-24th, 2024.