3GPP TSG-RAN WG3 Meeting #R3-125		R3-244636

Maastricht, Netherlands, Aug 19-23, 2024

Agenda Item:	11.2

Source: 	Rakuten

Title:  	Discussion on AIML based Network Slicing

Document for:	Discussion and decision

# Introduction

Following are the agreements from RAN3#123-bis meeting:

AI/ML-based network slicing model in NG-RAN node can generate following information as output:

-	Resource management within RRM policy (Internal output)

-	Slice aware mobility decisions (Internal output)

Following are the agreements from RAN3#124 meeting:

In Rel-19, predicted information from CN to NG-RAN is not considered.

Agree to improve the granularity of UE performance feedback in Rel-19 for slicing.

# Background

Network Slicing enables the creation of multiple virtual networks on a shared physical infrastructure. Each of these virtual networks, or "slices," can be tailored to meet specific requirements of different applications, services, or business verticals. This capability allows network operators to provide customized and optimized network experiences for a diverse range of use cases, from enhanced mobile broadband (eMBB) to ultra-reliable low-latency communications (URLLC) and massive machine-type communications (mMTC). Since network slicing is envisioned as an end-to-end feature encompassing the core network, transport network, and radio access network (RAN), these requirements must be met across all slice subnets throughout the lifecycle of a network slice, particularly on the RAN side. Exemplary slice performance requirements include throughput, energy efficiency, latency, and reliability.

And Service Level Agreement (SLA) is a formal contract between a service provider and a customer that defines the expected level of service, performance metrics, and responsibilities. In the context of network slicing, SLAs are crucial for ensuring that each slice meets the specific requirements of the applications or services it supports.

# Discussion

AI/ML can significantly enhance resource management in network slicing by enabling more intelligent, adaptive, and efficient allocation of resources. Optimizing network slicing on a per-UE per-slice basis requires detailed and specific information that can help tailor the network resources and configurations to meet the unique needs of each user and the services they are accessing.

Case 1: Measured/predicted per-UE per slice traffic information

The local node (with which the UE is currently attached) can measure and predict per UE traffic per slice and use this information as an input to the AIML training model. Per-UE per-slice traffic data is needed to ensure that the QoS requirements defined in Service Level Agreements (SLAs) are met for each user. In addition to the information collected from the local and neighbouring nodes, this data on a per UE per slice level with be critical for accurate predictions like resource management policy per slice or slice aware mobility decisions.

Monitoring Quality of Service (QoS) information per UE (User Equipment) per slice is critical for optimizing network slicing and ensuring that each slice meets its specific performance requirements. It provides the detailed insights needed to allocate resources efficiently, manage QoS, enhance user experiences, and ensure the overall performance and reliability of the network. This detailed QoS information in helpful to at the AI/ML entity to assist with management of resource allocation, service quality, and user experience.

Case 2: Measured/predicted per slice performance metric information

We also propose that the local node sends to the AIML entity the measured and predicted per slice performance metrics which can indicate the information on slice fulfilment. The per slice performance metrics can include slice level information on throughput, latency, packet loss, and mobility.

Throughput: Monitoring uplink (UL) and downlink (DL) UE throughput in the NG-RAN is crucial for ensuring end-user satisfaction and the optimal functioning and configuration of cells and scheduling features. The UE throughput per mapped 5QI will indicate the scheduling efficiency and QoS priority handling in the gNB. When network slicing is supported by the NG-RAN, multiple S-NSSAIs may be in use. In such cases, monitoring the UL and DL UE throughput for each S-NSSAI is important to identify specific performance issues.

Latency: Meeting low latency expectations for 5G services, such as URLLC, is a critical task for operators to fulfill service performance requirements. Given that uplink (UL) and downlink (DL) performance can vary, it is essential for operators to monitor UL and DL user plane latencies independently. By utilizing performance measurements that provide separate UL and DL user plane latency information per S-NSSAI, the AI/ML entity can address service performance issues specific to either the uplink or downlink.

Packet loss: Monitoring UL and DL packet loss in the NG-RAN is crucial, as packet loss can significantly affect the end-user experience for certain services. Measuring UL and DL packet loss, along with concurrent packet measurements, is valuable for evaluating, optimizing, and ensuring performance within the integrity area (user plane connection quality). Subcounters for each QoS level and supported S-NSSAI enable the AI/ML entity to accurately identify the high packet loss rates and recommend resource management and mobility decisions to address them.

Mobility: Handover based information is a key metric for evaluation of slice performance. Handover failures can lead to service interruptions, directly affecting user experience. The handover procedure encompasses handover preparation, resource allocation, and execution, and it is essential to monitor performance at each of these stages. Resources, such as PDU Session Resources, must be prepared and allocated for handovers in accordance with the QoS requirements for each S-NSSAI.

Proposal 1: RAN3 to consider additional information from the local node including measured/predicted per UE traffic per slice and measured/predicted per slice performance metrics

As in the local node, we propose the neighbouring nodes to send similar information to the AIML entity to be able to make holistic predictions for resource management per slice or slice aware mobility decisions. It is required for coordinated network slice level optimization e.g. radio resources management between neighbor gNBs. This information gives an indication of current and predicted SLA compliance and to verify which performance targets are being met and which are not being met. Using this information, the predictions from the AI/ML training entity can be more accurate for to meet the specific needs to the slices and corrective decisions can be made where lacking. This will enable optimized network slicing decisions for coordinated radio resource management and mobility optimizations across neighbouring gNB

Proposal 2: RAN3 to consider additional information from the neighbouring nodes including measured/predicted per UE traffic per slice and measured/predicted per slice performance metrics

Furthermore, to optimize the performance of AI/ML based network slicing model, we propose that the feedback of measured per slice fulfilment be considered to be collected from the gNBs. This provides continuous monitoring and evaluation of each network slice’s performance against its pre-defined SLA and QoS requirements. This measured data serves as a critical feedback to the AI/ML training.

Proposal 3: RAN3 to consider additional information in terms of measured per slice fulfilment from the gNBs as a feedback of AIML based Network slicing.

By leveraging more granular data, the AI/ML for Network Slicing can output more accurate predictions of radio resource status per slice, slice available capacity, resource decisions for RRM policies, and mobility decisions.

# Conclusion

We have the following proposals:

Proposal 1: RAN3 to consider additional information from the local node including measured/predicted per UE traffic per slice and measured/predicted per slice performance metrics

Proposal 2: RAN3 to consider additional information from the neighbouring nodes including measured/predicted per UE traffic per slice and measured/predicted per slice performance metrics

Proposal 3: RAN3 to consider additional information in terms of measured per slice fulfilment from the gNBs as a feedback of AIML based Network slicing.

# Reference

[1]. TR38.743 Study on enhancement of AI/ML for NG-RAN

[2] Chairman’s notes RAN3#123-bis

[3] Chairman’s notes RAN3#124