3GPP TSG-RAN3 Meeting #125	R3-244641

# Maastricht, Netherlands, 19-23 Aug, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0425 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on IAB-node authorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Lenovo, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2024-08-16 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - For an IAB-node, the IAB donor-CU receives the authorizations from 5GC or from another IAB donor CU, when the authorization status is received for the first time and “not authorized”, the IAB-donor-CU neither establishes the backhaul resources nor allocates any BAP address, TNL address or default BAP configuration for this IAB-node. However, when the authorization status is not received for the first time, different action shall be taken, e.g. the IAB-donor-CU attempts to hand over the UEs served by the IAB-node to other cell(s), releases the F1 interface towards the IAB-DU, and releases all backhaul resources (including the BAP address, TNL address and default BAP configuration) for this IAB-node. As we can see, different actions are taken by the IAB donor CU for the two cases. However, the two cases when the authorization status is received are not distinguished in the current description in 38.401. - The current description for the case when MT is dual-connected with two donors is incomplete and incorrect. Based on current specification, when the IAB-MT is NR dual-connected, two cases are described separately, i.e. case 1: the non-F1-terminating IAB-donor-CU is the MN, case 2: the F1-terminating IAB-donor-CU is the MN. - For case 1 (i.e. the non-F1-terminating IAB-donor-CU is the MN), it’s specified “the non-F1-terminating IAB-donor-CU sends the authorization status received from the 5GC to the F1-terminating IAB-donor-CU in the IAB-TRANPORT MIGRATION MODIFICATION REQUEST message.”. However the S-NODE ADDITION REQUEST instead of IAB-TRANPORT MIGRATION MODIFICATION REQUEST message is used in this scenario during the NR-DC establishment procedure. For case 2 (i.e. the MN is the F1-terminating IAB-donor-CU), it’s specified “upon reception of the authorization status, the IAB-node’s authorization procedure follows the same steps as described in clause 8.9.17.2.1.”. However, clause 8.9.17.2 and even 8.9.17.1 which is referred by clause 8.9.17.2 only describes the case when IAB-node connects to a single IAB-donor wherein the following steps are missing. Actually, these operations applies to all scenarios when the IAB node is served by two IAB-donors, not only to part of scenarios as described currently in 38.401. - The authorization status transfer between the two donors - F1-terminating donor informing the non-F1 terminating donor that the corresponding operation (e.g. handover of UEs) has completed - When the IAB-node is servied by two IAB-donors, it is specified “the non-F1-terminating IAB-donor-CU sends the authorization status received from the 5GC to the F1-terminating IAB-donor-CU in the IAB-TRANPORT MIGRATION MODIFICATION REQUEST message”. However, in some cases, e.g., if the received status is the same as the current status, there is no need for the non-F1 terminating donor to send received status to the F1 terminating donor. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that if the authorization status is received for the first time and “not authorized”, the IAB-donor-CU neither establishes the backhaul resources nor allocates any BAP address, TNL address or default BAP configuration for this IAB-node. - Update the description for the case when the IAB-MT is NR dual-connected with two IAB-donors. - Add “if needed” for the operation of non-F1-terminating IAB-donor-CU sending the authorization status received from the 5GC to the F1-terminating IAB-donor-CU. Impact analysis Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). This CR has impact on the functional point of view, the impact can be considered isolated because it only impacts the IAB node authorization. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 2 description of IAB node authorization is incorrect and incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.9.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

-------------------------------------------Start of change-------------------------------------------

8.9.17	IAB-node authorization

8.9.17.1	IAB-node in NSA

During the IAB-node integration procedure, the eNB receives the authorization status of the IAB-node from the EPC. The eNB forwards the authorization status to the IAB-donor-CU in the SGNB ADDITION REQUEST message. If the authorization status is received for the first time and “not authorized”, the IAB-donor-CU neither establishes the backhaul resources nor allocates any BAP address, TNL address or default BAP configuration for this IAB-node.

When the authorization status for the IAB-node changes, the EPC sends an updated authorization status to the IAB-MT’s eNB. The eNB forwards the authorization status to the IAB-donor-CU in the SGNB ADDITION REQUEST message or SGNB MODIFICATION REQUEST message.

In case the updated authorization status is “not authorized”, the IAB-donor-CU performs the following actions in this order: it attempts to hand over the UEs and descendant nodes served by the IAB-node to other cell(s), releases the F1 interface towards the IAB-DU, and releases all backhaul resources (including the BAP address, TNL address and default BAP configuration) for this IAB-node. The IAB-donor-CU may indicate to the eNB that the actions described above have been completed, by sending a SGNB MODIFICATION REQUIRED or SGNB RELEASE REQUIRED message with the corresponding cause value included. Then, the eNB may indicate to the EPC that the IAB-MT can be de-registered.

8.9.17.2	IAB-node with single IAB-donor in SA

8.9.17.2.1	IAB-node is single-connected

During the IAB-node network integration or RLF recovery, the IAB-donor-CU receives the authorization status of the IAB-node from the 5GC. Also, during the inter-CU topology adaptation procedure, the target IAB-donor-CU receives the authorization status of the IAB-node from the source IAB-donor-CU as well as from the 5GC when performing the Path Switch Request procedure. If the authorization status is received for the first time and “not authorized”, the IAB-donor-CU neither establishes the backhaul resources nor allocates any BAP address, TNL address or default BAP configuration for this IAB-node. When the authorization status for the IAB-node changes, the 5GC sends an updated authorization status to the IAB-donor-CU. When the authorization status received by the IAB-donor-CU changes, the IAB-donor-CU performs the SA equivalent of the steps described for NSA in clause 8.9.17.1.

In case the updated authorization status is “not authorized”, after actions described in clause 8.9.17.1 have been completed, the IAB-donor-CU may indicate to the 5GC that the IAB-MT can be de-registered.

8.9.17.2.2	IAB-node is NR dual-connected

In case the IAB-node is dual-connected to the IAB-donor-CU, the IAB-donor-CU receives the authorization status of the IAB-node from the 5GC. Upon reception of the authorization status, the IAB-donor-CU performs the same steps described in clause 8.9.17.2.1.

In case the IAB-node is dual-connected to a non-IAB-capable gNB and to an IAB-donor-CU, the MN receives the authorization status of the IAB-node from the 5GC. If the MN is the non-IAB-capable gNB and the SN is the IAB-donor-CU, the MN forwards the authorization status to the IAB-donor-CU in the S-NODE ADDITION REQUEST message or S-NODE MODIFICATION REQUEST message. Upon reception of the authorization status, the IAB-donor-CU performs the same steps described in clause 8.9.17.2.1. If the updated authorization status is “not authorized”, the SN may indicate to the MN that the actions of removing the UEs and descendant nodes, releasing the F1 interface and backhaul resources for the IAB-node have been completed, by sending a S-NODE MODIFICATION REQUIRED or S-NODE RELEASE REQUIRED message with the corresponding cause value included. Then, the MN may indicate to the 5GC that the IAB-MT can be de-registered. If the MN is the IAB-donor-CU and the SN is not the non-IAB-capable gNB, the IAB-donor-CU performs the same steps described in clause 8.9.17.2.1.

8.9.17.3	IAB-node is served by two IAB-donors in SA

In case the IAB-MT only connects to the non-F1-terminating IAB-donor-CU, the non-F1-terminating IAB-donor-CU sends the authorization status received from the 5GC to the F1-terminating IAB-donor-CU in the IAB-TRANPORT MIGRATION MODIFICATION REQUEST message if needed. In case the IAB-MT is NR dual-connected, the MN receives authorization status of the IAB-node from the 5GC. And then the MN can forward the authorization status to the SN.

Upon reception of the authorization status, the F1-terminating IAB-donor-CU performs the same steps described in clause 8.9.17.2.1. If the authorization status is “not authorized”, the F1-terminating IAB-donor-CU sends to the non-F1-terminating IAB-donor-CU an IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message requesting the release of all offloaded traffic, after which the non-F1-terminating IAB-donor-CU releases the offloaded traffic and all backhaul resources, BAP address, TNL address and default BAP configuration for the IAB-node. The non-F1-terminating IAB-donor-CU then may indicate to the 5GC that the IAB-MT can be de-registered.

-------------------------------------------End of change-------------------------------------------