3GPP TSG-RAN WG3 Meeting #125	R3-244643

Maastricht, NL, 19 - 23 Aug, 2024

Title:	Response to R3-244126, R3-244127, R3-244128, R3-244130

Agenda Item:	8.3

Source:	Huawei

Document for:	Response

# 1	Introduction

Besides the adaptation of the RRCRelease with resume indication solution (so called RAN2 solution), in previous meetings, RAN3 sent an LS [1] to RAN2 to ask them to confirm whether this network-based solution has any RAN2 impact or issue:

In addition, RAN3 has discussed a network-based solution, i.e., for UEs not supporting the RAN2 solution above, where the last serving gNB during the ongoing SDT procedure without anchor relocation, upon, e.g., receiving a DL-non SDT data, further relocates the UE context to the receiving gNB by sending an XnAP RETRIEVE UE CONTEXT RESPONSE message, and then the receiving gNB sends the RRCSetup message to the UE (as in TS 38.331, Figure 5.3.13.1-2) with ongoing SDT in RRC_INACTIVE state.

RAN3 assumes that the network-based solution would have impact on Xn signalling only, but would like to confirm with RAN2 whether this network-based solution has any RAN2 impact or issue.

For this meeting, paper [3] ~ [6] are submitted to introduce network-based solution. In this contribution, we our response accordingly.

# 2	Response

In RAN2#126 meeting, RAN2 discussed the network-based solution and agreed the following:

=> Reply to LS: - No specification impact (for Rel-17 and 18) - However, there are RAN2 issues, like data loss (explain the issue) and Uu signaling overhead due to security being cleared (explain that as from Rel-15 …)

However, during the “[POST126][033][SDT] LS reply to RAN3 (ZTE)” e-mail discussion, some additional issues were raised, therefore, the outcome of the email discussion is announced as: Postpone this reply LS and companies can discuss RAN2 impacts, if any, for this at the next meeting.

In [2] the additional RAN2 issues are summarized as follows:

- It was indicated that even though it was previously agreed that sending RRCSetup in response to SDT request is allowed, RAN2 had never considered that this would happen in the middle of an ongoing SDT procedure. It was assumed RRCSetup would be sent immediately upon SDT request, before any data between the UE and the network would be exchanged.

- Currently, the UE does not perform MAC reset upon reception of RRCSetup message. It was indicated this may have an impact on subsequent data transmission.

As RAN3 assumes that the solution should only impact Xn signalling, in case there are RAN2 impacts, this network-based solution may not be worth adopting, but before making such decision, RAN3 should wait for RAN2 feedback.

Proposal: RAN3 wait for RAN2 feedback on SDT network-based solution.

# 3	Conclusion

In this contribution, we introduced the current status of the RAN2 discussion on SDT network-based solution, and get the proposal as below:

Proposal: RAN3 wait for RAN2 feedback on SDT network-based solution.

# 4	Reference

[1] R3-242198 Reply LS on SDT signalling optimization for partial context transfer, Source: RAN3, To: RAN2

[2] R2-2406929 Impact of SDT signalling optimization for partial context transfer on RAN2, Huawei, HiSilicon

[3] R3-244126	Switch to RRC Connected after SDT Partial Context Transfer	Nokia, Ericsson	discussion

[4] R3-244127	Correction of switch to RRC connected after SDT Partial Context Transfer	Nokia, Ericsson	draftCR	38.300

[5] R3-244128	Correction of switch to RRC connected after SDT Partial Context Transfer	Nokia, Ericsson	CR	38.423

[6] R3-244130	Response LS on SDT signalling optimization for partial context transfer	Nokia, Ericsson	LS out