3GPP TSG-RAN WG3 Meeting #125	R3-244647

Maastricht, NL, 19 - 23 Aug, 2024

Title:	Response to R3-244220 on UE awareness of MBS QoE

Agenda Item:	8.3

Source:	ZTE Corporation

Document for:	Discussion and decision

1	Introduction

In previous meeting, companies have discussed the UE awareness issue for the MBS QoE and make no consensus on whether further enhancement is needed. The following progress has been made at the end of previous meeting:

The UE should be aware whether to conduct QMC for MBS in broadcast or in multicast mode.

FFS whether the UE is informed via RRC or via the QMC configuration container or existing solutions.

This paper further provide our views on this open issue based on our previous outcome and contributions received this meeting.

- Discussion

- As explained in [1] and previously received LS[2], OAM, CN and gNB know a specific QoE configuration is for the MBS service type(e.g. broadcast or multicast). More specifically, as explained in previous received LS[2],  SA5 has updated the QMCJob IE in section 4.3.54.2, TS 28.622 with the new added value mBSCommunicationServiceType shown below. By using this new added value, OAM and CN can know the specific MBS service type for a QoE configuration.

- ==== content in TS 28.622 ====

4.3.54.2	Attributes

The QMCJob IOC includes attributes inherited from Top IOC (defined in clause 4.3.29) and the following attributes:

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| serviceType | M | T | T | F | T |
| areaScope | CM | T | T | F | T |
| qoECollectionEntityAddress | M | T | T | F | T |
| pLMNTarget | CM | T | T | F | T |
| qoETarget | CM | T | T | F | T |
| qoEReference | M | T | T | F | T |
| jobId | O | T | T | T | T |
| sliceScope | CM | T | T | F | T |
| qMCConfigFile | M | T | T | F | T |
| mDTAlignmentInformation | O | T | T | F | T |
| availableRANqoEMetrics | O | T | T | F | T |
| mBSCommunicationServiceType | O | T | F | F | T |

4.4.1	Attribute properties

The following table defines the properties of attributes specified in the present document.

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| numberOfFiles | Number of files in a file collection. allowedValues: NA | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| SKIP UNRELATED CONTENT |  |  |
| mBSCommunicationServiceType | This IE indicates for which type of MBS communication service the QoE measurement configuration pertains to. See the clause 4.5.1 of TS 28.405 [50] for additional details. allowedValue: BROADCAST, MULTICAST | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |

- ==== content in TS 28.622 ====

- In addition, based on RAN3 definition, the MBS Communication Service Type IE has been introduced in NGAP messages which is used to indicate the type of MBS communication service for which the QoE measurement collection should be performed. By adding this IE into the QoE configuration, gNB knows whether a specific MBS service type is combined with a QoE configuration.

- ==== content in TS 38.413 ====

| MBS Communication Service Type | O |  | ENUMERATED (broadcast, multicast, ...) | This IE indicates the type of MBS communication service for which the QoE measurement collection should be performed. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

- ==== content in TS 38.413 ====

- Hence, as shown above and explained in [1], it is clear that OAM/CN and gNB have explicitly info on whether one QoE configuration is configured for a specific MBS service type(e.g., broadcast, multicast).

- Observation 1: OAM/CN and gNB have explicitly info on whether one QoE configuration is configured for a specific MBS service type(e.g., broadcast, multicast).

- Moreover, based on our understanding, [1] further explains that UE AS layer and APP layer can know whether an ongoing session is a MBS session by UE received info e.g., bearer info, MBS User Service parameters which is defined in clause 4.5.3, TS 26.502.

Observation 2: Both the UE AS layer and App layer know the the MBS service type(e.g., broadcast, multicast) of the ongoing MSB service.

- The goal of our discussion is that, UE can trigger a QoE measurement for a specific MBS service type(e.g. broadcast, multicast) when such MBS session start. The gap between our goal and the above observations is that if a UE can perform the QoE measurement when the specific MBS service type(e.g. broadcast, multicast) is used to transmit certain UP data(e.g. VR), UE shall received such configuration. Generally, 2 ways may be used to achieve such configuration: RRC or QoE configuration container(e.g. XML file).

- Alt 1: QoE configuration container:

- At least, based on the current spec(e.g. TS 26.114 for MTSI , TS 26.247 for Streaming, TS 26.118 for VR) there is no info defined in the QoE configuration container which is used to further restrict that this QoE shall be triggered only if specific MBS service type is used.

- Alt 2: QoE configuration in RRC:

- The current IEs designed for MBS QoE in TS 38.331 is yellow highlighted in annex. It is clear that the new introduced IE, AppLayerIdleInactiveConfig, can only indicate whether a QoE can be performed in RRC_INACTIVE and RRC_IDLE. Combined with the QoE configuration defined in Rel-17, this may indicate that this QoE measurement can be performed in RRC_CONNECTED, RRC_INACTIVE, and RRC_IDLE. Considering at current stage, only QoE for broadcast service can be performed regardless of RRC status, when UE receives the AppLayerIdleInactiveConfig, UE can easily deduce that this QoE configuration is for broadcast service.

- Observation 3: Based on the current specification, UE may perform QoE for broadcast service if the received QoE configuration contains AppLayerIdleInactiveConfig IE.

- Hence, based on our understanding, the current mechanism can not let UE know whether a QoE configuration is a Rel-17 QoE configuration or QoE configuration for multicast only.

- To have a short summary and to let all interested companies in the same page, we may use the following naive example to discuss this issue. How does each entities(e.g.,OAM/CN, gNB, UE) distinguish the following 3 QoE configurations:

- QoE configuration 1 which contains: QoE reference 1, service type A, no MBS related restriction,

- QoE configuration 2 which contains: QoE reference 2, service type A, for broadcast only,

- QoE configuration 3 which contains: QoE reference 3, service type A, for multicast only.

Based on our above explanation, each entities can distinguish the difference between above 3 configurations.

- OAM/CN can distinguish the difference of the these 3 QoE configurations by using mBSCommunicationServiceType IE which is defined in QMCJob IE.

  - QoE configuration 1: no mBSCommunicationServiceType IE.

  - QoE configuration 2: mBSCommunicationServiceType IE with the value “broadcast”.

  - QoE configuration 3: mBSCommunicationServiceType IE with the value  “multicast”.

- gNB can distinguish the difference between these 3 QoE configurations by checking MBS Communication Service Type IE.

  - QoE configuration 1: no MBS Communication Service Type IE.

  - QoE configuration 2: MBS Communication Service Type IE with the codepoint “broadcast”.

  - QoE configuration 3: MBS Communication Service Type IE with the codepoint “multicast”.

- UE can only distinguish the difference between QoE configuration 1 and 2 by using AppLayerIdleInactiveConfig IE.

  - QoE configuration 1: no AppLayerIdleInactiveConfig IE.

  - QoE configuration 2: has AppLayerIdleInactiveConfig IE.

  - QoE configuration 3: ?.

- Observation 4: Based on the current specification, UE can not distinguish whether a QoE configuration is for multicast or for QoE without MBS restriction.

From our point of view, RAN3 may further discuss whether this issue exist based on the above explanation and share our understanding/wondering to related WGs if necessary.

Proposal 1: RAN3 may further discuss whether this issue exist based on the above explanation and shares our understanding/confusion to related WGs if necessary.

A draft LS out is prepared in annex and can be considered as baseline.

3	Conclusion

- Observation 1: OAM/CN and gNB have explicitly info on whether one QoE configuration is configured for a specific MBS service type(e.g., broadcast, multicast).

Observation 2: Both the UE AS layer and upper layer knows the the MBS service type(e.g., broadcast, multicast) of the ongoing MSB service.

- Observation 3: Based on the current specification, UE may perform QoE for broadcast service if the received QoE configuration contains AppLayerIdleInactiveConfig IE.

- Observation 4: Based on the current specification, UE can not distinguish whether a QoE configuration is for multicast or for QoE without MBS restriction.

Proposal 1: RAN3 may further discuss whether this issue exist based on the above explanation and shares our understanding/confusion to related WGs if necessary.

4	Reference

[1]		R3-244220	Discussions on UE's awareness of  MBS Communication Service Type for QMC (Huawei)

[2]		R3-243028	LS on MBS Communication Service Type

Annex Draft LS out

3GPP TSG-RAN WG3 Meeting #125	R3-24xxxx

Maastricht, NL, 19 - 23 Aug, 2024

Title:	[DRAFT] Clarification  on MBS Communication Service Type

Response to:	R3-243028

Release:	Rel-18

Work Item:	NR_QoE_enh-Core

Source:	ZTE Corporation[shall be RAN3]

To:	RAN2,

Cc:	SA5, SA4, CT1, CT4

Contact person:	Liu Yansheng

Liu.yansheng@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1	Overall description

Thanks SA5 for informing RAN3 on the MBS communication service type progress. RAN3 further discusses the UE awareness issue based on received content and current specifications. In Rel-18, NR QoE supports to measure the existing service type when the UP data for such service type is transmitted via a specific MBS communication type(e.g. broadcast, multicast). RAN3 understands that one UE with such QoE configuration shall be aware on the whether the QoE measurement shall be triggered and collected for a specific MBS communication type(e.g. broadcast, multicast). However, based on the current mechanism, RAN3 understands UE may not be able to distinguish the difference between a QoE configuration for multicast and a QoE configuration for broadcast.

2	Actions

To RAN2

ACTION: 	RAN3 kindly asks RAN2 to take the above information into account and provide feedback if necessary

3	Dates of next TSG RAN WG 3 meetings

RAN3#125-bis		14 - 18 Oct 2024		Hefei, CN

RAN3#126		18 - 22 Nov 2024		Orlando, US

Annex TS 38.331

–	AppLayerIdleInactiveConfig

The IE AppLayerIdleInactiveConfig indicates parameters specific to application layer measurements applicable to RRC_IDLE and RRC_INACTIVE.

AppLayerIdleInactiveConfig information element

-- ASN1START

-- TAG-APPLAYERIDLEINACTIVECONFIG-START

AppLayerIdleInactiveConfig-r18 ::=   SEQUENCE {

measConfigAppLayerId-r18             MeasConfigAppLayerId-r17,

serviceType-r18                      ENUMERATED {streaming, mtsi, vr, spare5, spare4, spare3, spare2, spare1}  OPTIONAL, -- Need M

appLayerMeasPriority-r18             INTEGER (1..16)                                                           OPTIONAL, -- Need M

qoe-Reference-r18                    OCTET STRING (SIZE (6))                                                   OPTIONAL, -- Need M

qoe-MeasurementType-r18              ENUMERATED {sbased, mbased}                                               OPTIONAL, -- Need M

qoe-AreaScope-r18                    Qoe-AreaScope-r18                                                         OPTIONAL, -- Need M

mce-Id-r18                           OCTET STRING (SIZE (1))                                                   OPTIONAL, -- Need M

availableRAN-VisibleMetrics-r18      AvailableRAN-VisibleMetrics-r18                                           OPTIONAL, -- Need M

...

}

Qoe-AreaScope-r18 ::=                CHOICE {

cellGlobalIdList                     CellGlobalIdList-r16,

trackingAreaCodeList                 TrackingAreaCodeList-r16,

trackingAreaIdentityList             TrackingAreaIdentityList-r16,

plmn-IdentityList                    PLMN-IdentityList2-r16,

...

}

AvailableRAN-VisibleMetrics-r18 ::=  SEQUENCE {

appLayerBufferLevelList-r18          ENUMERATED {true}                                                         OPTIONAL, -- Need M

playoutDelayForMediaStartup-r18      ENUMERATED {true}                                                         OPTIONAL, -- Need M

...

}

-- TAG-APPLAYERIDLEINACTIVECONFIG-STOP

-- ASN1STOP

AppLayerIdleInactiveConfig field descriptions

appLayerBufferLevelList The field defines whether the buffer level for DASH streaming and VR service types can be collected as a RAN visible QoE metric from the UE.

qoe-AreaScope The field contains the area where the application layer measurement shall start if an application layer session starts.

qoe-MeasurementType Indicates whether the application layer measurement is signalling based or management based.

qoe-Reference Indicates the QoE Reference as defined in TS 28.405 [55], clause 5.2.

mce-id The field contains the Measurement Collection Entity Id.

playoutDelayForMediaStartup The field defines whether the playout delay for DASH streaming and VR service types can be collected as a RAN visible QoE metric from the UE.

–	AppLayerMeasConfig

The IE AppLayerMeasConfig indicates configuration of application layer measurements.

# AppLayerMeasConfig information element

-- ASN1START

-- TAG-APPLAYERMEASCONFIG-START

AppLayerMeasConfig-r17 ::=           SEQUENCE {

measConfigAppLayerToAddModList-r17   SEQUENCE (SIZE (1..maxNrofAppLayerMeas-r17)) OF MeasConfigAppLayer-r17     OPTIONAL, -- Need N

measConfigAppLayerToReleaseList-r17  SEQUENCE (SIZE (1..maxNrofAppLayerMeas-r17)) OF MeasConfigAppLayerId-r17   OPTIONAL, -- Need N

rrc-SegAllowedSRB4-r17               ENUMERATED {enabled}                                                       OPTIONAL, -- Need R

...,

[[

rrc-SegAllowedSRB5-r18               ENUMERATED {enabled}                                                       OPTIONAL, -- Need R

idleInactiveReportAllowed-r18        ENUMERATED {enabled}                                                       OPTIONAL  -- Need R

]]

}

MeasConfigAppLayer-r17 ::=           SEQUENCE {

measConfigAppLayerId-r17             MeasConfigAppLayerId-r17,

measConfigAppLayerContainer-r17      OCTET STRING (SIZE (1..8000))                                              OPTIONAL, -- Need N

serviceType-r17                      ENUMERATED {streaming, mtsi, vr, spare5, spare4, spare3, spare2, spare1}   OPTIONAL, -- Need M

pauseReporting-r17                   BOOLEAN                                                                    OPTIONAL, -- Need M

transmissionOfSessionStartStop-r17   BOOLEAN                                                                    OPTIONAL, -- Need M

ran-VisibleParameters-r17            SetupRelease {RAN-VisibleParameters-r17}                         OPTIONAL, -- Cond ServiceType

...,

[[

reportingSRB-r18                     ENUMERATED {srb4, srb5}                                                    OPTIONAL, -- Cond QoENRDC

appLayerMeasPriority-r18             INTEGER (1..16)                                                      OPTIONAL, -- Cond Need M

appLayerIdleInactiveConfig-r18       SetupRelease {AppLayerIdleInactiveConfig-r18}                              OPTIONAL  -- Need M

]]

}

RAN-VisibleParameters-r17 ::=        SEQUENCE {

ran-VisiblePeriodicity-r17           ENUMERATED {ms120, ms240, ms480, ms640, ms1024}                            OPTIONAL, -- Need S

numberOfBufferLevelEntries-r17       INTEGER (1..8)                                                             OPTIONAL, -- Need R

reportPlayoutDelayForMediaStartup-r17 BOOLEAN                                                                   OPTIONAL, -- Need M

...,

[[

ran-VisibleReportingSRB-r18          ENUMERATED {srb4, srb5}                                              OPTIONAL  -- Cond QoENRDC

]]

}

-- TAG-APPLAYERMEASCONFIG-STOP

-- ASN1STOP

AppLayerMeasConfig field descriptions

appLayerMeasPriority The field indicates the priority of the application layer measurement configuration, where a higher value indicates lower priority. If the field is not configured, the application layer measurement configuration has the lowest priority.

idleInactiveReportAllowed The field indicates whether transmission of application layer measurement configurations and reports for application layer measurements applicable to RRC_IDLE and/or RRC_INACTIVE is allowed in the cell where the field is received. If the field is not configured, transmission of application layer measurement reports and configurations for RRC_IDLE/RRC_INACTIVE is not allowed.

measConfigAppLayerContainer The field contains configuration of application layer measurements, see Annex L (normative) in TS 26.247 [68], clause 16.5 in TS 26.114 [69] and TS 26.118 [70].

measConfigAppLayerId The field contains the identity of the application layer measurements. When application layer measurements are configured for an SCG, the measConfigAppLayerId is obtained according to TS 38.423 [37], clauses 8.3.1 and 8.3.3.

pauseReporting The field indicates whether the transmission of measReportAppLayerContainer is paused or not. Value true indicates the transmission of measReportAppLayerContainer is paused; value false indicates the transmission of measReportAppLayerContainer is not paused.

ran-VisibleParameters The field indicates whether RAN visible application layer measurements shall be reported or not.

ran-VisibleReportingSRB The field indicates the SRB to be used for transmission of RAN visible application layer measurement reports.

reportingSRB The field indicates the SRB to be used for transmission of encapsulated application layer measurement reports.

rrc-SegAllowedSRB4 This field indicates that RRC segmentation of MeasurementReportAppLayer is enabled on SRB4. The field is only configured for an MCG. It may be present only if the UE supports RRC segmentation of the MeasurementReportAppLayer message.

rrc-SegAllowedSRB5 This field indicates that RRC segmentation of MeasurementReportAppLayer is enabled on SRB5. The field is only configured for an SCG. It may be present only if the UE supports RRC segmentation of the MeasurementReportAppLayer message.

serviceType Indicates the type of application layer measurement. Value streaming indicates Quality of Experience Measurement Collection for streaming services (see TS 26.247 [68]), value mtsi indicates Quality of Experience Measurement Collection for MTSI (see TS 26.114 [69]) and value vr indicates Quality of Experience Measurement Collection for VR service (see TS 26.118 [70]). The network always configures serviceType when application layer measurements are initially configured and at fullConfig.

transmissionOfSessionStartStop Value true indicates that the UE shall transmit indications when the measurement session in the application layer starts and stops. Value false indicates that the UE shall not transmit any session status indications. The UE transmits a session start indication upon configuration of this field set to value true if a session already has started in the application layer.

RAN-VisibleParameters field descriptions

numberOfBufferLevelEntries The field contains the maximum number of buffer level entries that can be reported for RAN visible application layer measurements. This field is also used by application layer to calculate the interval of RAN visible buffer level measurement, which is equal to the periodicity of RAN visible application layer measurements reporting divided by numberOfBufferLevelEntries.

ran-VisiblePeriodicity The field indicates the periodicity of RAN visible application layer measurements reporting. Value ms120 indicates 120 ms, value ms240 indicates 240 ms and so on. If this field is absent, the periodicity of RAN visible application layer measurements reporting is the same as the reporting periodicity indicated in measConfigAppLayerContainer.

reportPlayoutDelayForMediaStartup The field indicates whether the UE shall report Playout Delay for Media Startup for RAN visible application layer measurements.

| Conditional Presence | Explanation |
| --- | --- |
| QoENRDC | This field is optionally present, Need M, when QoE for an NR-DC configuration is configured, i.e. when either QoE for an SCG is configured or when SRB5 is configured. Otherwise, it is absent. |
| ServiceType | This field is optionally present, Need M, when serviceType is set to streaming or vr. Otherwise, it is absent. |