3GPP TSG-RAN WG3 Meeting #125	R3-244703

Maastricht, NL, 19th – 23rd Aug 2024

Agenda Item:	16.3

Source:	ZTE (moderator)

Title:	Summary of Offline Discussion on CB: # AIoT2_CNRANSignalling

Document for:	Approval

- Introduction

CB: # AIoT2_CNRANSignalling

- Start with R3-244059, check the open issues above

(moderator - ZTE)

Summary of offline disc R3-244703

- For the Chairman’s Notes

R3-244792 is agreed

- Discussion-First round

RAN3 starts with inventory-only procedure. Discuss the content in Inventory/Command.

The AIoT CN to select suitable reader (AIoT enabled gNB or AIoT enabled UE reader) for both topology 1 and topology 2?

Assistance information from AIoT CN to AIoT RAN?

  - Content of Inventory procedure

Content of Inventory request message:

1) Session id

2) Reader id

3) Device id

4) Periodicity

5) Other content, if any

Question 1: Which content of Inventory request message do you agree? Any new content?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes for all |  |
| Nokia | Partly ok | Only Ok for 1/ mandatory, 2/ optional and 3/ optional. 4/ is FFS. I think the parameter “minimum number of responses“ should also be added so that the gNB knows when it can consider to stop and complete. |
| Samsung | Partly ok | 1, 3 - ok; 2, 4 - FFS. For 2, need to understand the use case: for initial inventory, the reader information has not been obtained in the network yet so no reader id can be signalled; for subsequent inventory (i.e. the network has stored the reader information for the targeted inventoried devices), needs to further discuss which node stores the reader information in advance, AIoT (enabled) RAN or AIoT CN. |
| Qualcomm | 1 – Confirm requirement first 2 – No 3 – Already agreed 4 - Yes | - First, we need to confirm that there is a requirement to support multiple parallel inventory sessions. Even if there is such a requirement, question is are these parallel inventory sessions intended for the same filter criteria/set of devices or is this a duplicate inventory session. So, we propose to discuss the requirement first and have this FFS. FFS if there is a requirement to support multiple parallel inventory sessions. If yes, are these sessions intended for different filter criteria/set of devices or are these duplicate inventory sessions. - If AIoT CN selects the Reader(s), there might be no need to indicate Reader ID in the Inventory Request message - Device ID(s) was already agreed in RAN3 last meeting. Periodicity can be useful as an “assistance information” to trigger multiple rounds of inventory from the Reader to the AIoT device at a certain periodicity |
| Huawei | Partly ok | 1 mandatory, 2 optional, 3 optional (it has already been agreed) 4 FFS 5 Assistance Information (optional) Estimated Number of devices, e.g., to enable CFRA, to assists resource allocation. |
| Xiaomi | Partly OK | - Session ID, Yes - Reader ID, FFS, it’s not clear what’s the reader ID, whther it’s gNB ID or something else. - Device ID, FFS, we agreed the device idenfication, it can be a filter (e.g. MASK-like) information for device to decide whther response to inventory, whether the device ID can be aware by gNB is FFS. - Peridodicity, Yes - minimum number of responses, can be optional. |
| NEC | Partly OK | - We support 1)&2)&3). - For 4), we think it is necessary, but if it is triggered by reader or CN should be further discussed. - We also support to add Estimated Number of devices, which is helpful to optimize the inventory procedure. |
| CATT | Partly OK - need clarify - yes, optional - yes, already agreed - need clarify requirement | For Session ID, similar view with QC, it seems there will be no praillel inventory procedures over AIoT Radio for a reader. Then in the XX interface, introduce session id may need RAN to buffer the request and handle it later, whether need to support cancellation? For Reader ID, ok, should be optional, e.g. for topology 1, it’s used in case of AIoT RAN node holds multiple readers, and AioT CN selects part of the reader for inventory. |
| LGE | Partly OK | OK for 1), 2), and 3) FFS for 4) |
| CMCC | Partly OK | 1) Session id, OK 2) Reader id, further discussion case by case 3) Device id, OK, agreed previous meeting. 4) Periodicity, optional |
| China Telecom |  | OK for 1) and 3) FFS for 2) and 4). We still unclear the meaning of reader ID |

Content of Inventory response message:

1) Session id

2) Reader id

3) Indication whether Inventory Request was successful or not

Question 2: Which content of Inventory response message do you agree? Any new content?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes for all |  |
| Nokia | Yes for both. |  |
| Samsung | See comments | Not sure whether the Inventory response is needed. Still do not see the strong need to have Inventory response message. A class2 Inventory request can be another alternative. |
| Qualcomm | 1 – Confirm requirement first 2 – Not in this message 3 - Yes | Inventory Response should be a simple response messge indicating whether inventory request was successful or failed. Therefore added 3). Detailed info (e.g. Reader ID) can be included in Inventory Report. |
| Huawei | Yes for 1) FFS for 2) | Session ID, mandatory Reader ID, FFS, does it mean partial success? For 3), principle ok, anyway response should indicate to go or not to go, but this seems to be stage 3 details. |
| Xiaomi | Partly yes | 1) Session id, yes 2) Reader id, FFS, not sure what’s the reader ID 3) Indication whether Inventory Request was successful or not, FFS, this can be done by design a failure response. |
| NEC | See comments | - Support 1) &2). - For 3), share view with Xiaomi. |
| CATT | - pending - no yes | 1)whether Session ID is included is up to the inventory request 2)Reader ID, not in this message, in the inventory report. - 3)should be ok for the class 1 inventory procedure, used to indicate RAN could not perform the inventory request from AIoT CN. |
| LGE | Partly OK | OK for 1) and 2) FFS for 3), we agree with Xiaomi |
| CMCC | Partly yes | - OK - Include reader ID should be discussed case by case. - OK |
| China Telecom |  | Ok for 1) FFS for 2) 3) is no need |

Content of Inventory report message:

1) Reader id

2) Device id

3) Enrichment info (e.g., signal strength measurements by Reader)

4) Session id

Question 3: Which content of Inventory report message do you agree? Any new content?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes for all |  |
| Nokia | Yes for both. |  |
| Samsung | Partly ok | 2 – ok; 1 – FFS, see comments in Q1. |
| Qualcomm | 1 – Confirm requirement first 2 – Yes | 1 – Same response as Q1 in Inventory Request 2 – Reader ID can be useful to have an association of Reader-devices at the AIoT CN node for subsequent inventory/command. Also can be useful for use case 1 in locating the AIoT device. 3 – Added this as we had proposed in the paper. This info can be useful for assisting the AIoT CN node by providing some signal strength measurements at Reader side. Can also be useful for use case 2 in locating the AIoT device. |
| Huawei | 1 FFS 2 Mandatory 3. not now 4 Mandatory | 1) FFS 2) device ID is carried as NAS PDU 3) not now 4) corresponding to the request/response, in the report message, the session ID should also be included. |
| Xiaomi | Parlty OK | 1) Reader id, FFS, not sure what reader ID is. 2) Device id, Yes, but this can be a continaer 3) Enrichment info (e.g., signal strength measurements by Reader), can be discussed in locating topic. 4) Session id, yes |
| NEC | Yes for all. |  |
| CATT | 1 yes 2 yes 3 FFS 4 pending | If Session id is introduced, all the inventory request/response, report should include this IE. We need to clarify the use case why session id is needed. |
| LGE | Partly OK | OK for 1), 2), and 4) FFS for 3) |
| CMCC | Parlty OK | 1) Reader id, Further discussion 2) Device id, OK 3) Enrichment info, NOT, at least not now. 4) Session id, OK |
| China Telecom |  | 1) FFS for reader ID - We understand Device id may be encoded as a container - Not sure this information is useful at this stage. Session id is OK. |

  - Overall procedure for inventory-only

RAN3 starts with inventory-only procedure. Discuss the content in Inventory/Command.

Overall procedure for AIoT Topology 1

Figure 1: Overall procedure for AIoT Topology 1

1. When the AIoT CN decides to initiate inventory procedure, it sends Inventory request message to the AIoT RAN node.

2. The AIoT RAN node sends Inventory response message to the AIoT CN.

Note： In step 2, if the AIoT RAN node sends Inventory failure message to the AIoT CN, this Inventory procedure ends.

3. The AIoT RAN node initiates inventory procedure at AIoT radio interface accordingly.

Note: This step is defined by RAN2.

4/4b. After receiving part or full of inventory result from devices, the AIoT RAN node sends a single or multiple Inventory reports to the AIoT CN including the received inventory result.

Question 4: Do you agree with the above figure and text description? Please modify it if any

| Companies | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes for all |  |
| Nokia | Yes for all |  |
| Samsung | Ok, but | Another alternative without step 2 should also be considered. |
| Qualcomm | Yes with the suggested editorial clarifications | 1. The AIoT CN sends an Inventory request message to the AIoT RAN node. 2. The AIoT RAN node sends an Inventory response message to the AIoT CN. Note： In step 2, the AIoT RAN node may instead send an Inventory failure message to the AIoT CN Indicating that the inventory procedure could not be initiated towards the AIoT device(s). 3. The AIoT RAN node initiates inventory procedure towards the AIoT device(s) over the AIoT radio interface.. Editor’s Note: This step is defined by RAN2. 4a/4b. After receiving inventory result from devices, the AIoT RAN node may send one or or multiple Inventory reports towards the AIoT CN including the received inventory result. |
| Huawei | Ok | See update above. |
| Xiaomi | Generally ok, but | some revision comments: - Remove the line in the box in step 3, only keep the box, we don’t know whether it’s only 2 steps or 4 steps inventory, this is RAN1/2 scope. - There must be an inventory procedure between AIoT RAN and AIoT devices, then there will be step 4b. |
| NEC | OK |  |
| CATT | Ok with QC’s update version |  |
| LGE | OK |  |
| CMCC | Generally fine |  |
| China Telecom | OK |  |

Overall procedure for AIoT Topology 2

Figure 2: Overall procedure for RRC based AIoT Topology 2

1. When the AIoT CN decides to initiate inventory procedure, it sends Inventory request message to the AIoT RAN node.

Note: Before initiating inventory procedure, the AIoT CN selects suitable UE/reader.

2. The AIoT RAN node sends Inventory response message to the AIoT CN.

Note： In step 2, if the AIoT RAN node sends Inventory failure message to the AIoT CN, this Inventory procedure ends.

3. The gNB initiates inventory procedure at Uu interface and AIoT radio interface accordingly.

Note: This step is defined by RAN2.

4/4b. After receiving part or full of inventory result from UE, the gNB sends a single or multiple Inventory reports to the AIoT CN including the received inventory result.

Question 5: Do you agree with the above figure and text description? Please modify it if any

| Companies | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes for all |  |
| Nokia | OK to include as one potential option |  |
| Samsung | Ok, but | See comment in Q4. |
| Qualcomm | Yes with editorial suggestions | 3. The gNB requests the AIoT enabled UE(s) to trigger inventory procedure towards the AIoT device(s). Editor’s Note: This step is defined by RAN2. Please also make changes to steps 1, 2, 4 as suggested for T1 |
| Huawei | Ok, but | The procedures over AIoT Radio are initiated by the AIoT enabled UE, not the AIoT enabled gNB, therefore, better to change step 3 to two steps. |
| Xiaomi | Ok but | Same comments in Q4 |
| NEC | OK, but | The AIoT enabled UE should be involved in step 1 and step 2 (it may related with RAN2 but necessary) because the AIoT enabled UE is the customer of inventory request. |
| CATT | Ok, but | Agree with QC’s update. |
| LGE | OK |  |
| CMCC | Ok, but | Agree with QC’s update. |
| China Telecom | OK | Agree with QC’s update |

Figure 3: Overall procedure for NAS/UP based AIoT Topology 2

1. When the AIoT CN decides to initiate inventory procedure, it sends Inventory request message included in NAS PDU/PDU session, to the UE/reader.

Note: Before initiating inventory procedure, the AIoT CN selects suitable UE/reader.

2.  The UE sends Inventory response message included in NAS PDU/PDU session, to the AIoT CN.

Note： In step 2, if the UE sends Inventory failure message to the AIoT CN, this Inventory procedure ends.

3.  The UE initiates inventory procedure at AIoT radio interface accordingly.

Note: This step is defined by RAN2.

4/4b. After receiving part or full of inventory result from UE, the UE sends a single or multiple Inventory reports included in NAS PDU/PDU session, to the AIoT CN.

Question 6: Do you agree with the above figure and text description? Please modify it if any

| Companies | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes for all |  |
| Nokia | OK to include as one possible option but | Replace “UE/ reader” by “AIoT enabled UE” in step 1. |
| Samsung | Ok, but | See comment in Q4. In addition, the content of Step 2: Inventory request -> Inventory response. |
| Qualcomm | Yes with editorial suggestions | 3. The gNB requests the AIoT enabled UE(s) to trigger inventory procedure towards the AIoT device(s). Editor’s Note: This step is defined by RAN2. Please also make changes to steps 1, 2, 4 as suggested for T1 |
| Huawei | ok |  |
| Xiaomi | Ok but | Same comments in Q4 |
| NEC | OK |  |
| CATT | ok |  |
| LGE | OK |  |
| CMCC | OK |  |
| China Telecom | OK |  |

- Conclusion, Recommendations

- References

- R3-244104 (TP for TR 38.769) Signalling and Procedures for Topology 1 (Huawei)	other

- R3-244428 Inventory, Command and device context management for Ambient IoT (Qualcomm Incorporated) discussion

- R3-244060 Discussion on unified procedure for AIoT topology 1 and 2 (ZTE Corporation) discussion

- R3-244059 (TP for TR38.769) NG interface impact for Ambient-IoT (ZTE Corporation)	other

- R3-244105 (TP for TR 38.769) Signalling and Procedures for Topology 2 (Huawei)	other

- R3-244158 RAN-CN interface impact on ambient IoT (NEC)	discussion

- R3-244184 (TP to TR 38.769) Network signalling aspects for Topology 1 (CATT)	other

- R3-244185 (TP to TR 38.769) Network signalling aspects for Topology 2 (CATT)	other

- R3-244192 (TP for TR 38.769) AIoT interface impacts between RAN and CN (Xiaomi)	other

- R3-244328 Inventory procedure of ambient IoT (Lenovo) discussion

- R3-244329 Context Management and Data Transport of Ambient IOT (Lenovo)	discussion

- R3-244389 [TP for TR 38.769] Elements for AIoT RAN – AIoT CN communication and overall signalling (Ericsson)	other

- R3-244421 Inventory Signalling Reusing Paging procedures for AIoT Topology 2 (Nokia )	discussion

- R3-244422 [TP for TR 38.769] Inventory Signalling reusing Paging procedures for AIoT Topology 2 (Nokia )	other

- R3-244423 [TP for TR 38.769] Data Transport and Context Management for AIoT (Nokia )	other

- R3-244556 Discussion on RAN-CN interface impact for Ambient IoT (Samsung)	discussion

- R3-244564 Discussion on RAN-CN interface for Ambient IoT (China Telecom)	discussion

- R3-244594 Discussion on paging for Ambient IoT (CMCC)	discussion

- R3-244595 Discussion on Device Context Management and Data Transfer (CMCC)	discussion

- R3-244607 Discussion on NG impact for Ambient IoT (LG Electronics)	discussion