3GPP TSG-RAN WG3 Meeting #125	R3-244749

Maastricht, Netherlands, 19th – 23rd August 2024

Title: 	TP on Improved granularity for Energy Cost

Source: 	Ericsson

Agenda item:	11.4

Document Type:	Discussion and decision

# TP to TR 38.743

## 5.3	Energy saving enhancements

------------------------------------------- Start of changes -------------------------------------------

## 5.3.1	Use case description

A description of the AI/ML-based Network Energy Saving use case as such is presented in TR 37.817. Based on the conclusions in TR 37.817, a metric called Energy Cost was introduced during Rel-18 normative work; this metric is defined as an index representing the energy consumption at the NG-RAN node and can be exchanged between NG-RAN nodes over the Xn interface upon request with a per NG-RAN node granularity. However, a node-level Energy Cost metric might be too coarse to provide a good understanding of the energy impact of AI/ML-based Network Energy Saving actions in NG-RAN nodes handling multiple cells, so possible solutions to improve Energy Cost granularity could be beneficial.

## 5.3.2	Solutions and standard impacts

During Rel-18 normative work, the Energy Cost (EC) metric was defined on an NG-RAN node-level granularity even though the AI/ML Energy Saving actions that we assumed in Rel-18 were on a cell-level granularity, e.g., cell switch-off actions. Enhancements to the node-level EC to a finer granularity could be beneficial, assuming that such enhancements enable network-level energy saving.

The following approaches to improve the EC granularity were discussed:

- EC per group of Cells based on energy consumption measurements associated to the hardware serving the group of cells

- EC per Cell based on energy consumption estimations

- EC per one or more HO event based on energy consumption estimations

- EC per gNB-DU based on energy consumption measurements associated to the hardware serving the gNB-DU

------------------------------------------- End of changes ------------------------------------------