3GPP TSG-RAN WG3 Meeting #125	R3-244767

# Maastricht, NL, 19 - 23 August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1485 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for DL PSI based discard |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson, Lenovo, CMCC, Qualcomm, Xiaomi, Nokia, Nokia Shanghai Bell，China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_Ph3-Core |  |  |  |  |  | Date: |  |  | 2024-08-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To support DL PSI based discard, regarding the following agreements: PSI Discard coordination: RAN3 to introduce new notification over F1AP and F1-U for DL PSI Discard. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the PSI based SDU Discard DL IE to the UE CONTEXT SETUP REQUEST and the UE CONTEXT MODIFICATION REQUEST messages. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The DL PSI based discard cannot be implemented in case of CU-DU split. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.2, 8.3.4.2, 9.2.2.1, 9.2.2.7, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.425 CR 0156 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGES START

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB, DRB, BH RLC channel, Uu Relay RLC channel, PC5 Relay RLC channel, and SL DRB configuration. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure. Except for RACH based SDT and UE configured with BWP specific ServingCellMO, the gNB-CU shall perform RRC Reconfiguration or RRC connection resume to send UE to the RRC_CONNECTED state as described in TS 38.331 [8], and in this case, the CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8]. In the cases of RACH based SDT procedure and UE configured with BWP specific ServingCellMO, the CellGroupConfig IE shall be ignored by the gNB-CU.

------ Unchanged part skipped ------

If the DRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the QoS Flow Mapping Indication IE is included in the DRB To Be Setup List IE for a QoS flow, the gNB-DU may take it into account that only the uplink or downlink QoS flow is mapped to the indicated DRB. If the SDT RLC Bearer Configuration IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, use it for packet transmission belonging to the SDT DRB indicated by the DRB ID IE. If the DRB Mapping Info IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the DRB Mapping Info IE for the DRB identified by the DRB ID IE and the Uu Relay RLC channel identified by the DRB Mapping Info IE. The gNB-DU shall use the mapping information stored for the mapping of DRB data to Uu Relay RLC channel.

If the PSI based SDU Discard UL IE is included in the DRB To Be Setup List IE and the value is set as "start", the gNB-DU shall, if supported, take it into account to perform UL PSI based SDU discarding activation or deactivation for the indicated DRB as defined in TS 38.321 [16].

If the PSI based SDU Discard DL IE is included in the DRB To Be Setup List IE, the gNB-DU shall, if supported, take it into account to provide suggestion on whether to activate or deactivate DL PSI based SDU discarding for the indicated DRB.

For each GBR DRB, if the Alternative QoS Parameters Sets IE is included in the GBR QoS Flow Information IE in the UE CONTEXT SETUP REQUEST message, gNB-DU shall, if supported, behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [3].

------ Unchanged part skipped ------

NEXT CHANGE

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

------ Unchanged part skipped ------

If the DRB To Be Setup List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in the TS 38.401 [4]. If the DRB Mapping Info IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the DRB Mapping Info IE, if present, for the DRB identified by the DRB ID IE and the Uu Relay RLC channel identified by the DRB Mapping Info. The gNB-DU shall use the mapping information stored for the mapping of DRB data to Uu Relay RLC channel.

If the PSI based SDU Discard UL IE is included in the DRB To Be Setup List IE or the DRB To Be Modified List IE and the value is set as "start", the gNB-DU shall, if supported, take it into account to perform UL PSI based SDU discarding activation or deactivation for the indicated DRB as defined in TS 38.321 [16].

If the PSI based SDU Discard DL IE is included in the DRB To Be Setup List IE or the DRB To Be Modified List IE, the gNB-DU shall, if supported, take it into account to provide suggestion on whether to activate or deactivate DL PSI based SDU for the indicated DRB.

If the BH Information IE is included in the UL UP TNL Information to be setup List IE or the Additional PDCP Duplication TNL List IE for a DRB, the gNB-DU shall, if supported, use the indicated BAP Routing ID and BH RLC channel for transmission of the corresponding GTP-U packets to the IAB-donor, as specified in TS 38.340 [30].

------ Unchanged part skipped ------

NEXT CHANGE

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| ------ Unchanged part skipped ------ |  |  |  |  |  |  |
| >>>>ECN Marking or Congestion Information Reporting Request | O |  | 9.3.1.321 |  | YES | ignore |
| >>>>PSI based SDU Discard UL | O |  | ENUMERATED (start, stop, …) | Indicates whether UL PSI based SDU discard is (re)configured or released for the DRB. The codepoint “start” means that UL PSI based discarding is (re)configured, while the codepoint “stop” means that UL PSI based discarding is released. Up to 8 DRBs can be set as “start”. | YES | ignore |
| >>>>PSI based SDU Discard DL | O |  | ENUMERATED (configured, not-configured, …) | Indicates whether DL PSI based SDU discard is configured or not for the DRB. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| ------ Unchanged part skipped ------ |  |  |  |  |  |  |

NEXT CHANGE

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| ------ Unchanged part skipped ------ |  |  |  |  |  |  |
| >>>>ECN Marking or Congestion Information Reporting Request | O |  | 9.3.1.321 |  | YES | ignore |
| >>>>PSI based SDU Discard UL | O |  | ENUMERATED (start, stop, …) | Indicates whether UL PSI based SDU discard is (re)configured or released for the DRB. The codepoint “start” means that UL PSI based discarding is (re)configured, while the codepoint “stop” means that UL PSI based discarding is released. Up to 8 DRBs can be set as “start”. | YES | ignore |
| >>>>PSI based SDU Discard DL | O |  | ENUMERATED (configured, not-configured, …) | Indicates whether DL PSI based SDU discard is configured or not for the DRB. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| ------ Unchanged part skipped ------ |  |  |  |  |  |  |

NEXT CHANGE

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

------ Unchanged part skipped ------

id-PDUSetQoSParameters,

id-N6JitterInformation,

id-ECNMarkingorCongestionInformationReportingRequest,

id-ECNMarkingorCongestionInformationReportingStatus,

id-ERedcap-Bcast-Information,

id-NeedForInterruptionInfoNR,

id-LTMCells-ToBeReleased-Item,

id-SCPAC-Request,

id-MobileIAB-Barred,

id-F1UTunnelNotEstablished,

id-S-CPACLowerLayerReferenceConfigRequest,

id-MusimCandidateBandList,

id-PSIbasedSDUdiscardUL,

id-SIB22-message,

id-U2URLCChannelQoS,

id-SL-PHY-MAC-RLC-ConfigExt,

id-UL-RSCP,

id-BW-Aggregation-Request-Indication,

id-ReportingGranularitykminus1,

id-ReportingGranularitykminus1additionalpath,

id-ReportingGranularitykminus2,

id-ReportingGranularitykminus2additionalpath,

id-ReportingGranularitykminus3,

id-ReportingGranularitykminus3additionalpath,

id-ReportingGranularitykminus4,

id-ReportingGranularitykminus4additionalpath,

id-ReportingGranularitykminus5,

id-ReportingGranularitykminus5additionalpath,

id-ReportingGranularitykminus6,

id-ReportingGranularitykminus6additionalpath,

id-TimingReportingGranularityFactorExtended,

id-PosValidityAreaCellList,

id-SymbolIndex,

id-AggregatedPosSRSResourceIDList,

id-PhaseQuality,

id-PRSBandwidthAggregationRequestIndication,

id-AggregatedPRSResourceSetList,

id-MeasuredFrequencyHops,

id-TxHoppingConfiguration,

id-AggregatedPosSRSResourceSetList,

id-ValidityAreaSpecificSRSInformation,

id-PeerUE-ID,

id-MeasBasedOnAggregatedResources,

id-SIB23-message,

id-PointA,

id-SCS-SpecificCarrier,

id-NR-PCI,

id-E-CID-MeasuredResultsAssociatedInfoList,

id-XR-Bcast-Information,

id-MaxDataBurstVolume,

id-PSIbasedSDUdiscardDL,

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

maxnoofBPLMNsNR,

maxnoofDLUPTNLInformation,

maxnoofNrCellBands,

maxnoofULUPTNLInformation,

maxnoofQoSFlows,

maxnoofSliceItems,

maxnoofSIBTypes,

maxnoofSITypes,

maxCellineNB,

maxnoofExtendedBPLMNs,

------ Unchanged part skipped ------

DRB-Information	::=	SEQUENCE {

dRB-QoS		QoSFlowLevelQoSParameters,

sNSSAI		SNSSAI,

notificationControl		NotificationControl		OPTIONAL,

flows-Mapped-To-DRB-List	Flows-Mapped-To-DRB-List,

iE-Extensions	ProtocolExtensionContainer { { DRB-Information-ItemExtIEs } }	OPTIONAL

}

DRB-Information-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-ECNMarkingorCongestionInformationReportingRequest		CRITICALITY ignore	EXTENSION ECNMarkingorCongestionInformationReportingRequest		PRESENCE optional	}|

{	ID id-PSIbasedSDUdiscardUL		CRITICALITY ignore	EXTENSION PSIbasedSDUdiscardUL		PRESENCE optional	}|

{	ID id-PSIbasedSDUdiscardDL		CRITICALITY ignore	EXTENSION PSIbasedSDUdiscardDL		PRESENCE optional	},

...

}

DRBs-Modified-Item	::= SEQUENCE {

dRBID							DRBID,

lCID								LCID		OPTIONAL,

dLUPTNLInformation-ToBeSetup-List		DLUPTNLInformation-ToBeSetup-List,

iE-Extensions	ProtocolExtensionContainer { { DRBs-Modified-ItemExtIEs } }	OPTIONAL,

...

}

------ Unchanged part skipped ------

PDUSetQoSInformation	::= SEQUENCE {

pduSetDelayBudget						ExtendedPacketDelayBudget		OPTIONAL,

pduSetErrorRate							PacketErrorRate					OPTIONAL,

pduSetIntegratedHandlingInformation		ENUMERATED {true, false, ...}	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { PDUSetQoSInformation-ExtIEs } }	OPTIONAL

}

PDUSetQoSInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PSIbasedSDUdiscardUL ::= ENUMERATED {start, stop, ...}

PSIbasedSDUdiscardDL ::= ENUMERATED {configured, not-configured, ...}

------ Unchanged part skipped ------

NEXT CHANGE

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

------ Unchanged part skipped ------

id-ReportingGranularitykminus6additionalpath		ProtocolIE-ID ::= 828

id-AggregatedPosSRSResourceSetList					ProtocolIE-ID ::= 829

id-RequestedSRSPreconfigurationCharacteristics-List	ProtocolIE-ID ::= 830

id-SRSPreconfiguration-List							ProtocolIE-ID ::= 831

id-SRSInformation									ProtocolIE-ID ::= 832

id-ValidityAreaSpecificSRSInformation				ProtocolIE-ID ::= 833

id-E-CID-MeasuredResultsAssociatedInfoList			ProtocolIE-ID ::= 834

id-XR-Bcast-Information								ProtocolIE-ID ::= 835

id-MaxDataBurstVolume 								ProtocolIE-ID ::= 836

id-TAInformation-List								ProtocolIE-ID ::= 837

id-NonIntegerDRXCycle								ProtocolIE-ID ::= 838

id-PointA											ProtocolIE-ID ::= 839

id-SCS-SpecificCarrier								ProtocolIE-ID ::= 840

id-NR-PCI											ProtocolIE-ID ::= 841

id-PeerUE-ID											ProtocolIE-ID ::= 842

id-EarlySyncServingCellInformation					ProtocolIE-ID ::= 843

id-RANSharingAssistanceInformation					ProtocolIE-ID ::= 844

id-LTMCFRAResourceConfig-List						ProtocolIE-ID ::= 845

id-F1U-PathFailure									ProtocolIE-ID ::= 846

id-MeasBasedOnAggregatedResources					ProtocolIE-ID ::= 847

id-SIB23-message									ProtocolIE-ID ::= 848

id-PSIbasedSDUdiscardDL								ProtocolIE-ID ::= xxx

END

-- ASN1STOP

CHANGES END