3GPP TSG-RAN WG3 Meeting #125	R3-244783

Maastricht, NL, 19th – 23rd August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0157 | rev | 1 | Current version: | 18.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Positioning SRS BW Aggregation and Tx Hopping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Ericsson, ZTE, Samsung, Qualcomm Incorporated, China Telecom, Huawei, Nokia, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-07-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. SRS for Positioning with Tx Frequency Hopping for RedCap UEs, RAN1 agreed that the number of hops: ENUMERATED {2,3,4,5,6} (new IE). However, in the NRPPa, the value range of Number of Hops is defined as INTEGER(1..6), which should be corrected. 2. For Positioning SRS BW Aggregation, RAN1/RAN2 agreed that the number of “aggregated combinations” is up to 32, and for each combination, only one SRS resource set from each of the 2 or 3 carriers are aggregated, e.g. CC#1 SRS resource set 1 + CC#2 SRS resource set 2 + CC#3 SRS resource set 3. In NRPPa, the Aggregated Positioning SRS Resource Set List IE includes at most 48 SRS resource sets to be aggregated in one combination, which equal to 3CCs * 16 SRS Resource sets per CC. It’s not well aligned with the design of TS 38.331, which may cause erroneous during SRS configuration. To make alignment with NR-Uu, and make the IE encoding more clear, it’s proposed to move the Aggregated Positioning SRS Resource Set List IE to the top level of the SRS Configuration IE. And make corresponding change to Positioning Activation/Deactivation procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Correct value range for number of Tx Hopping. - Redefine the IE “Aggregated Positioning SRS Resource Set List” to support up to 32 combinations, and move the IE to the top level of the SRS Configuration IE. Corresponding changes are also provided to POSITIONING ACTIVATION REQUEST and POSITIONING DEACTIVATION. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it affects the Positioning SRS BW Aggregation related function and Redcap Positioning related function. The CR has impact from functional and protocol point of view. The CR is non-backwards compatible from protocol point of view. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IE encoding for SRS BW aggregation, Redcap Positioning are not clear which may cause confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.9, 8.2.10, 9.1.1.17, 9.1.1.20, 9.2.28, 9.2.32, 9.2.94, 9.2.100, 9.3.4, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.473 CR1466 |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1, merge the CR from R3-244397. |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of the change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 8.2.9	Positioning Activation

#### 8.2.9.1	General

The Positioning Activation procedure is initiated by the LMF to request the NG-RAN node to activate semi-persistent or trigger aperiodic UL SRS transmission by the UE. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.9.2	Successful Operation

Figure 8.2.9.2-1: Positioning Activation procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING ACTIVATION REQUEST message to the NG-RAN node.

For semi-persistent UL SRS, the POSITIONING ACTIVATION REQUEST message includes an indication of the UL SRS resource set to be activated and may include the spatial relation for the semi-persistent UL SRS resource to be activated. For aperiodic UL SRS, if the SRS Resource Trigger IE is included in the POSITIONING ACTIVATION REQUEST message, the NG-RAN node shall take the value of this IE into account when triggering aperiodic SRS transmission by the UE.

If the Activation Time IE is included in the POSITIONING ACTIVATION REQUEST message, the NG-RAN node shall take the indicated value as the LMF’s requested time for activation of the UE’s SRS transmission.

Following successful activation of UL SRS transmission in the UE, the NG-RAN node shall respond with a POSITIONING ACTIVATION RESPONSE message. If the POSITIONING ACTIVATION RESPONSE message includes the System Frame Number and/or the Slot Number IEs, the LMF shall consider that the respective information indicates the activation time of SRS transmission by the UE.

If the Aggregated Positioning SRS Resource Set List IE is included in POSITIONING ACTIVATION REQUEST message and SRS bandwidth aggregation is configured in the UE, the NG-RAN node shall, if supported, activate the indicated SRS resource set(s) in the UE.

[…]

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 8.2.10	Positioning Deactivation

#### 8.2.10.1	General

The Positioning Deactivation procedure is initiated by the LMF to indicate to the NG-RAN node that UL SRS transmission should be deactivated in the UE. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.10.2	Successful Operation

Figure 8.2.10.2-1: Positioning Deactivation procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING DEACTIVATION message to the NG-RAN node. This message shall include an indication of the UL SRS resource set(s) to be deactivated or release all the related resources.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.1.1.17	POSITIONING ACTIVATION REQUEST

This message is sent by the LMF to cause the NG RAN node to activate/trigger UL SRS transmission by the UE.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| CHOICE SRS type | M |  |  |  | YES | reject |
| >Semi-persistent |  |  |  |  |  |  |
| >>SRS Resource Set ID | M |  | 9.2.33 | This IE is ignored if the Aggregated Positioning SRS Resource Set List IE below is present. | - |  |
| >>SRS Spatial Relation | O |  | Spatial Relation Information 9.2.34 | This IE is ignored if the Spatial Relation Information per SRS Resource IE is present. | YES | ignore |
| >>Spatial Relation Information per SRS Resource | O |  | 9.2.60 |  | YES | ignore |
| >>Aggregated Positioning SRS Resource Set List | O |  | 9.2.94 |  | YES | ignore |
| >Aperiodic |  |  |  |  |  |  |
| >>Aperiodic | M |  | ENUMERATED(true,…) |  | - |  |
| >>SRS Resource Trigger | O |  | 9.2.35 |  | - |  |
| >>Aggregated Positioning SRS Resource Set List | O |  | 9.2.94 |  | YES | ignore |
| Activation Time | O |  | Relative Time 1900 9.2.36 | Indicates the start time when the SRS activation is requested | YES | ignore |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.1.1.20	POSITIONING DEACTIVATION

This message is sent by the LMF to cause the NG RAN node to deactivate UL SRS transmission or release all the transmission by the UE.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| CHOICE Abort Transmission | M |  |  |  | YES | ignore |
| >Deactivate SRS Resource Set |  |  |  |  |  |  |
| >>SRS Resource Set ID | M |  | 9.2.33 |  | - |  |
| >Release ALL |  |  | NULL | the NG-RAN node configures UE to stop transmitting SRS for the positioning purpose |  |  |
| >Deactivate aggregated SRS Resource Set |  |  |  |  |  |  |
| >>Aggregated Positioning SRS Resource Set List | M |  | 9.2.94 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.2.28	SRS Configuration

This information element contains the SRS configuration configured by the NG-RAN node for the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| SRS Carrier List |  | 1 |  |  | - |  |
| >SRS Carrier List Item |  | 1..<maxnoSRS-Carriers> |  |  | - |  |
| >>Point A | M |  | INTEGER (0..3279165) | NR ARFCN | - |  |
| >>Uplink Channel BW-PerSCS-List |  | 1 |  | Corresponds to sCS-SpecificCarrierList in TS 38.331 [13] | - |  |
| >>>SCS Specific Carrier |  | 1. .<maxnoSCSs> |  |  | - |  |
| >>>>Offset To Carrier | M |  | INTEGER(0..2199,…) | First usable RB to Point A in the number of PRBs | - |  |
| >>>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120,…, kHz480, kHz960) |  | - |  |
| >>>>Carrier Bandwidth | M |  | INTEGER(1..275,…) |  | - |  |
| >>Active UL BWP |  | 1 |  | Only the configuration in the active UL BWP is needed. | - |  |
| >>>Location And Bandwidth | M |  | INTEGER(0..37949,…) | Corresponds to information provided in locationAndBandwidth contained in BWP IE as defined in TS 38.331 [13] | - |  |
| >>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120,…, kHz480, kHz960) |  | - |  |
| >>>Cyclic Prefix | M |  | ENUMERATED(Normal, Extended) |  | - |  |
| >>>Tx Direct Current Location | M |  | INTEGER(0..3301,…) |  | - |  |
| >>>Shift7dot5kHz | O |  | ENUMERATED(true,…) |  | - |  |
| >>>SRS Config |  | 1 |  | Corresponds to information provided in SRS-Config IE as defined in TS 38.331 [13] | - |  |
| >>>>SRS Resource List |  | 0..<maxnoSRS-Resources> |  |  | - |  |
| >>>>>SRS Resource | M |  | 9.2.29 | Corresponds to information provided in SRS-Resource contained in SRS-Config IE as defined in TS 38.331 [13] | - |  |
| >>>>Positioning SRS Resource List |  | 0..< maxnoSRS-PosResources> |  |  | - |  |
| >>>>>Positioning SRS Resource | M |  | 9.2.30 | Corresponds to information provided in SRS-PosResource contained in SRS-Config IE as defined in TS 38.331 [13] | - |  |
| >>>>SRS Resource Set List |  | 0..<maxnoSRS-ResourceSets> |  |  | - |  |
| >>>>>SRS Resource Set | M |  | 9.2.31 | Corresponds to information provided in SRS-ResourceSet contained in SRS-Config IE as defined in TS 38.331 [13] | - |  |
| >>>>Positioning SRS Resource Set List |  | 0..<maxnoSRS-PosResourceSets> |  |  | - |  |
| >>>>>Positioning SRS Resource Set | M |  | 9.2.32 | Corresponds to information provided in SRS-PosResourceSet contained in SRS-Config IE as defined in TS 38.331 [13] | - |  |
| >>NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID of the cell that contains the SRS carrier | - |  |
| Aggregated Positioning SRS Resource Set List | O |  | 9.2.94 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-Carriers | Maximum no of carriers for SRS. Value is 32. |
| maxnoSCSs | Maximum no of SCS spacings for a carrier. Value is 5. |
| maxnoSRS-Resources | Maximum no of SRS resources per UL BWP. Value is 64. |
| maxnoSRS-PosResources | Maximum no of positioning SRS resources per UL BWP. Value is 64. |
| maxnoSRS-ResourceSets | Maximum no of SRS resource sets per UL BWP. Value is 16. |
| maxnoSRS-PosResourceSets | Maximum no of positioning SRS resource sets per UL BWP. Value is 16. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.2.32	Positioning SRS Resource Set

This information element indicates a positioning SRS resource set in the UE for UL SRS transmission.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Positioning SRS Resource Set ID | M |  | INTEGER(0..15) |  | - |  |
| Positioning SRS Resource ID List |  | 1..<maxnoSRS-PosResourcePerSet> |  |  | - |  |
| >Positioning SRS Resource ID | M |  | INTEGER(0..63) |  | - |  |
| CHOICE Resource Type | M |  |  |  | - |  |
| >periodic |  |  |  |  |  |  |
| >>PosperiodicSet | M |  | ENUMERATED(true,…) |  | - |  |
| >semi-persistent |  |  |  |  |  |  |
| >>Possemi-persistentSet | M |  | ENUMERATED(true,…) |  | - |  |
| >aperiodic |  |  |  |  |  |  |
| >>SRS Resource Trigger | M |  | INTEGER(1..3) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-PosResourcePerSet | Maximum no of positioning SRS resources per positioning SRS resource set. Value is 16. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.2.94	Aggregated Positioning SRS Resource Set List

This information element is used to indicate the aggregated Positioning SRS Resource Set List.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Aggregated SRS Positioning Resource Set List |  | 1 |  |  |
| >Aggregated SRS Positioning Resource Set Item |  | 1..< maxnoAggregatedPosSRSCombinations> |  |  |
| >>Combined Positioning SRS Resource Set List |  | 1 |  |  |
| >>>Combined Positioning SRS Resource Set Item |  | 2..< maxnoaggregatedPosSRS-ResourceSets> |  |  |
| >>>>Point A | M |  | INTEGER (0..3279165) | NR ARFCN |
| >>>>NR PCI | O |  | INTEGER(0..1007) |  |
| >>>>Positioning SRS Resource Set ID | M |  | INTEGER(0..15) |  |
| >>>>SCS Specific Carrier |  | 1 |  |  |
| >>>>>Offset To Carrier | M |  | INTEGER (0..2199, …) |  |
| >>>>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, … , kHz480, kHz960) |  |
| >>>>>Carrier Bandwidth | M |  | INTERGER (1..275, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoaggregatedPosSRS-ResourceSets | Maximum no of aggregated SRS Positioning Resource Sets among 2~3 Carriers. Value is 3. |
| maxnoAggregatedPosSRSCombinations | Maximum no of Aggregated SRS Positioning Resource Set Combinations. Value is 32. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.2.100	Tx Hopping Configuration

This information element indicates the SRS Tx frequency hopping configuration.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Overlap Value | M |  | ENUMERATED(rb0, rb1, rb2, rb4) |  |
| Number of Hops | M |  | INTEGER(2..6) |  |
| Slot Offset for Remaining Hops List |  | 1 |  |  |
| >Slot Offset for Remaining Hops Item |  | 1..<maxnoofHopsMinusOne> |  |  |
| >>CHOICE slot offset remaining hops | M |  |  |  |
| >>>aperiodic |  |  |  |  |
| >>>>Slot Offset | O |  | INTEGER(1..32) |  |
| >>>>Start Position | O |  | INTEGER(0..13) | In symbols |
| >>>semi-persistent |  |  |  |  |
| >>>>SRS Periodicity | M |  | 9.2.99 |  |
| >>>>Offset | M |  | INTEGER(0..81919, …) | In slots |
| >>>>Start Position | O |  | INTEGER(0..13) | In symbols |
| >>>periodic |  |  |  |  |
| >>>>SRS Periodicity | M |  | 9.2.99 |  |
| >>>>Offset | M |  | INTEGER(0..81919, …) |  |
| >>>>Start Position | O |  | INTEGER(0..13) | In symbols |

| Range bound | Explanation |
| --- | --- |
| maxnoofHopsMinusOne | Maximum no of hops that can be configured for positioning SRS transmission minus one. Value is 5. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.3.4	PDU Definitions

-- **************************************************************

--

#### -- POSITIONING ACTIVATION REQUEST

--

-- **************************************************************

PositioningActivationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningActivationRequestIEs} },

...

}

PositioningActivationRequestIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-SRSType					CRITICALITY reject	TYPE SRSType					PRESENCE mandatory	} |

{ ID id-ActivationTime			CRITICALITY ignore	TYPE RelativeTime1900				PRESENCE optional	}

,

...

}

SRSType ::= CHOICE {

semipersistentSRS				SemipersistentSRS,

aperiodicSRS					AperiodicSRS,

choice-Extension				ProtocolIE-Single-Container { { SRSType-ExtIEs} }

}

SRSType-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

SemipersistentSRS ::= SEQUENCE {

sRSResourceSetID			SRSResourceSetID,

iE-Extensions				ProtocolExtensionContainer { {SemipersistentSRS-ExtIEs} } OPTIONAL,

...

}

SemipersistentSRS-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-SRSSpatialRelation						CRITICALITY ignore	EXTENSION SpatialRelationInfo					PRESENCE optional}|

{ ID id-SRSSpatialRelationPerSRSResource		CRITICALITY ignore	EXTENSION SpatialRelationPerSRSResource		PRESENCE optional}|

{ ID id-PosSRSResourceSet-Aggregation-List		CRITICALITY ignore TYPE PosSRSResourceSet-Aggregation-List		PRESENCE optional},

...

}

AperiodicSRS ::= SEQUENCE {

aperiodic					ENUMERATED{true,...},

sRSResourceTrigger			SRSResourceTrigger OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {AperiodicSRS-ExtIEs} } OPTIONAL,

...

}

AperiodicSRS-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-PosSRSResourceSet-Aggregation-List		CRITICALITY ignore TYPE PosSRSResourceSet-Aggregation-List		PRESENCE optional},

...

}

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<<<<<<<<<<<<<< unchanged texts omitted >>>>>>>>>>>>>>>

maxFreqLayers,

maxnoPRSTRPs,

maxNumResourcesPerAngle,

maxnoAzimuthAngles,

maxnoElevationAngles,

maxnoVACell,

maxnoaggregatedPosSRS-Resources,

maxnoaggregatedPosSRS-ResourceSets,

maxnoAggPosPRSResourceSets,

maxnoofTimeWindowSRS,

maxnoofTimeWindowMeas,

maxnoPreconfiguredSRS,

maxnoofHopsMinusOne,

maxnoAggCombinations,

maxnoAggregatedPosSRSCombinations,

<<<<<<<<<<<<<< unchanged texts omitted >>>>>>>>>>>>>>>

#### -- A

AbortTransmission ::= CHOICE {

deactivateSRSResourceSetID		SRSResourceSetID,

releaseALL						NULL,

choice-extension				ProtocolIE-Single-Container { { AbortTransmission-ExtIEs } }

}

AbortTransmission-ExtIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-PosSRSResourceSet-Aggregation-List	CRITICALITY ignore TYPE PosSRSResourceSet-Aggregation-List		PRESENCE mandatory },

...

}

<<<<<<<<<<<<<< unchanged texts omitted >>>>>>>>>>>>>>>

#### -- P

<<<<<<<<<<<<<< unchanged texts omitted >>>>>>>>>>>>>>>

PosSRSResourceSet-Item ::= SEQUENCE {

possrsResourceSetID				INTEGER(0..15),

possRSResourceIDPerSet-List		PosSRSResourceIDPerSet-List,

posresourceSetType				PosResourceSetType,

iE-Extensions		ProtocolExtensionContainer { { PosSRSResourceSet-Item-ExtIEs} }	OPTIONAL,

...

}

PosSRSResourceSet-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PosSRSResourceSet-Aggregation-List ::= SEQUENCE (SIZE (1..maxnoAggregatedPosSRSCombinations)) OF PosSRSResourceSet-Aggregation-Item

PosSRSResourceSet-Aggregation-Item ::= SEQUENCE {

combined-posSRSResourceSet-List			Combined-PosSRSResourceSet-List,

pointA				INTEGER (0..3279165),

pCI-NR				INTEGER(0..1007)		OPTIONAL,

possrsResourceSetID	INTEGER(0..15),

iE-Extensions		ProtocolExtensionContainer { { PosSRSResourceSet-Aggregation-Item-ExtIEs} }	OPTIONAL,

...

}

PosSRSResourceSet-Aggregation-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

Combined-PosSRSResourceSet-List ::= SEQUENCE (SIZE (2..maxnoaggregatedPosSRS-ResourceSets)) OF Combined-PosSRSResourceSet-Item

Combined-PosSRSResourceSet-Item::= SEQUENCE {

pointA				INTEGER (0..3279165),

pCI-NR				INTEGER(0..1007)		OPTIONAL,

possrsResourceSetID	INTEGER(0..15),

scs-specificCarrier	SCS-SpecificCarrier,

iE-Extensions						ProtocolExtensionContainer { { Combined-PosSRSResourceSet-Item-ExtIEs} } OPTIONAL,

...

}

Combined-PosSRSResourceSet-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

#### -- S

<<<<<<<<<<<<<< unchanged texts omitted >>>>>>>>>>>>>>>

SRSConfiguration ::= SEQUENCE {

sRSCarrier-List		SRSCarrier-List,

iE-Extensions			ProtocolExtensionContainer { { SRSConfiguration-ExtIEs } } OPTIONAL,

...

}

SRSConfiguration-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-PosSRSResourceSet-Aggregation-List	CRITICALITY ignore EXTENSION PosSRSResourceSet-Aggregation-List		PRESENCE optional },

...

}

#### -- T

<<<<<<<<<<<<<< unchanged texts omitted >>>>>>>>>>>>>>>

TxHoppingConfiguration ::= SEQUENCE {

overlapValue						ENUMERATED {rb0, rb1, rb2, rb4},

numberOfHops						INTEGER (2..6),

slotOffsetForRemainingHopsList		SlotOffsetForRemainingHopsList,

iE-extensions		ProtocolExtensionContainer { { TxHoppingConfiguration-ExtIEs } }	OPTIONAL,

...

}

TxHoppingConfiguration-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

<<<<<<<<<<<<<< unchanged texts omitted >>>>>>>>>>>>>>>

maxPRS-ResourcesPerSet						INTEGER ::= 64

maxNoSSBs									INTEGER ::= 255

maxnoofPRSresourceSet						INTEGER ::= 8

maxnoofPRSresource							INTEGER ::= 64

maxnoofULAoAs								INTEGER ::= 8

maxNoPathExtended							INTEGER ::= 8

maxnoARPs									INTEGER ::=	16

maxnoUETEGs									INTEGER ::= 256

maxnoTRPTEGs								INTEGER ::= 8

maxFreqLayers								INTEGER ::= 4

maxNumResourcesPerAngle						INTEGER ::= 24

maxnoAzimuthAngles							INTEGER ::= 3600

maxnoElevationAngles						INTEGER ::= 1801

maxnoPRSTRPs								INTEGER ::= 256

maxnoVACell									INTEGER ::= 32

maxnoaggregatedPosSRS-Resources				INTEGER ::= 3

maxnoaggregatedPosSRS-ResourceSets			INTEGER ::= 3

maxnoAggPosPRSResourceSets					INTEGER ::= 3

maxnoofTimeWindowSRS						INTEGER ::= 16

maxnoofTimeWindowMeas						INTEGER ::= 16

maxnoPreconfiguredSRS						INTEGER ::= 16

maxnoofHopsMinusOne							INTEGER ::= 5

maxnoAggCombinations						INTEGER ::= 2

maxnoAggregatedPosSRSCombinations			INTEGER ::= 32

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of the change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>