3GPP TSG-RAN WG3 Meeting #125	R3-244792

Maastricht, NL, 19th – 23rd Aug 2024

Title: 	(TP for TR38.769) NG interface impact for Ambient-IoT

Source: 	ZTE Corporation

Agenda item:	16.3

Document for:	Discussion and Decision

- Text Proposal

================================Start of change===============================

6.4	Impacts on CN-RAN interface

Editor’s note: Corresponds to the first RAN3 objective in the SID, to identify necessary impacts on signaling and procedures for CN-RAN interface.

6.4.1	Information exchanged between AIoT CN and AIoT RAS

6.4.1.1	Inventory

Inventory can be sent by the AIoT CN for a single device, or a group of devices, or all devices.

The Inventory Request from the AIoT CN to the AIoT RAS, may include the following:

(1)	AIoT Device Identification (to find a single device, a group of devices, or all devices)

Note 1:	The definition of this identification is out of RAN3 scope.

Editor’s Note 1: It is FFS whether AIoT RAS needs to interpret/store/process it.

(2)	Scope of inventory request (e.g., a certain area in which the inventory is to be triggered)

Multiple individual AIoT Device IDs (one ID per device) can be provided to the AIoT CN via a single Inventory Report.

Editor’s Note 2: It is up to SA2 whether device ID is sent transparent or not.

6.4.1.2	Command

Command can be sent by the AIoT CN for a single device.

Editor’s Note 1: it is FFS for command on a group of devices, or all devices.

Editor’s Note 2: it is FFS whether AIoT RAS can remain agnostic of the type of request from the AIoT CN (need to differentiate command and inventory)

6.4.1.3	Overall procedures of Inventory procedure

Figure 6.4.1.3-1: Overall procedure for AIoT Topology 1

Figure 6.4.1.3-2: Overall procedure for RRC based AIoT Topology 2

Figure 6.4.1.3-3: Overall procedure for NAS/UP based AIoT Topology 2

================================End of change===============================