3GPP TSG-RAN3 Meeting #125	R3-244801

# Maastricht, Netherlands, 19-23 Aug, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0426 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on mobile IAB-node authorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Lenovo, Xiaomi, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_mobile_IAB-Core |  |  |  |  |  | Date: |  |  | 2024-08-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - it is specified that “During the mobile IAB-MT migration procedure and the mobile IAB-node RLF recovery procedure, the target/new RRC-terminating IAB-donor-CU receives the authorization status of the mobile IAB-node from the source/initial RRC-terminating IAB-donor-CU as well as from the 5GC during Path Switch Request procedure. ”. As we know, the mobile IAB-MT migration procedure comprises mobile IAB-MT migration via NG handover and mobile IAB-MT migration via Xn handover as captured in clause 8.23.1 and clause 8.23.2 respectively. So the above sentence is incorrect since it only applies to the mobile IAB-MT migration via Xn handover case. - For a mobile IAB-node, the IAB donor-CU receives the authorizations from 5GC or from another IAB donor CU, when the authorization status is received for the first time and “not authorized”, the IAB-donor-CU neither establishes the backhaul resources nor allocates any BAP address, TNL address or default BAP configuration for this mobile IAB-node. However, when the authorization status is not received for the first time, different action shall be taken, e.g. the IAB-donor-CU attempts to hand over the UEs served by the mobile IAB-node to other cell(s), releases the F1 interface towards the mobile IAB-DU, and releases all backhaul resources (including the BAP address, TNL address and default BAP configuration) for this mobile IAB-node. As we can see, different actions are taken by the IAB donor CU for the two cases. However, the two cases when the authorization status is received are not distinguished in the current description in 38.401. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add “ via Xn handover ” to clarify that during the mobile IAB-MT migration via Xn handover procedure, the target/new RRC-terminating IAB-donor-CU receives the authorization status of the mobile IAB-node from the source/initial RRC-terminating IAB-donor-CU as well as from the 5GC during Path Switch Request procedure. - Clarify that if the authorization status is received for the first time and the status is “not authorized”, the IAB-donor-CU neither establishes the backhaul resources nor allocates any BAP address, TNL address or default BAP configuration for this mobile IAB-node. Impact analysis Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). This CR has impact on the functional point of view, the impact can be considered isolated because it only impacts the mobile IAB-node authorization. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 2 description of mobile IAB node authorization is incorrect and incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.9.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - |  |  |  |  |  |  |  |  |

| This CR's revision history: | - Rev 1:Keep the first two changes and remove other changes based on discussion at the RAN3#125 meeting. - Rev 2: Keep the first change based on discussion at the RAN3#125 meeting. |
| --- | --- |

-------------------------------------------Start of change-------------------------------------------

8.9.14	Mobile IAB-node authorization

During the mobile IAB-node integration procedure or mobile IAB-MT migration via NG handover, the RRC-terminating IAB-donor-CU receives the authorization status of the mobile IAB-node from the 5GC. During the mobile IAB-MT migration via Xn handover procedure and the mobile IAB-node RLF recovery procedure, the target/new RRC-terminating IAB-donor-CU receives the authorization status of the mobile IAB-node from the source/initial RRC-terminating IAB-donor-CU as well as from the 5GC during Path Switch Request procedure. If the authorization status is “not authorized”, the RRC-terminating IAB-donor-CU does not establish any backhaul resources. Additionally, the RRC-terminating IAB-donor-CU does not allocate any BAP address, TNL address(es) or default BAP configuration for this mobile IAB-node. If the authorization status for the mobile IAB-node changes, the 5GC sends an updated authorization status to the RRC-terminating IAB-donor-CU.

-------------------------------------------End of change-------------------------------------------