3GPP TSG-RAN WG3 Meeting #125	R3-244840

Maastricht, NL, 19 - 23 August, 2024

Title: 	(TP for TR 38.769) Locating Ambient-IoT device

Source: 	CMCC

Agenda item:	16.4

Document for:	Discussion & Approval

1. Introduction

This contribution provides the TP for TR 38.769 on locating Ambient-IoT device based on online and offline discussion.

2. Text Proposal for TR 38.769

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 6.8	Locating ambient IoT devices

Editor’s note 1: Proximity determination may be in a 6.8.x sub-clause, or another arrangement, depending on how the study proceeds.

Editor’s note 2: Corresponds to the third RAN3 objective in the SID, to identify potential solutions for locating an Ambient IoT device with no specification impact, e.g., reusing existing user location report, or minimal specification impact to convey location information to core network.

### 6.8.1	General

AIoT device location information may be used for the following purposes:

(a) improving the AIoT operation itself, e.g. by AIoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the AIoT service.

Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes.

For topology 1, AIoT RAN node ID can be considered as a location of AIoT device. For topology 2, UE ID of the AIoT-enabled UE can be considered as a location of AIoT device.

Editor’s Note 1: How to know the “reader” location is FFS. Whether to use more than one “readers” within one AIoT RAN for location purposes is FFS.

Editor’s Note 2: Signalling details on how to provide the location information of the AIoT device to the AIoT CN needs further study.

Editor’s Note 3: Introducing finer granularity for locating an Ambient device besides the “reader ID granularity” needs further study.

Editor’s Note 4: Analysising the gap between the positioning requirements in the SID and the feasibility for AIoT devices.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>