3GPP TSG-RAN WG3 Meeting #125	R3-244862

Maastricht, NL, from 19th to 23rd 2024

Agenda Item:	11.4

Source:	ZTE Corporation, Huawei, Ericsson, InterDigital, Nokia,CATT, NEC, Lenovo, Telecom Italia, LG Electronics, Deutsche Telekom, Samsung, CMCC, Jio

Title:	(TP to TR 38.743) AI/ML for NG-RAN Rel-18 Leftovers

Document for:	Text Proposal

1	Introduction

This TP follows discussions in R3-244700.

2	Text Proposal

5	Rel-18 Leftovers and solutions

5.1	Mobility optimization for NR-DC

5.1.1	Use case description

Mobility in NR-DC can be optimized by means of AI/ML.

Mobility Optimization for NR-DC is studied by assuming inference at the MN only. The main use case is limited to Dual Connectivity only and Conditional Dual Connectivity procedures are out of scope.

5.1.2	Potential Standard impacts

The Dual Connectivity procedures (e.g., SN Addition, MN-initiated SN Change) are enhanced to trigger the collection of measured UE performance.

5.2	Split architecture support for Rel-18 use cases

5.2.1	Use case description

The split architecture should be enhanced to support the Rel-18 use cases, e.g, Load Balancing, Energy Saving, and Mobility Optimization.

In case of CU-DU architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU(-CP);

-	AI/ML Model Training and Model Inference are both located in the gNB-CU(-CP).

5.2.2	Potential Standard impacts

The following standard impacts are listed for subsequent Rel-19 normative work compared to what was specified during Rel-18:

-	The details of signaling measured UE performance metrics from gNB-DU to gNB-CU-CP and/or from gNB-CU-UP to gNB-CU-CP need further discussion during normative phase.

-	Measured Energy Cost from gNB-DU to gNB-CU.

5.3	Energy saving enhancements

Editor Note: Capture the description and its potential standard impacts.

5.4	Continuous MDT collection targeting the same UE across RRC states

5.4.1	Use Case Description

The problem of continuous data collection for management-based MDT can be described as follows: a UE in the NG-RAN can be configured with management-based Logged MDT when in RRC_Idle and RRC_Inactive states and with management-based Immediate MDT when in RRC_Connected state. Differently from signalling-based MDT, in management-based MDT, a UE is not uniquely identified in the MDT activation. Therefore, when a UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive (during which Logged MDT data have been collected) or when a UE is handed over between gNBs, the network does not have standardized means to select again the same UE for continuous MDT for subsequent MDT data collection.

The Data Collection continuity in this scenario can be split into two tasks as below:

-	Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

-	Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

5.4.2	Potential Standard impacts

Potential solutions addressing the problems above can be discussed during normative phase.