3GPP TSG-RAN3 Meeting #126	R3-247149

Orlando, USA, Nov 18th – 22nd, 2024

Agenda Item:	16.3

Source: 	Qualcomm Incorporated

Title: 	Inventory and Command related procedures for Ambient IoT

Document for:	Discussion/Decision

# Background

In this paper, we provide our views on AIoT related procedures e.g., Inventory, Command based on the progress and open issues identified in the previous WG meetings.

# Discussion

## 2.1 General

In the draft TR, RAN2 added following definitions for Inventory and Command:

Inventory: The service provided by the network to discover and acquire the identifier of A-IoT device(s).

Command: The service provided by the network to send the operation instruction to the A-IoT device (e.g. read, write, etc.).

Further, RAN2 is discussing several procedures such as:

- Inventory-only

- Command-only

- Inventory and Command

In a prior meeting, RAN3 agreed to start with the Inventory-only procedure as shown below and captured several candidate procedures for A-IoT Inventory for Topology 1 and Topology 2 in the TR.

RAN3 starts with inventory-only procedure. Discuss the content in Inventory/Command.

In addition to the Inventory-only use case, we think RAN3 should also study and capture the candidate procedures for Command-only and Inventory and Command use case as well, for both T1 and T2. Therefore, the following is proposed:

In addition to Inventory-only use case, RAN3 should study and capture the candidate procedures for Topology 1 and Topology 2 (RRC/NAS/user-plane based) for the following use cases as well:

  - Command-only

  - Inventory and Command

Currently, section 6.5 the TR has the following structure for capturing Inventory/Command:

- 6.5 Impacts on CN-RAN Interface

  - 6.5.1 Information exchanged between A-IoT CN and A-IoT RAN

    - 6.5.1.1. Inventory

    - 6.5.1.2 Command

  - 6.5.2 Signaling and Procedures for Topology 1

    - 6.5.2.1 Candidate procedures for A-IoT Inventory for Topology 1

  - 6.5.3 Signaling and Procedures for Topology 2

    - 6.5.2.1 Candidate procedures for A-IoT Inventory for Topology 2

Section 6.5 in the TR covers candidate procedures for both Topology 1 and Topology 2 and end-to-end call flows (not just focuses on the “CN-RAN” interface or information exchanged between A-IoT CN and “A-IoT RAN”). Therefore, the structure of section 6.5 should be reworked and therefore the following is proposed:

Section 6.5 in the TR covers candidate procedures for both Topology 1 and Topology 2 and end-to-end call flows (not just focuses on the “CN-RAN” interface or information exchanged between A-IoT CN and “A-IoT RAN”). Therefore, the structure of section 6.5 should be reworked as follows:

  - 6.5 A-IoT procedures

    - 6.5.1 Inventory

      - 6.5.1.1. Description

      - 6.5.1.2 Candidate procedures for A-IoT Inventory for Topology 1

      - 6.5.1.3 Candidate procedures for A-IoT Inventory for Topology 2

    - 6.5.2 Command

      - 6.5.2.1. Description

      - 6.5.2.2 Candidate procedures for A-IoT Command for Topology 1

      - 6.5.2.3 Candidate procedures for A-IoT Command for Topology 2

In section 6.5, use the terminology “Reader” as a shorthand to refer to A-IoT RAN node in case of T1 and A-IoT enabled UE in case of T2, wherever commonalities apply to both T1 and T2.

## 2.2 Inventory-only

In the TR, the following is captured by RAN2 for the “inventory-only” use case:

For the detailed use case of “inventory-only”, it is supported by the procedure with step A and step B as baseline.

- Step A: A-IoT paging. Based on the service request, the reader sends the A-IoT paging message indicating device(s) that need to respond.

- Step B: D2R data transmission. Triggered A-IoT device(s) perform the device ID transmission via the A-IoT random access procedure or without using the A-IoT random access procedure. See sub-clause 6.2.4.

SA2 is discussing aspects such as how the Application Function (AF) initiates the inventory process and what kind of information it provides to the AIoTF. For example, AF may provide the following:

- Filter Criteria (e.g., product category, area) to limit the inventory to AIoT Devices matching those criteria (e.g., only Pepsi cans in a warehouse).

- a periodicity value to request the inventory to be performed periodically.

- a list of reader IDs to limit the inventory to specific Readers.

Here a Reader is used to refer to the A-IoT RAN node in case of Topology 1 and the A-IoT enabled UE in case of Topology 2. AIoTF might limit the inventory to specific Readers (if AF requests it) or can select Readers on its own. Irrespective of that, once the Reader(s) are selected, the AIoTF sends the Inventory Request to those Reader(s).

Each Reader that received the Inventory Request from the AIoTF performs the Inventory procedure towards the AIoT device. The Readers then collects the Device IDs and provides the Device IDs to the AIoTF. The Readers may further provide some enrichment data e.g., the Reader ID to the AIoTF.

The inventory-only procedure call flow is captured in the Figure 1 below.

Figure 1: Inventory-only procedure

We therefore propose the following:

In the Inventory Request sent by the AIoTF to the Reader, AIoTF can provide an Inventory Periodicity value which refers to the periodicity with which the Reader should send Inventory Report to the AIoTF

In the Inventory Request sent by the AIoTF to the Reader, AIoTF can provide expected number of AIoT device(s) that will respond to assist in resource scheduling

In the Inventory Report sent by the Reader to the AIoTF, the Reader can include Enrichment data (e.g., the Reader ID, signal strength measurements by the Reader) in addition to the Device ID(s)

This would be needed only if there is a requirement that there might be multiple inventory sessions in parallel i.e., the AIoTF selects the same reader for inventory but for different purposes

There were also proposals in the previous meeting to include a “Session ID” or “AIoT transaction ID” to the Inventory Request/Response/Report message to identify a certain AIoT transaction. Before agreeing to any such enhancements, we should first discuss the requirement.

It is not clear whether an AIoTF via implementation can ensure that a second inventory request is not sent towards a Reader before the first inventory procedure involving the Reader is complete

In case AIoTF might send a second inventory request even before the first inventory procedure is complete, Reader behavior needs to be defined e.g., (and this needs coordination with RAN2/SA2)

- Alt 1: Reader should reject the second inventory request and inform the AIoTF

- Alt 2: Reader stores the second inventory request and performs the second inventory towards the AIoT devices only after the first inventory is complete (sequential inventory)

- Alt 3: Reader processes the second inventory request and performs the second inventory towards the AIoT devices even before the first inventory is complete (i.e., parallel inventory)

To keep it simple, we propose the following:

RAN3 assumes that AIoTF (e.g., via implementation) can ensure that a second inventory request is not sent towards a Reader before a first inventory procedure involving the Reader is complete.

We discussed Radio Resource management for NAS/UP based solutions in T2 in detail in our companion paper R3-247148 along with several options/call flows (e.g., based on UE Request or CN Request). We propose to capture the call flows for Inventory depicting how the A-IoT resource configuration is determined e.g., based on A-IoT assistance info provided by A-IoT enabled UE.

For NAS-based solution and user-plane based solution, enhance the existing call flow to show that the A-IoT enabled gNB determines the A-IoT resource configuration based on “A-IoT assistance info” provided by the A-IoT enabled UE over Uu after receiving the Inventory Request over NAS/UP.

## 2.3 Command-only

In the TR, the following is captured by RAN2 for the “command-only” use case:

For the detailed use case of “command-only”:

-	It can be also supported by the baseline procedure with step A, step B, step C1 and step C2.

- Step A: A-IoT paging. Based on the service request, the reader sends the A-IoT paging message indicating device(s) that need to respond.

  - NOTE 1:	In the sub-clause 6.2, the term of “A-IoT paging message” is equal to the “(initial) trigger message”. For simplification, only the former is used.

- Step B: D2R data transmission. Triggered A-IoT device(s) perform the device ID transmission via the A-IoT random access procedure or without using the A-IoT random access procedure.

- Step C1: Possible R2D data transmission (e.g. for sending the command).

- Step C2: Possible D2R data transmission (e.g. the corresponding response to command).

As can be seen from here in the TR on the Command-only use case, the Command can also be sent towards a group of devices(s) e.g., an AF might want to read water usage from all water meters in a certain area. Therefore, we propose RAN3 to also study the use case when Command is sent towards a group of devices as well in addition to targeting a single device. Hence, the following is proposed:

RAN3 agrees to study the use case where Command is sent towards a group of devices meeting a certain filter criterion (and not just targeting a single device)

Upon receiving the Command from the AF, the AIoTF should determine the list of Readers to use for sending the Command taking the Reader IDs (if provided by the AF) and any stored information (e.g., about the last known Reader for specific Device IDs) into account.

The AIoTF then should provide the Command and the Device IDs or Filter Criteria (whichever has been provided by the AF) to the Readers.

Each Reader that receives the Command request from the AIoTF, sends the Command to the AIoT Device(s) e.g., in step 8 after AIoT paging and an AIoT random access procedure. The AIoT Device(s) that respond to the Reader provide a Command Response which the Reader forwards it to the AIoTF. The Readers may optionally include Enrichment Data. The above procedure is captured in Figure 2.

Figure 2: Command-only procedure

Therefore, the following is proposed:

Upon receiving Command from AF, AIoTF sends at least the following to the Reader in a “Command Request” message:

- Command instruction (e.g., Read/Write)

- Filter Criteria (e.g., device identifier(s) or a product code for which the Command is to be sent)

- Assistance info (e.g., expected response)

After completing the Command procedure over the A-IoT radio interface, the Reader sends at least the following to the AIoTF in a “Command Report” message:

  - Command Response(s)

  - Enrichment data (e.g., Reader ID, signal strength)

Multiple Command Response(s) can be provided by the Reader to the AIoTF via a single Command Report.

Similar to Inventory, capture in TR the candidate procedures for Command for Topology 1 and Topology 2 (RRC/NAS/user-plane based) showing the Command Request/Response/Report messages and how A-IoT radio resource management is done for Command.

The following aspects are left to normative phase:

  - Whether the Inventory/Command procedures can be modeled as a single message or need separate messages

  - Whether the call flows for T1 and T2 can be merged into a single call flow (e.g., using the terminology “Reader” or need separate call flows)

  - Whether the Inventory/Command messages are carried as “upper layer” information or carried explicitly

Agree the TP to 38.769 in Annex.

# Summary

2.1 General

Proposal 1:	In addition to Inventory-only use case, RAN3 should study and capture the candidate procedures for Topology 1 and Topology 2 (RRC/NAS/user-plane based) for the following use cases as well:

a. Command-only

b. Inventory and Command

Proposal 2:	Section 6.5 in the TR covers candidate procedures for both Topology 1 and Topology 2 and end-to-end call flows (not just focuses on the “CN-RAN” interface or information exchanged between A-IoT CN and “A-IoT RAN”). Therefore, the structure of section 6.5 should be reworked as follows:

o	6.5 A-IoT procedures

 6.5.1 Inventory

6.5.1.1. Description

6.5.1.2 Candidate procedures for A-IoT Inventory for Topology 1

6.5.1.3 Candidate procedures for A-IoT Inventory for Topology 2

6.5.2 Command

6.5.2.1. Description

6.5.2.2 Candidate procedures for A-IoT Command for Topology 1

6.5.2.3 Candidate procedures for A-IoT Command for Topology 2

Proposal 3:	In section 6.5, use the terminology “Reader” as a shorthand to refer to A-IoT RAN node in case of T1 and A-IoT enabled UE in case of T2, wherever commonalities apply to both T1 and T2.

2.2 Inventory-only

Proposal 4:	In the Inventory Request sent by the AIoTF to the Reader, AIoTF can provide an Inventory Periodicity value which refers to the periodicity with which the Reader should send Inventory Report to the AIoTF

Proposal 5:	In the Inventory Request sent by the AIoTF to the Reader, AIoTF can provide expected number of AIoT device(s) that will respond to assist in resource scheduling

Proposal 6:	In the Inventory Report sent by the Reader to the AIoTF, the Reader can include Enrichment data (e.g., the Reader ID, signal strength measurements by the Reader) in addition to the Device ID(s)

Proposal 7:	RAN3 assumes that AIoTF (e.g., via implementation) can ensure that a second inventory request is not sent towards a Reader before a first inventory procedure involving the Reader is complete.

Proposal 8:	For NAS-based solution and user-plane based solution, enhance the existing call flow to show that the A-IoT enabled gNB determines the A-IoT resource configuration based on “A-IoT assistance info” provided by the A-IoT enabled UE over Uu after receiving the Inventory Request over NAS/UP.

2.3 Command-only

Proposal 9:	RAN3 agrees to study the use case where Command is sent towards a group of devices meeting a certain filter criterion (and not just targeting a single device)

Proposal 10:	Upon receiving Command from AF, AIoTF sends at least the following to the Reader in a “Command Request” message:

oCommand instruction (e.g., Read/Write)

oFilter Criteria (e.g., device identifier(s) or a product code for which the Command is to be sent)

oAssistance info (e.g., expected response)

Proposal 11:	After completing the Command procedure over the A-IoT radio interface, the Reader sends at least the following to the AIoTF in a “Command Report” message:

a.Command Response(s)

b.Enrichment data (e.g., Reader ID, signal strength)

Proposal 12:	Multiple Command Response(s) can be provided by the Reader to the AIoTF via a single Command Report.

Proposal 13:	Similar to Inventory, capture in TR the candidate procedures for Command for Topology 1 and Topology 2 (RRC/NAS/user-plane based) showing the Command Request/Response/Report messages and how A-IoT radio resource management is done for Command.

Proposal 14:	The following aspects are left to normative phase:

a. Whether the Inventory/Command procedures can be modeled as a single message or need separate messages

b. Whether the call flows for T1 and T2 can be merged into a single call flow (e.g., using the terminology “Reader” or need separate call flows)

c. Whether the Inventory/Command messages are carried as “upper layer” information or carried explicitly

Proposal 15:	Agree the TP to 38.769 in Annex.

# 4 Annex: TP to 38.769

## 6.5	A-IoT procedures

### 6.5.1	Inventory

#### 6.5.1.1	Description

Inventory can be sent by the A-IoTF for a single device, or a group of devices, or all devices.

The Inventory Request from the A-IoTF to the Reader (i.e., the A-IoT RAN node in topology 1 and the A-IoT enabled UE in topology 2), may include the following:

(1)	A-IoT Device Identification (to find a single device, a group of devices, or all devices). This can also be a filter criteria (e.g., a certain type of product codes for which the inventory is triggered)

Note 1:	The definition of this identification is out of RAN3 scope.

Editor’s Note 1: It is FFS whether the Reader needs to interpret/store/process it.

(2)	Scope of inventory request (e.g., a certain area in which the inventory is to be triggered)

(3) Inventory periodicity e.g., the periodicity with which the Reader should send Inventory Report to the A-IoTF.

Multiple individual A-IoT Device IDs (one ID per device) can be provided from the Reader to the A-IoTF via a single Inventory Report.

In the Inventory Report, the Reader may further provide some enrichment data (e.g., the Reader ID, signal strength measurements by the Reader) to the A-IoTF.

The A-IoTF may store the association between the Reader ID and A-IoT device ID(s) obtained from Inventory Report for subsequent AIoT signaling.

Editor’s Note 2: It is up to SA2 whether device ID is sent transparently or not.

RAN3 assumes that AIoTF (e.g., via implementation) can ensure that a second inventory request is not sent towards a Reader before a first inventory procedure involving the Reader is complete.

Figure 6.5.3.1-2: Message flow for A-IoT Inventory in Topology 2 (if NAS/UP based solution is used)

### 6.5.2	Command

#### 6.5.2.1	Description

Command can be sent by the A-IoTF for a single device, or a group of devices or all devices.

In the Command Request sent from AIoTF to the Reader, the following may be included:

(1) Command instruction (e.g., Read/Write/Disable)

(2) Filter Criteria (e.g., a certain area or certain product types for which Command is to be sent)

(3) Device identifier (e.g., if Command is sent to a specific A-IoT device)

(4) Assistance information (e.g., expected response size)

Each Reader that receives the Command request from the AIoTF sends the Command to the AIoT Device(s).

After performing Command procedure over the A-IoT radio, the Reader sends at least the following to the AIoTF node in a “Command Report” message including the following:

(1) Command Response(s)

(2) Enrichment data (e.g., Reader ID, signal strength)

Multiple Command Response(s) can be provided by the Reader to the AIoTF via a single Command Report.

Editor’s Note 2: it is FFS whether A-IoT RAN node can remain agnostic of the type of request from the A-IoT CN (need to differentiate command and inventory)