3GPP TSG RAN WG3 Meeting #126	R3-247170

Orlando, U.S.A., 18 - 22 November, 2024

Title: 	Discussion on support of intra-SN SCPAC in MN format

Source: 	ZTE Corporation

Agenda item:	8.1

Document for:	Discussion and Decision

- Introduction

In this meeting, RAN3 has received an LS (R3-247012/ R2-2409359) from RAN2, as below.

1. Overall Description: RAN2 thanks RAN3 for the reply LS on inter-node RRC message for intra-SN SCPAC configured by the MN. Regarding the following questions, RAN2 has discussed and would like to provide the responses as below: Q1: Are both CG-Config and CG-CandidateList related to same configuration of intra-SN subsequent CPAC in MN format, and must be sent together in the same SN Modification Required XnAP message? RAN2 response: From RAN2 perspective, the CG-Config and CG-CandidateList are related to different part of UE configurations of intra-SN SCPAC configured by the MN, and not required to be sent together in the same SN Modification Required XnAP message. Q2: If both RRC messages must be sent together, is it feasible to include both CG-Config message and CG-CandidateList messages in a single RRC message (new or existing), which could then be transferred in a single SN Modification Required XnAP message? RAN2 response: RAN2 understands that two XnAP messages can be sent from the SN to the MN to transfer CG-Config message and CG-CandidateList messages separately. RAN2 requests RAN3 to consider the above answers for the stage-2 procedure changes for the Intra-SN SCPAC with MN involvement applicable.

This paper addresses to resolve this issue based on the above LS.

- Discussion

Based on the LS above, CG-Config message and CG-CandidateList messages can be send via multiple XnAP messages. In order to ensure the MN can collect both CG-Config message and CG-CandidateList messages from the SN, the SN needs to send at least two XnAP messages to the MN upon initiation the preparation of intra-SN SCPAC with MN involvement procedure. In RAN2’s discussion, the following options are proposed:

•	Option 1: SN sends two SN Modification Required XnAP messages to the MN, i.e. one includes CG-Config message and the other includes CG-CandidateList message.

•	Option 2: SN sends CG-Config message and CG-CandidateList message separately via SN Modification Required XnAP message and SN Modification Request Acknowledge XnAP message.

In our understanding, Option 2 seems more natural. However, we need to note that currently, the MN initiated SN modification procedure is optionally performed (TS 37.340 clause 10.20 Figure 10.20-3). If we want to use the SN Modification Request Acknowledge XnAP message to transfer CG-Config message or CG-CandidateList message, this procedure should become mandatory. Besides, some corresponding descriptions may need to be added.

[TS 37.340] SN initiated intra-SN subsequent CPAC with MN involvement This procedure is initiated by the SN for intra-SN subsequent CPAC with MN involvement. Figure 10.20-3: Intra-SN subsequent CPAC - SN initiated with MN involvement Figure 10.20-3 shows an example signalling flow for intra-SN subsequent CPAC initiated by the SN with MN involvement: 1. The SN initiates the conditional SN modification procedure by sending the SN Modification Required message, which contains an intra-SN subsequent CPAC initiation indication. The message includes a list of PSCell(s) to prepare and associated execution conditions proposed for the initial execution of subsequent CPAC and execution conditions proposed for the following execution of subsequent CPAC, and for each prepared PSCell, the SN decides SCG SCells and provides the new corresponding SCG radio resource configuration to the MN in an NR RRCReconfiguration** message contained in the SN Modification Required message. The SN may include an indication that the SCG radio resource configuration of a prepared PSCell is a complete candidate configuration, i.e. that it is not a delta configuration with respect to the reference SCG configuration. 2/3. The MN initiated SN Modification procedure may be triggered by SN Modification Required message, e.g. when an SN security key change needs to be applied. NOTE 12: For SN terminated bearers to be setup for which PDCP duplication with CA is configured in NR MCG side, the SN allocates up to 4 separate Xn-U bearers and the MN provides a logical channel ID for primary or split secondary path to the SN via the nested MN-initiated SN modification procedure. <----------------Omit--------------------------->

Observation 1: For option 2, the MN shall initiate SN modification procedure after receiving SN modification required message from Source SN.

In the current S-NODE MODIFICATION REQUIRED message in TS38.423, there is an IE S-CPAC Request, which is used to indicate that SN modification is for S-CPAC preparation.

[TS38.423] 8.3.4 S-NG-RAN node initiated S-NG-RAN node Modification 8.3.4.1 General This procedure is used by the S-NG-RAN node to modify the UE context in the S-NG-RAN node. The procedure uses UE-associated signalling. ------------------------------------<Skip unrelated part>-------------------------------------- If the S-CPAC Request IE set to "initiation" is included in the S-NODE MODIFICATION REQUIRED message, the M-NG-RAN node shall, if supported, consider that the procedure is triggered for intra-SN S-CPAC preparation in MN RRC format, as described in TS 37.340 [8]. ------------------------------------<Skip unrelated part>-------------------------------------- #### 9.1.2.8 S-NODE MODIFICATION REQUIRED This message is sent by the S-NG-RAN node to the M-NG-RAN node to request the modification of S-NG-RAN node resources for a specific UE. Direction: S-NG-RAN node M-NG-RAN node. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3.1; YES; reject. M-NG-RAN node UE XnAP ID; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the M-NG-RAN node; YES; reject. S-NG-RAN node UE XnAP ID; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the S-NG-RAN node; YES; reject. Cause; M; 9.2.3.2; YES; ignore. ------------------------------------<Skip unrelated part>--------------------------------------. S-NG-RAN node to M-NG-RAN node Container; O; OCTET STRING; Includes the CG-Config message or the CG-CandidateList message as defined in subclause 11.2.2 of TS 38.331 [10].; YES; ignore. ------------------------------------<Skip unrelated part>--------------------------------------. S-CPAC Request; O; ENUMERATED (initiation, ...); Indicates that SN modification is for S-CPAC preparation. This IE may be sent from the source SN.; YES; reject. PDU Sessions List To Be Released - UPError; O; 9.2.1.41; YES; ignore.

So, if the MN receives the S-NODE MODIFICATION message with the IE S-CPAC Request, the MN knows that this modification request is for intra-SN S-CPAC in MN format. Then, the MN initiates SN medication procedure.

Observation 2: According to TS38.423, if receiving SN modification required message including IE S-CPAC Request, the MN shall initiate SN modification procedure.

[TS38.423] 9.1.2.6 S-NODE MODIFICATION REQUEST ACKNOWLEDGE This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node’s request to modify the S-NG-RAN node resources for a specific UE. Direction: S-NG-RAN node M-NG-RAN node. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3.1; YES; reject. M-NG-RAN node UE XnAP ID; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the M-NG-RAN node; YES; ignore. S-NG-RAN node UE XnAP ID; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the S-NG-RAN node; YES; ignore. ------------------------------------<Skip unrelated part>--------------------------------------. S-NG-RAN node to M-NG-RAN node Container; O; OCTET STRING; Includes the CG-Config message or the CG-CandidateList message as defined in subclause 11.2.2 of TS 38.331 [10].; YES; ignore. Admitted Split SRBs; O; ENUMERATED (srb1, srb2, srb1&2, ...); Indicates admitted SRBs; YES; ignore. ------------------------------------<Skip unrelated part>-------------------------------------.

Proposal 1: Select option 2 to support intra-SN SCPAC in MN format, i.e., SN sends CG-Config message and CG-CandidateList message separately via SN Modification Required XnAP message and SN Modification Request Acknowledge XnAP message.

- Conclusion

Based on the above analysis, we provide the following observations proposals, and corresponding CRs are also provided.

Observation 1: For option 2, the MN shall initiate SN modification procedure after receiving SN modification required message from Source SN.

Observation 2: According to TS38.423, if receiving SN modification required message including IE S-CPAC Request, the MN shall initiate SN modification procedure.

Proposal 1: Select option 2 to support intra-SN SCPAC in MN format, i.e., SN sends CG-Config message and CG-CandidateList message separately via SN Modification Required XnAP message and SN Modification Request Acknowledge XnAP message.

The corresponding CRs (R3-247171, R3-247172) and draft LS (R3-247173) are also provided.