3GPP TSG RAN WG3 Meeting #126	R3-247186

Orlando, U.S.A., 18 - 22 November, 2024

Title:	(TP to TR 38.769) Completion of Signaling and Procedures for Topology 2

Agenda Item:	16.3

Source:	ZTE Corporation

Document for:	other

- Introduction

In the last RAN3 #125bis meeting, we have agreed to capture Signaling and Procedures for Topology 1 and for RRC based Topology 2 into TR 38.769 (R3-245849).

This TP is used to modify the Signaling and Procedures for RRC based Topology 2 and for NAS based Topology 2.

- Discussion

  - RRC based Topology 2

The following figure is captured in the TR 38.769, for RRC based Toplogy 2.

6.5.3.1.1 RRC solution Figure 6.5.3.1-1: Message flow for A-IoT Inventory in Topology 2 - RRC-based solution

The step 2b can also be performed before step 1c/2a, i.e., the A-IoT enable gNB can send Inventory response/failure message to AIoT CN before it triggers RRC inventory procedure.

Proposal 1: AIoT enable gNB can send XX inventory response/failure message to AIoT CN, before it triggers RRC Inventory procedure.

  - NAS based Topology 2

In the cosigned TP (R3-247185 Signaling and Procedures for CN assistant NAS/UP based Topology 2), the AIoT CN firstly initiated CN assistant AIoT radio resource request procedure, then it sends Inventory request to the AIoT UE including the received AIoT radio resources.

Figure 6.5.3.1-3: Message flow for CN assistant A-IoT Inventory in Topology 2 - NAS/UP based solution

In the above figure, for the signalling for CN assistant A-IoT Inventory in Topology 2 - NAS/UP based solution, one candidate solution is that AIoT resource request procedure and AIoT inventory procedure can be handled in sequence.

In this TP, we provide another candidate soltion for NAS based soltion, i.e., AIoT resource request procedure and AIoT inventory procedure can be handled in band, as below.

Figure 1: Message flow for CN assistant A-IoT Inventory in Topology 2 - NAS based solution (Option xx)

In above figure 1, the candidate solution on AIoT resource request procedure and AIoT inventory procedure handling in band is only used for NAS based soltion in Topology 2. However, SA2 has not agreed NAS based Topolgy 2. So, if NAS based Topology 2 is not finally agreed in SA2, the signalling and procedure for NAS based Topology 2 as above figure 1, shall be removed.

Moreover, in above figure 1, after receiving Step 1, if the AIoT-enabled gNB cannot configure the AIoT resource, it shall send AIoT handling failure messag to AIoT CN in step 3. In this case, the AIoT-enabled gNB shall not delivery the received NAS PDU to the AIoT-enabled UE. It is a new gNB behaviour.

Proposal 2: RAN3 agrees another solution for CN assistant A-IoT Inventory in Topology 2 – NAS based solution, i.e., AIoT resource request procedure and AIoT inventory procedure are handled in band.

3	Text Proposal

-----------Start of the Change------------

6.5.3.1	Candidate procedures for A-IoT Inventory for Topology 2

6.5.3.1.1	RRC solution

Figure 6.5.3.1-1: Message flow for A-IoT Inventory in Topology 2 - RRC-based solution

1a.	The A-IoT CN sends an Inventory request message to the A-IoT enabled gNB

1b.	The A-IoT enabled gNB allocates and coordinates usage of A-IoT radio resources.

1c/2a	RRC communication with the A-IoT enabled UE takes place.

Editor’s Note 1:	RRC based communication is only depicted schematically, details in RAN2 FFS.

2b.	The A-IoT-enabled gNB sends an Inventory response message to the A-IoT CN.

NOTE 1: In step 2b, the A-IoT-enabled gNB may instead send an Inventory failure message to the A-IoT CN indicating that the inventory procedure could not be initiated towards the A-IoT device(s). Then, this procedure ends.

NOTE 2: The step 2b can be ahead of step 1c/2a.

3.	The A-IoT-enabled gNB requests the A-IoT-enabled UE(s) to trigger inventory procedure towards the A-IoT device(s).

4a/4b.	After receiving inventory result reported from the A-IoT enabled UEs, the A-IoT-enabled gNB may send one or multiple Inventory reports towards the A-IoT CN including the received inventory result.

NOTE 3: Steps 4a/4b may happen in parallel with Step 3 for different A-IoT devices.

Editor’s Note 2:	Step 4a/4b between A-IoT-enable UE and A-IoT-enabled gNB can be refined by RAN2.

-----------Next of the Change------------

6.5.3.1.2	NAS/UP solution

Editor’s Note 1: Future discussions on A-IoT Inventory will take place based on the following message flow, working on the content of the messages including ownership, associated functions, scope, etc.

Figure 6.5.3.1-4: Message flow for CN assistant A-IoT Inventory in Topology 2 - NAS based solution

1.	The A-IoT CN sends XX AIoT handing request message to the A-IoT enabled gNB, including AIoT resource request information and NAS PDU (conveying Inventory request message)

2.	The A-IoT enabled gNB allocates and coordinates the usage of A-IoT radio resources.

3.	The A-IoT enabled gNB sends XX AIoT handing response message to the A-IoT CN.

NOTE 1:  In step 3, the A-IoT enabled gNB may instead send an XX AIoT handing failure message to the A-IoT CN indicating that the AIoT handing procedure could not be initiated towards the A-IoT enabled UE. Then A-IoT enabled gNB shall not delivery NAS PDU to the A-IoT enabled UE by DL RRC message, and the procedure ends.

4.  The A-IoT enabled gNB sends DL RRC message to the A-IoT enabled UE, including Inventory request message and A-IoT radio resources configured at step 2.

5. The A-IoT-enabled gNB requests the A-IoT-enabled UE(s) to trigger inventory procedure towards the A-IoT device(s).

6/6b. The A-IoT enabled UE may send Inventory report message encapsulated in NAS PDU towards the A-IoT CN including the received inventory result, by one or more UL RRC messages.

NOTE 2: Steps 6a/6a may happen in parallel with Step 5 for different A-IoT devices.

NOTE 3: In step 6, the A-IoT enabled UE may instead send an Inventory failure message encapsulated in NAS PDU towards the A-IoT CN, by UL RRC message.

-----------End of the Change------------