3GPP TSG-RAN WG3 Meeting #126	R3-247190

Orlando, US, 18 - 22 Nov, 2024

Title:	(TP to TR 38.769) Protocol Stacks for Topology 2 NAS/UP based solutions

Agenda Item:	16.2

Source:	Huawei, CMCC, ZTE, CATT, NEC, Xiaomi, Lenovo, China Telecom, Samsung, Qualcomm

Document for:	other

# Introduction

In this paper, we propose to capture the leftover details on UP based and NAS based solutions as provided in section 3.

# 2	Reference

[1] 3GPP TR 23.700-13 Study on Architecture support of Ambient power-enabled Internet of Things, (Release 19) V1.1.0 (2024-10)

# 3	Text Proposal

-----------Start of the Change------------

#### 6.4.2.1	Solutions for Topology 2

##### 6.4.2.1.0	General

To support Topology 2, the following solutions are to be studied for conveying A-IoT upper layer information:

-	RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

-	NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

-	UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

NOTE 1:	The protocol stack for each solution option does not illustrate A-IoT CN internal architecture and how A-IoT upper layer information is transported, if any. Details are subject to SA2 agreements.

NOTE 2:	The selection among these solutions is subject to SA2 conclusion.

##### 6.4.2.1.1	Solution1: RRC based solution

Upon receiving XXAP: A-IoT related message from A-IoT CN, the A-IoT-enabled gNB transmits the related information towards the A-IoT-enabled UE via NR Uu RRC, and vice versa.

Figure 6.4.2.1.1-1: RRC based solution of Topology 2

##### 6.4.2.1.2	Solution 2: NAS based solution

The A-IoT related messages between the A-IoT CN and the A-IoT-enabled UE are carried via A-IoT-enabled UE’s DL/UL NAS packets, the A-IoT-enabled gNB handles the A-IoT-enabled UE’s NAS packets as legacy, i.e., using DL NAS Transport and UL NAS Transport procedures over NGAP.

Figure 6.4.2.1.2-1: NAS based solution of Topology 2

NOTE 1: The communication between A-IoT CN and the A-IoT-enabled UE via A-IoT related messages and the presence of AIoT-AP protocol are up to SA2 decision.

NOTE 2: Control and coordination of AIoT radio resources should be enabled in NAS based solution, there are different options to achieve this purpose, as described in 6.5.1.3.

NOTE 3: The definition and description of AIoTF and AIoT-AP refers to TR 23.700-13 [7].

##### 6.4.2.1.3	Solution 3: UP based solution

The A-IoT related messages between the A-IoT CN and the A-IoT-enabled UE are carried via A-IoT-enabled UE’s PDU Session, the A-IoT-enabled gNB handles the A-IoT-enabled UE’s user plane data as legacy, i.e., over NG-U GTP-U tunnels.

Figure 6.4.2.1.3-1: UP based solution of Topology 2

NOTE 1: The communication between A-IoT CN and the A-IoT-enabled UE via A-IoT related messages and the presence of AIoT-AP/Transport/IP protocols are up to SA2 decision.

NOTE 2: Control and coordination of AIoT radio resources should be enabled in UP based solution, there are different options to achieve this purpose, as described in 6.5.1.3.

NOTE 3: The definition and description of AIoTF and AIoT-AP/Transport/IP refers to TR 23.700-13 [7].

# 4	Annex

In SA2 TR [1], in section section 8.1 captures the Interim Conclusion on Key Issue #1, the following interim conclusions can be found:

### 8.1.3 Architecture to Support Topology 2 #### 8.1.3.1 General Two options will be specified for Topology 2: - User-plane option as defined in clause 8.1.3.2 - RRC-based option as defined in clause 8.1.3.3 Editor’s note: Whether a NAS-based option is needed in addition is FFS. #### 8.1.3.2 User-plane option The following principles apply: - As depicted in Figure 8.1.3.2-1, the UE Reader connects to the AIoTF based on the AIoT Application Protocol (AIoT-AP) using an IP PDU Session between the UE and the UPF as transport. The related protocol stack is shown in Figure 8.1.3.2-2. The AIoT AP protocol will support procedures and information to be exchanged as specified by RAN2, RAN3 and SA2. Figure 8.1.3.2-1: User-plane architecture for Topology 2 Figure 8.1.3.2-2: Protocol stack for the user-plane architecture for Topology 2 NOTE 1: Which transport protocol to use for AIoT-AP can be decided by CT1 in coordination with RAN3. NOTE 2: Security for AIoT-AP is assumed to be defined by SA3. Editor’s note: Further details are FFS. Editor’s note: The terminology, e.g. AIoT RAN is FFS.