3GPP TSG-RAN WG3 Meeting #126	R3-247193

Orlando, USA, 18 - 22 Nov, 2024

Title:	Consideration on A-IoT Device Locating

Agenda Item:	16.4

Source:	Huawei

Document for:	other

# 1	Introduction

After RAN3#125 bis meeting, regarding to A-IoT Device Locating, there are four Editor’s Notes needs to be solved:

Editor’s Note 1:	How to know the “reader” location is FFS. Whether to use more than one “readers” within one A-IoT RAN for location purposes is FFS.

Editor’s Note 2:	Signalling details on how to provide the location information of the A-IoT device to the A-IoT CN needs further study.

Editor’s Note 3:	Introducing finer granularity for locating an Ambient device besides the “reader ID granularity” needs further study.

Editor’s Note 4:	Analysing the gap between the positioning requirements in the SID and the feasibility for A-IoT devices.

In this paper, we provide our further analyses on Ambient IoT device locating, try to solve these FFSs.

# 2	Discussion

## 2.1	How to know the “reader” location?

There is an Editor’s note 1 in [1] includes two FFSs, the first one is how to know the “reader” location.

Editor’s Note 1:	How to know the “reader” location is FFS. Whether to use more than one “readers” within one A-IoT RAN for location purposes is FFS.

In Topology 1, considering that the A-IoT RAN does not move, there is no need to update its location to the A-IoT CN frequently. Therefore, the A-IoT CN can get the A-IoT RAN’s location (may with per TRP information) based on e.g., OAM configuration, or A-IoT XX Setup procedure.

Proposal 1: In topology 1, the A-IoT CN can get the A-IoT RAN’s location (may with per TRP information) based on e.g., OAM configuration.

In Topology 2, the A-IoT CN is able to get the A-IoT-enabled UE’s location by existing mechanisms, there is no need to introduce new method to enable this in this study.

Proposal 2: In topology 2, the A-IoT CN is able to get the A-IoT-enabled UE’s location by existing mechanisms.

## 2.2	How to provide the location information of the A-IoT device to the A-IoT CN?

There are following agreements about AIoT Device Locating captured in current pCR [1]:

For topology 1, A-IoT RAN node ID can be considered as a location of A-IoT device. In Topology 1, an A-IoT RAN node may serve one or more readers. For topology 2, UE ID of the A-IoT-enabled UE can be considered as a location of A-IoT device.

And in last meeting, there is the following new agreement not captured in pCR yet:

The following principle applies to Topology1 and Topology2: For both purpose a) and b), identifier of the reader may be reported to AIoT CN in the Inventory Report message.

From the above agreements, we can see that A-IoT RAN node ID can be considered as a location of A-IoT device in topology 1, and UE ID of the A-IoT-enabled UE can be considered as a location of A-IoT device in topology 2, but as mentioned in the following Editor’s Note 2, signalling details on how to provide the location information of the A-IoT device to the A-IoT CN needs further study:

Editor’s Note 2:	Signalling details on how to provide the location information of the A-IoT device to the A-IoT CN needs further study.

In Topology 1, during the setup of XX interface, the A-IoT RAN will provide the A-IoT RAN node ID to the A-IoT CN, afterwards, during the A-IoT services related procedures after XX setup procedure, as the A-IoT CN knows which A-IoT RAN it’s communicating with, therefore the A-IoT RAN does not need to report the A-IoT RAN node ID to the A-IoT CN during these A-IoT services related procedures.

Proposal 3: In topology 1, the A-IoT RAN informs the A-IoT RAN node ID to the A-IoT CN during the setup of XX interface.

From the above agreements, we can see that an A-IoT RAN node may serve one or more readers. In last meeting, it was agreed that identifier of the reader may be reported to AIoT CN in the Inventory Report message. Based on the agreement, during the Inventory or the command after inventory procedure, the A-IoT RAN node can include Reader/TRP ID within the Inventory Report message towards the A-IoT CN.

Proposal 4: In topology 1, the A-IoT RAN may include Reader/TRP ID within the Inventory Report message towards the A-IoT CN.

In Topology 2 RRC based solution, the A-IoT-enabled gNB will forward the information received from A-IoT-enabled UE to the A-IoT CN. Based on above agreement, i.e., identifier of the UE reader may be reported to AIoT CN in the Inventory Report message, the A-IoT-enabled gNB can report the UE ID of the A-IoT-enabled UE towards the A-IoT CN via the Inventory Report message.

In Topology 2 NAS/UP based solutions, there is no need for the A-IoT-enabled gNB to provide any location information to the A-IoT CN, as the A-IoT procedures terminated between the A-IoT-enabled UE and the A-IoT CN directly, CN knows which A-IoT-enabled UE the Ambient IoT device accessing from.

Proposal 5: In topology 2 RRC based solution, the A-IoT-enabled gNB includes the Ambient IoT device location information (UE ID of the A-IoT-enabled UE) within the Inventory Report message.

Proposal 6: In topology 2 NAS/UP based solutions, the A-IoT CN knows which A-IoT-enabled UE the device comes from by default.

## 2.3	Finer granularity?

In Editor’s Note 1, there is also another FFS about whether to use more than one “readers” within one A-IoT RAN for location purposes is FFS:

Editor’s Note 1:	How to know the “reader” location is FFS. Whether to use more than one “readers” within one A-IoT RAN for location purposes is FFS.

On the other hand, there are two Editor’s notes related to finer granularity:

Editor’s Note 3:	Introducing finer granularity for locating an Ambient device besides the “reader ID granularity” needs further study.

Editor’s Note 4:	Analysing the gap between the positioning requirements in the SID and the feasibility for A-IoT devices.

In last meeting, the following was made:

The following principle applies to Topology1 and Topology2: While for purpose b), whether the reader ID based granularity of the AIoT device location is sufficient can be further checked.

In the SID [2], the device positiong/locating related objectives are copied as below:

Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). - RAN3-led: - … - Identify potential solutions for locating an Ambient IoT device with no specification impact, e.g. reusing existing user location report, or minimal specification impact to convey location information to core network.

For Editor’s Note 1 and Editor’s Note 3, based on the discussions in previous meetings [3], there are lack of support for the complex solutions such as utilization of more than one “reader” for location purposes and introducing finer granularity for locating an Ambient device. From our view, there is no need to discuss these in the study, as we need to figure out the basic solutions first. On the other hand, based on the SID, reader granularity of the AIoT device location is clearly sufficient.

For Editor’s Note 4, it is clear that the requirement from the SID is to identify solutions with no or minimal specification impact, and it is noted that the existing user location report is in per cell per TAI granularity, in the discussion so far, RAN3 concluded to support reader ID granularity, and the related solutions do not have device impact, therefore, we do not see the gap mentioned in the Editor’s note 4.

Proposal 7: “Use more than one “readers” within one A-IoT RAN for location purposes”, and “Introduce finer granularity for locating an Ambient device besides the reader ID granularity” are not considered in this release.

Proposal 8: reader granularity of the AIoT device location is clearly sufficient based on the SID.

Proposal 9: Remove the Editor’s Notes 1~4.

# 3	Conclusion

- In this contribution, we further discussed the signaling and procedures for Topology 2, and got the following proposals:

- Proposal 1: In topology 1, the A-IoT CN can get the A-IoT RAN’s location (may with per TRP information) based on e.g., OAM configuration.

- Proposal 2: In topology 2, the A-IoT CN is able to get the A-IoT-enabled UE’s location by existing mechanisms.

- Proposal 3: In topology 1, the A-IoT RAN informs the A-IoT RAN node ID to the A-IoT CN during the setup of XX interface.

- Proposal 4: In topology 1, the A-IoT RAN may include Reader/TRP ID within the Inventory Report message towards the A-IoT CN.

- Proposal 5: In topology 2 RRC based solution, the A-IoT-enabled gNB includes the Ambient IoT device location information (UE ID of the A-IoT-enabled UE) within the Inventory Report message.

- Proposal 6: In topology 2 NAS/UP based solutions, the A-IoT CN knows which A-IoT-enabled UE the device comes from by default.

- Proposal 7: “Use more than one “readers” within one A-IoT RAN for location purposes”, and “Introduce finer granularity for locating an Ambient device besides the reader ID granularity” are not considered in this release.

- Proposal 8: reader granularity of the AIoT device location is clearly sufficient based on the SID.

- Proposal 9: Remove the Editor’s Notes 1~4.

# 4. Reference

- [1] R3-247036 (BL pCR to TR 38.769) Study on solutions for Ambient IoT in NR, Huawei, CMCC

- [2] RP-240826 Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA

- [3] R3-244705 Summary of Offline Discussion – CB: # AIoT3_Location, CMCC