3GPP TSG-RAN WG3 Meeting #126	R3-247221

Orlando, US, November 18th – 22nd, 2024

Agenda item:		20.2

Title:		Discussion on AI/ML based Positioning Accuracy 		Enhancements

Source:		NEC

Document for:		Discussion and Decision

- Introduction

The work items pertaining on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface were officially approved in RAN #102 and updated in RAN #105. The details concerning the positioning use case after RAN #105 have been finalized as follows [1].

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

Based on the progress of RAN1 and RAN2, this contribution addresses the following topic:

- Model monitoring for case 3a

- Discussion

2.1 Model monitoring for Case 3a

In the context of model monitoring of case 3a, RAN1 #118 has agreed two options for model performance metric calculation for label-based monitoring, and the final selection of Option A and Option B is pending RAN3 confirmation [2].

##### Agreement @RAN1#118 For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.

Besides, RAN2 #125b meeting [3] has also made the agreement for model performance monitoring of case 3a:

Agreements: 1 RAN2 to consider an RRC configuration to configure radio measurements and the related reporting to enable data collection for NW-side training 2 For AI/ML based beam management, RAN2 assumes the L1 measurement framework shall be used for configuring the input data of the NW side AI/ML model inference. FFS if further enhancements are needed 3 There is no specification impact associated to gNB-side model inference, depending on further RAN1 input. 4 FFS whether there is specification impact associated to gNB-side model monitoring. 5 For POS, RAN2 assumes gNB or LMF could perform performance monitoring for case 3a and LMF is responsible for the performance monitoring for case 3b and wait for any further inputs from other WGs 6 For POS, RAN2 assumes that NRPPa is used for the signalling between gNB and LMF for case 3a and 3b and the detailed signalling design is up to RAN3

Observed from these agreements, it is evident that both Option A and Option B are acceptable to RAN1 and RAN2 for model monitoring of case 3a.

RAN3 #125bis meeting has reached a work assumption that RAN3 will proceed with Option A. The detail is [4]:

Case 3a agreements: - LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design. - How the LMF obtains the ground truth label and related data information is outside of RAN3 scope. - NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). The details of the measurements are still pending and subject to further discussion in RAN1. - WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

It is reasonable to go with Option A, as it does not need to transmit the model output from NR-RAN to LMF. Moreover, if the monitoring decision is made at NG-RAN side (the entity for making monitoring decision is same as the entity that deploy with AI/ML model), Option A show less signal interaction than Option B.

An essential issue for Option A is how to obtain the label before NG-RAN node going to calculate this monitoring metric. Based on RAN1’s previous agreement for training data generation for case 3a, LMF rather than gNB is the entity for generated the label and its related data.

Agreement @RAN1 #116b For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope.

If NG-RAN node is going to performs monitoring metric calculation for label-based model monitoring, a reasonable way is NG-RAN node requires the transmission of ground truth label from LMF to it, as gNB may not be capability to generate the label. Therefore, the way for obtaining the ground truth label at NG-RAN node should be identified before confirming this WA.

Confirm the work assumption that RAN3 will proceed with Option A with providing the way that obtain the ground truth label at NG-RAN node by:

LMF transmit the ground truth label to NG-RAN

For Option B, it needs to transfer the model output from NG-RAN node to LMF, which is already supported in existing NRPPa message. Therefore, supporting Option B may have less specification impact at least from the perspective of monitoring metric calculation. It should be noted that the agreement reached in RAN1 only refer to determine the entity for calculating monitoring metric, but not refer to determine the entity for making monitoring decision, and RAN1 still do not reach an agreement regarding the entity for making monitoring decision.

The agreement reached in RAN1 only refer to determine the entity for calculating monitoring metric, but not refer to determine the entity for making monitoring decision.

Based on this observation, it should not close the door for Option B as it still very likely that LMF will make the final monitoring decision because LMF can control everything for positioning procedure. If RAN1 agree that monitoring decision is made at LMF side, there exist no any specification impact for Option B but exist specification impact for Option A, i.e., transfer of calculated metric.

Figure 1: Procedure for Option A and Option B if LMF make monitoring decision

Option B have less signal interaction between NR-RAN node and LMF if LMF is the entity for making monitoring decision for case 3a.

Therefore, whether support Option B should depend on which entity will make the final monitoring decision.

Wait more discussions about which entity make the monitoring decision for case 3a before determining whether to support Option B.

To send an LS to RAN1 to inform RAN3 agreement on Option A/Option B selection.

- Conclusion

In this contribution, we discussed the issues of AI/ML for positioning accuracy enhancement. Observations and proposals are summarized as following:

Observation 1:	The agreement reached in RAN1 only refer to determine the entity for calculating monitoring metric, but not refer to determine the entity for making monitoring decision.

Observation 2:	Option B have less signal interaction between NR-RAN node and LMF in LMF is the entity for making monitoring decision for case 3a.

Proposal 1:	Confirm the work assumption that RAN3 will proceed with Option A with providing the way that obtain the ground truth label at NG-RAN node by:

−	LMF transmit the ground truth label to NG-RAN

Proposal 2:	Wait more discussions about which entity make the monitoring decision for case 3a before determining whether to support Option B.

Proposal 3:	To send an LS to RAN1 to inform RAN3 agreement on Option A/Option B selection.

# References

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #105.

- Chairman’s notes, 3GPP RAN1 #118.

- Chairman’s notes, 3GPP RAN2 #125b.

- RAN3_125bis_agenda_20241018_1430_EOM, 3GPP RAN3 #125b.

3GPP TSG-RAN WG3 Meeting #126	R3-xxxxxx

Orlando, US, 18th – 22nd, November 2024

Title:	[draft] LS on AI/ML based positioning accuracy enhancement

Response to:	-

Release:	Rel-19

Work Item:	NR_AIML_air-Core

Source:	NEC (to be RAN3)

To:	RAN1

Cc:	RAN2

Contact person:	Hui Ma

hui.ma@emea.nec.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1. Overall Description:

RAN3 has discussed the issue for the selection between Option A and Option B for model performance metric calculation of case 3a according to the following agreement from RAN1.

##### Agreement @RAN1#118 For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.

RAN3 concluded that Option x should be supported in normative work.

# 2. Actions:

To RAN1

ACTION: 	RAN3 kindly ask RAN1 to take the above information into account .

3. Date of Next RAN3 Meetings:

TSG RAN WG3 Meeting #127             17th – 21nd February 2025, Athens, GR.

TSG RAN WG3 Meeting #127bis           7th – 11st February 2025, CN.