3GPP TSG-RAN WG3 Meeting #126	             R3-247245

Orlando, USA, 18th – 22nd Nov 2024

Agenda Item:	8.1

Source:	CMCC

Title:	Discussion on PDU Set Information Marking Support without QoS parameters

Document for:	Discussions & Approval

# Introduction

RAN3 received SA2 LS on PDU Set Information Marking Support without QoS parameters [1], the following questions related to RAN3 were raised.

SA2 would like to update RAN 2 and RAN3 on the latest progress of Rel-19 FS_XRM_Ph2: At the CN side, the PDU Set information marking on the GTP-U header of the downlink packets can be supported, even when the PDU Set QoS parameters are absent. Related to this, SA2 has agreed the following: - If the PDU Set QoS parameters are absent, but the PDU Set Information marking is supported, the SMF sends NG-RAN a new PDU Set Information Marking Support Indicator to indicate that DL PDU Set Information Marking is supported. - Based on the received PDU Set Information Marking Support Indicator from the CN, the NG-RAN sends information to the SMF to activate the PDU Set Information Marking for the PDU Set based handling (e.g., PSI based discarding due to congestion). Based on the NG-RAN provided information, the SMF instructs the PSA UPF to perform PDU Set information marking in the GTP-U header. Q1:SA2 would like RAN2 to provide feedback (e.g. the usefulness) on the PDU Set based handling without PDU set QoS parameters. SA2 would like to understand what information is sent by the NG-RAN to the SMF. Some companies think the NG-RAN can respond with a PDU Set Based Handling Support Indication, to indicate that PDU Set based handling is supported and to activate the PDU Set information marking. Some other companies think the NG-RAN could dynamically request the SMF to activate or deactivate the PDU Set Information marking at any time with an NG-RAN initiated procedure. Q2:SA2 would like RAN3 feedback which approach to support in Rel-19.

# Discussions

As described in the TS 23. 502 [3], in response to PDU Set QoS parameters, NG-RAN includes the PDU Set Based Handling Support Indication in N2 SM information as defined in clause 5.37.5.3 of TS 23.501 [2]:

The SMF, by sending PDU Set QoS parameters to the NG-RAN as described in clause 5.7.7.1, requests the NG-RAN to activate PDU Set QoS handling for a given QoS Flow and the NG-RAN provides the SMF with a PDU Set Based Handling Support Indication if the PDU Set based handling is supported. Based on this indication, SMF may activate the PDU Set identification and marking in the PSA UPF.

Currently, the PDU Set Based Handling Support Indication is sent by the NG-RAN triggered by receiving the PDU Set QoS parameters from AMF/SMF. In the case of PDU Set QoS parameters are absent, it has been agreed in SA2 that: The SMF sends NG-RAN a new PDU Set Information Marking Support Indicator to indicate that DL PDU Set Information Marking is supported.

From RAN perspective, it is useful for the NG-RAN to be aware of the capability of PDU Set Information Marking supported in the UPF, thus the above agreement is supported. After receiving this new PDU Set Information Marking Support Indicator, the NG-RAN can respond with PDU Set Based Handling Support Indication immediately if it wants to proceed with the PDU Set based handling in the RAN side. Or firstly, the NG-RAN acknowledges that it is aware of the capability of PDU Set Information Marking supported in the UPF as a response to the new PDU Set Information Marking Support Indicator.  Afterwards, if it wishes to start or stop the PDU Set based handling, it dynamically requests the SMF to activate/deactivate the PDU Set information marking at the UPF.

In absence of PDU Set QoS parameters, the SMF needs to send a new PDU Set Information Marking Support Indicator to the NG-RAN to make the NG-RAN aware of the capability of PDU Set Information Marking supported in the UPF.

In response to the new PDU Set Information Marking Support Indicator, the NG-RAN can respond with the PDU Set Based Handling Support Indication immediately if it wants to proceed with the PDU Set based handling in the RAN side.

In response to the new PDU Set Information Marking Support Indicator, the NG-RAN can acknowledge its awareness of the capability of PDU Set Information Marking supported in the UPF.

The NG-RAN can dynamically request the SMF to activate/deactivate the PDU Set information marking at the UPF.

# Conclusions

In this submission, we discussed the PDU Set Information Marking Support without QoS parameters. The following proposals are presented:

- In absence of PDU Set QoS parameters, the SMF needs to send a new PDU Set Information Marking Support Indicator to the NG-RAN to make the NG-RAN aware of the capability of PDU Set Information Marking supported in the UPF.

- In response to the new PDU Set Information Marking Support Indicator, the NG-RAN can respond with the PDU Set Based Handling Support Indication immediately if it wants to proceed with the PDU Set based handling in the RAN side.

- In response to the new PDU Set Information Marking Support Indicator, the NG-RAN can acknowledge its awareness of the capability of PDU Set Information Marking supported in the UPF.

- The NG-RAN can dynamically request the SMF to activate/deactivate the PDU Set information marking at the UPF.

# References

R3-247113, LS for PDU Set Information Marking Support without QoS parameters.

﻿System architecture for the 5G System (5GS) (3GPP TS 23.501 version 18.7.0 Release 18).

Procedures for the 5G System (5GS) (3GPP TS 23.502 version 18.7.0 Release 18).