3GPP TSG-RAN WG3 Meeting #126                             R3-247290

Orlando, US, 18th – 22nd, November, 2024

Source:	CATT

Title:	Further discussion on SDT network based solution

Agenda Item:	8.3

Document for:	Discussion and Decision

Introduction

In the Rel-17 SDT, handling of DL non-SDT in case of SDT without anchor relocation is relied on RRCRelease + subsequent paging to trigger another RRC Resume as legacy behaviors designed for RRC_INACTIVE. However, this is not so efficient, in Rel-18, RAN3 further discussed and suggest RAN2 to further consider an optimization to let UE trigger RRC Resume Request automatically after receives RRCRelease.

In the LS [1], RAN2 confirms the Uu-based solution for non-SDT handling in case of SDT with partial context transfer case is supported. Accordingly, RAN3 completed the Uu-based solution and endorsed the corresponding CRs.

Beyond the Uu-based solution, some companies in RAN3 also wonders if feasible to define a network based solution for non-SDT handling in case of SDT with partial context transfer case, which provide another option for the NG-RAN, especially when the UE does not support the Uu-based solution. RAN3 discussed but could not decide whether there’s any extra impact to the UE behaviours, therefore, RAN3 sent an LS to RAN2 [2] to check the NR-Uu impact.

In RAN3#125bis, RAN2 replied the LS in [3]. RAN3 discussed and summarized in the SoD[4]:

No issue on AMF redirecting NG connections (i.e., NG connection from old gNB to new gNB), reuse the legacy procedure. Call flow may need to be checked.

No issue for DL data loss by data forwarding.

The network based solution does not apply to UL data/signaling and DL MBS configuration scenario.

Whether MAC reset is needed for network-based solution needs to be further checked.

In this contribution, we further discuss the feasibility of the network based solution, taking the information of the RAN2 LS into account.

Discussion

In the RAN3#123bis, RAN3 discussed and agreed the Uu-based solution and endorsed the CRs accordingly. And RAN3 discussed a network-based solution, i.e., for UEs not supporting the Uu-based solution as defined by RAN2. However, it’s not clear to RAN3 whether there’s any extra impact to the UE behaviours, that’s why RAN3 sent the LS to RAN2 to further check.

RAN3 would like to thank RAN2 for the reply LS on SDT signalling optimization for partial context transfer (R3-240015 / R2-2313759).

RAN3 has endorsed related CRs for TS 38.300 and TS 38.401, to adopt RAN2 solution based on RRCRelease with resume indication solution.

In addition, RAN3 has discussed a network-based solution, i.e., for UEs not supporting the RAN2 solution above, where the last serving gNB during the ongoing SDT procedure without anchor relocation, upon, e.g., receiving a DL-non SDT data, further relocates the UE context to the receiving gNB by sending an XnAP RETRIEVE UE CONTEXT RESPONSE message, and then the receiving gNB sends the RRCSetup message to the UE (as in TS 38.331, Figure 5.3.13.1-2) with ongoing SDT in RRC_INACTIVE state.

RAN3 assumes that the network-based solution would have impact on Xn signalling only, but would like to confirm with RAN2 whether this network-based solution has any RAN2 impact or issue.

Following the LS, RAN2 discussed the issue a few meetings, not easy to reach the consensus on whether there’s any impact to the UE behaviours, whether there’s any issue to support network based solution. Finally, RAN2 replied the LS to RAN3 in [3]:

RAN2 would like to thank RAN3 for the reply LS on SDT signalling optimization for partial context transfer in R2-2404123 (R3-242198).

With regards to the network-based solution discussed in RAN3, RAN2 would like to inform RAN3 that:

- If RRCSetup is received during SDT, the UE will discard any pending UL data and set up a new RRC Connection. Thus, the pending UL data will be lost.

- If RRCSetup is received during SDT, the UE will discard the current INACTIVE AS context and AS security context which requires RRC establishment related signalling to establish AS context and AS security context again

In addition, RAN2 also discussed whether there is any issue if MAC is not reset upon receiving RRCSetup after data is exchanged during SDT. RAN2 discussed whether the network can handle this scenario without performing a MAC reset by network implementation or if a MAC reset is needed. Some companies in RAN2 are not sure whether network implementation can handle this without MAC reset. RAN2 would like to add that if a MAC reset is needed, this would be a non-backward compatible change to Rel-17 and is not desirable.

In the existing specification, it is not clearly specified how to handle the RRCSetup during SDT transmission procedure.

According to the reply LS [3], RAN2 would like to inform RAN3 that:

- If RRCSetup is received during SDT, the UE will discard any pending UL data and set up a new RRC Connection. Thus, the pending UL data will be lost.

- If RRCSetup is received during SDT, the UE will discard the current INACTIVE AS context and AS security context which requires RRC establishment related signalling to establish AS context and AS security context again

Observation 1: When UE receives RRCSetup during SDT, the UE will discard any pending UL data.

Observation 2: When UE receives RRCSetup during SDT, the UE will discard the current INACTIVE AS context and AS security context which requires RRC establishment related signalling to establish AS context and AS security context again.

The discard UL data could be resent by upper layer after NAS recovery. Discard of pending UL data, reestablishment of the AS security context are same as legacy when UE receives RRCSetup in RRC_INACTIVE, no extra impact to UE behavior.

Proposal 1: For network based solution, no issue for the UE to discard the pending UL data, establish the new AS context and AS security context, just like it receives the RRCSetup in normal inactive state.

Whether MAC reset is needed for network-based solution needs to be further checked.

In the Reply LS [3], RAN2 also pointed out the potential “MAC reset issue”, as below:

In addition, RAN2 also discussed whether there is any issue if MAC is not reset upon receiving RRCSetup after data is exchanged during SDT. RAN2 discussed whether the network can handle this scenario without performing a MAC reset by network implementation or if a MAC reset is needed. Some companies in RAN2 are not sure whether network implementation can handle this without MAC reset. RAN2 would like to add that if a MAC reset is needed, this would be a non-backward compatible change to Rel-17 and is not desirable.

According to existing specification, MAC is not reset when UE receives the RRCSetup in RRC_INACTIVE. Following the legacy UE behavior as above, and not introducing any extra enhancement to NR Uu, MAC is not reset during SDT without anchor relocation case.

Observation 3: MAC is not reset when UE receives RRCSetup in RRC_INACTIVE for normal RRC Resume case.

In the network side, the MAC process is proceed in the same receiving gNB before and after the RRCSetup, we understand it’s possible to continue the MAC processes between the UE and the receiving gNB before and after RRCSetup for SDT without anchor relocation case. Network can handle this without MAC reset, which could be well aligned with the MAC process in the UE.

Proposal 2: Network could continue the MAC processes without MAC reset before and after RRCSetup, which is well aligned with MAC process in the UE.

The network based solution only requires introducing an indicator in the Retrieve UE Context Response, to trigger the receiving gNB make proper behaviours on handling of DL non-SDT data.

Observation 4: Only minor impact to support network based solution, i.e. introduce an indicator in the Retrieve UE Context Response.

As above, only minor specification impact to support the network based solution, and no key issue has been identified from UE and network. It’s proposed to make support of network based solution as an alternative solution on handling of DL non-SDT in case of SDT without anchor relocation.

Corresponding draftCR for TS 38.300 is provided in [5], CR to XnAP is provided in [6]. RAN3 is requested to discuss and agree the CRs on support of SDT network based solution.

Proposal 3: RAN3 is requested to discuss and agree the XnAP CR and Stage 2 draftCR to make support of SDT network based solution.

Conclusion

In this contribution, we further discussed the SDT signalling optimization for partial context transfer according to the RAN2 LS reply. Based on the discussion, the following observations and proposal are provided:

Observation 1: When UE receives RRCSetup during SDT, the UE will discard any pending UL data.

Observation 2: When UE receives RRCSetup during SDT, the UE will discard the current INACTIVE AS context and AS security context which requires RRC establishment related signalling to establish AS context and AS security context again.

Proposal 1: For network based solution, no issue for the UE to discard the pending UL data, establish the new AS context and AS security context, just like it receives the RRCSetup in normal inactive state.

Observation 3: MAC is not reset when UE receives RRCSetup in RRC_INACTIVE for normal RRC Resume case.

Proposal 2: Network could continue the MAC processes without MAC reset before and after RRCSetup, which is well aligned with MAC process in the UE.

Observation 4: Only minor impact to support network based solution, i.e. introduce an indicator in the Retrieve UE Context Response.

Proposal 3: RAN3 is requested to discuss and agree the XnAP CR and Stage 2 draftCR to make support of SDT network based solution.

Reference

- R3-240015 / R2-2313759 Reply LS on SDT signalling optimization for partial context transfer (RAN2 to RAN3)

- R3-242198 Reply LS on SDT signalling optimization for partial context transfer (RAN3 to RAN2)

- R3-245012 Reply LS on SDT signalling optimization for partial context transfer (RAN2 to RAN3)

- R3-245685 Summary of Offline Discussion on CB: # 3_SDTNetworkBasedSolution, ZTE (moderator)

- R3-247291 Support of SDT network based solution (draftCR to TS 38.300), CATT

- R3-247292 Support of SDT network based solution (CR to TS 38.423), CATT