3GPP TSG-RAN WG3 Meeting #126	R3-247314

Orlando, US, 18th – 22nd November 2024

Agenda item:	16.3

Source:	Xiaomi, Huawei, ZTE, CATT, CMCC

Title:	(TP for TR 38.769) Procedures for A-IoT Command

Document for:	Other

# 1	Introduction

In this contribution, a TP including the procedures for A-IoT command related operations within topology 1 and topology 2 (only for RRC-based) is provided based on the progresses of RAN2 and RAN3, and it’s noted that there’s no progress for this topic from SA2 based on their interim conclusion.

According to RAN3’s progress, the A-IoT inventory procedures for Topology 1 and Topology 2 (only for RRC-based solution) are stable and are captured in RAN3 BL pCR R3-245849.

From RAN2 perspectives, the procedures for “inventory and command” and “command only” cases are both captured in RAN2 BL pCR R2-2407984.

Referring RAN2’s latest TP to pCR in R2-2407984, both the “inventory and command” and “command only” can be supported by the procedures with Step A, Step B, Step C1 and Step C2 in below overall AS procedures between A-IoT device and reader. In addition, the “command only” may also be supported with Step A’ and C2, but the feasibility is still FFS. (Details can be found in the Annex)

Figure 1 overall AS procedures between A-IoT device and reader in R2-2407984

Based on the above, we have the following observations and proposed two command procedures for Topology 1 in section 2.

Observation 1, both “inventory and command” and “command only” cases are supported from RAN2 perspective.

Observation 2, for both “inventory and command” and “command only” cases, step A A-IoT paging, step B D2R data transmission and step C data transmission can be used.

Observation 3, for “command only” case, it’s still FFS on whether step A’ A-IoT paging and step C2 possible D2R data transmission can be used, and the feasibility needs to be confirmed by SA3, SA2 and RAN2.

Proposal 1, RAN3 agrees the TP in section 2.

# 2	TP to BL pCR for TR 38.769

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 6.5.2.x1	Candidate procedures for A-IoT Command in Topology 1

Figure 6.5.2.x1-1: Message flow for A-IoT Command in Topology 1

1.	The Inventory procedures are performed in step 1, which are the same as in Figure 6.5.2.1-1 in step 1-4.

NOTE 1: Step 1 shall be performed for “A-IoT Inventory and Command” case, and may not be performed for “Command-only” case, the feasibility of the “Command-only” case is subject to SA2, SA3 and RAN2.

2a.	The A-IoT CN node sends a Command Request message to the A-IoT RAN.

2b.	The A-IoT RAN node allocates and coordinates the usage of A-IoT radio resources.

3.	The A-IoT RAN node performs A-IoT command procedures towards the A-IoT device over the A-IoT radio interface.

NOTE 2: For “A-IoT Inventory and Command” case, the A-IoT command procedures in step 3 can be data transmission procedure, refer to the Step C in Figure 6.3.1-1, the details subject to RAN2.

NOTE 3: For “Command-only” case, the A-IoT command procedures in step 3 can be A-IoT paging, D2R data transmission and data transmission procedures, refer to Step A to Step C in Figure 6.3.1-1, the details subject to RAN2.

4.	The A-IoT RAN node sends a Command Response message to the A-IoT CN, if any command result is received from A-IoT device, the A-IoT RAN node may include the command result in the Command response message.

NOTE 4: In step 4, the A-IoT RAN node may instead send a Command Failure message to the A-IoT CN indicating that the command procedure is failed towards the A-IoT device.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 6.5.3.x1	Candidate procedures for A-IoT Command for Topology 2

##### 6.5.3.x1.1	RRC solution

Figure 6.5.3.x1-1: Message flow for A-IoT Command in Topology 2 (RRC-based solution)

1. The Inventory procedures are performed in step 1, which are the same as in Figure 6.5.3.1-1 in step 1-4.

NOTE 1: Step 1 shall be performed for “A-IoT Inventory and Command” case, and may not be performed for “Command-only” case, the feasibility of the “Command-only” case is subject to SA2, SA3 and RAN2.

2a.	The A-IoT CN sends a Command Request message to the A-IoT enabled gNB.

2b.	The A-IoT enabled gNB allocates and coordinates usage of A-IoT radio resources.

2c.	The A-IoT enabled gNB sends a RRC Command Request message to the A-IoT enabled UE.

Editor’s Note 1:	RRC based communication is only depicted schematically, details in RAN2 FFS.

3.	The A-IoT-enabled UE performs A-IoT command procedures at A-IoT interface towards the A-IoT device over the A-IoT radio interface.

NOTE 2: For “A-IoT Inventory and Command” case, the A-IoT command procedures in step 3 can be data transmission procedure, refer to the Step C in Figure 6.3.1-1, the details subject to RAN2.

NOTE 3: For “Command-only” case, the A-IoT command procedures in step 3 can be A-IoT paging, D2R data transmission and data transmission procedures, refer to Step A to Step C in Figure 6.3.1-1, the details subject to RAN2.

4a.	A-IoT enabled UE sends a Command Response message to A-IoT enabled gNB, if the command result is received from A-IoT device, the A-IoT enabled UE may include the command result in the Command Response message.

4b. The A-IoT enabled gNB sends a Command Response message to the A-IoT CN, it may include the command result in the Command Response message, if any.

NOTE 5: In step 4b, the A-IoT enabled gNB may instead send a Command Failure message to the A-IoT CN indicating that the command procedure is failed towards the A-IoT device.

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>

# 3	Annex (Reference from R2-2407984)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Reference start >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 6.3.1	General aspects and overall procedure

The study aims that the design on the A-IoT radio interface between reader and A-IoT device is common for Topology 1 and Topology 2. The difference of topologies is transparent to the A-IoT devices and has no impact on A-IoT devices. Unless explicitly stated, the descriptions in clause 6.3 apply to all A-IoT device types and both Topology 1 and Topology 2.

Figure 6.3.1-1 Overall AS procedures between A-IoT device and reader

The overall AS procedures can be formulated as:

-	Step A: A-IoT paging. Based on the service request, the reader sends the A-IoT paging message indicating device(s) that need to respond. See clause 6.3.3.

NOTE 1:	In the clause 6.3, the term of “A-IoT paging message” is equal to the “(initial) trigger message”. For simplification, only the former is used.

-	Step B: D2R data (device ID) transmission. Triggered A-IoT device(s) perform the device ID transmission via the A-IoT random access procedure or without using the A-IoT random access procedure. See clause 6.3.4 (and 6.3.5).

-	Step C1: Possible R2D data transmission (e.g., for sending the command). See clause 6.3.5.

-	Step C2: Possible D2R data transmission (e.g., the corresponding response to command). See clause 6.3.5.

Then, above AS procedure can support indoor inventory and indoor command use cases by the following manners:

-	For the detailed use case of “inventory-only”, it is supported by the procedure with Step A and Step B as baseline.

-	For the detailed use case of “inventory and command”, it is supported by the procedure with Step A, Step B, Step C1 and Step C2, as baseline.

NOTE 2:	For the use case of “inventory and command”, it does not imply that the A-IoT paging message includes both the inventory and command and it does not imply the inventory and command are received by the reader at the same time from upper layer.

-	For the detailed use case of “command-only”:

-	It can be also supported by the baseline procedure with Step A, Step B, Step C1 and Step C2.

-	In addition, another candidate to support this use case is following, whose feasibility still depends on the conclusion from [7] and [8]:

-	Step A’: A-IoT paging. Based on the service request, the reader sends the A-IoT paging message including the command, indicating device(s) to process/respond the command.

-	Step C2: Possible D2R data transmission (e.g., the device ID or the corresponding response to command), via the A-IoT random access procedure or without using the A-IoT random access procedure.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Reference end >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>