3GPP TSG-RAN WG3 Meeting #126	R3-247319

Orlando, US, 18th – 22nd November 2024

Agenda item:	20.2

Source:	Xiaomi

Title:	(TP to BL CR for TS 38.455) Support of gNB-side model (case 3a)

Document for:	Other

# 1	Introduction

In this contribution, we’d like to discus the data collection of ground truth label info and performance moniotring for case 3a based on the following progress in RAN3 and provied the corresponding TP in the Annex.

Case 3a: The AI/ML model (inference and training functions) is at the gNB. LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design. How the LMF obtains the ground truth label and related data information is outside of RAN3 scope. NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). The details of the measurements are still pending and subject to further discussion in RAN1. WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model. Check progress in RAN1 and solution discussion…

# 2	Discussion

Figure 1 illustrates the potential deployment scenario for case 3a, the gNB performs AI training, inference and management.

Figure 1 deployment scenario 1 for case 3b (gNB is responsible for management)

## Data collection and correlation

RAN3 agreed that LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. For AI model training, shown in the figure 2, there are multiple data sets to be used as training data input. One data set includes the part A parameters (i.e., channel measurement related info) and the part B parameters (i.e., ground truth label related info), and the part A paraments and part B parameters in one data set should be correlated from UE and timing perspective, considering the part A are from TRP, part B are from LMF, RAN3 needs to discuss how to correlate part A parameters and part B parameters.

Figure 2 AI/ML model training

Regarding how to correlate part A and part B, we think it depends on the how and when the gNB request the LMF for the part B and when gNB generate the part A, there may be two cases:

- Option 1, part A is generated first in legacy positioning, after that, gNB requests LMF to provide the corresponding part B from LMF. gNB has already performed and collected some measurements in legacy NG-RAN assisted positioning procedures, and it wants to use the measurement data for AI model training, then it can request LMF for the specific UE in a specific measurement. It means that the gNB can request part B after part A generation.

- Option 2, gNB request AMF for AI training data collection, LMF initiates the positioning procedures, by sending gNB the measurement request message, and including the corresponding ground truth label info. It means that the part B is obtained by gNB before part A generation.

In both cases, the gNB needs to send request to LMF to obtain part B, RAN3 needs to discuss which message is to be used to carry this request. According to the principle of NRPPa transaction as specified in TS 38.305 as below, only LMF can initiates the NRPPa transaction, which means that the gNB can not initiate NRPPa transaction by itself.

### 7.2.1 NRPPa procedures <<<<<<skip the relevant parts>>>>>> Parallel transactions between the same LMF and NG-RAN node are supported; i.e. a pair of LMF and NG-RAN node may have more than one instance of an NRPPa procedure in execution at the same time. For possible extensibility, the protocol is considered to operate between a generic "access node" (e.g. gNB, ng-eNB) and a "server" (e.g. LMF). An NRPPa transaction is only initiated by the server.

In Option 1, the possible call flow is shown in figure 3, the gNB receives the measurement request from LMF first in legacy way, and then performs the measurements and obtains part A, after that, it includes the request of the ground truth label in the measurement response/report message, the LMF can provide the ground truth label info for that measurement (which correlated with measurement ID) in a new NRPPa transaction, a new NRPPa message may need to be introduced, the whole procedures follow the principle of the NRPPa transaction. In this case, the part A and part B in one data set can be correlated by measurement ID and timestamps for a specific NRPPa transaction.

Figure 3 Option 1 part A generation first and then request part B from LMF

In Option 2, the possible call flow is shown in figure 4, the gNB can request LMF for part B by using the TRP information exchange procedure, when LMF sends the TRP information request to gNB, it indicate the availability of the data collection, and if needed, gNB can include the data collection required in the TRP response message. LMF performs the UE/PRU selection for the data collection requirements and initiates the legacy positioning procedures. The LMF can provide the part B along with the measurement configuration of part A, so that the part A and part B in one data set can be correlated as they are triggered and transferred in a same message.

Figure 4 Option 2 configure part A and obtain part B in same message

Observation 1, part A parameters and part B parameters in one data set should be correlated from UE and timing perspective, to make sure the part A channel measurements are correlated to a right part B ground truth.

Observation 2, gNB needs to request LMF to obtain the part B, but gNB cannot initiate a NRPPa transaction by itself, which means that the gNB’s request should be included in a NRPPa procedural response message.

Proposal 1, RAN3 discuss the following two cases for gNB request part B and correlate part A and part B.

- Option 1, part A generation first and then request part B from LMF, gNB includes the request of part B in measurement response/report message, where the corelated part A is generated in the same measurement. (Potential impacts are in Annex A.)

- Option 2, configure part A and obtain part B in same message, gNB include the request of part B in TRP information response message, LMF includes the part B in a measurement request message, where the measurements of the correlated part A are configured. (Potential impacts are in Annex B.)

## Performance monitoring

WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

According to TR 38.843 [1], it can be noted that Performance monitoring procedure includes performance evaluation and decision-making procedure for AI/ML functionality, it should also be noted that one functionality can include multiple models.

The following procedure can be considered for defining core requirements: - Performance monitoring procedure, including performance evaluation and decision-making procedure for AI/ML functionalities/models - Functionality/Model management procedure, including functionality/model selection/activation/deactivation, and functionality/model switching/fallback/transfer/delivery/update - Latency/interruption requirement for above procedures The following LCM related requirements can be considered: - Model/Functionality select/switch/activate/deactivate/fallback - Model/Functionality monitoring

In our views, performance monitoring includes the following phases:

- phase a, monitoring data collection

- phase b, performance evaluation

- phase c, decision-making

For phase a, according our RAN1 discussion, the monitoring data can include model input data, output and ground truth label shown in below figure.

gNB itself can obtain the input data and output data of the AI inference model, as the model inference is located in gNB, while gNB needs request LMF the ground truth label for performance monitoring, we think the same mechanism as training data collection can be used.

For phase b, the performance metrics are used for performance evaluation, which is not in 3GPP scope, it can be done by implementation.

For phase c, gNB make the decisions according to the result of performance evaluation, the decision can be Model/Functionality select/switch/activate/deactivate/fallback. In our view, gNB can perform the model level management (e.g., switch model 1 to model 3 if model 1’s performance is not good), while LMF can decide whether to use AI functionality (if all the models in gNB has low performance, gNB can indicate LMF).

Based on the above, the following call flows can be used to illustrate the performance monitoring and management. In step 2-3, the LMF initiate the AI positioning functionality, gNB may have multiple AI models for this functionality, gNB can select one of the models for positioning. In step 4, gNB performs the performances monitoring including 3 phases, it may notify LMF the performance result in AI functionality level in step 5 and LMF may decide whether to fallback to legacy positioning in step 6.

Figure 5 performance monitoring call flow

In general, we think AI-based positioning can be treated as one kind of positioning method and can be selected by LMF, according to the LCS requirements.

gNB is responsible for model/functionality LCM performance monitoring and management, gNB can perform model selection/(de)-activation by itself, gNB can also indicate the performance evaluation result to LMF, the quality indictor in the measurement report can be re-used as performance evaluation results.

Observation 3, AI functionality/model performance monitoring includes three phases:

- phase a, monitoring data collection

- phase b, performance evaluation

- phase c, decision-making

Observation 4, performance monitoring data includes input data, output data from gNB, and ground truth label from LMF, where the same mechanism as training data collection can be used.

Observation 5, performance metrics calculation with monitoring data can be gNB implementation.

Proposal 2, turn the WA to agreement: NG-RAN node performs monitoring metric calculation for its own model.

Observation 6, one AI functionality may include one or more AI models.

Proposal 3, NG-RAN deicide model management (e.g., model selection, switch) according to the performance evaluation result by itself.

Proposal 4, RAN3 agree that AI-based positioning functionality can be treated as a positioning method, and LMF decide whether to use AI functionality according to the LCS service requirements.

Proposal 5, RAN3 agree that the gNB can indicate the performance evaluation result to LMF.

## RAN split architecture

If RAN split architecture is supported, we think the AI model training and inference can be deployment in gNB-CU, which is aligned with AI/ML for NG-RAN.

Proposal 6, if RAN split architecture is supported, the AI/Model training, inference and management should be deployed in gNB-CU.

# 3	Conclusion

## Data collection and correlation

Observation 1, part A parameters and part B parameters in one data set should be correlated from UE and timing perspective, to make sure the part A channel measurements are correlated to a right part B ground truth.

Observation 2, gNB needs to request LMF to obtain the part B, but gNB cannot initiate a NRPPa transaction by itself, which means that the gNB’s request should be included in a NRPPa procedural response message.

Proposal 1, RAN3 discuss the following two cases for gNB request part B and correlate part A and part B.

- Option 1, part A generation first and then request part B from LMF, gNB includes the request of part B in measurement response/report message, where the corelated part A is generated in the same measurement. (Potential impacts are in Annex A.)

- Option 2, configure part A and obtain part B in same message, gNB include the request of part B in TRP information response message, LMF includes the part B in a measurement request message, where the measurements of the correlated part A are configured. (Potential impacts are in Annex B.)

## Performance monitoring

Observation 3, AI functionality/model performance monitoring includes three phases:

- phase a, monitoring data collection

- phase b, performance evaluation

- phase c, decision-making

Observation 4, performance monitoring data includes input data, output data from gNB, and ground truth label from LMF, where the same mechanism as training data collection can be used.

Observation 5, performance metrics calculation with monitoring data can be gNB implementation.

Proposal 2, turn the WA to agreement: NG-RAN node performs monitoring metric calculation for its own model.

Observation 6, one AI functionality may include one or more AI models.

Proposal 3, NG-RAN deicide model management (e.g., model selection, switch) according to the performance evaluation result by itself.

Proposal 4, RAN3 agree that AI-based positioning functionality can be treated as a positioning method, and LMF decide whether to use AI functionality according to the LCS service requirements.

Proposal 5, RAN3 agree that the gNB can indicate the performance evaluation result to LMF.

## RAN split architecture

Proposal 6, if RAN split architecture is supported, the AI/Model training, inference and management should be deployed in gNB-CU.

# 4	Reference

[1] 3GPP TR 38.843 V18.0.0 (2023-12), Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

# 5	Annex A	Text Proposal to BL BR (Option 1)

<<<<<<<<<<<<<<<<<<<< Change Start >>>>>>>>>>>>>>>>>>>>

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| OTDOA Information Exchange | OTDOA INFORMATION REQUEST | OTDOA INFORMATION RESPONSE | OTDOA INFORMATION FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Measurement | MEASUREMENT REQUEST | MEASUREMENT RESPONSE | MEASUREMENT FAILURE |
| Positioning Activation | POSITIONING ACTIVATION REQUEST | POSITIONING ACTIVATION RESPONSE | POSITIONING ACTIVATION FAILURE |
| PRS Configuration Exchange | PRS CONFIGURATION REQUEST | PRS CONFIGURATION RESPONSE | PRS CONFIGURATION FAILURE |
| Measurement Preconfiguration | MEASUREMENT PRECONFIGURATION REQUIRED | MEASUREMENT PRECONFIGURATION CONFIRM | MEASUREMENT PRECONFIGURATION REFUSE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Error Indication | ERROR INDICATION |
| Assistance Information Control | ASSISTANCE INFORMATION CONTROL |
| Assistance Information Feedback | ASSISTANCE INFORMATION FEEDBACK |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Measurement Report | MEASUREMENT REPORT |
| Measurement Update | MEASUREMENT UPDATE |
| Measurement Abort | MEASUREMENT ABORT |
| Measurement Failure Indication | MEASUREMENT FAILURE INDICATION |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| Measurement Activation | MEASUREMENT ACTIVATION |
| SRS Information Reservation Notification | SRS INFORMATION RESERVATION NOTIFICATION |
| Data Collection Information | DATA COLLECTION INFORMATION |

<<<<<<<<<<<<<<<<<<<< Next Change>>>>>>>>>>>>>>>>>>>>

### 8.5.1	Measurement

#### 8.5.1.1	General

The Measurement procedure allows the LMF to request one or more TRPs in the NG-RAN node to perform and report positioning measurements. This procedure applies only if the NG-RAN node is a gNB.

#### 8.5.1.2	Successful Operation

Figure 8.5.1.2.1: Measurement procedure. Successful operation.

The LMF initiates the procedure by sending a MEASUREMENT REQUEST message to the NG-RAN node, indicating in the TRP Measurement Request List IE the TRP(s) from which measurements are requested. The NG-RAN node shall use the included information to configure positioning measurements by the indicated TRP(s). If at least one of the requested measurements has been successful for at least one of the TRPs, the NG-RAN node shall reply with a MEASUREMENT RESPONSE message including the TRP Measurement Response List IE.

If the Report Characteristics IE is set to "OnDemand", the NG-RAN node shall return the corresponding measurement results in the MEASUREMENT RESPONSE message, and the LMF shall consider that this reporting has been terminated by the NG-RAN node. If the Report Characteristics IE is set to "Periodic", the NG-RAN node shall initiate the corresponding measurements, and it shall reply with the MEASUREMENT RESPONSE message without including any measurement results in the message. The NG-RAN node shall then periodically initiate the Measurement Report procedure for the corresponding measurements, with the requested reporting periodicity.

If the Measurement Beam Information Request IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall include the Measurement Beam Information IE in the TRP Measurement Result IE of the MEASUREMENT RESPONSE message.

If the Measurement Quality IE is included in the TRP Measurement Result IE in the MEASUREMENT RESPONSE message, the LMF may take it into account as the TRP estimate of the measurement quality. If the Measurement Quality IE includes the Zenith Quality IE, the LMF may take it into account within the angle measurement quality.

If the Timing Reporting Granularity Factor IE is included in the TRP Measurement Quantities IE in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account when configuring measurements including UL RTOA and gNB Rx-Tx Time Difference.

If the System Frame Number IE and/or the Slot Number IE are included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, consider that the respective information indicates the activation time of SRS transmission.

If the Report Characteristics IE is set to "OnDemand" and the Response Time IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, return the corresponding measurement results in the MEASUREMENT RESPONSE message within the indicated time.

If the Measurement Characteristics Request Indicator IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, take the requested measurement characteristics into account when configuring measurements, and include the requested information, if available, in the MEASUREMENT RESPONSE message.

If the Number of TRP Rx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP Rx TEGs for the indicated TRP, and report the corresponding UL-RTOA and/or gNB Rx-Tx time difference measurements.

If the Number of TRP RxTx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP RxTx TEGs with the same TRP Tx TEG for the indicated TRP, and report the corresponding gNB Rx-Tx time difference measurements.

If the Measurement Time Occasion IE is included in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account as the number of SRS measurement time occasions for a measurement instance.

If the Time Window Information Measurement List IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, measure the UL SRS resources from the UE within the indicated time window(s).

Interaction with the Measurement Report procedure:

If the Report Characteristics IE is set to "Periodic" and the Measurement Amount IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, take it into account for sending the MEASUREMENT REPORT message.

Interaction with the Data Collection Information procedure:

If the Data Collection Required IE is included in the MEASUREMENT RESPONSE message, the LMF shall, if supported, provide the requested data associated with the measurements for sending the DATA COLLECTION INFORMATION message.

<<<<<<<<<<<<<<<<<<<< Next Change>>>>>>>>>>>>>>>>>>>>

### 8.5.2	Measurement Report

#### 8.5.2.1	General

The Measurement Report procedure allows the NG-RAN node to report positioning measurements to the LMF. This procedure applies only if the NG-RAN node is a gNB.

#### 8.5.2.2	Successful Operation

Figure 8.5.2.2.1: Measurement Report procedure. Successful operation.

The NG-RAN node initiates the procedure by sending a MEASUREMENT REPORT message to the LMF. The MEASUREMENT REPORT message contains the measurement results according to the associated measurement configuration.

Interaction with the Data Collection Information procedure:

If the Data Collection Required IE is included in the MEASUREMENT REPORT message, the LMF shall, if supported, provide the requested data associated with the measurements for sending the DATA COLLECTION INFORMATION message.

<<<<<<<<<<<<<<<<<<<< Next Change>>>>>>>>>>>>>>>>>>>>

### 8.5.x	Data Collection Information

#### 8.5.x.1	General

The Data Collection Information procedure allows the LMF to provide the NG-RAN node the required data for gNB-side model training and performance monitoring. This procedure applies only if the NG-RAN node is a gNB.

#### 8.5.x.2	Successful Operation

Figure 8.5.x.2.1: Data Collection Information procedure. Successful operation.

Upon reception of the DATA COLLECTION INFORMATION message, the NG-RAN Node can use the provided data for gNB-side model training and performance monitoring.

#### 8.5.x.3	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change>>>>>>>>>>>>>>>>>>>>

#### 9.1.4.2	MEASUREMENT RESPONSE

This message is sent by the NG-RAN node to report positioning measurements.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Response List |  | 0..1 |  |  | YES | reject |
| >TRP Measurement Response Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>TRP Measurement Result | M |  | 9.2.37 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |
| Data Collection Required | O |  | ENUMERATED(ground truth label, …, FFS) |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change>>>>>>>>>>>>>>>>>>>>

#### 9.1.4.4	MEASUREMENT REPORT

This message is sent by the NG-RAN node to report positioning measurements for the target UE.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Response List |  | 1 |  |  | YES | reject |
| >TRP Measurement Response Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>TRP Measurement Result | M |  | 9.2.37 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| Data Collection Required | O |  | ENUMERATED(ground truth label, …, FFS) |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change>>>>>>>>>>>>>>>>>>>>

#### 9.1.4.X	DATA COLLECTION INFORMATION

This message is sent by the LMF to provide the required data for gNB-side model training and performance monitoring.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| >>Ground Truth Label Information | O |  | FFS |  |  |  |

<<<<<<<<<<<<<<<<<<<< Change End >>>>>>>>>>>>>>>>>>>>

# 4	Annex B	Text Proposal to BL BR (Option 2)

<<<<<<<<<<<<<<<<<<<< Change Start >>>>>>>>>>>>>>>>>>>>

### 8.2.8	TRP Information Exchange

#### 8.2.8.1	General

The purpose of the TRP Information Exchange procedure is to allow the LMF to request the NG-RAN node to provide detailed information for TRPs hosted by the NG-RAN node. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.8.2	Successful Operation

Figure 8.2.8.2-1: TRP Information Exchange procedure, successful operation

The LMF initiates the procedure by sending a TRP INFORMATION REQUEST message. The NG-RAN node responds with a TRP INFORMATION RESPONSE message that contains the requested TRP information.

If the TRP List IE is included in the TRP INFORMATION REQUEST message, the NG-RAN node should include in the TRP INFORMATION RESPONSE message, the requested information for all TRPs included in the TRP List IE.

If the TRP List IE is not included in the TRP INFORMATION REQUEST message, the NG-RAN node should include the requested information for all TRPs hosted by the NG-RAN node in the TRP INFORMATION RESPONSE message

If the PRS Muting IE is included in the PRS Configuration IE in the TRP INFORMATION RESPONSE message, the LMF may take it into account as the muting information for the given PRS resource set.

If the QCL Info IE is included in the PRS Configuration IE in the TRP INFORMATION RESPONSE message, the LMF may take it into account for the given PRS resource list.

If the DL-PRS Resource Coordinates IE is included in the Geographical Coordinates IE in the TRP Information IE in the TRP INFORMATION RESPONSE message, the LMF may take it into account as the DL PRS Resource Coordinates relative to the TRP coordinate.

If the Mobile IAB-MT UE ID IE is included in the TRP Information IE in the TRP INFORMATION RESPONSE message, the LMF shall, if supported, use this information to determine an updated location of the Mobile TRP as specified in TS 23.273 [20].

If the TRP Information Type Item IE is set to 'mobile trp location info', the NG-RAN node shall, if supported, derive the location of the Mobile TRP as specified in TS 23.273 [20] and include the Mobile TRP Location Information in the TRP INFORMATION RESPONSE message.

If the Data Collection Available Indication IE is included in the TRP INFORMATION REQUEST message, the NG-RAN node should include the Data Collection Required IE in the TRP INFORMATION RESPONSE message to indicate LMF whether it wants to collect data.

<<<<<<<<<<<<<<<<<<<< Next Change>>>>>>>>>>>>>>>>>>>>

#### 9.1.1.14	TRP INFORMATION REQUEST

This message is sent by an LMF to request information for TRPs hosted by an NG-RAN node.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| TRP List |  | 0 ..1 |  |  | YES | ignore |
| >TRP Item |  | 1 .. <maxnoTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>PRS Bandwidth Aggregation Request Indication | O |  | ENUMERATED(true, …) |  | YES | ignore |
| TRP Information Type List |  | 1 |  |  | YES | reject |
| >TRP Information Type Item |  | 1 .. <maxnoTRPInfoTypes> |  |  | EACH | reject |
| >>TRP Information Type Item | M |  | ENUMERATED (nr pci, ng-ran cgi, nr arfcn, prs config, ssb config, sfn init time, spatial direction info, geo-coordinates, …, trp type, on-demand prs, trp tx teg, beam antenna info, mobile trp location info, common ta) |  | - |  |
| Data Collection Available Indication | O |  | ENUMERATED(ground truth label, …, FFS) |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change>>>>>>>>>>>>>>>>>>>>

#### 9.1.1.15	TRP INFORMATION RESPONSE

This message is sent by an NG-RAN node to convey TRP information to an LMF.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| TRP Information List |  | 1 |  |  | YES | ignore |
| >TRP Information Item | M | 1 .. <maxnoTRPs> |  |  | - |  |
| >>TRP Information | M |  | 9.2.25 |  | - |  |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |
| Data Collection Required | O |  | ENUMERATED(ground truth label, …, FFS) |  | YES | ignore |

Editor note: FFS on whether gNB provide more information for data collection requirements.

<<<<<<<<<<<<<<<<<<<< Next Change>>>>>>>>>>>>>>>>>>>>

## 8.5	Measurement Information Transfer

### 8.5.1	Measurement

#### 8.5.1.1	General

The Measurement procedure allows the LMF to request one or more TRPs in the NG-RAN node to perform and report positioning measurements. This procedure applies only if the NG-RAN node is a gNB.

#### 8.5.1.2	Successful Operation

Figure 8.5.1.2.1: Measurement procedure. Successful operation.

The LMF initiates the procedure by sending a MEASUREMENT REQUEST message to the NG-RAN node, indicating in the TRP Measurement Request List IE the TRP(s) from which measurements are requested. The NG-RAN node shall use the included information to configure positioning measurements by the indicated TRP(s). If at least one of the requested measurements has been successful for at least one of the TRPs, the NG-RAN node shall reply with a MEASUREMENT RESPONSE message including the TRP Measurement Response List IE.

If the Report Characteristics IE is set to "OnDemand", the NG-RAN node shall return the corresponding measurement results in the MEASUREMENT RESPONSE message, and the LMF shall consider that this reporting has been terminated by the NG-RAN node. If the Report Characteristics IE is set to "Periodic", the NG-RAN node shall initiate the corresponding measurements, and it shall reply with the MEASUREMENT RESPONSE message without including any measurement results in the message. The NG-RAN node shall then periodically initiate the Measurement Report procedure for the corresponding measurements, with the requested reporting periodicity.

If the Measurement Beam Information Request IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall include the Measurement Beam Information IE in the TRP Measurement Result IE of the MEASUREMENT RESPONSE message.

If the Measurement Quality IE is included in the TRP Measurement Result IE in the MEASUREMENT RESPONSE message, the LMF may take it into account as the TRP estimate of the measurement quality. If the Measurement Quality IE includes the Zenith Quality IE, the LMF may take it into account within the angle measurement quality.

If the Timing Reporting Granularity Factor IE is included in the TRP Measurement Quantities IE in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account when configuring measurements including UL RTOA and gNB Rx-Tx Time Difference.

If the System Frame Number IE and/or the Slot Number IE are included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, consider that the respective information indicates the activation time of SRS transmission.

If the Report Characteristics IE is set to "OnDemand" and the Response Time IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, return the corresponding measurement results in the MEASUREMENT RESPONSE message within the indicated time.

If the Measurement Characteristics Request Indicator IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, take the requested measurement characteristics into account when configuring measurements, and include the requested information, if available, in the MEASUREMENT RESPONSE message.

If the Number of TRP Rx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP Rx TEGs for the indicated TRP, and report the corresponding UL-RTOA and/or gNB Rx-Tx time difference measurements.

If the Number of TRP RxTx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP RxTx TEGs with the same TRP Tx TEG for the indicated TRP, and report the corresponding gNB Rx-Tx time difference measurements.

If the Measurement Time Occasion IE is included in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account as the number of SRS measurement time occasions for a measurement instance.

If the Time Window Information Measurement List IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, measure the UL SRS resources from the UE within the indicated time window(s).

If the Label Information IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use the information for gNB-side model training.

<<<<<<<<<<<<<<<<<<<< Next Change>>>>>>>>>>>>>>>>>>>>

### 8.5.x	Measurement Data Collection Indication

#### 8.5.x.1	General

The Measurement Data Collection Indication procedure allows the NG-RAN node to notify the LMF Measurement Data collection is required for gNB-side model training. This procedure applies only if the NG-RAN node is a gNB.

#### 8.5.x.2	Successful Operation

Figure 8.5.x.2.1: Measurement Data Collection Indication procedure. Successful operation.

Upon reception of the MEASUREMENT DATA COLLECTION INDICATION message, the LMF shall select UEs with consent to initiate the UL related positioning measurement for gNB-side model training.

#### 8.5.x.3	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change>>>>>>>>>>>>>>>>>>>>

#### 9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.2.66 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms, extended) | The codepoint 120ms, 240ms, 480ms, 1024ms, 2048ms, 1min, 6min, 12min, 30min, and 60min are not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >>TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…, Multiple UL-AoA, UL SRS-RSRPP, UL-RSCP) | The UL-RSCP measurement is applicable only when the UL-RTOA and/or gNB-RxTxTimeDiff measurement(s) is also requested. | - |  |
| >>Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16]. This IE is ignored when the Timing Reporting Granularity Factor Extended IE is included. | - |  |
| >>Timing Reporting Granularity Factor Extended | O |  | INTEGER (-6..-1, …) | Value -6 corresponds to kminus6, value -5 corresponds to kminus5 and so on, see TS 38.133 [16] | - |  |
| >>Ground Truth Label Information | O |  | FFS |  |  |  |

<<<<<<<<<<<<<<<<<<<< Change End >>>>>>>>>>>>>>>>>>>>