3GPP TSG-RAN WG3 Meeting #126	R3-247322

Orlando, US, 18 – 22 Nov, 2024

Agenda Item:	10.4

Source:	Nokia

Title:	MHI/UHI Enhancement for SCG Deactivation/Activation

Document for:	Discussion

# 1 Introduction

In this Tdoc with discuss some aspects of MHI/UHI enhancements with SCG Activation/de-activation time.

# 2 Discussion

### 2.1 Incoming LS from RAN2

RAN2 sent the following LS [1] to RAN3:

RAN2 agreed that it is beneficial for the network to have information about time spent in the PSCell in activated state vs. deactivated state:

RAN2#127: It is beneficial for the network to have information about time spent in the PSCell in activated state vs. deactivated state.

For how to collect the time spent information, there are following two options:

Option 1: Network based solution: The network collects the information about time spent in the PSCell in activated or deactivated state.

Option 2: UE based solution: The UE includes the information about time spent in the PSCell in activated vs deactivated state in MHI (Mobility History Information).

RAN2 kindly asks RAN3 to analyse whether Network based solution is sufficient enough for network optimization, or whether a UE based solution is needed.

This topic was discussed during the last RAN3 #125bis meeting but there was no consensus between the companies.

### 2.2 SCG activation and deactivation

The mechanism for SCG activation and deactivation is introduced in Rel-17 to enable reasonable UE battery consumption while having fast usage of SCG when (NG)EN-DC or NR-DC is configured. While the SCG is deactivated, there is no transmission via SCG RLC bearers. Only the NR SCG can be deactivated, and all SCG SCell(s) are in deactivated state while the SCG is deactivated.

SCG activation/deactivation can be triggered by UL or DL traffic on Data Radio Bearers (DRBs) with an SCG Radio Link Control (RLC) bearer (split or SCG bearer)  by MN, SN or UE based on some activation/deactivation condition that is no clearly specified in 37.340 , In the  following picture shows an example of how this could work assuming the activation/deactivation condition is based on a traffic threshold.

The concept of SCG activation and deactivation is most efficient for traffic where bursts of data large enough to benefit from SCG transmission are separated by periods of lower activity, during which the SCG can be deactivated. As the energy consumption in SCG active state is high, the criterion deciding upon SCG activation and deactivation shall be carefully selected. The simplest way is a UE traffic threshold where periods with data traffic above the threshold correspond to SCG activated state, while periods with data traffic below the threshold correspond to SCG deactivated state. The threshold may be in principle configured for each UE or group of UEs with similar traffic profile. In case the threshold is set as too conservatively low it may mean that SCG activated period contains also bursts of data which are not large enough to benefit from SCG transmission, hence leading to unnecessary power consumption. In case the threshold is set as too aggressively high it may mean that not all bursts of data which are large enough benefit from SCG transmission which may lead to QoS degradation from end user perspective, though energy consumption may be reduced in this case.

Currently, the UHI and MHI for SCG include no information about SCG activation and deactivation. Despite majority of companies consider the information about the time of SCG activation and/or deactivation could help the network understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity in our understanding, due to possible different setting of traffic thresholds between the PSCells of different nodes this information may be insufficient. The detailed information about relative time in SCG activated state may therefore need to be extended with some more info. For instance, if the relative time in SCG activated state is quite low, it means that the Dual Connectivity is possibly not so necessary for the UE, and the network might need not configure DC for the UE, to reduce the UE’s battery consumption and the signaling overhead of DC configuration at network side. However, such decision may not be optimal if the traffic threshold for the PSCells(s) reported within the MHI is set as too aggressively high. On the other hand, if the relative time in SCG activated state is quite high, it possibly means that the Dual Connectivity is highly demanded, and the MN could configure and/or activate the SCG more efficiently, which again may be not optimal if traffic threshold is set as too conservatively low.

Observation 1: The information of SCG activation and/or deactivation, e.g., the relative time in SCG activated state together with some more assistance information related to the condition(s) for SCG activation/deactivation, could help the network understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity.

### 2.3 Network-based solution and UE-based solution

The main argument to have a UE-based solution based on MHI is that the NW-based solution (the UHI stored) is lost upon RRC state transitions. It is due to the NW cannot maintain the PSCell info because it removes all the UE context while UE leaves connected state. Therefore, some of the companies think, the network-based solution is insufficient for the optimization of dual connectivity configuration, and the UE-based solution is needed. We tend to agree with this conclusion however in relation to MHI/UHI enhancement with the information about the time of SCG activation and/or deactivation to help the network understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity in our understanding, due to possible different setting of traffic thresholds between the PSCells of different nodes, this information is insufficient as described in previous chapter.

Proposal 1: It is proposed to include in MHI the information of SCG activation/deactivation, e.g., the ratio of time in SCG activated state together with some more assistance information related to the condition (s) for SCG activation/deactivation.

Proposal 2: RAN3 to send LS to RAN2 asking for the implementation of UE based solution including assistance information related to the condition(s) for SCG activation/deactivation

# 3 Conclusion

In this contribution the following observation and proposal are made:

Observation 1: The information of SCG activation and/or deactivation, e.g., the relative time in SCG activated state together with some more assistance information related to  related to the condition(s) for SCG activation/deactivation, could help the network understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity.

Proposal 1: It is proposed to include in MHI the information of SCG activation/deactivation, e.g., the ratio of time in SCG activated state together with some more  related to the condition(s) for SCG activation/deactivation.

Proposal 2: RAN3 to send LS to RAN2 asking for the implementation of UE based solution including assistance information related to the condition(s) for SCG activation/deactivation

# Reference

[1]	R3-245009, RAN2 LS on MHI enhancement solution for SCG deactivation/activation