3GPP TSG-RAN WG3 Meeting #126	R3-247365

Orlando, USA, 18 - 22 November, 2024

Agenda Item:	12.3

Source:	Huawei

Title:	(TP for TS 38.300/38.413) Architecture and functional split for NR Femto

Document for:	Discussion

1	Introduction

In last meeting RAN3 achieved the following agreements for the NR Femto [1], and agreed the TP for Femto architecture and protocol stack [2].

Apply the following changes in the text below:

For NR Femto, the NG-C interface is defined as the interface:

-	Between the NR Femto GW and the Core Network;

-	Between the NR Femto and the NR Femto GW;

-	Between the NR Femto and the Core Network;

Add a note for Figure 4.X.3.2-1, stating that it is FFS whether the figure is needed.

Add an editor´s note stating that the following sentence will be captured in details in the functional split section:

“Any protocol function associated to a UE-dedicated-procedure shall reside within the NR Femto and the AMF only.”

Correct the sentence as follows:

“Any protocol function associated to a UE-dedicatedassociated-procedure shall reside within the NR Femto and the AMF only.”

Define an acronym for SeGW.

Correct the following figure caption as follows:

Figure 4.X.3.2-1: Control plane for NG-C Interface for NR Femto to AMF without the NR Femto GW

Figure 4.X.3.2-2: Control plane for NG-C Interface for NR Femto to AMF with the NR Femto GW

WA: an NR Femto may have more than one cell.

This contribution further discusses the architecture, functional split, node ID, and the mobility issues for NR Femto.

2	Discussion

2.1 UP protocol stack

It was captured in the agreed BL CR that “based on implementation, the transport of NG-U between the NR Femto and the 5GC may be optionally concentrated in the NR Femto GW.” The NR Femto GW does not handle the GTP-U layer, so there is no difference for the UP protocol stack between the cases with the GW and without the GW. We can add in the BL CR for TS 38.300 the user plane protocol stack as shown in Fig. 1.

Fig 1. User plane protocol stack in NR Femto

- RAN3 to capture the UP protocol stack for NR Femto.

2.2 Functional split

The functional split of NR Femto, NR Femto GW, and AMF can generally refer to the functional split in HeNB. The NR Femto GW relays UE-associated messages between the Femto node and AMF, and terminates non-UE-associated messages towards the Femto node and towards the AMF.

The stage 3 impact introduced by the functional split is summarized as follows:

- Assistance information for AMF selection/determination in INITIAL UE MESSAGE and PATH SWITCH REQUEST.

Since the NR Femto node is not directly connected to the AMF if connect via NR femto GW, the UE’s AMF should be selected by the Femto GW but not Femto node. So, the Femto GW can be provided with some assistance information for the AMF selection. Specifically, if the UE has already registered and the GUAMI and GUAMI Type are included in the RRCSetupComplete, the Femto node should add the GUAMI and GUAMI Type into INITIAL UE MESSAGE. Otherwise if the UE has not registered but indicate the requested S-NSSAI in RRCSetupComplete, the requested S-NSSAI should be included in the INITIAL UE MESSAGE. These information is needed for the INITIAL UE MESSAGE from the Femto node to the Femto GW, but not needed for the INITIAL UE MESSAGE from the Femto GW to the AMF.

In case of Xn based handover, if the target gNB is a Femto node, the source AMF’s GUAMI is needed in the PATH SWITCH REQUEST, for the Femto GW serving the target Femto node to determine the source AMF to forward the PATH SWITCH REQUEST message. Similarly the source AMF’s GUAMI is needed from the Femto node to the Femto GW, but not needed from the Femto GW to the AMF.

Since the S-NSSAI is newly introduced in 5G compared with 4G, it is particularly proposed:

- The NR Femto node forwards the UE’s requested S-NSSAI to the NR Femto GW in INITIAL UE MESSAGE. The NR Femto GW may select a proper AMF accordingly.

- AMF UE NGAP ID assigned by AMF in INITIAL CONTEXT SETUP REQUEST, HANDOVER REQUEST and PATH SWITCH REQUEST ACK.

Similar as LTE HeNB, when a UE initially accesses the Femto node or is handed over to the Femto node, the UE NGAP ID allocated by the UE’s serving AMF needs to be included in the INITIAL CONTEXT SETUP REQUEST (for initial access), HANDOVER REQUEST (for NG handover) and PATH SWITCH REQUEST ACK (for Xn handover), for the subsequent Xn handover. Specifically, for the subsequent Xn handover, the NR Femto GW will indicate the AMF UE NGAP ID to the target Femto node, so that the target Femto node can further include that in the PATH SWITCH REQUEST message, for the AMF to identify the UE.

- AMF ID indication in OVERLOAD START and OVERLOAD STOP.

The GUAMI List is included in the OVERLOAD START and OVERLOAD STOP, for indicating the Femto node the overloaded AMF IDs.

- GW Context Release Indication in UE CONTEXT RELEASE REQUEST

Similar as LTE HeNB, the GW Context Release Indication should be included in the UE CONTEXT RELEASE REQUEST. This IE can be used by the source NR femto node to indicates to the NR femto GW that the UE has been successfully handed over to a target NG-RAN node. With such indication, if the NR femto GW determines that the UE is no longer served by an NR femto node attached to it, it terminates the UE Context Release Request procedure and releases the UE context.

We provide the detailed functional split description in the TPs for TS 38.300 and 38.413.

- RAN3 to agree the TP for TS 38.300 and 38.413 to capture spec impact introduced by the functional split for NR Femto.

We also observe that in TS 36.300 the HeNB GW is responsible for some determination and verification, such as the HeNB identity verification, CSG mode determination and verification, and CSG ID verification. The Access control related part will be particularly discussed in our paper [3]. Genrally the verification of gNB ID in NG SETUP REQUEST, PWS RESTART INDICATION and PWS FAILURE INDICATION is implemented by the AMF. If the Femto GW is considered, the Femto node’s NG interface is terminated to the Femto GW, and it is straightforward the NR Femto GW verifies the Femto node ID. That needs to be checked with SA3 for confirmation.

- RAN3 assumes NR Femto GW verifies the Femto node ID in NG SETUP REQUEST, PWS RESTART INDICATION and PWS FAILURE INDICATION, and check with SA3 for confirmation.

2.3 gNB ID for NR femto node

In the last meeting we have a WA that “WA: an NR Femto may have more than one cell”, as a normal gNB, which is different from the LTE HeNB. In TS 36.300, it is captured “one HeNB serves only one cell.” And we also observed that in TS 36.413, the HeNB ID is represented as the ECGI of the cell served by the HeNB.

---Start of excerpt from TS 36.413---

#### 9.2.1.37	Global eNB ID

This information element is used to globally identify an eNB (see TS 36.401 [2]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.2.3.8 |  |
| CHOICE eNB ID | M |  |  |  |
| >Macro eNB ID |  |  |  |  |
| >>Macro eNB ID | M |  | BIT STRING (SIZE(20)) | Equal to the 20 leftmost bits of the Cell Identity IE contained in the E-UTRAN CGI IE (see subclause 9.2.1.38) of each cell served by the eNB. |
| >Home eNB ID |  |  |  |  |
| >>Home eNB ID | M |  | BIT STRING (SIZE(28)) | Equal to the Cell Identity IE contained in the E-UTRAN CGI IE (see subclause 9.2.1.38) of the cell served by the eNB. |
| >Short Macro eNB ID |  |  |  |  |
| >> Short Macro eNB ID | M |  | BIT STRING (SIZE(18)) | Equal to the 18 leftmost bits of the Cell Identity IE (see subclause 9.2.1.38) of each cell served by the eNB. |
| >Long Macro eNB ID |  |  |  |  |
| >> Long Macro eNB ID | M |  | BIT STRING (SIZE(21)) | Equal to the 21 leftmost bits of the Cell Identity IE (see subclause 9.2.1.38) of each cell served by the eNB. |

---End of excerpt from TS 36.413---

---Start of excerpt from TS 38.413---

9.3.1.6	Global gNB ID

This IE is used to globally identify a gNB (see TS 38.300 [8]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.3.3.5 |  |
| CHOICE gNB ID | M |  |  |  |
| >gNB ID |  |  |  |  |
| >>gNB ID | M |  | BIT STRING (SIZE(22..32)) | Equal to the leftmost bits of the NR Cell Identity IE contained in the NR CGI IE of each cell served by the gNB. |

---End of excerpt from TS 38.413---

- One HeNB serves only one cell, and the HeNB ID is the ECGI of the cell served by the HeNB.

Now, the NR Femto node may serve more than one cells. It is unreasonable to still use the NCGI of a served cell as the Femto ID. And for stage 3, we need to discuss how to identify the NR femto node. A straightforward solution is just to reuse the global gNB ID for NR Femto node, considering the length of the Global gNB ID is 22-32bits , there may be a risk/concern that the range cannot support the large number of NR femto nodes. Alternatively, if the scalability is a valid issue, dedicated NR Femto node ID may be also considered.

It is proposed:

- RAN3 to investigate how to design the gNB ID for NR femto node, i.e., whether to introduce dedicated NR Femto Node ID, or reuse the Global gNB ID.

2.4 NG mobility impact

It is noted in the 36.300 that “if routing ambiguities are to be avoided, a TAI used in a HeNB GW should not be reused in another HeNB GW”, which means only one HeNB GW can be deployed in a TA. During NG handover, if the target eNB is a HeNB, the MME sends the HANDOVER REQUEST message to the HeNB GW in the UE’s TA.

Techinically such mechanism can be resued in NR Femto, but that depends on whether we still want to restrict the implementation that only one Femto GW can be deployed in a TA. In our view, such restriction is too strong, and considering the coverage of gNB may be smaller than eNB, more Femto nodes may be deployed in a TA causing heavy workload to the Femto GW. Therefore, RAN3 need to discuss how to find the correct Femto GW during NG handover, i.e., how the AMF knows the association of the Femto GW and its corresponding Femto node.

- RAN3 to discuss how the AMF knows the association of the Femto GW and the Femto node, or to agree at most one Femto GW can be deployed in a TA.

3	Conclusion

This paper focus on the architecture, functional split, Femto ID, and NG mobility in the NR femto, and provide the RAN3 to agree the TP for TS 38.300 to capture the UP protocol stack for NR Femto.

Issue of architecture

- RAN3 to agree the TP for TS 38.300 to capture the UP protocol stack for NR Femto.

Issue of functional split

- The NR Femto node forwards the UE’s requested S-NSSAI to the NR Femto GW in INITIAL UE MESSAGE. The NR Femto GW selects proper AMF accordingly.

- RAN3 to agree the TP for TS 38.300 and 38.413 to capture spec impact introduced by the functional split for NR Femto.

- RAN3 assumes NR Femto GW verifies the Femto node ID in NG SETUP REQUEST, PWS RESTART INDICATION and PWS FAILURE INDICATION, and check with SA3 for confirmation.

Issue of Femto ID

- One HeNB serves only one cell, and the HeNB ID is the ECGI of the cell served by the HeNB.

- RAN3 to investigate how to design the gNB ID for NR femto node, i.e., whether to introduce dedicated NR Femto Node ID, or reuse the Global gNB ID.

Issue of NG mobility impact

- RAN3 to discuss how the AMF knows the association of the Femto GW and the Femto node, or to agree at most one Femto GW can be deployed in a TA.

4	Reference

- RAN3 #125b chairman notes.

- R3-245754, TP for BL CR on Introduction of NR Femto Architecture and Protocol Aspects, Nokia, TMO US, AT&T, Verizon Wireless, BT, Charter, Huawei, LG Electronics, Samsung, Lenovo, Baicells, RAN3 #125b.

- R3-247366, (TP for TS 38.300) Access control for NR Femto, Huawei, RAN3 #126.

Annex 1——TP for TS 38.300

Start of Change

4	Overall Architecture and Functional Split

## 4.X	Support of NR Femtos

### 4.X.1	Architecture

Figure 4.X.1-1 shows a logical architecture for the NR Femto that has a set of NG interfaces to connect the NR Femto to the 5GC.

Figure 4.X.1-1: NR Femto Logical Architecture

NOTE: the SeGW is out of RAN scope.

The NR Femto may directly connect to the 5GC. The NG-RAN architecture may also deploy an NR Femto Gateway (NR Femto GW) to allow the concentration of the NG-C interface between the NR Femto and the 5GC. Based on implementation, the transport of NG-U between the NR Femto and the 5GC may be optionally concentrated in the NR Femto GW.

For NR Femto, the NG-C interface is defined as the interface:

-	Between the NR Femto GW and the Core Network;

-	Between the NR Femto and the NR Femto GW;

-	Between the NR Femto and the Core Network;

The NR Femto GW appears to the AMF as a gNB. The NR Femto GW appears to the NR Femto as an AMF. The NG interface between the NR Femto and the 5GC is the same regardless whether the NR Femto is connected to the 5GC via an NR Femto GW or not.

The functions supported by the NR Femto shall be the same as those supported by a gNB (with possible exceptions e.g. NNSF when the NR Femto connects via the NR Femto GW) and the procedures run between an NR Femto and the 5GC shall be the same as those between a gNB and the 5GC.

Xn-connectivity is supported between NR Femtos and between NR Femtos and gNBs, independent of whether any of the involved NR Femtos is connected to an NR Femto GW.

<<<<<<<<<<<<<<<<<<<< Start of changes >>>>>>>>>>>>>>>>>>>>

4.X.2	Functional Split

A NR Femto hosts the same functions as a gNB as described in clause 4.2, with the following additional specifics in case of connection to the NR Femto GW:

-	Discovery of a suitable Serving NR Femto GW;

-	A NR Femto shall only connect to a single NR Femto GW at one time, namely no NG Flex function shall be used at the NR Femto:

-	The NR Femto will not simultaneously connect to another NR Femto GW, or another AMF.

-	The TAC and PLMN ID used by the NR Femto shall also be supported by the NR Femto GW;

-	Selection of an AMF at UE attachment is hosted by the NR Femto GW instead of the NR Femto. Upon reception of the GUAMI from a UE, the NR Femto shall include it in the INITIAL UE MESSAGE message; upon reception of the GUAMI Type from the UE, the NR Femto shall also include it in the message if supported and supported by the NR Femto GW.

-	NR Femtos may be deployed without network planning. A NR Femto may be moved from one geographical area to another and therefore it may need to connect to different NR Femto GWs depending on its location;

-	Signalling the GUAMI of the Source AMF to the NR Femto GW in the PATH SWITCH REQUEST message.

The NR Femto GW hosts the following functions:

-	Relaying UE-associated NGAP messages between the AMF serving the UE and the NR Femto, except the UE CONTEXT RELEASE REQUEST message received from the NR Femto with an explicit GW Context Release Indication. In that case, the NR Femto GW terminates the NG UE Context Release Request procedure and releases the UE context if it determines that the UE identified by the received UE NGAP IDs is no longer served by an NR Femto attached to it. Otherwise it ignores the message.

-	In case of INITIAL CONTEXT SETUP REQUEST message and NG HANDOVER REQUEST message, informing the NR Femto about any GUAMI corresponding to the serving AMF, the AMF UE NGAP ID assigned by the AMF and the AMF UE NGAP ID assigned by the NR Femto GW for the UE. In case of PATH SWITCH REQUEST ACKNOWLEDGE message, informing the NR Femto about the AMF UE NGAP ID assigned by the AMF and the AMF UE NGAP ID assigned by the NR Femto GW for the UE.

-	Terminating non-UE associated NG application part procedures towards the NR Femto and towards the AMF.

Editor’s Note: Whether the NR Femto GW verifies the identity of NR Femto in case of NG SETUP REQUEST message, PWS RESTART INDICATION message and PWS FAILURE INDICATION message is pending SA3 confirmation.

-	Upon receiving an OVERLOAD START/STOP message, the NR Femto GW should send the OVERLOAD START/STOP message towards the NR Femto(s) including in the message the identities of the affected AMF node. The NR Femto uses this information received from the OVERLOAD START message to identify to which traffic the above defined rejections shall be applied. The NR Femto shall apply the defined rejections until reception of an OVERLOAD STOP message applicable to this traffic, or until the NR Femto receives a further OVERLOAD START message applicable to the same traffic, in which case it shall replace the ongoing overload action with the newly requested one.

NOTE:	If a NR Femto GW is deployed, non-UE associated procedures shall be run between NR Femtos and the NR Femto GW and between the NR Femto GW and the AMF.

-	Optionally transport NG-U interface with the NR Femto and with the UPF.

-	Supporting TAC and PLMN ID used by the NR Femto.

-	Xn interfaces shall not be established between the NR Femto GW and other nodes.

-	Routing the NG PATH SWITCH REQUEST message towards the AMF based on the GUAMI of the source AMF received from the NR Femto.

<<<<<<<<<<<<<<<<<<< next change >>>>>>>>>>>>>>>>>>>>

4.X.3	Interfaces

4.X.3.1	Protocol Stack for NG User Plane

The figure below show the NG-U protocol stack between NR Femto and UPF.

Figure 4.X.3.1-1: User plane for NG Interface between NR Femto and UPF

4.X.3.2	Protocol Stacks for NG Control Plane

The two figures below show the NG-C protocol stack with and without the NR Femto GW.

When the NR Femto GW is not present (Fig. 4.x.3.2-1), all the NGAP procedures are terminated at the NR Femto and the AMF.

Figure 4.X.3.2-1: Control plane for NG Interface for NR Femto to AMF without the NR Femto GW

Figure 4.X.3.2-2: Control plane for NG Interface for NR Femto to AMF with the NR Femto GW

End of Change

Annex 2——TP for TS 38.413

Start of Change

8.3.1.2	Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the PNI-NPN Area Scope of MDT IE is included in the MDT Configuration-NR IE included in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall, if supported, use it to derive the MDT area scope for MDT measurement collection in PNI-NPN areas. Upon reception of the PNI-NPN Area Scope of MDT IE, the NG-RAN node shall consider that the area scope for MDT measurement collection in PNI-NPN areas is defined only by the areas included in the PNI-NPN Area Scope of MDT IE.

If the AMF UE NGAP ID 2 IE is contained in the INITIAL CONTEXT SETUP REQUEST message, the gNB shall, if supported, store this information in the UE context and use it for subsequent Xn handovers.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

8.4.2.2	Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Partially Allowed NSSAI IE is contained in the HANDOVER REQUEST message, the NG-RAN node shall, if supported, deduce from it the partially allowed network slices for the UE, store and replace any previously received Partially Allowed NSSAI and use it as specified in TS 23.501 [9].

If the MBS Support Indicator IE is included in the Handover Request Acknowledge Transfer IE in the HANDOVER REQUEST ACKNOWLEDGE message, the SMF shall, if supported, handle this information as specified in TS 23.247 [44].

The AMF UE NGAP ID 2 IE shall only be contained in the HANDOVER REQUEST message according to subclause 4.X.2 of TS 38.300 [8].If the AMF UE NGAP ID 2 IE is present, the target gNB shall store this information in the UE context and use it for subsequent Xn handovers.

If the ECN Marking or Congestion Information Reporting Status IE is included in the Handover Request Acknowledge Transfer IE, the SMF shall, if supported, use it to deduce if ECN marking at NG-RAN or ECN marking at UPF or congestion information reporting is active or not active as described in TS 23.501 [9].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

8.4.4.2	Successful Operation

Figure 8.4.4.2-1: Path switch request: successful operation

The NG-RAN node initiates the procedure by sending the PATH SWITCH REQUEST message to the AMF. Upon reception of the PATH SWITCH REQUEST message the AMF shall, for each PDU session indicated in the PDU Session ID IE, transparently transfer the Path Switch Request Transfer IE to the SMF associated with the concerned PDU session.

Upon reception of the PATH SWITCH REQUEST message the AMF shall deactivate MT communication handling if already activated, as specified in TS 23.502 [10].

If the RRC Resume Cause IE is included in the PATH SWITCH REQUEST message, the AMF shall, if supported, use it as described in TS 23.502 [10] for User Plane CIoT 5GS Optimisation when the NG-RAN node is an ng-eNB.

If the RedCap Indication IE or the eRedCap Indication IE is included in the PATH SWITCH REQUEST message, the AMF shall, if supported, consider the UE respectively as a RedCap UE or an eRedCap UE that was previously served by a E-UTRA cell, and use the IE according to TS 23.501 [9].

After all necessary updates including the UP path switch have been successfully completed in the 5GC for at least one of the PDU session resources included in the PATH SWITCH REQUEST, the AMF shall send the PATH SWITCH REQUEST ACKNOWLEDGE message to the NG-RAN node and the procedure ends.

The list of accepted QoS flows shall be included in the PATH SWITCH REQUEST message within the Path Switch Request Transfer IE. The SMF shall handle this information as specified in TS 23.502 [10].

If the GUAMI of the AMF currently serving the UE is available at the gNB (see TS 38.300 [8]) the gNB shall include the Source AMF GUAMI IE within the PATH SWITCH REQUEST message.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Ranging and Sidelink Positioning Service Information IE is contained in the PATH SWITCH REQUEST ACKNOWLEDGE message, the NG-RAN node shall, if supported, update the Ranging and Sidelink Positioning service information for the UE accordingly. If the Ranging and Sidelink Positioning Authorized IE within the Ranging and Sidelink Positioning Service Information IE is set to "not authorized", the NG-RAN node shall, if supported, initiate actions to ensure that the UE is no longer accessing the Ranging and Sidelink Positioning service.

If the AMF UE NGAP ID 2 IE is contained in the PATH SWITCH REQUEST ACKNOWLEDGE message, the gNB shall store this information in the UE context and use it for subsequent Xn handovers.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

8.6.1.2	Successful Operation

Figure 8.6.1.2-1: Initial UE message

The NG-RAN node initiates the procedure by sending an INITIAL UE MESSAGE message to the AMF. The NG-RAN node shall allocate a unique RAN UE NGAP ID to be used for the UE and the NG-RAN node shall include this identity in the INITIAL UE MESSAGE message.

The NAS-PDU IE contains a UE – AMF message that is transferred without interpretation in the NG-RAN node.

In case of network sharing, the selected PLMN is indicated by the PLMN Identity IE within the TAI IE included in the INITIAL UE MESSAGE message, or by the Serving PLMN IE within the NR NTN TAI Information IE included in the same message for NTN.

When the NG-RAN node has received from the radio interface the 5G-S-TMSI IE, it shall include it in the INITIAL UE MESSAGE message.

If the AMF Set ID IE is included in the INITIAL UE MESSAGE message this indicates that the message is a rerouted message and the AMF shall, if supported, use the IE as described in TS 23.502 [10].

If the gNB does not support NNSF and the gNB has received from the radio interface the GUAMI IE, the gNB may include it in the INITIAL UE MESSAGE message. If the gNB does not support NNSF and the gNB has received from the radio interface the GUAMI Type IE, the gNB may include it in the INITIAL UE MESSAGE message. If the gNB does not support NNSF and the gNB has received from the radio interface the Requested S-NSSAI IE, the gNB may include it in the INITIAL UE MESSAGE message.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

8.7.7.2	Successful Operation

Figure 8.7.7.2-1: Overload start

The NG-RAN node receiving the OVERLOAD START message shall assume the AMF from which it receives the message as being in an overloaded state.

If the Overload Action IE is included the AMF Overload Response IE within the OVERLOAD START message, the NG-RAN node shall use it to identify the related signalling traffic. When the Overload Action IE is set to

-	"reject RRC connection establishments for non-emergency mobile originated data transfer" (i.e., reject traffic corresponding to RRC cause "mo-data", "mo-SMS", "mo-VideoCall" and "mo-VoiceCall" in TS 38.331 [18] or "mo-data" and "mo-VoiceCall" in TS 36.331 [21]), or

-	"reject RRC connection establishments for signalling" (i.e., reject traffic corresponding to RRC cause "mo-data", "mo-SMS", "mo-signalling", "mo-VideoCall" and "mo-VoiceCall" in TS 38.331 [18] or "mo-data", "mo-signalling" and "mo-VoiceCall" in TS 36.331 [21]), or

-	"only permit RRC connection establishments for emergency sessions and mobile terminated services" (i.e., only permit traffic corresponding to RRC cause "emergency" and "mt-Access" in TS 38.331 [18] or in TS 36.331 [21]), or

-	"only permit RRC connection establishments for high priority sessions and mobile terminated services" (i.e., only permit traffic corresponding to RRC cause "highPriorityAccess", "mps-PriorityAccess", "mcs-PriorityAccess" and "mt-Access" in TS 38.331 [18] or "highPriorityAccess", "mo-ExceptionData" and "mt-Access" in TS 36.331 [21]),

the NG-RAN node shall:

-	if the AMF Traffic Load Reduction Indication IE is included in the OVERLOAD START message, reduce the signalling traffic by the indicated percentage,

-	otherwise ensure that only the signalling traffic not indicated as to be rejected is sent to the AMF.

If the Overload Start NSSAI List IE is included in the OVERLOAD START message, the NG-RAN node shall:

-	if the Slice Traffic Load Reduction Indication IE is present, reduce the signalling traffic by the indicated percentage for the UE(s) whose requested NSSAI only include S-NSSAI(s) contained in the Overload Start NSSAI List IE, and the signalling traffic indicated as to be reduced by the Overload Action IE in the Slice Overload Response IE if the IE is present,

-	otherwise ensure that only the signalling traffic from UE(s) whose requested NSSAI includes S-NSSAI(s) other than the ones contained in the Overload Start NSSAI List IE, or the signalling traffic not indicated as to be reduced by the Overload Action IE in the Slice Overload Response IE for the UE(s) if the requested NSSAI matched, is sent to the AMF.

If the GUAMI List IE is present, the gNB shall, if supported, use this information to identify to which traffic the above defined rejections shall be applied.

If an overload action is ongoing and the gNB receives a further OVERLOAD START message, the gNB shall replace the ongoing overload action with the newly requested one. If the GUAMI List IE is present, the gNB replaces applicable ongoing actions according to TS 38.300 [8], clause 4.X.2.

If an overload control is ongoing and the NG-RAN node receives a further OVERLOAD START message, the NG-RAN node shall replace the contents of the previously received information with the new one.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

8.7.8.2	Successful Operation

Figure 8.7.8.2-1: Overload stop

The NG-RAN node receiving the OVERLOAD STOP message shall assume that the overload situation at the AMF from which it receives the message has ended and shall resume normal operation for the applicable traffic towards this AMF.

If the GUAMI List IE is present, the gNB shall, if supported, use this information to identify which traffic to cease rejecting, and proceed according to TS 38.300 [8], clauses 4.X.2. If no particular overload action is ongoing for a particular GUAMI value, the gNB shall ignore this value.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

9.2.2.1	INITIAL CONTEXT SETUP REQUEST

This message is sent by the AMF to request the setup of a UE context.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| Old AMF | O |  | AMF Name 9.3.3.21 |  | YES | reject |
| UE Aggregate Maximum Bit Rate | C-ifPDUsessionResourceSetup |  | 9.3.1.58 |  | YES | reject |
| Core Network Assistance Information for RRC INACTIVE | O |  | 9.3.1.15 |  | YES | ignore |
| GUAMI | M |  | 9.3.3.3 |  | YES | reject |
| PDU Session Resource Setup Request List |  | 0..1 |  |  | YES | reject |
| >PDU Session Resource Setup Request Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>PDU Session NAS-PDU | O |  | NAS-PDU 9.3.3.4 |  | - |  |
| >>S-NSSAI | M |  | 9.3.1.24 |  | - |  |
| >>PDU Session Resource Setup Request Transfer | M |  | OCTET STRING | Containing the PDU Session Resource Setup Request Transfer IE specified in subclause 9.3.4.1. | - |  |
| >>PDU Session Expected UE Activity Behaviour | O |  | Expected UE Activity Behaviour 9.3.1.94 | Expected UE Activity Behaviour for the PDU Session. | YES | ignore |
| Allowed NSSAI | M |  | 9.3.1.31 | Indicates the S-NSSAIs permitted by the network | YES | reject |
| UE Security Capabilities | M |  | 9.3.1.86 |  | YES | reject |
| Security Key | M |  | 9.3.1.87 |  | YES | reject |
| Trace Activation | O |  | 9.3.1.14 |  | YES | ignore |
| Mobility Restriction List | O |  | 9.3.1.85 |  | YES | ignore |
| UE Radio Capability | O |  | 9.3.1.74 |  | YES | ignore |
| Index to RAT/Frequency Selection Priority | O |  | 9.3.1.61 |  | YES | ignore |
| Masked IMEISV | O |  | 9.3.1.54 |  | YES | ignore |
| NAS-PDU | O |  | 9.3.3.4 |  | YES | ignore |
| Emergency Fallback Indicator | O |  | 9.3.1.26 |  | YES | reject |
| RRC Inactive Transition Report Request | O |  | 9.3.1.91 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.3.1.68 |  | YES | ignore |
| Redirection for Voice EPS Fallback | O |  | 9.3.1.116 |  | YES | ignore |
| Location Reporting Request Type | O |  | 9.3.1.65 |  | YES | ignore |
| CN Assisted RAN Parameters Tuning | O |  | 9.3.1.119 |  | YES | ignore |
| SRVCC Operation Possible | O |  | 9.3.1.128 |  | YES | ignore |
| IAB Authorized | O |  | 9.3.1.129 |  | YES | ignore |
| Enhanced Coverage Restriction | O |  | 9.3.1.140 |  | YES | ignore |
| Extended Connected Time | O |  | 9.3.3.31 |  | YES | ignore |
| UE Differentiation Information | O |  | 9.3.1.144 |  | YES | ignore |
| NR V2X Services Authorized | O |  | 9.3.1.146 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.3.1.147 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.148 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.149 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| PC5 QoS Parameters | O |  | 9.3.1.150 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| CE-mode-B Restricted | O |  | 9.3.1.155 |  | YES | ignore |
| UE User Plane CIoT Support Indicator | O |  | 9.3.1.160 |  | YES | ignore |
| RG Level Wireline Access Characteristics | O |  | OCTET STRING | Specified in TS 23.316 [34]. Indicates the wireline access technology specific QoS information corresponding to a specific wireline access subscription. | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.168 |  | YES | ignore |
| UE Radio Capability ID | O |  | 9.3.1.142 |  | YES | reject |
| Time Synchronisation Assistance Information | O |  | 9.3.1.220 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.3.1.223 |  | YES | ignore |
| Target NSSAI Information | O |  | 9.3.1.229 |  | YES | ignore |
| UE Slice Maximum Bit Rate List | O |  | 9.3.1.231 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.3.1.233 |  | YES | ignore |
| 5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.148 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.3.1.234 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| Network Controlled Repeater Authorized | O |  | 9.3.1.245 |  | YES | ignore |
| Aerial UE Subscription Information | O |  | 9.3.1.246 |  | YES | ignore |
| NR A2X Services Authorized | O |  | 9.3.1.247 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.248 |  | YES | ignore |
| NR A2X UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.148 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE A2X UE PC5 Aggregate Maximum Bit Rate | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.149 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| A2X PC5 QoS Parameters | O |  | 9.3.1.249 | This IE applies only if the UE is authorized for A2X services. | YES | ignore |
| Mobile IAB Authorized | O |  | 9.3.1.259 |  | YES | ignore |
| Partially Allowed NSSAI | O |  | 9.3.1.261 | Indicates the S-NSSAIs partially permitted by the network. | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.269 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| AMF UE NGAP ID 2 | O |  | AMF UE NGAP ID 9.3.3.1 | This IE indicates the AMF UE NGAP ID assigned by the AMF. | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

9.2.2.4	UE CONTEXT RELEASE REQUEST

This message is sent by the NG-RAN node to request the release of the UE-associated logical NG-connection over the NG interface.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| PDU Session Resource List |  | 0..1 |  |  | YES | reject |
| >PDU Session Resource Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| GW Context Release Indication | O |  | 9.3.1.x |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

9.2.3.4	HANDOVER REQUEST

This message is sent by the AMF to the target NG-RAN node to request the preparation of resources.

Direction: AMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| Handover Type | M |  | 9.3.1.22 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| UE Aggregate Maximum Bit Rate | M |  | 9.3.1.58 |  | YES | reject |
| Core Network Assistance Information for RRC INACTIVE | O |  | 9.3.1.15 |  | YES | ignore |
| UE Security Capabilities | M |  | 9.3.1.86 |  | YES | reject |
| Security Context | M |  | 9.3.1.88 |  | YES | reject |
| New Security Context Indicator | O |  | 9.3.1.55 |  | YES | reject |
| NASC | O |  | NAS-PDU 9.3.3.4 | Refers to either the “Intra N1 mode NAS transparent container” or the “S1 mode to N1 mode NAS transparent container”, the details of the IE definition and the encoding arespecified in TS 24.501 [26]. | YES | reject |
| PDU Session Resource Setup List |  | 1 |  |  | YES | reject |
| >PDU Session Resource Setup Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>S-NSSAI | M |  | 9.3.1.24 |  | - |  |
| >>Handover Request Transfer | M |  | OCTET STRING | Containing the PDU Session Resource Setup Request Transfer IE specified in subclause 9.3.4.1. | - |  |
| >>PDU Session Expected UE Activity Behaviour | O |  | Expected UE Activity Behaviour 9.3.1.94 | Expected UE Activity Behaviour for the PDU Session. | YES | ignore |
| Allowed NSSAI | M |  | 9.3.1.31 | Indicates the S-NSSAIs permitted by the network. | YES | reject |
| Trace Activation | O |  | 9.3.1.14 |  | YES | ignore |
| Masked IMEISV | O |  | 9.3.1.54 |  | YES | ignore |
| Source to Target Transparent Container | M |  | 9.3.1.20 |  | YES | reject |
| Mobility Restriction List | O |  | 9.3.1.85 |  | YES | ignore |
| Location Reporting Request Type | O |  | 9.3.1.65 |  | YES | ignore |
| RRC Inactive Transition Report Request | O |  | 9.3.1.91 |  | YES | ignore |
| GUAMI | M |  | 9.3.3.3 |  | YES | reject |
| Redirection for Voice EPS Fallback | O |  | 9.3.1.116 |  | YES | ignore |
| CN Assisted RAN Parameters Tuning | O |  | 9.3.1.119 |  | YES | ignore |
| SRVCC Operation Possible | O |  | 9.3.1.128 |  | YES | ignore |
| IAB Authorized | O |  | 9.3.1.129 |  | YES | reject |
| Enhanced Coverage Restriction | O |  | 9.3.1.140 |  | YES | ignore |
| UE Differentiation Information | O |  | 9.3.1.144 |  | YES | ignore |
| NR V2X Services Authorized | O |  | 9.3.1.146 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.3.1.147 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.148 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.149 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| PC5 QoS Parameters | O |  | 9.3.1.150 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| CE-mode-B Restricted | O |  | 9.3.1.155 |  | YES | ignore |
| UE User Plane CIoT Support Indicator | O |  | 9.3.1.160 |  | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.168 |  | YES | ignore |
| UE Radio Capability ID | O |  | 9.3.1.142 |  | YES | reject |
| Extended Connected Time | O |  | 9.3.3.31 |  | YES | ignore |
| Time Synchronisation Assistance Information | O |  | 9.3.1.220 |  | YES | ignore |
| UE Slice Maximum Bit Rate List | O |  | 9.3.1.231 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.3.1.233 |  | YES | ignore |
| 5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.148 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.3.1.234 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| Aerial UE Subscription Information | O |  | 9.3.1.246 |  | YES | ignore |
| NR A2X Services Authorized | O |  | 9.3.1.247 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.248 |  | YES | ignore |
| NR A2X UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.148 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE A2X UE PC5 Aggregate Maximum Bit Rate | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.149 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| A2X PC5 QoS Parameters | O |  | 9.3.1.249 | This IE applies only if the UE is authorized for A2X services. | YES | ignore |
| Mobile IAB Authorized | O |  | 9.3.1.259 |  | YES | ignore |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is a mobile IAB-MT. | YES | ignore |
| Partially Allowed NSSAI | O |  | 9.3.1.261 | Indicates the S-NSSAIs partially permitted by the network. | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.269 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| AMF UE NGAP ID 2 | O |  | AMF UE NGAP ID 9.3.3.1 | This IE indicates the AMF UE NGAP ID assigned by the AMF. | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

9.2.3.8	PATH SWITCH REQUEST

This message is sent by the NG-RAN node to inform the AMF of the new serving NG-RAN node and to transfer some NG-U DL tunnel termination point(s) to the SMF via the AMF for one or multiple PDU session resources.

Direction: NG-RAN node  AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| Source AMF UE NGAP ID | M |  | AMF UE NGAP ID 9.3.3.1 |  | YES | reject |
| User Location Information | M |  | 9.3.1.16 |  | YES | ignore |
| UE Security Capabilities | M |  | 9.3.1.86 |  | YES | ignore |
| PDU Session Resource to be Switched in Downlink List |  | 1 |  |  | YES | reject |
| >PDU Session Resource to be Switched in Downlink Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>Path Switch Request Transfer | M |  | OCTET STRING | Containing the Path Switch Request Transfer IE specified in subclause 9.3.4.8. | - |  |
| PDU Session Resource Failed to Setup List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resource Failed to Setup Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>Path Switch Request Setup Failed Transfer | M |  | OCTET STRING | Containing the Path Switch Request Setup Failed Transfer IE specified in subclause 9.3.4.15. | - |  |
| RRC Resume Cause | O |  | RRC Establishment Cause 9.3.1.111 |  | YES | ignore |
| RedCap Indication | O |  | 9.3.1.228 |  | YES | ignore |
| eRedCap Indication | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| Source AMF GUAMI | O |  | GUAMI 9.3.3.3 | This IE indicates the source AMF. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

9.2.3.9	PATH SWITCH REQUEST ACKNOWLEDGE

This message is sent by the AMF to inform the NG-RAN node that the path switch has been successfully completed in the 5GC.

Direction: AMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | ignore |
| UE Security Capabilities | O |  | 9.3.1.86 |  | YES | reject |
| Security Context | M |  | 9.3.1.88 |  | YES | reject |
| New Security Context Indicator | O |  | 9.3.1.55 |  | YES | reject |
| PDU Session Resource Switched List |  | 1 |  |  | YES | ignore |
| >PDU Session Resource Switched Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>Path Switch Request Acknowledge Transfer | M |  | OCTET STRING | Containing the Path Switch Request Acknowledge Transfer IE specified in subclause 9.3.4.9. | - |  |
| >>PDU Session Expected UE Activity Behaviour | O |  | Expected UE Activity Behaviour 9.3.1.94 | Expected UE Activity Behaviour for the PDU Session. | YES | ignore |
| PDU Session Resource Released List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resource Released Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>Path Switch Request Unsuccessful Transfer | M |  | OCTET STRING | Containing the Path Switch Request Unsuccessful Transfer IE specified in subclause 9.3.4.20. | - |  |
| Allowed NSSAI | M |  | 9.3.1.31 | Indicates the S-NSSAIs permitted by the network. | YES | reject |
| Core Network Assistance Information for RRC INACTIVE | O |  | 9.3.1.15 |  | YES | ignore |
| RRC Inactive Transition Report Request | O |  | 9.3.1.91 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Redirection for Voice EPS Fallback | O |  | 9.3.1.116 |  | YES | ignore |
| CN Assisted RAN Parameters Tuning | O |  | 9.3.1.119 |  | YES | ignore |
| SRVCC Operation Possible | O |  | 9.3.1.128 |  | YES | ignore |
| Enhanced Coverage Restriction | O |  | 9.3.1.140 |  | YES | ignore |
| Extended Connected Time | O |  | 9.3.3.31 |  | YES | ignore |
| UE Differentiation Information | O |  | 9.3.1.144 |  | YES | ignore |
| NR V2X Services Authorized | O |  | 9.3.1.146 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.3.1.147 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.148 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.149 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| PC5 QoS Parameters | O |  | 9.3.1.150 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| CE-mode-B Restricted | O |  | 9.3.1.155 |  | YES | ignore |
| UE User Plane CIoT Support Indicator | O |  | 9.3.1.160 |  | YES | ignore |
| UE Radio Capability ID | O |  | 9.3.1.142 |  | YES | reject |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.168 | This IE is ignored if the Management Based MDT PLMN Modification List IE is present. | YES | ignore |
| Time Synchronisation Assistance Information | O |  | 9.3.1.220 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.3.1.233 |  | YES | ignore |
| 5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.148 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.3.1.234 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| Management Based MDT PLMN Modification List | O |  | MDT PLMN Modification List 9.3.1.243 |  | YES | ignore |
| IAB Authorized | O |  | 9.3.1.129 |  | YES | ignore |
| Aerial UE Subscription Information | O |  | 9.3.1.246 |  | YES | ignore |
| NR A2X Services Authorized | O |  | 9.3.1.247 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.248 |  | YES | ignore |
| NR A2X UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.148 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE A2X UE PC5 Aggregate Maximum Bit Rate | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.149 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| A2X PC5 QoS Parameters | O |  | 9.3.1.249 | This IE applies only if the UE is authorized for A2X services. | YES | ignore |
| Mobile IAB Authorized | O |  | 9.3.1.259 |  | YES | ignore |
| Partially Allowed NSSAI | O |  | 9.3.1.261 | Indicates the S-NSSAIs partially permitted by the network. | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.269 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| AMF UE NGAP ID 2 | O |  | AMF UE NGAP ID 9.3.3.1 | This IE indicates the AMF UE NGAP ID assigned by the AMF. | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

9.2.5.1	INITIAL UE MESSAGE

This message is sent by the NG-RAN node to transfer the initial layer 3 message to the AMF over the NG interface.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| NAS-PDU | M |  | 9.3.3.4 |  | YES | reject |
| User Location Information | M |  | 9.3.1.16 |  | YES | reject |
| RRC Establishment Cause | M |  | 9.3.1.111 |  | YES | ignore |
| 5G-S-TMSI | O |  | 9.3.3.20 |  | YES | reject |
| AMF Set ID | O |  | 9.3.3.12 |  | YES | ignore |
| UE Context Request | O |  | ENUMERATED (requested, ...) |  | YES | ignore |
| Allowed NSSAI | O |  | 9.3.1.31 |  | YES | reject |
| Source to Target AMF Information Reroute | O |  | 9.3.3.27 |  | YES | ignore |
| Selected PLMN Identity | O |  | PLMN Identity 9.3.3.5 | Indicates the selected PLMN id for the non-3GPP access. | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) | Indication of an IAB node | YES | reject |
| CE-mode-B Support Indicator | O |  | 9.3.1.156 |  | YES | reject |
| LTE-M Indication | O |  | 9.3.1.157 |  | YES | ignore |
| EDT Session | O |  | ENUMERATED (true, …) |  | YES | ignore |
| Authenticated Indication | O |  | ENUMERATED (true, …) | Indicates the FN-RG has been authenticated by the access network. | YES | ignore |
| NPN Access Information | O |  | 9.3.3.46 |  | YES | reject |
| RedCap Indication | O |  | 9.3.1.228 |  | YES | ignore |
| Selected NID | O |  | NID 9.3.3.42 | Indicates together with the Selected PLMN Identity IE, the selected SNPN Identity for the non-3GPP access. | YES | ignore |
| Mobile IAB Node Indication | O |  | ENUMERATED (true, ...) | Indicates that the UE is a mobile IAB-node | YES | reject |
| Partially Allowed NSSAI | O |  | 9.3.1.261 | Indicates the S-NSSAIs partially permitted by the network. | YES | ignore |
| eRedCap Indication | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| GUAMI | O |  | GUAMI 9.3.3.3 |  | YES | ignore |
| GUAMI Type | O |  | ENUMERATED (native, mapped, …) |  | YES | ignore |
| Requested S-NSSAI | O |  | 9.3.1.y | Indicates the S-NSSAIs requested by the UE. | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

9.2.6.14	OVERLOAD START

This message is sent by the AMF and is used to indicate to the NG-RAN node that the AMF is overloaded.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF Overload Response | O |  | Overload Response 9.3.1.104 |  | YES | reject |
| AMF Traffic Load Reduction Indication | O |  | Traffic Load Reduction Indication 9.3.1.106 |  | YES | ignore |
| Overload Start NSSAI List |  | 0..1 |  |  | YES | ignore |
| >Overload Start NSSAI Item |  | 1..<maxnoofSliceItems> |  |  | - |  |
| >>Slice Overload List | M |  | 9.3.1.107 |  | - |  |
| >>Slice Overload Response | O |  | Overload Response 9.3.1.104 |  | - |  |
| >>Slice Traffic Load Reduction Indication | O |  | Traffic Load Reduction Indication 9.3.1.106 |  | - |  |
| GUAMI List |  | 0..1 |  |  | YES | ignore |
| >GUAMI List Item |  | 1..<maxnoofGUAMIs> |  |  | - |  |
| >>GUAMI | M |  | 9.3.3.3 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofGUAMIs | Maximum no. of GUAMIs. Value is 1024. |

9.2.6.15	OVERLOAD STOP

This message is sent by the AMF and is used to indicate that the AMF is no longer overloaded.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| GUAMI List |  | 0..1 |  |  | YES | ignore |
| >GUAMI List Item |  | 1..<maxnoofGUAMIs> |  |  | - |  |
| >>GUAMI | M |  | 9.3.3.3 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofGUAMIs | Maximum no. of GUAMIs. Value is 1024. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

9.3.1.x	GW Context Release Indication

This information element is set by the gNB to provide an indication that the AMF may release any resources related to the signalled NG UE context (see TS 38.300 [8]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| GW Context Release Indication | M |  | ENUMERATED (true, …) | This IE indicates to the AMF that the gNB has successfully performed an Xn HO for the UE to a target gNB. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

9.3.1.y	Requested S-NSSAI

This IE indicates the S-NSSAI requested by the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Requested S-NSSAI Item |  | 1..<maxnoofRequestedS-NSSAIs> |  |  |
| >S-NSSAI | M |  | 9.3.1.24 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRequestedS-NSSAIs | Maximum no. of S-NSSAIs requested by the UE. Value is 8. |

End of Change