# 3GPP TSG-RAN WG3 Meeting #126										R3-247390

# Orlando, USA, 18 – 22 November 2024

Agenda Item:	13.2

Source:	CATT

Title:	Discussion for Inter-CU LTM

Document for:	Decision

- Introduction

In this paper, we will analyse the potential issues in inter-CU LTM.

2. Discussion

2.1 Multiple UE association

WA: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

The error handling of multiple UE associations need to be considered.

In last meeting, we finally reach the WA using separate LTM request message for each candidate cell, i.e., parallel request. And another issue is when parallel request toward the same candidate gNB, whether the source and candidate gNB can assign different Source/target NG-RAN node UE XnAP ID reference pair for the same UE.

Support multiple associations may give more flexibility, however, there are several potential issues need to be handle. The candidate gNB doesn’t know different Sourcet NG-RAN node UE XnAP ID reference are point to the same UE, which will impact candidate resource reserved, and complexity UE context handling. What’s more, and target node can not realize when the error occurred.

To simply the procedure, one way is request source cell and candidate cell to assign the same Source/Target NG-RAN node UE XnAP ID reference pair each time for LTM.

Proposal 1: In LTM, when parallel request, the source and the candidate cell shall always assign the same Source NG-RAN node UE XnAP ID reference and Target NG-RAN node UE XnAP ID reference for the same IE.

Proposal 2: If assume source and candidate shall always assign the same Source/target NG-RAN node UE XnAP ID reference, error handling can follow the legacy mechanism.

2.2 UE Association in-between candidate CUs

Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. When and how to establish the UE association is FFS.

For when to establish the association among candidate cells, there are three options on table.

- Option 1: During the LTM preparation phase, the candidate gNB allocates UE XnAP ID and send to other candidate gNBs (e.g. via initial source gNB).

- Option 2: After each cell switch, the new serving gNB updates the Source node UE XnAP ID and send it to other candidate gNBs, and then the candidate gNB-CUs reply with the Target node UE XnAP ID.

- Option 3: After each cell switch, the old serving gNB transfers the UE XnAP ID(s) used by the candidate gNBs to the new serving gNB.

For the option 1, it is the quickest way for subsequent LTM, however, it increases the overload of initial candidate cell preparation heavily. For the option 3, it requests the new source gNB(gNB2) to reuse the UE XnAP ID pairs assigned by the old source gNB (gNB3), which may facing the source node UE XnAP ID collision.

We prefer to use the option 2. When UE execute cell switch form source gNB(gNB1)and access to the new source gNB(gNB2), the gNB2 establish UE association with other candidate cell. This procedure can contain other functions at least:

- Update the KgNB no matter which solution is adopted by SA3;

- Inform other candidate cells which cell is the current serving cell;

Proposal 3: After each cell switch, the new serving gNB and candidate gNBs establish the UE association. i.e., option 2.

Proposal 4: Along with establish the UE association, the following function may also include in the same message:

- Update the KgNB no matter which solution is adopted by SA3;

- Inform other candidate cells which cell is the current serving cell;

2.3 L1 measurement and report configuration

Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase.

For SSB, RAN3 have get the conclusion to follow Rel-18 mechanism. Except SSB, Rel-19 LTM also agreed to support perdic CSI-RS, accordingly, RAN3 shall also design the signalling for CSI-RS. Considering CSI-RS is a UE-specific RS, it is not possible for CU to obtain it in advance anymore.

Thus, source node shall request the candidate node to provide CSI-RS for specified UE in the first round candidate cell request, e.g., HO REQUEST message for XnAP. And the candidate node shall contain the CSI-RS of the requested cell by HO REQUEST ACK message.

Proposal 5: The source CU request candidate gNB to provide CSI-RS for specified UE by HO REQUEST message, and candidate CU shall contain the CSI-RS of the requested cell by HO REQUEST ACK.

2.4 Early sync

For the early sync information request, for avoiding early sync confliction, candidate DU shall provide RACH configuration in per source DU in Rel-18. Considering the same logic, in inter-CU case, besides DU ID, CU ID should also be transferred in early sync information request. However, as a gNB can not explore its own architecture to neighbour gNB, it is reasonable to let candidate gNB provides the early sync resource per source gNB, and source gNB-CU further split the resource into per-gNB-DU granularity, if needed.

Proposal 6: Candidate gNB provide RACH configuration per source gNB per candidate cell, and source gNB-CU further split the resource into per-gNB-DU granularity, if needed.

2.5 TA validity

TA validity is discussed in Rel.18 and decided to leave it to source DU implementation. In Rel.19, companies re-raise concern about TA validity due to longer transmit latency among Xn interface.

Back to the legacy TA mechanism, TA validity is maintained by TAT, which is configured by network and inform to UE according to SIB1.

38.321, 5.2 RRC configures the following parameters for the maintenance of UL time alignment: - timeAlignmentTimer (per TAG) which controls how long the MAC entity considers the Serving Cells to the associated TAG to be uplink time aligned for the TAG; 38.331, 5.3.3.2 1> else: 2> apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1; 2> apply the default MAC Cell Group configuration as specified in 9.2.2; 2> apply the CCCH configuration as specified in 9.1.1.2; 2> apply the timeAlignmentTimerCommon included in SIB1;

In Rel.18, though not specific explicitly, the basic way to maintain TA validity is to start a timer at source side, the range of the timer usually as the same level with TAT due to they are for the same purpose. As defined in TS38.331, TAT is defined as ENUMERATED and the minimum value is 500ms. Usually, latency between the Xn interfaces can be seen as 10ms. It is easy to see the latency of Xn interface is much less than the TAT, i.e., TA validity time. Thus, the validity of TA is still possible to be decided at source DU, and let it to implementation as REL.18.

Proposal 7: TA validity assurance is up to network implementation in inter-CU LTM, Source DU is in charge of checking the TA validity.

To avoid unnecessary early sync, in Rel-18, we enable source to transfer the obtained TA values in Cell Switch Notification message. In intra-DU scenario, the movement range of UE is limited, TA still meaningful in target cell in a high possibility. However, extend to inter-CU case, UE movement ranger lager than before, it is hardly to say TA obtained at source is still workable in target side, no matter in source or in target. Using an unsuitable TA may lead to HO failure, the drawback of transferring TA among different CU outweigh the potential advantages.

Proposal 8: Due to TA is relative with UE location, transferring TA values among different gNB is not support in inter-CU LTM.

2.6 Synchronization among candidate cells

To support subsequent inter-CU LTM, at least the below information should below information should be synchronize among candidate cells.

- TCI state information list

- Configuration ID list

- Early sync information

- The data forwarding information

This procedure can be done by the second round interaction between source CU and candidate CUs, also can be done the after UE switch to the new DU. Whatever, this information synchronize should be finished no later than the UE access to the new serving cells.

Proposal 9: Synchronization among candidate cells can be triggered at the preparation phase to all candidate CUs, or triggered after UE execute LTM to the new serving DUs only.

2.7 Reference configuration

How the source gNB-CU sends the reference configuration to all candidate gNBs is pending on RAN2 progress.

As RAN2 discussed in last meeting, only support one reference configuration for inter-CU LTM. RAN3 can follow RAN2 agreements.

Proposal 10: Support one reference configuration for inter-CU LTM.

2.8 Path switch

The purpose of the Path Switch Request procedure is to establish a UE associated signalling connection to the 5GC. And usually, it should be finished as soon as UE access to the target gNB. In LTM, when UE access to the candidate DU, the candidate DU shall send a access success message to the CU among F1AP, which means UE had access to the target gNB. It is propose target cell triggers the PATH SWITCH REQUEST to the CN once target CU received the access success message.

Proposal 11: It is propose target cell triggers the PATH SWITCH REQUEST to the CN once target CU received the access success message.

2.9 LTM Cancel

Introduce a new procedure for candidate gNB-initiated LTM cancellation.

For the open issue about candidate cell initial cancellation, to make the procedure clear, it is better to introduce a new message. we prefer to have a more generally name for this message, that is, not only limited to LTM, to avoid the same discussion will happen in further discussion for candidate node triggered cancellation, e.g, Mobility Cancel, Candidate Cancel.

Proposal 12: Introduce a new message for candidate cell initial cancel for LTM, the name of the message shall be generally name instead of only limited to LTM, e.g, Mobility Cancel, Candidate Cancel.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

Proposal 1: In LTM, when parallel request, the source and the candidate cell shall always assign the same Source NG-RAN node UE XnAP ID reference and Target NG-RAN node UE XnAP ID reference for the same IE.

Proposal 2: If assume source and candidate shall always assign the same Source/target NG-RAN node UE XnAP ID reference, error handling can follow the legacy mechanism.

Proposal 3: After each cell switch, the new serving gNB and candidate gNBs establish the UE association. i.e., option 2.

Proposal 4: Along with establish the UE association, the following function may also include in the same message:

- Update the KgNB no matter which solution is adopted by SA3;

- Inform other candidate cells which cell is the current serving cell;

Proposal 5: The source CU request candidate gNB to provide CSI-RS for specified UE by HO REQUEST message, and candidate CU shall contain the CSI-RS of the requested cell by HO REQUEST ACK.

Proposal 6: Candidate gNB provide RACH configuration per source gNB per candidate cell, and source gNB-CU further split the resource into per-gNB-DU granularity, if needed.

Proposal 7: TA validity assurance is up to network implementation in inter-CU LTM, Source DU is in charge of checking the TA validity.

Proposal 8: Due to TA is relative with UE location, transferring TA values among different gNB is not support in inter-CU LTM.

Proposal 9: Synchronization among candidate cells can be triggered at the preparation phase to all candidate CUs, or triggered after UE execute LTM to the new serving DUs only.

Proposal 10: Support one reference configuration for inter-CU LTM.

Proposal 11: It is propose target cell triggers the PATH SWITCH REQUEST to the CN once target CU received the access success message.

Proposal 12: Introduce a new message for candidate cell initial cancel for LTM, the name of the message shall be generally name instead of only limited to LTM, e.g, Mobility Cancel, Candidate Cancel.

4. Reference

[1]	 R3-244272 Discussion for Inter-CU LTM