# 3GPP TSG-RAN WG3 Meeting #126										R3-247391

# Orlando, USA, 18 – 22 November 2024

Agenda Item:	13.2

Source:	CATT

Title:	Discussion for Inter-CU LTM in DC

Document for:	Decision

- Introduction

Based on the previous meeting, RAN2 had made some conclusion about DC scenario, and extracted as below:

RAN2#126

An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

RAN2#127

Inter-CU SCG LTM:

- Inter-CU SCG LTM preparation can be initiated by source SN.

- The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message.

- For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN.

- RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works.

- Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19.

Inter-CU MCG LTM:

- SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

- Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied).

- For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

RAN2#127bis

Agreements on inter-CU LTM

- The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

- For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

- The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3.

- Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline:

- The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part.

- FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations.

- The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message.

- Upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed.

- The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration.

- The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part.

- The MN can request an SCG reference configuration from any of the involved SNs.

5.	For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN.

6.	The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells.

7.	The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN.

8.	In order to support subsequent inter-CU SCG LTM, the MN needs to transfer, during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN.

9.	Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message.

10.	Re-use legacy LTM Cell Switch Command MAC CE for inter-SN LTM.

11.	RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen:

- Inter-MN LTM and intra-SN LTM

- Inter-SN LTM and intra-MN LTM

We start RAN3 discussion based on the above agreements.

2. Discussion

2.1 Simultaneous MCG LTM and SCG LTM Configuration

Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time.

In the previous discussion, it is confirm that for inter-CU LTM is not configured in both MCG and SCG at the same time. The first issue is whether the MCG LTM and SCG LTM have different priority. There are three possible options:

- Option 1: MCG LTM take the high priority

- Option 2: SCG LTM take the high priority

- Option 3: Flexible priority, the LTM configuration triggered first take the high priority

For the option 1, MCG LTM have a high priority, thus, once the MN decided to trigger inter-CU MCG LTM, the MN shall inform UE and related SN to release the prepared configuration for inter-CU SCG LTM, if configured.

For the option 2, SCG LTM have a high priority, thus, once source SN decided to trigger inter-CU LTM(the only case confirmed by RAN2), it shall inform the MN, the MN shall release the prepared configuration for inter-CU MCG LTM, if configured.

For the option 3, it pend on which configuration is triggered earlier. If Inter-CU SCG LTM triggered earlier than inter-CU MCG LTM (source SN send corresponding request to MN), the Inter-CU SCG LTM has higher priority than Inter-CU MCG LTM. If Inter-CU MCG triggered earlier than inter-CU SCG LTM (source MN send corresponding request to candidate MN), Inter-CU MCG LTM has higher priority than Inter-CU SCG LTM. We slightly prefer option 3.

Proposal 1: Consider the priority of MCG LTM and SCG LTM, slight prefer option3,

-	Option 1: MCG LTM take the high priority

-	Option 2: SCG LTM take the high priority

-	Option 3: Flexible priority, the LTM configuration triggered first take the high priority

If flexible priority is adopted, the coordination between MN and SN can realized by current procedure.

- When the inter-CU SCG LTM triggered earlier, the source SN shall send a required message to the MN, and MN shall not trigger inter-CU MCG as it already know the SCG LTM is prepared.

- When the inter-CU MCG LTM triggered earlier, the MN shall response a reject message with a suitable cause.

Proposal 2: If flexible priority is adopted, the coordination between MN and SN can realized by current procedure.

Proposal 3: Using a suitable cause value for MN informing SN inter-CU MCG LTM has already triggered, check whether MCG LTM triggered can be reused.

For the coexistence issue, RAN2 confirmed the following two cases shall be considered. And network implement shall to ensure not execution simulation.

- Inter-MN LTM and intra-SN LTM

- Inter-SN LTM and intra-MN LTM

This issue also exists in Rel-18, but rel-18 not introduce interface interaction. In Rel-19, we can further discussion whether introduce any interaction between MN and SN, or just ensure simultaneous by network implementation, e.g, OAM.

Proposal 4: RAN3 discuss whether need introduce interface interaction for ensuring MN and SN LTM execution simulation.

2.2 Inter-CU SCG LTM

For the inter-CU SCG LTM, RAN2 had down the scope and only focus on SN initiate SCG LTM. The source SN shall trigger the request of LTM candidate cells to MN, and then the MN request candidate SN to allocate resource for the UE.

The SN CHANGE REQUIRED message can be reused for request LTM candidate cell(s) from source SN to MN. And the SN ADDITION REQUEST message can be reused for request LTM candidate cell(s) from MN to candidate SN.

Proposal 5: Enhance SN CHANGE REQUIRED message for SN initiate SCG LTM candidate cell preparation from source SN to MN.

Proposal 6: Enhance SN ADDITION REQUEST for SN initiate SCG LTM candidate cell preparation from MN to candidate SN.

There are two possible ways for requesting candidate cell preparation:

- Single message: Similar with S-CPAC, source SN sends recommended candidate Pscells and the upper limit for the number of PSCells that can be prepared by each candidate SN to the MN. MN requests each candidate SN(s) to allocate resources for UE, and the candidate SN decides the list of PSCell(s) to prepared.

- Parallel messages: Similar with LTM, source SN sends the request candidate PScell to the MN, and MN requests candidate SN to allocate resources for UE, candidate SN decided whether accept or reject the request.

The main different is whether enable source SN/MN request multiple candidate cells by one message. As the message between MN and SN already support containing multiple cells in one message, we prefer to supporting source SN/MN request multiple cells in one request message.

Proposal 7：Support source SN/MN request multiple candidate cells by one request message.

In RAN2 previous discussion, they had discussed the procedure of interaction among MN, source-SN and candidate-SN. And based on their agreements 5, 6, 7, 8 from RAN2#127bis meeting, the procedure shall perform as below (considering the RS as CSI-RS).

Fig.1 Basic procedure of inter-CU SCG LTM based on RAN2 agreements

In above figure, step4, step5, step6 are enable source SN generate the common CSI-RESOURCE configuration, and then send the common configuration to the candidate SN via MN.

However, from signalling designation perspective, MN also can generate common CSI-RESOURCE configuration after step 3, which is quicker than transfer all the information to source SN. As RAN2 send a LS about their agreement in R2-2409378, RAN3 may reply LS to RAN2 asking whether it is possible enable MN generate common CSI-RESOURCE.

Proposal 8: RAN3 may reply LS to RAN2 asking whether it is possible enable MN generate common CSI-RESOURCE for inter-CU SCG LTM.

2.3 Inter-CU MCG LTM

For Inter-MN PCell change, the issue need to be discussed handle of SCG, e.g, SCG release ot SCG kept.

For our understanding, if the candidate MN cannot keep the previous SCG, it shall not configuration any other SCGs, i.e., the previous SCG shall be released.

The similar procedure is discussed at CHO with SCG.

1. The source MN starts the conditional handover procedure by initiating the X2 Handover Preparation procedure including MCG configuration and, if the UE is configured with an SCG, SCG configuration. The source MN may include the (source) SN UE X2AP ID, SN ID, the UE context in the (source) SN and the Conditional Handover Information Request IE in the Handover Request message. NOTE 3: In case of the CHO with/without SN change, the source MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration, if configured, before step 1. 2. If the candidate MN decides to keep the UE context in the SN, the candidate MN sends the SgNB Addition Request message to the SN including the SN UE X2AP ID as a reference to the UE context in the SN that was established by the source MN. If the candidate MN decides to change the SN allowing delta configuration, the candidate MN sends the SgNB Addition Request message to the candidate SN including the UE context in the source SN that was established by the source MN. Otherwise, the candidate MN may send the SgNB Addition Request message to the candidate SN including neither the SN UE X2AP ID nor the UE context in the source SN that was established by the source MN.

We suggest taking this procedure as baseline for the discussion of LTM candidate SCG kept/release. When considering inter-CU MCG LTM, the source MN can indicate the SCG information to candidate MN, and it depend on the candidate MN to decide whether kept the previous SCG and send feedback the source MN.

Proposal 9: Take SCG kept procedure in CHO with SCG as baseline for the discussion of LTM candidate SCG kept/release.

Proposal 10: It is propose that the candidate MN decide whether kept the current SCG and send feedback the source MN

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

Proposal 1: Consider the priority of MCG LTM and SCG LTM, slight prefer option3,

-	Option 1: MCG LTM take the high priority

-	Option 2: SCG LTM take the high priority

-	Option 3: Flexible priority, the LTM configuration triggered first take the high priority

Proposal 2: If flexible priority is adopted, the coordination between MN and SN can realized by current procedure.

Proposal 3: Using a suitable cause value for MN informing SN inter-CU MCG LTM has already triggered, check whether MCG LTM triggered can be reused.

Proposal 4: RAN3 discuss whether need introduce interface interaction for ensuring MN and SN LTM execution simulation.

Proposal 5: Enhance SN CHANGE REQUIRED message for SN initiate SCG LTM candidate cell preparation from source SN to MN.

Proposal 6: Enhance SN ADDITION REQUEST for SN initiate SCG LTM candidate cell preparation from MN to candidate SN.

Proposal 7：Support source SN/MN request multiple candidate cells by one request message.

Proposal 8: RAN3 may reply LS to RAN2 asking whether it is possible enable MN generate common CSI-RESOURCE for inter-CU SCG LTM.

Proposal 9: Take SCG kept procedure in CHO with SCG as baseline for the discussion of LTM candidate

SCG kept/release.

Proposal 10: It is propose that the candidate MN decide whether kept the current SCG and send feedback the source MN

4. Reference

[1]	RP-240299 Revised WID: NR mobility enhancements Phase 4

[2]	R2-2409378 LS on RAN2 agreements for inter-CU LTM