3GPP TSG-RAN WG3 Meeting #126	R3-247402

Orlando, US, 18 - 22 Nov, 2024	Revision of R3-245218

Title: 	(TP for TS 38.473) Discussion on on-demand SSB SCell operation

Source: 	Huawei

Agenda item:	17.2

Document Type:	 Other

# 1. Introduction

In the RAN#102 meeting, a new Work Item for network energy saving enhancement has been approved in [1] including the following objective on on-demand SSB SCell operation:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

Previous RAN3-125 bis meeting discussed the following aspects but without conclusions.

- The gNB-CU indicates the allowed cell list for on-demand SSB to gNB-DU?

- The gNB-CU indicates the on-demand SSB mode to gNB-DU?

- The information provided from DU to CU, e.g., on-demand SSB state

In this contribution, we provide our considerations on on-demand SSB SCell operation, based on RAN1 progress.

# 2. Discussion

## 2.1 Scenarios and related RAN1 progress

In previous RAN1 and RAN2 meetings, several agreements were reached regarding the on-demand SSB SCell operation. The relevant agreements pertaining to higher layers are given as follows.

Agreement at RAN1-116bis meeting For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - One signal - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed. Agreement at RAN1-116 meeting Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study: - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted. FFS: Which scenario the above applies for For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios. - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321)

RAN1 has agreed upon four use cases for on-demand SSB SCell operation (Scenario #2 and Scenario #2A combined with case #1 and case #2), which encompass situations both before and at the moment of SCell activation and when SCell is activated. The figure and table below illustrate these cases in detail.

Figure 1: The illustration of potential cases in [R2-2405122]

|  | Case #1 | Case #2 |
| --- | --- | --- |
| Scenario #2 | No always-on SSB First on-demand SSB MAC CE command, then MAC CE SCell Activation command | SSB transmitted at long periodicity First on-demand SSB MAC CE command, then MAC CE SCell Activation command |
| Scenario #2A | No always-on SSB on-demand SSB command and MAC CE SCell Activation command simultaneously | SSB transmitted at long periodicity on-demand SSB command and MAC CE SCell Activation command simultaneously |

It can also be noted that RAN1 and RAN2 will continue discussions on additional uses cases, such as scenario 3A and 3B. RAN3 can monitor the progress and continue to work on this.

Observation 1: RAN3 should focus on the two agreed cases (Case #1 and Case #2) and two scenarios (Scenario #2 and Scenario #2A) for on-demand SSB until RAN1 proposes new agreements.

## 2.2 Potential RAN3 impacts

### 2.2.1 Case 1 – No Always-on SSB Transmission

For Case #1, the SSB is not continuously transmitted. The operation of on-demand SSB SCell involves three potential triggering methods as detailed in [1]:

- Using an existing signal or channel for UE uplink wake-up signals,

- Cell on/off status communicated via backhaul,

- SCell activation/deactivation signaling.

According to the current XnAP specification, the procedure for such cell activation/deactivation via backhaul is already supported, allowing an NG-RAN node to request the activation of a cell by a neighboring NG-RAN node. For the case discussed, the existing cell activation message can be re-used without changes to the specification. With regards to the SMTC when the SCell SSB broadcast status changes, RAN3 can be pending to RAN2 further progress, then decide if there is any impact.

For Case #1, with no always-on SSB transmission, RAN3 to agree no additional modifications to the Xn specification for cell (de)activation.  With regards to the SMTC or L3 measurement, RAN3 can be pending to other WG progress.

For the CU/DU split architecture, the general on-demand SSB procedure over F1 is illustrated in Figure 2 below.

Figure 2: The general on-demand SSB procedure

In our view, the DU itself can decide whether and when to broadcast SSB transmission as well as broadcast periodicity. For example, when the SCell load is low, the DU could transit into the on-demand SCell state and stop SSB broadcast. Conversely, it can exit the on-demand SSB Cell state and return to the normal Cell state.

On the other hand, given that the CU has a general system view, it is beneficial for the CU to provide Cells allowed to be in on-demand SSB operation to the DU especially considering the on-demand SSB broadcast could be quite dynamic.  This approach aligns with the R18 inter-node beam activation, where the CU can provide a list of cells allowed to be deactivated.

The CU can send the allowed cells for on-demand SSB operation to the DU. And it is up to the DU to determine whether and when to broadcast SSB transmission as well as broadcast periodicity.

Secondly, when the UE accesses the CU and is configured with the SCell, the CU should indicate the on-demand SSB transmission status of this SCell to the UE. As discussed in previous RAN1 meetings, the RRC and MAC CE based signaling to indicate on-demand SSB transmission is supported.

Agreement at RAN1#118 meeting For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed. Agreement at RAN2#127 meeting Scenarios on OD-SSB: 1. RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B. OD-SSB transmission indication: 2. RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration. 3. New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A. Agreement at RAN2#127bis meeting 1. No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required. 2. Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A. 3. NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

From the above agreements, it can be observed that both the RRC and MAC CE may be used to manage on-demand SSB transmission. For RRC based signalling to indicate the activation/deactivation state on-demand SSB transmission, the DU can decide the SSB configuration per SCell and notify the CU, which then conveys this information to the UE. RAN3 can consider this could be included in the RRC container, which is to be decided by RAN2.

For RRC based signalling to indicate activation/deactivation state of OD-SSB configuration, the DU can decide and convey it the CU included in the RRC container.

For MAC CE based signalling, the DU can send the MAC CE to the UE as indicated in the above two scenarios. Note RAN1 is discussing whether to use the MAC CE as a way of deactivation for on-demand SSB.

- When the DU sends the MAC CE to broadcast SSB with the SSB periodicity, this SCell becomes a normal cell.  RAN3 can discuss whether the DU should notify the CU.

- When the DU decides to stop the SSB broadcast, RAN3 can also discuss if the notification to the CU is needed.

For MAC CE based signalling, when the DU decides to broadcast SSB with SSB periodicity or stop the broadcast, RAN3 to discuss whether and how the CU should be notified.

- In addition, for scenario#2, when the DU sends the on-demand SSB activation MAC CE to the UE, the UE may report the L3 measurements to the CU directly. RAN3 can discuss whether the CU can decide to activate the SCell for the UE or not, and indicate to the DU. Since the L3 measurement is still being discussed in other WGs, RAN3 can be pending till further progress was made.

For scenario#2, after the DU sends the on-demand SSB activation MAC CE to the UE, RAN3 to discuss whether the CU can indicate to the DU for SCell activation upon L3 measurement reports from the UE.

### 2.2.2 Case 2 - Always-on SSB is periodically transmitted on the cell

For Case #2, the always-on SSB is periodically transmitted on the SCell. From RAN3 perspective, this cell can be considered as a normal working cell. For example, this SCell can be already activated for one UE while being deactivated for another.  Also, RAN1 agreed that the always-on SSB could be multiplexed with on-demand SSB.

Agreement at RAN1-118bis meeting For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), study at least the following Mux-Cases. - Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB - Mux-Case #2: Always-on SSB and on-demand SSB overlap at least in time or frequency domain

Over Xn interface, neighbouring NG-RAN nodes can treat this cell as a normal served cell, i.e., without deactivation indication. As a result, there is no need to activate such cell. Similarly, over F1 interface, the CU can treat this cell as an activated and “in service” cell. Similar to case 1, RAN3 can wait for RAN2 further progress of the SMTC when the SSB periodicity is changed.

For case #2 (Always-on SSB is periodically transmitted on the cell) on-demand SSB, RAN3 to agree that no Xn specification impact for cell (de)activation.  With regards to the SMTC or L3 measurement, RAN3 can be pending to other WG progress.

- For the F1 impact, it is suggested that RAN3 can focus on the case1 first, then decide if any agreements can be applied to case 2 as well.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals.

Observation 1: RAN3 should focus on the two agreed cases (Case #1 and Case #2) and two scenarios (Scenario #2 and Scenario #2A) for on-demand SSB until RAN1 proposes new agreements.

- For Case #1, with no always-on SSB transmission, RAN3 to agree no additional modifications to the Xn specification for cell (de)activation.  With regards to the SMTC or L3 measurement, RAN3 can be pending to other WG progress.

The CU can send the allowed cells for on-demand SSB operation to the DU. And it is up to the DU to determine whether and when to broadcast SSB transmission as well as broadcast periodicity.

For RRC based signalling to indicate activation/deactivation state of OD-SSB configuration, the DU can decide and convey it the CU included in the RRC container.

For MAC CE based signalling, when the DU decides to broadcast SSB with SSB periodicity or stop the broadcast, RAN3 to discuss whether and how the CU should be notified.

For scenario#2, after the DU sends the on-demand SSB activation MAC CE to the UE, RAN3 to discuss whether the CU can indicate to the DU for SCell activation upon L3 measurement reports from the UE.

For case #2 (Always-on SSB is periodically transmitted on the cell) on-demand SSB, RAN3 to agree that no Xn specification impact for cell (de)activation.  With regards to the SMTC or L3 measurement, RAN3 can be pending to other WG progress.

- The TP for TS 38.473 to implement the Proposal 2 is provided in Annex.

# 4. Reference

- RP-241650 Revised WID: Enhancements of network energy savings for NR, Ericsson

- Chair notes RAN1#117 eom4.doc

- TS 38.423 V18.0.0

- TS 38.473 V18.0.0

# 5. TP for TS 38.473

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall activate the cell indicated by NR CGI IE and reconfigure the physical cell identity for which the NR PCI IE is included.

If the SSBs within the cell to be Activated List IE is included in the Cells to be Activated List Item IE within the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, only activate those SSB beams indicated by the SSB Index IE.

If at least one requested SSB beam in the SSBs within the cell to be Activated List IE is activated, the gNB-DU includes the Cells with SSBs Activated List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message. The gNB-CU shall consider that the SSB beams indicated by the SSBs activated List IE as activated.

If Cells to be Deactivated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall deactivate the cell indicated by NR CGI IE.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is included in the GNB-CU CONFIGURATION UPDATE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose.

If the Cells Allowed to On-Demand SSB List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to initiate on-demand SSB for the indicated cells for network energy saving purpose.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >>gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| >>MBS Broadcast Neighbour Cell List | O |  | 9.3.1.226 |  | YES | ignore |
| >>SSBs within the cell to be Activated List | O |  | 9.3.1.326 | List of SSB beams within the cell requested to be activated. | YES | reject |
| Cells to be Deactivated List |  | 0..1 |  | List of cells to be deactivated | YES | reject |
| >Cells to be Deactivated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| gNB-CU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-CU. | YES | ignore |
| Extended gNB-CU Name | O |  | 9.3.1.206 |  | YES | ignore |
| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Cells Allowed to On-Demand SSB List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be On-Demand SSB List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

TimeWindowInformation-Measurement-List,

SRSPosRRCInactiveValidityAreaConfig,

SRSReservationType,

RequestedSRSPreconfigurationCharacteristics-List,

SRSPreconfiguration-List,

Broadcast-MRBs-Transport-Request-Item,

TAInformation-List,

NonIntegerDRXCycle,

AggregatedPosSRSResourceSetList,

F1U-PathFailure,

Cells-Allowed-to-on-demand-ssb-List-Item

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-SRSPreconfiguration-List,

id-SRSInformation,

id-TAInformation-List,

id-NonIntegerDRXCycle,

id-AggregatedPosSRSResourceSetList,

id-RANSharingAssistanceInformation,

id-F1U-PathFailure,

id-Cells-Allowed-to-on-demand-ssb-List,

id-Cells-Allowed-to-on-demand-ssb-List-Item,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofIndividualF1ConnectionsToReset

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

GNBCUConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { GNBCUConfigurationUpdateIEs} },

...

}

GNBCUConfigurationUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE	TransactionID											PRESENCE mandatory	}|

{ ID id-Cells-to-be-Activated-List				CRITICALITY reject	TYPE	Cells-to-be-Activated-List							PRESENCE optional	}|

{ ID id-Cells-to-be-Deactivated-List			CRITICALITY reject	TYPE	Cells-to-be-Deactivated-List						PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Add-List		CRITICALITY ignore	TYPE	GNB-CU-TNL-Association-To-Add-List				PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Remove-List	CRITICALITY ignore	TYPE	GNB-CU-TNL-Association-To-Remove-List			PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Update-List	CRITICALITY ignore	TYPE	GNB-CU-TNL-Association-To-Update-List			PRESENCE optional	}|

{ ID id-Cells-to-be-Barred-List					CRITICALITY ignore	TYPE	Cells-to-be-Barred-List								PRESENCE optional	}|

{ ID id-Protected-EUTRA-Resources-List			CRITICALITY reject	TYPE	Protected-EUTRA-Resources-List					PRESENCE optional	}|

{ ID id-Neighbour-Cell-Information-List			CRITICALITY ignore	TYPE	Neighbour-Cell-Information-List					PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info			CRITICALITY ignore	TYPE	Transport-Layer-Address-Info						PRESENCE optional	}|

{ ID id-UL-BH-Non-UP-Traffic-Mapping			CRITICALITY reject	TYPE	UL-BH-Non-UP-Traffic-Mapping						PRESENCE optional	}|

{ ID id-BAPAddress								CRITICALITY ignore  TYPE 	BAPAddress												PRESENCE optional }|

{ ID id-CCO-Assistance-Information				CRITICALITY ignore	TYPE 	CCO-Assistance-Information							PRESENCE optional	}|

{ ID id-CellsForSON-List						CRITICALITY ignore	TYPE	CellsForSON-List										PRESENCE optional }|

{ ID id-gNB-CU-Name								CRITICALITY ignore	TYPE	GNB-CU-Name												PRESENCE optional	}|

{ ID id-Extended-GNB-CU-Name					CRITICALITY ignore	TYPE	Extended-GNB-CU-Name									PRESENCE optional	}|

{ ID id-Cells-Allowed-to-be-Deactivated-List			CRITICALITY ignore	TYPE	Cells-Allowed-to-be-Deactivated-List		PRESENCE optional	}

{ ID id-Cells-Allowed-to-on-demand-ssb-List			CRITICALITY ignore	TYPE	Cells-Allowed-to-on-demand-ssb-List		PRESENCE optional	},

...

}

Cells-to-be-Deactivated-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-to-be-Deactivated-List-ItemIEs } }

GNB-CU-TNL-Association-To-Add-List		::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-CU-TNL-Association-To-Add-ItemIEs } }

GNB-CU-TNL-Association-To-Remove-List	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-CU-TNL-Association-To-Remove-ItemIEs } }

GNB-CU-TNL-Association-To-Update-List	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-CU-TNL-Association-To-Update-ItemIEs } }

Cells-to-be-Barred-List			::= SEQUENCE(SIZE(1.. maxCellingNBDU)) OF ProtocolIE-SingleContainer { { Cells-to-be-Barred-ItemIEs } }

Cells-Allowed-to-be-Deactivated-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-Allowed-to-be-Deactivated-List-ItemIEs } }

Cells-Allowed-to-be-Deactivated-List-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-Allowed-to-be-Deactivated-List-Item	CRITICALITY ignore	TYPE	Cells-Allowed-to-be-Deactivated-List-Item	PRESENCE mandatory },

...

}

Cells-Allowed-to-on-demand-ssb-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-Allowed-to-on-demand-ssb-List-ItemIEs } }

Cells-Allowed-to-on-demand-ssb-List-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-Allowed-to-on-demand-ssb-List-Item	CRITICALITY ignore	TYPE	Cells-Allowed-to-on-demand-ssb-List-Item	PRESENCE mandatory },

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Cells-With-SSBs-Activated-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-Allowed-to-be-Deactivated-List-Item ::= SEQUENCE {

nRCGI		NRCGI,

iE-Extensions				ProtocolExtensionContainer { { Cells-Allowed-to-be-Deactivated-List-ItemExtIEs} }	OPTIONAL,

...

}

Cells-Allowed-to-be-Deactivated-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-Allowed-to-on-demand-ssb-List-Item ::= SEQUENCE {

nRCGI		NRCGI,

iE-Extensions				ProtocolExtensionContainer { { Cells-Allowed-to-on-demand-ssb-List-Item-ItemExtIEs} }	OPTIONAL,

...

}

Cells-Allowed-to-on-demand-ssb-List-Item-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-BarringExemptionforEmerCallInfo					ProtocolIE-ID ::= 849

id-SIB17bis-message									ProtocolIE-ID ::= 850

id-ReportingIntervalIMs								ProtocolIE-ID ::= 851

id-Transmission-Bandwidth-asymmetric				ProtocolIE-ID ::= 852

id-TagIDPointer										ProtocolIE-ID ::= 853

id-Cells-Allowed-to-on-demand-ssb-List				ProtocolIE-ID ::= aaa

id-Cells-Allowed-to-on-demand-ssb-List-Item			ProtocolIE-ID ::= bbb