3GPP TSG-RAN WG3 Meeting #126	R3-247425

Orlando, USA, Nov.18–22, 2024

Agenda item:	 10.2

Source:		Lenovo

Title:		(TP for SON BLCR for 37.340) Discussion on MRO for subsequent CPAC and CHO with candidate SCG(s)

Document for:		Discussion and Decision

1	Introduction

In RAN3#123bis, we have achieved following agreements:

Work on the scenarios of failure in S-CPAC. The optimization of non-failure scenarios (e.g., near failure and ping-pong) is not excluded.

MRO for CHO with candidate SCG failure and near failure cases.

RAN3 focuses on NR-DC for MRO for CHO with candidate SCG in R19.

R19 SON/MDT solution discussion is based on R18 work.

In RAN3#124, we further agreed that:

CPAC failure scenarios and detection mechanism captured in stage2 used as baseline.

Do not distinguish between fast MCG recovery/no-fast MCG recovery for now (to simplify use cases).

RAN3 will start with the failure scenarios with UEs configured with CHO with candidate SCGs.

In RAN3#125, we further agreed that:

MRO for CHO with Candidate SCG(s)

MRO for Case 1a, 1b, 2a, 2b, 3a, 3b, 7a, 7b, 8a, 8b, 9 will be addressed.

MRO for case 4/5/6 is in the scope.

FFS whether there is any specification impact for case 4/5/6.

Case 1c, 2c, 2c, 3c, 7c, 8c are FFS.

The RLF report includes: The type of the first fulfilled execution condition e.g. CPAC or CHO, fulfilment means all events of the type are met.

MRO for S-CPAC:

Reusing SCG FAILURE INFORMATION REPORT message over Xn for MN to report SCG failure of SCPAC to the concerned SN.

In RAN3#125bis, we further agreed that:

MRO for others:

MRO for CHO with Candidate SCG(s)

RLF report is enhanced to including

  - Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered

  - Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered

  - The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

  - The fulfilled events before the RLF is encountered?

In this paper, we would further discuss the details of MRO for subsequent CPAC and CHO with candidate SCG(s).

2	Discussion

  - 2.1 MRO for subsequent CPAC

For MN initiated subsequent CPAC, the MN initially triggers the candidate cell preparation of subsequent CPAC procedure and generates the execution conditions for the initial execution of subsequent CPAC (e.g. CPA or CPC). For MN initiated subsequent CPAC, the candidate SN generates the execution conditions for the following execution of subsequent CPAC when the candidate SN prepares the candidate SCG configuration(s) for candidate PSCell(s).

For SN initiated subsequent CPAC, the source SN initially triggers the candidate cell preparation of subsequent CPAC procedure and generates the execution conditions for the initial execution of subsequent CPAC. For SN initiated subsequent CPAC, the candidate SN generates the execution conditions for the following execution of subsequent CPAC when the candidate SN prepares the candidate SCG configuration(s) for candidate PSCell(s). For SN initiated intra-SN subsequent CPAC, the source SN generates the execution conditions for the following execution of subsequent CPAC when the source SN prepares the candidate SCG configuration(s) for candidate PSCell(s).

Similar as legacy CPAC, MN determines whether and where to forward the S-CPAC failure related information when receiving SCGfailureInformation message from the UE.

For MN initiated subsequent CPAC,

- if the suitable PSCell is not one of the candidate PSCells provided by MN at S-CPAC preparation, the MN performs the final MRO related optimisation e.g. optimize the candidate PSCell list, in case of initial execution of subsequent CPAC (e.g. CPA or CPC) or following execution of subsequent CPAC;

- if the suitable PSCell is one of the candidate PSCells provided by MN at S-CPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, the MN sends the SCG FAILURE INFORMATION REPORT message to candidate or target SN for candidate PSCell selection optimization, in case of initial execution of subsequent CPAC or following execution of subsequent CPAC;

- otherwise, MN performs the final MRO related optimisation e.g. optimize the execution conditions for the initial execution of subsequent CPAC, in case of initial execution of subsequent CPAC, or, the MN may forward the SCG FAILURE INFORMATION REPORT message to source SN of following execution of subsequent CPAC for optimising the execution conditions for the following execution of subsequent CPAC, in case of following execution of subsequent CPAC.

For SN initiated subsequent CPAC, the MN sends the SCG Failure Information Report message to SN initiating the subsequent CPAC procedure,

- if the suitable PSCell is not one of the candidate PSCells provided by SN initiating the subsequent CPAC procedure, the SN initiating the subsequent CPAC procedure performs the final MRO related optimisation e.g. optimize the candidate PSCell list, in case of initial execution of subsequent CPAC (e.g. CPA or CPC) or following execution of subsequent CPAC;

- if the suitable PSCell is one of the candidate PSCells provided by SN initiating the subsequent CPAC procedure, but not one of the candidate PSCells selected by the candidate or target SN, SN initiating the subsequent CPAC procedure indicates the MN, then, MN sends the SCG FAILURE INFORMATION REPORT message to candidate or target SN for candidate PSCell selection optimization, in case of initial execution of subsequent CPAC or following execution of subsequent CPAC;

- otherwise, SN initiating the subsequent CPAC procedure performs the final MRO related optimisation e.g. optimize the execution conditions for the initial execution of subsequent CPAC, in case of initial execution of subsequent CPAC, or, SN initiating the subsequent CPAC procedure indicates the MN to send the SCG FAILURE INFORMATION REPORT message to source SN of following execution of subsequent CPAC for optimising the execution conditions for the following execution of subsequent CPAC, in case of following execution of subsequent CPAC.

The corresponding TP for TS37.340 is provided in Annex.

Proposal 1: For MN initiated subsequent CPAC,

- if the suitable PSCell is not one of the candidate PSCells provided by MN at S-CPAC preparation, the MN performs the final MRO related optimisation e.g. optimize the candidate PSCell list, in case of initial execution of subsequent CPAC (e.g. CPA or CPC) or following execution of subsequent CPAC;

- if the suitable PSCell is one of the candidate PSCells provided by MN at S-CPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, the MN sends the SCG FAILURE INFORMATION REPORT message to candidate or target SN for candidate PSCell selection optimization, in case of initial execution of subsequent CPAC or following execution of subsequent CPAC;

- otherwise, MN performs the final MRO related optimisation e.g. optimize the execution conditions for the initial execution of subsequent CPAC, in case of initial execution of subsequent CPAC, or, the MN may forward the SCG FAILURE INFORMATION REPORT message to source SN of following execution of subsequent CPAC for optimising the execution conditions for the following execution of subsequent CPAC, in case of following execution of subsequent CPAC.

Proposal 2: For SN initiated subsequent CPAC, the MN sends the SCG Failure Information Report message to SN initiating the subsequent CPAC procedure,

- if the suitable PSCell is not one of the candidate PSCells provided by SN initiating the subsequent CPAC procedure, the SN initiating the subsequent CPAC procedure performs the final MRO related optimisation e.g. optimize the candidate PSCell list, in case of initial execution of subsequent CPAC or following execution of subsequent CPAC;

- if the suitable PSCell is one of the candidate PSCells provided by SN initiating the subsequent CPAC procedure, but not one of the candidate PSCells selected by the candidate or target SN, SN initiating the subsequent CPAC procedure indicates the MN, then, MN sends the SCG FAILURE INFORMATION REPORT message to candidate or target SN for candidate PSCell selection optimization, in case of initial execution of subsequent CPAC or following execution of subsequent CPAC;

- otherwise, SN initiating the subsequent CPAC procedure performs the final MRO related optimisation e.g. optimize the execution conditions for the initial execution of subsequent CPAC, in case of initial execution of subsequent CPAC, or, SN initiating the subsequent CPAC procedure indicates the MN to send the SCG FAILURE INFORMATION REPORT message to source SN of following execution of subsequent CPAC for optimising the execution conditions for the following execution of subsequent CPAC, in case of following execution of subsequent CPAC.

When a SCG failure happens in subsequent CPAC procedure, as analyzed above, if execution conditions are not proper, different node performs optimisation in case of different execution phase, i.e. in case of initial execution of subsequent CPAC, the node initiating S-CPAC procedure optimizes the execution conditions for the initial execution of subsequent CPAC, in case of following execution of subsequent CPAC, the MN forwards the SCG FAILURE INFORMATION REPORT message to source SN of following execution of subsequent CPAC for optimising the execution conditions for the following execution of subsequent CPAC. To enable MN to forward SCG FAILURE INFORMATION REPORT message correctly, it is benecifical to enable MN know whether the failure happens in initial execution of subsequent CPAC or following execution of subsequent CPAC. Based on existing SCGfailureInformation message from the UE, MN can’t know this. Therefore, an indication concerning the SCG Failure Information message is for initial execution of subsequent CPAC or following execution of subsequent CPAC may be included in the SCG Failure Information message‎.

Proposal 3: An indication concerning the SCG Failure Information message is for initial execution of subsequent CPAC or following execution of subsequent CPAC may be included in the SCG Failure Information message.

Similar as legacy CPAC, it may be possible that source SN has released UE context when receiving the SCG Failure Information Report message, to enable source SN to identify the received SCG failure information, subsequent CPAC configuration (e.g. candidate PSCell list for S-CPAC, S-CPAC execution condition(s)) may be included in the SCG Failure Information Report message.

Proposal 4: Include subsequent CPAC configuration (e.g. candidate PSCell list for S-CPAC, S-CPAC execution condition(s)) in the SCG Failure Information Report message.

  - 2.2 MRO for CHO with candidate SCG(s)

    - 2.2.1 MRO for connection failure case

| Cases | Figure: CHO with candidate SCG(s) is configured | Brief Description |
| --- | --- | --- |
| Case 1 |  | Failure happens before CHO and/or CPAC execution. 1a) RLF@srcPCell 1b) SCG failure 1c) MCG failure+SCG failure |
| Case 2 |  | CPC/CPA execution condition is met. Failure happens before CHO execution condition is met. 2a) RLF@srcPCell 2b) SCG failure 2c) MCG failure + SCG failure |
| Case 3 |  | CHO execution condition is met. No CHO only or CHO with SCG configuration. Failure happens before CPAC execution condition is met. 3a) RLF@srcPCell 3b) SCG failure 3c) MCG failure + SCG failure |
| Case 4 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. CHO execution to a candidate PCell fails. |
| Case 5 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. RLF shortly after CHO execution success |
| Case 6 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. SCG failure shortly after CHO execution success |
| Case 7 |  | Both CHO and CPAC execution conditions are met. 7a) CHO execution to a candidate PCell failure. 7b) CPAC to a candidate PSCell failure. 7c) Execution to SCG failure and toMCG failure. |
| Case 8 |  | Failure happens shortly after CHO and CPAC execution success 8a) RLF@targetPCell 8b) SCG failure 8c) MCG failure + SCG failure |
| Case 9 |  | Failure due to the execution conditions for candidate PCell and the associated candidate PSCell cannot be met simultaneously. 9a) RLF@srcPCell 9b) SCG failure 9c) MCG failure + SCG failure |

It was agreed in RAN3#125 meeting that MRO for case 4/5/6 is in the scope and it was FFS whether there is any specification impact for case 4/5/6. In case of 4/5/6, configuration of CHO only is configured to the UE together with configuration of CHO with candidate SCG(s) configuration, the UE performs CHO only execution due to CHO with candidate SCG(s) can’t be triggered (e.g. CHO execution condition of CHO with candidate SCG(s) is fulfilled but CPAC execution condition of CHO with candidate SCG(s) is not fulfilled, CPAC execution condition of CHO with candidate SCG(s) is fulfilled but CHO execution condition of CHO with candidate SCG(s) is not fulfilled, neither CHO execution condition nor CPAC execution condition of CHO with candidate SCG(s) is fulfilled). To improve mobility performance, it is beneficial for network to know the reason about why CHO with candidate SCG(s) together with CHO only is configured but CHO only is performed instead of CHO with candidate SCG(s), e.g. only CHO execution condition or only CPAC execution condition of CHO with candidate SCG(s) is fulfilled, or neither CHO execution condition nor CPAC execution condition of CHO with candidate SCG(s) is fulfilled. RAN3#125 meeting agreed the RLF report includes the type of the first fulfilled execution condition e.g. CPAC or CHO. This agreement applies to the case 4/5, for case 6, SCG failure information message also needs to include the type of the first fulfilled execution condition of CHO with candidate SCG(s) if any e.g. CPAC or CHO.

Proposal 5: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) if any, can be stored and reported by the UE in SCG failure information message.

RAN3#125bis meeting further agreed to include the identifier of candidate PCell(s) and/or PSCell(s) that fulfilled execution conditions when/before the RLF is encountered in RLF report. For case 6, it is also needed to include identifier of candidate PCell(s) which met the configured CHO execution conditions and/or identifier of candidate PSCell(s) which met the configured CPAC execution conditions before the failure is encountered in the SCG failure information message.

Proposal 6: Identifier of candidate PCell(s) which met the configured CHO execution conditions and/or identifier of candidate PSCell(s) which met the configured CPAC execution conditions before connection failure is encountered can be stored and reported by the UE in SCG failure information message.

For case 7b and case 8b, SCG failure happens after CHO with candidate SCG(s) is triggered. To analyse whether the failure is caused due to inappropriate CHO execution condition and/or CPAC execution condition, it is beneficial to store and report time duration between the fulfilled CHO execution condition and the fulfilled CPAC execution condition in SCG failure information message.

Proposal 7: Time duration between the fulfilled CHO execution condition and the fulfilled CPAC execution condition can be stored and reported by the UE in SCG failure information message.

Whether to include the fulfilled events before the RLF is encountered in RLF report is an issue left from last RAN3#125bis meeting, based on agreements achieved so far in both RAN2 and RAN3, the RLF report includes the first met execution condition, the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), the elapsed time between the point in time of the first fulfilled condition and RLF, identifier of candidate PCell(s) and/or PSCell(s) that fulfilled execution conditions when/before the RLF is encountered, these information is sufficient for MRO analysis and optimisation. Including the fulfilled events before the RLF is encountered in RLF report is complex and redundant.

Proposal 8: No need to include the fulfilled events before the RLF is encountered in RLF report.

For the left case 1c, 2c, 3c, 7c and 8c, they are complex since both MCG failure and SCG failure happen, regarding TU limitation, we should prioritize the single failure cases, if time allows, the concurrent failure cases can be further discussed later.

Proposal 9: Deprioritize Case 1c, 2c, 3c, 7c and 8c.

    - 2.2.2 MRO for near failure successful case

When CHO with candidate SCG(s) is executed, a near-failure successful PCell change and/or a near-failure successful PSCell addition or PSCell change may happen. For sub-optimal successful CHO execution, to help network know how to modify the configuration of the CHO with candidate SCG(s) procedure, legacy SHR trigger condition(s) can be reused, and SHR can be enhanced, e.g. the type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) and the time duration between two fulfilled execution conditions (e.g. the time elapsed between CHO execution condition is fulfilled and the associated CPAC execution condition is fulfilled) can be stored and reported by the UE in SHR.

Proposal 10: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SHR.

Similarly, for any sub-optimal successful CPAC, legacy SPR trigger condition(s) can be reused, and SPR can be enhanced, e.g. the type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) and the time duration between two fulfilled execution conditions (e.g. the time elapsed between CHO execution condition is fulfilled and the associated CPAC execution condition is fulfilled) can be stored and reported by the UE in SPR.

Proposal 11: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SPR.

In case the SPR is retrieved in a node different from source MN that sent the SPR configuration to the UE or target MN that to which the UE successfully access, since SPR trigger condition(s) may be generated by target MN or target SN, for SPR forwarding, there are two potential options:

- Option 1: the receiving node that fetches the SPR from the UE first forwards the SPR to source MN, then source MN forwards the SPR to target MN, if needed target MN forwards the SPR to the respective SN(s) which should perform the SPR optimization (e.g. corresponding target SN that generates the SPR trigger condition that triggers the SPR).

- Option 2: the receiving node that fetches the SPR from the UE forwards the SPR to target MN, then, if needed target MN forwards the SPR to the respective SN(s) which should perform the SPR optimization (e.g. corresponding target SN that generates the SPR trigger condition that triggers the SPR).

In Option 1, to enable the receiving node to forward the SPR to source MN, the UE should store and report cell ID of source PCell in the SPR, furthermore, the UE should store and report cell ID of target PCell in the SPR for forwarding the SPR from source MN to target MN. Compared with Option 1, in Option 2, the UE should only store and report cell ID of target PCell in the SPR, which has fewer spec impacts. RAN3 should discuss the SPR forwarding mechanism for CHO with candidate SCG(s).

Proposal 12: RAN3 to discuss SPR forwarding for CHO with candidate SCG(s):

- Option 1: the receiving node that fetches the SPR from the UE first forwards the SPR to source MN, then source MN forwards the SPR to target MN, if needed target MN forwards the SPR to the respective SN(s) which should perform the SPR optimization (e.g. corresponding target SN that generates the SPR trigger condition that triggers the SPR).

- Option 2: the receiving node that fetches the SPR from the UE forwards the SPR to target MN, then, if needed target MN forwards the SPR to the respective SN(s) which should perform the SPR optimization (e.g. corresponding target SN that generates the SPR trigger condition that triggers the SPR).

3	Conclusion

In this paper, the details of MRO for subsequent CPAC and CHO with candidate SCG(s) are discussed. We have the following proposals:

MRO for subsequent CPAC

Proposal 1: For MN initiated subsequent CPAC,

- if the suitable PSCell is not one of the candidate PSCells provided by MN at S-CPAC preparation, the MN performs the final MRO related optimisation e.g. optimize the candidate PSCell list, in case of initial execution of subsequent CPAC (e.g. CPA or CPC) or following execution of subsequent CPAC;

- if the suitable PSCell is one of the candidate PSCells provided by MN at S-CPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, the MN sends the SCG FAILURE INFORMATION REPORT message to candidate or target SN for candidate PSCell selection optimization, in case of initial execution of subsequent CPAC or following execution of subsequent CPAC;

- otherwise, MN performs the final MRO related optimisation e.g. optimize the execution conditions for the initial execution of subsequent CPAC, in case of initial execution of subsequent CPAC, or, the MN may forward the SCG FAILURE INFORMATION REPORT message to source SN of following execution of subsequent CPAC for optimising the execution conditions for the following execution of subsequent CPAC, in case of following execution of subsequent CPAC.

Proposal 2: For SN initiated subsequent CPAC, the MN sends the SCG Failure Information Report message to SN initiating the subsequent CPAC procedure,

- if the suitable PSCell is not one of the candidate PSCells provided by SN initiating the subsequent CPAC procedure, the SN initiating the subsequent CPAC procedure performs the final MRO related optimisation e.g. optimize the candidate PSCell list, in case of initial execution of subsequent CPAC (e.g. CPA or CPC) or following execution of subsequent CPAC;

- if the suitable PSCell is one of the candidate PSCells provided by SN initiating the subsequent CPAC procedure, but not one of the candidate PSCells selected by the candidate or target SN, SN initiating the subsequent CPAC procedure indicates the MN, then, MN sends the SCG FAILURE INFORMATION REPORT message to candidate or target SN for candidate PSCell selection optimization, in case of initial execution of subsequent CPAC or following execution of subsequent CPAC;

- otherwise, SN initiating the subsequent CPAC procedure performs the final MRO related optimisation e.g. optimize the execution conditions for the initial execution of subsequent CPAC, in case of initial execution of subsequent CPAC, or, SN initiating the subsequent CPAC procedure indicates the MN to send the SCG FAILURE INFORMATION REPORT message to source SN of following execution of subsequent CPAC for optimising the execution conditions for the following execution of subsequent CPAC, in case of following execution of subsequent CPAC.

Proposal 3: An indication concerning the SCG Failure Information message is for initial execution of subsequent CPAC or following execution of subsequent CPAC may be included in the SCG Failure Information message.

Proposal 4: Include subsequent CPAC configuration (e.g. candidate PSCell list for S-CPAC, S-CPAC execution condition(s)) in the SCG Failure Information Report message.

MRO for CHO with candidate SCG(s)

Proposal 5: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) if any, can be stored and reported by the UE in SCG failure information message.

Proposal 6: Identifier of candidate PCell(s) which met the configured CHO execution conditions and/or identifier of candidate PSCell(s) which met the configured CPAC execution conditions before connection failure is encountered can be stored and reported by the UE in SCG failure information message.

Proposal 7: Time duration between the fulfilled CHO execution condition and the fulfilled CPAC execution condition can be stored and reported by the UE in SCG failure information message.

Proposal 8: No need to include the fulfilled events before the RLF is encountered in RLF report.

Proposal 9: Deprioritize Case 1c, 2c, 3c, 7c and 8c.

Proposal 10: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SHR.

Proposal 11: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SPR.

Proposal 12: RAN3 to discuss SPR forwarding for CHO with candidate SCG(s):

- Option 1: the receiving node that fetches the SPR from the UE first forwards the SPR to source MN, then source MN forwards the SPR to target MN, if needed target MN forwards the SPR to the respective SN(s) which should perform the SPR optimization (e.g. corresponding target SN that generates the SPR trigger condition that triggers the SPR).

- Option 2: the receiving node that fetches the SPR from the UE forwards the SPR to target MN, then, if needed target MN forwards the SPR to the respective SN(s) which should perform the SPR optimization (e.g. corresponding target SN that generates the SPR trigger condition that triggers the SPR).

Reference

[1] TS37.340

Annex for TS37.340

### 10.18.3	Conditional PSCell addition or change failure

One of the functions of self-optimization for CPAC is to detect CPAC failures that occur due to Too late CPC execution or Too early CPC/CPA execution, or CPC/CPA execution to wrong PSCell. These problems are defined as follows:

-	Too Late CPC Execution: UE receives CPC configuration/S-CPAC configuration, while a SCG failure occurs before CPC execution condition is satisfied; a suitable PSCell different from source PSCell is found based on the measurements reported from the UE.

-	Too Early CPC/CPA Execution: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; in case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE.

-	CPC/CPA Execution to wrong PSCell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE. There are two sub-cases for wrong candidate PSCell list selection:

-	if the suitable PSCell is one of the candidate target PSCells provided by the node initiating the CPC/S-CPAC or by the MN initiating the CPA, but not one of the candidate PSCells selected by the candidate or target SN, it is wrong target PSCell selection at the candidate or target SN;

-	else, it is wrong candidate PSCell list selection at the node initiating the CPC/S-CPAC or at the MN initiating the CPA.

In the definition above, the "successful CPC/CPA execution" refers to the UE state, namely the successful completion of the RA procedure.

Detection mechanism

The MN performs the initial analysis when SCGFailureInformation is received from the UE. In the first step, MN verifies whether intra-SN PSCell change has been triggered in the last serving SN. In case the intra-SN PSCell change has been triggered in the last serving SN, the MN forwards the SCG Failure Information Report message to this last serving SN, which performs the final root cause analysis. In case of no intra-SN PSCell change, the MN determines the type of PSCell addition/change, e.g., whether it is CPA or CPC in case of conditional mobility, if CPC whether it is MN initiated or SN initiated.

The detailed detection mechanisms for Too Late CPC Execution, Too Early CPC/CPA Execution and CPC/CPA Execution to wrong PSCell are carried out in the NG-RAN node which initiates the CPC/CPA or S-CPAC procedure:

-	Too Late CPC Execution: if CPC/S-CPAC is configured and there is no recent CPC execution for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt) , and there is a suitable PSCell different from the PSCell where the UE is located at the time of the failure occurrence.

-	Too Early CPC/CPA Execution: there is a recent CPC/CPA execution for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and in case of CPC, the source PSCell is a suitable PSCell, in case of CPA, no suitable PSCell is found.

-	CPC/CPA Execution to wrong PSCell: there is a recent CPC/CPA execution for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the suitable PSCell is not the source PSCell or the target PSCell.

The "UE reported timer" above indicates the time elapsed since the CPC/CPA execution until connection failure.

For CPA or MN initiated CPC or initial S-CPAC/following S-CPC initiated by MN, if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, MN sends the SCG Failure Information Report message to the candidate or target SN, which perform the final MRO related optimisation. Otherwise, the MN performs the final MRO related optimisation in case of CPA or MN initiated CPC or initial S-CPAC/following S-CPC initiated by MN, or MN sends the SCG FAILURE INFORMATION REPORT message to source SN of following S-CPC for final MRO related optimisation in case of following S-CPC initiated by MN.

For SN initiated CPC or initial/following S-CPC initiated by SN, the MN sends the SCG Failure Information Report message to SN initiating the CPC or S-CPAC procedure, and SN initiating the CPC or S-CPAC procedure performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the SN initiating the CPC or S-CPAC procedure, but not one of the candidate PSCells selected by the candidate or target SN, the SN initiating the CPC or S-CPAC procedure indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN. Otherwise, the SN initiating the CPC or S-CPAC procedure performs the final MRO related optimisation in case of SN initiated CPC or initial/following S-CPC initiated by SN, or SN initiating the S-CPAC procedure indicates the MN to send the SCG FAILURE INFORMATION REPORT message to source SN of following S-CPC for final MRO related optimisation in case of following S-CPC initiated by SN.