3GPP TSG-RAN WG3 Meeting #126	R3-247434

Orlando, USA, 18 - 22 November, 2024

Agenda item:	13.2

Source: 	Lenovo

Title: 	General aspects of inter-CU SCG LTM

Document for:	Discussion and Approval

1. Introduction

In this contribution, we discuss the remaining issues of inter-CU SCG LTM related to RAN3 impact.

2. Discussion

The revised work item of NR mobility enhancements phase 4 at the RAN#105 meeting has the following objectives [1].

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

The WID includes the following scenarios on the NR-DC:

- Scenario 1: Inter-MN PCell change, where the configurations at the SN are either releases or reconfigured.

- Scenario 2: Inter-SN PSCell change, where the configurations at the MN are unchanged.

In our view, the scenario 2 is simpler than the scenario 1, and most of the principles proposed by inter-CU LTM without DC could be reused. In addition, the scenario 2 avoids unnecessary tight coordination between MN and SN. In contrast, the scenario 1 is more complex. For example, if the SN is released, this information should be propagated to all other candidate MNs that prepare the LTM candidate configurations relying on using the SN, to avoid a misalignment between the UE configuration and the LTM candidate configuration.

Therefore, we propose to prioritize the scenario 2 under inter-CU LTM work in Rel-19.

Proposal 1: Prioritize the case where CU is acting as SN and MN is unchanged under inter-CU LTM work in Rel-19, i.e., inter-CU SCG LTM.

For the inter-CU SCG LTM, RAN2 agreed only SN-initiated inter-SN LTM is supported in Rel-19 at RAN2#127 meeting [2].

Agreements on inter-CU LTM Inter-CU SCG LTM: - Inter-CU SCG LTM preparation can be initiated by source SN. - The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message. - For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN. - RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works. - Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19.

For the SN initiated inter-SN LTM, the source SN initially triggers the candidate cell preparation of inter-SN LTM procedure by sending a SN Change Required message to the MN. In existing S-CPAC functionality, a S-CPAC Request IE was introduced in the SN Change Required message to indicate the SN change is for S-CPAC. A similar approach could be used to enable inter-CU SCG LTM, where an indicator is introduced in the SN Change Required message to indicate the SN change is for inter-CU SCG LTM.

Proposal 2: An indicator is introduced in the SN Change Required message to indicate that the SN change is for inter-CU SCG LTM.

On receipt of the SN Change Required message, the MN requests each candidate SN to allocate resources for the UE by means of the SN Addition procedures. In existing S-CPAC functionality, a S-CPAC Request Information IE was introduced in the SN Addition Request message to indicate the SN addition is for S-CPAC preparation. A similar approach could be used to enable inter-CU SCG LTM, where an indicator is introduced in the SN Addition Request message to indicate the SN Addition is for inter-CU SCG LTM.

Proposal 3: An indicator is introduced in the SN Addition Request message to indicate that the SN addition is for inter-CU SCG LTM.

In the inter-CU LTM without DC, the source gNB generates a common LTM CSI resource configuration outside of the LTM candidate configuration and sends the common LTM CSI resource configuration to the candidate gNB. The common LTM CSI resource configuration includes the RS configurations of multiple LTM candidate cells, and it is used by the candidate gNB to prepare the LTM CSI report configuration for each candidate cell.

A similar approach could be used to enable inter-CU SCG LTM. That is, the source SN generates a common LTM CSI resource configuration and sends it to the candidate SN via MN. The candidate SN then generates the LTM CSI report configuration for each candidate cell, based on the received common LTM CSI resource configuration. This approach is also agreed by RAN2 at RAN2#127bis meeting [4].

Agreements on inter-CU LTM: - The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells. - The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN. - In order to support subsequent inter-CU SCG LTM, the MN needs to transfer, during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN.

Like the R18 S-CPAC SCG reference configuration, the common LTM CSI resource configuration could be included in the CG-Config IE (S-NG-RAN node to M-NG-RAN node Container IE) in the SN Changed Required message from the source SN to the MN, and then be included in the CG-ConfigInfo IE (M-NG-RAN node to S-NG-RAN node Container IE) in the SN Addition Request message from the MN to the candidate SN.

Proposal 4: The common LTM CSI resource configuration for inter-CU SCG LTM is included in the CG-Config IE in the SN Changed Required message from the source SN to the MN, and in the CG-ConfigInfo IE in the SN Addition Request message from the MN to the candidate SN.

At RAN2#126 meeting [3], RAN2 agreed:

Agreements on inter-CU LTM: Clarification of DC - An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

To avoid simultaneous LTM configurations at both MN and SN, a simple approach is to inform the other node that LTM is to be configured. For example, MN indicates SN about MCG LTM configuration, while SN indicates MN about SCG LTM configuration. With this information, MN or SN can avoid configuring the LTM.

Proposal 5: MN indicates SN about MCG LTM configuration, to avoid simultaneous MCG and SCG LTM configurations.

Proposal 6: SN indicates MN about SCG LTM configuration, to avoid simultaneous MCG and SCG LTM configurations.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals.

Proposal 1: Prioritize the case where CU is acting as SN and MN is unchanged under inter-CU LTM work in Rel-19, i.e., inter-CU SCG LTM.

Proposal 2: An indicator is introduced in the SN Change Required message to indicate that the SN change is for inter-CU SCG LTM.

Proposal 3: An indicator is introduced in the SN Addition Request message to indicate that the SN addition is for inter-CU SCG LTM.

Proposal 4: The common LTM CSI resource configuration for inter-CU SCG LTM is included in the CG-Config IE in the SN Changed Required message from the source SN to the MN, and in the CG-ConfigInfo IE in the SN Addition Request message from the MN to the candidate SN.

Proposal 5: MN indicates SN about MCG LTM configuration, to avoid simultaneous MCG and SCG LTM configurations.

Proposal 6: SN indicates MN about SCG LTM configuration, to avoid simultaneous MCG and SCG LTM configurations.

4. Reference

- RP-242356, Revised work item: NR mobility enhancements phase 4.

- RAN2#127 meeting minutes.

- RAN2#126 meeting minutes.

- RAN2#127bis meeting minutes.