3GPP TSG-RAN WG3 Meeting #126	R3-247438

Orlando, USA, 18 - 22 Nov, 2024

Agenda item:	16.2

Source: 	Lenovo

Title: 	(TP to TR 38.769) Protocol Stack of UP based Solution for Topology 2

Document for:	Discussion and Approval

- Introduction

This paper discusses and provides text proposal for protocol stack of UP based solution for topology 2.

- Discussion

For user-plane option, the following interim conclusions have been made as in TR 23.700-13 v1.1.0.

---------------------------------------------------------------------------------------------------------------------------------

8.1.3.2	User-plane option

The following principles apply:

-	As depicted in Figure 8.1.3.2-1, the UE Reader connects to the AIoTF based on the AIoT Application Protocol (AIoT-AP) using an IP PDU Session between the UE and the UPF as transport. The related protocol stack is shown in Figure 8.1.3.2-2. The AIoT AP protocol will support procedures and information to be exchanged as specified by RAN2, RAN3 and SA2.

Figure 8.1.3.2-1:	User-plane architecture for Topology 2

Figure 8.1.3.2-2:	Protocol stack for the user-plane architecture for Topology 2

NOTE 1:	Which transport protocol to use for AIoT-AP can be decided by CT1 in coordination with RAN3.

NOTE 2:	Security for AIoT-AP is assumed to be defined by SA3.

Editor’s note:	Further details are FFS.

Editor’s note:	The terminology, e.g. AIoT RAN is FFS.

---------------------------------------------------------------------------------------------------------------------------------

To focus on RAN side protocol, the following text proposal for protocol stack of UP based solution for topology 2 can be captured into TR 38.769:

The upper layer protocol stacks between AIOTF and UE reader, and between AIOTF and AIOT device are not shown in the figure which can refer to SA2’s TR 23.700-13 v1.1.0.

3. Conclusion

This paper discusses the protocol stack for protocol stack of UP based solution for topology 2 and we propose to capture the protocol stack into TR 38.769.

4. Text Proposal

6.4.2.1.x	Solution x: UP based solution

The A-IoT related messages between the A-IoT CN and the A-IoT-enabled UE are carried via A-IoT-enabled UE’s PDU session.

Figure 6.4.2.1.x-1: UP based solution of Topology 2