3GPP TSG-RAN WG3 #126	R3-247480

# Orlando, USA, 18th-22nd November 2024

Agenda Item:	17.3

Source:	Ericsson

Title: 	Discussion on gaps in Cell-A coverage for OD-SIB1 and automatic recovery

Document for:	Discussion and Approval

# 1	Background

A work item on “Enhancements of network energy savings for NR” was agreed in [1]. The Work Item Description (WID) was revised in [2]. According to the revised WID, the objectives of the work item are the following:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we discuss issues and solutions regarding Objective 2 of the work item (see highlighted text above), focusing on how to ensure coverage overlap of NES Cell(s) and Cell A.

# 2	Discussion

We discuss a potential issue of on-demand SIB1 (OD-SIB1) transmission, namely a scenario where the UE cannot acquire the UL WUS configuration. For reasons of simplicity, we consider non-split architecture.

## 2.1	General scenario

Figure 1 illustrates an example deployment scenario for OD-SIB1 transmission, where the “NES Cell” of gNB1 is meant to operate in OD-SIB1 transmission mode and the “Cell A” of gNB2 provides the UL Wake-Up Signal (WUS) configuration (that is needed to request SIB1 transmission in the NES Cell) to UEs. The scenario corresponds to Case 2 as defined by RAN1, assumed for normative work according to the WID.

Figure 1: An example deployment scenario for OD-SIB1 transmission (corresponding to Case 2).

We assume that, before the NES Cell operates in OD-SIB1 transmission mode, gNB1 provides the UL WUS configuration of its NES Cell to gNB2 in step 1. In step 2, gNB2 provides the UL WUS configuration of the NES Cell to UEs camping on (or connected to) Cell A. In step 3, when needed, a UE (in RRC Idle/Inactive mode) sends the UL WUS to the NES Cell and, in response, the NES Cell transmits SIB1 in step 4.

## 2.2	Discussion on indication of missing UL WUS configuration

So far, it was assumed that the NES Cell is fully covered by one other cell (Cell A), and that the NES-Cell gNB knows which cell that is, i.e., which cell must assist as Cell A to ensure that the UL WUS configuration is provided to UEs in the entire coverage area of the NES Cell. This, however, cannot be taken for granted. In reality, the coverage area of a cell typically has a very irregular shape, e.g., in cities, due to various obstructions. Besides, the coverage area of a cell can change over time, e.g., due to Coverage and Capacity Optimization (CCO).

Observation 1: Because the coverage area of a Cell A (or NES Cell) can change over time (e.g., due to performing CCO), the coverage overlap of Cell A and a NES Cell can also vary.

Figure 2 visualizes an example scenario, where a part of the coverage area of the NES Cell is not covered by Cell A. However, the missing coverage can be provided by another cell, here called Cell X. In this particular case, the UE knows about the NES Cell but cannot find (any) Cell A. Hence, it cannot camp on the NES Cell. Such incomplete Cell-A coverage of the NES Cell effectively reduces the coverage area of a NES Cell, causing suboptimal performance and a waste of energy for both network and UE, since the UE cannot connect to the nearest cell when needed.

Observation 2: A lack of Cell-A coverage in the coverage area of the NES Cell causes suboptimal performance and a waste of energy for both network and UE.

Figure 2: An example of how a missing UL WUS configuration indication can reach the NES-Cell gNB.

It is thus very important that 3GPP provides means to detect and make the NES-Cell gNB aware of an incomplete provisioning of UL WUS configuration, so that such problem can be avoided and  resolved. For this purpose, the UE needs to report to the network when the UE finds a missing UL WUS configuration, The reporting could be either via Cell X, or later via the NES Cell or Cell A, in case the UE moves into the coverage area of Cell A.

To support detection of issues with UL WUS configuration provisioning, it is essential that a UE can detect that a cell is a NES Cell, i.e., a cell using OD-SIB1 transmission (e.g., from the SSB/MIB of the cell itself). A UE must also be able to inform the network about a missing UL WUS configuration. A UE that is not able to camp on the NES Cell and out of coverage of a Cell A,  can only report the observation of a missing UL WUS configuration to the network via Cell X (gNB3 in Figure 2), even though only gNB1 can improve the situation, e.g., by initiating Cell X to provide UL WUS configuration. As depicted in Figure 2, after Cell X is made aware of an incomplete UL WUS configuration provisioning, the Cell-X gNB can send the indication of a missing UL WUS configuration to the NES-Cell gNB, so that the NES-Cell gNB can take necessary action to improve the situation, e.g., by selecting more cells to act as Cell A, or turning off OD-SIB1 transmission. Such inter-node signalling is in RAN3 scope and should be already addressed in Rel-19.

Furthermore, such inter-node signalling is also necessary for enabling automatic Cell-A relations. For example, an operator may deliberately deploy NES cells without manually configuring the NES-Cell/Cell-A relations, and through the missing UL WUS configuration reporting, the relations are automatically set up by the different NG-RAN nodes. This can simplify NES-Cell deployment for the operator. Note, however, to enable this automatic function, the NES cell must be able to identify itself as a NES cell.

In RAN1#118bis, RAN1 made the following agreement:

Agreement

UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A

This agreement not only makes it difficult for a UE (as exemplified in Figure 2) to detect faulty configuration if a Cell A has not been visited but also hinders the automatic NES-Cell deployment.

Observation 3: Inter-node signaling concerning missing UL WUS configuration provisioning is needed for a NES-Cell gNB to detect and remedy lack of Cell A coverage. This is in RAN3 scope and should be addressed in Rel-19.

Observation 4: Inter-node signaling concerning missing UL WUS configuration provisioning can also be used for automatic NES-Cell/Cell-A relation establishment and can simplify NES-Cell deployment for the operator.

Observation 5: A NES Cell needs to identify itself as a NES cell without relying on a Cell A so that UEs can efficiently detect missing UL WUS configuration, and so that automatic NES-Cell/Cell-A relations can be set up.

Proposal 1: RAN3 to conclude the scenario of incomplete UL WUS configuration provisioning due to incomplete coverage of a NES Cell by a Cell A(s) should be tackled in Rel-19.

Proposal 2: RAN3 to discuss inter-node signaling for indicating to a NES-Cell gNB that a missing UL WUS configuration event has occurred.

Proposal 3: RAN3 to support signaling that enables automatic deployment of NES cells and setup of NES-Cell/Cell-A relations.

Proposal 4: RAN3 to support NES Cell identifying itself as a NES cell without having to rely on a Cell A. Send LS to RAN1 and RAN 2 for NES Cell self-identification.

# 3	Proposals

In this contribution, we discussed issues and high-level solutions for Objective 2 of the work item mentioned above and made the following observations and proposals:

Observation 1: Because the coverage area of a Cell A (or NES Cell) can change over time (e.g., due to performing CCO), the coverage overlap of Cell A and a NES Cell can also vary.

Observation 2: A lack of Cell-A coverage in the coverage area of the NES Cell causes suboptimal performance and a waste of energy for both network and UE.

Observation 3: Inter-node signaling concerning missing UL WUS configuration provisioning is needed for a NES-Cell gNB to detect and remedy lack of Cell A coverage. This is in RAN3 scope and should be addressed in Rel-19.

Observation 4: Inter-node signaling concerning missing UL WUS configuration provisioning can also be used for automatic NES-Cell/Cell-A relation establishment and can simplify NES-Cell deployment for the operator.

Observation 5: A NES Cell needs to identify itself as a NES cell without relying on a Cell A so that UEs can efficiently detect missing UL WUS configuration, and so that automatic NES-Cell/Cell-A relations can be set up.

Proposal 1: RAN3 to conclude the scenario of incomplete UL WUS configuration provisioning due to incomplete coverage of a NES Cell by a Cell A(s) should be tackled in Rel-19.

Proposal 2: RAN3 to discuss inter-node signaling for indicating to a NES-Cell gNB that a missing UL WUS configuration event has occurred.

Proposal 3: RAN3 to support signaling that enables automatic deployment of NES cells and setup of NES-Cell/Cell-A relations.

Proposal 4: RAN3 to support NES Cell identifying itself as a NES cell without having to rely on a Cell A. Send LS to RAN1 and RAN 2 for NES Cell self-identification.

# 4	References

[1]	RP-234065	New WID: Enhancements of network energy savings for NR

[2]	RP-242354	Revised WID: Enhancements of network energy savings for NR

[3]	R3-245737 	Summary of offline disc at RAN3#125-bis, Ericsson (Moderator)