3GPP TSG-RAN WG3 #126																		       R3-247483

Orlando, USA, 18th – 22nd November 2024

Agenda Item:	13.2

Source:	Ericsson

Title:	Comprehensive analysis of inter-CU LTM

Document for:	Discussion, Agreement

# Introduction

In this contribution, we continue to discuss XnAP signaling design for inter-CU LTM based on the agreements and open points captured at previous RAN3 meeting. The corresponding TP for XnAP capturing the proposals is in R3-247484.

Preparation phase

Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase.

WA: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

The error handling of multiple UE associations need to be considered.

Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

How the source gNB-CU sends the reference configuration to all candidate gNBs is pending on RAN2 progress.

Confirm the name of the new procedure as “LTM Configuration Update”.

Introduce a new procedure for candidate gNB-initiated LTM cancellation.

The LTM Configuration IDs are allocated by the source CU.

Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. When and how to establish the UE association is FFS.

Early Synchronization phase

WA: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

Furthermore, RAN3 received an LS from RAN2 in R3-247111, including a number of agreements related to NR-DC support for inter-CU LTM.

RAN2#127 agreements:

Agreements on inter-CU LTM (…) Inter-CU SCG LTM: 10. Inter-CU SCG LTM preparation can be initiated by source SN. 11. The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message. 12. For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN. 13. RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works. 14. Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19. Inter-CU MCG LTM: 15. SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration). 16. Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied). 17. For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

RAN2#127-bis agreements:

Agreements on inter-CU LTM (…) 3. The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3. 4. Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline: - The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part. - FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations. - The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message. - Upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed. - The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration. - The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part. - The MN can request an SCG reference configuration from any of the involved SNs. 5. For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN. 6. The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells. 7. The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN. 8. In order to support subsequent inter-CU SCG LTM, the MN needs to transfer ,during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN. 9. Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message. 10. Re-use legacy LTM Cell Switch Command MAC CE for inter-SN LTM. 11. RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen: - Inter-MN LTM and intra-SN LTM - Inter-SN LTM and intra-MN LTM

Consequently, in this contribution, we also assess the network signaling necessary to support inter-CU LTM with NR-DC.

# Discussion

## Initial LTM configuration

At previous meeting, the following was captured with respect to initial LTM configuration:

WA: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

The error handling of multiple UE associations need to be considered.

Let’s assume that separate Handover Preparation procedures are used for each LTM candidate cell and that we have 2 candidate cells in one candidate gNB. When the source CU sends the first HANDOVER REQUEST message, it assigns a first Source NG-RAN node UE XnAP ID and receives from the candidate gNB the corresponding Target NG-RAN node UE XnAP ID. At this point, a first UE association is established, pointing to the first LTM candidate cell (first LTM cell relation). The source CU now sends the second HANDOVER REQUEST for the same UE, and the candidate gNB could assign the same or a different Target NG-RAN node UE XnAP ID. With different gNB implementations, nothing precludes that different pairs of “Source NG-RAN node UE XnAP ID, Target NG-RAN node UE XnAP ID” are chosen for the same UE. If that happens, there will be a second UE association established for the same UE, pointing to the second LTM candidate cell (second LTM cell relation).

Continuing with the example, now we have two established UE associations, each one tightly coupled with one LTM candidate cell, and based on the current Xn baseline where the LTM Configuration Update is executed for one specific UE association (see tabular below), every time an update is needed that concerns the UE, which UE association is to be used? And, if an update is needed for all the LTM candidate cells of one candidate gNB, multiple LTM Configuration Updates are needed. Since all the UE associations are pointing to the same UE, this leads to redundant signaling and very complex error handling.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1 | YES | reject |
| NG-RAN node 2 UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2 | YES | reject |
| Target Cell Global ID (FFS) | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| Early Sync Information | O |  | FFS |  | YES | ignore |
| Early Sync Information for SUL | O |  | FFS |  | YES | ignore |
| LTM Information Request | O |  | 9.2.1.xx1 |  | YES | ignore |

At previous meeting we have proposed an alternative, with a single Handover Preparation procedure to establish one UE association and multiple LTM candidate cells for one UE in one round.

As a compromise, we propose another way to avoid the complex error handling, and still use one Handover Preparation to prepare only one LTM candidate cell. When the first Handover Preparation is done, to prepare the first LTM candidate cell controlled by the target gNB, the UE association is established and the pair “Source NG-RAN node UE XnAP ID, Target NG-RAN node UE XnAP ID” is created. When the following Handover Preparation procedures are done, to prepare the other LTM candidate cells controlled by the same target CU (second, third, etc.), the source CU signals to the target CU the Target NG-RAN node UE XnAP ID already assigned by the target CU for the UE. With this information, the target CU can understand that there is an already existing UE association for the UE and avoids assigning a new Target NG-RAN node UE XnAP ID. It seems therefore possible, to resolve the error handling concerning multiple UE associations in stage 3.

Therefore, we propose to confirm the WA “For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.” and resolve the error handling concerning multiple UE associations in stage 3.

- Turn the WA into an agreement to use one Handover Preparation procedure to manage one LTM candidate cell, and resolve the error handling concerning multiple UE association in the stage 3.

The following agreement from last RAN3 meeting: “Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.” is captured in the TP for Xn in the target CU.

Regarding the FFS: “How the source gNB-CU sends the reference configuration to all candidate gNBs is pending on RAN2 progress.” according to the incoming LS from RAN2 in R3-247111, RAN2 agreed to support only one reference configuration for inter-CU LTM:

Agreements on inter-CU LTM Reference configuration: 1. Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported. CSI resource and report configuration: 2. The Rel-18 signaling structure for LTM CSI resource and report configuration is reused for inter-CU LTM, i.e. a common CSI resource configuration and cell-specific CSI report configuration. 3. The source CU is responsible to generate the common CSI resource configuration. (…)

Based on that, we consider the FFS resolved, and we have captured the corresponding Xn impact in R3-247484.

- RAN3 to reuse a single reference configuration for the XnAP signaling.

## General principles for Early UL Sync signaling design

We think the following general principles should guide our Xn signaling design for Early UL Sync in case of inter-CU LTM:

- A first general principle is that Xn signaling is independent from gNB internal architecture. From a logical perspective, a peer gNB is seen as a “black-box”, i.e. a gNB cannot assume that a peer gNB is a split gNB or a non-split gNB. For this reason, the Xn signaling between gNBs is (and must be kept) unaware of the logical nodes constituting the gNBs themselves (gNB-DU, gNB-CU). In LTM rel-18 solution, F1 signaling has been enhanced to include IEs directly associated to one of the logical nodes of a gNB (e.g., “LTM gNB-DU ID”), which is a legitimate choice in F1 signaling design. The presence of the LTM gNB-DU ID IE allows e.g. the source DU to associate a list of Preamble Index to a certain candidate DU. A similar mechanism is needed for inter-CU LTM, but the source DU does not need to know that certain Preamble Indexes are associated to a certain candidate DU. What the source DU needs to know is that a certain group of Preamble Indexes can be treated independently from another group of Preamble Indexes. To achieve that a new identifier can be used, that will essentially identify the “provider” of the information. In that way we keep the Xn signaling independent of the gNB internal architecture i.e., whether a gNB is a split gNB or a non-split gNB. For this reason, for the purpose of requesting and providing Early UL Sync configuration via Xn, we support to introduce a new identifier, e.g., an “Early UL Sync Configuration Provider ID”.

- For inter-CU LTM, Xn has no notion of the logical nodes of a gNB (gNB-DU, gNB-CU).

- Keep the existing principle that Xn AP Identities and F1 AP Identities are independent.

- For inter-CU LTM, transfer via Xn a new identifier (e.g., an “Early UL Sync Configuration Provider ID”).

- A second general principle stems from the functionality (inter-CU LTM) that we intend to support. LTM aims to minimize mobility interruption times and in doing so, it makes use of some information (e.g., TA values) that can be short-lived. Therefore, transfer of such information should avoid unnecessary delay. In LTM rel-18 solution, F1 signaling has been enhanced to transfer all the TA values for all the candidate cells in a single message between CU and source DU (TS 38.401, Figure 8.2.1.5-1: Inter-gNB-DU LTM, step 15). The procedures to transfer TA values are non-UE associated. This means that if RACH-less LTM is wanted, the source DU will have to wait before sending the LTM Cell Switch Command to a certain UE until all the candidate DUs relevant for that UE have provided their corresponding TA value to the CU. For intra-CU LTM, this solution was deemed acceptable. With inter-CU LTM, there is an additional delay to account for, which is the sending of TA values from candidate gNB to source gNB via Xn, plus - if the candidate gNB is a split-gNB - the F1 delay between the candidate DU and the candidate CU. If a candidate gNB has multiple candidate LTM cells controlled by the same candidate DU, before the candidate cells’ TA values reach the source DU, we have 3 delays to consider: 1) the delay of the existing F1 procedure running at the candidate gNB which groups the TA values of the candidate cells potentially associated to multiple UEs and transfer them to the candidate CU, 2) the Xn delay, and 3) the delay of the existing F1 procedure running at the source gNB to transfer the TA values from the source CU to the source DU for potentially multiple UEs. In our view, the Xn signaling should at least allow to send the TA values of certain UE without having to wait for other TA values associated to other UEs. This is a change compared to F1 signaling but can help to avoid unnecessary delays in transferring TA values which can become outdated before they are used.

- For inter-CU LTM, unnecessary delay should be avoided over Xn when designing the signalling (e.g., to transfer TA values).

A certain TA value is always associated to only one UE, therefore the UE associated identities assigned by the source gNB and the candidate target gNB should be included in the message, to uniquely identify the UE to which the TA value corresponds to.

- Include the UE XnAP IDs of the source and target gNB in the message for TA information transfer.

Some companies proposed to use a non-UE associated message to transfer more than one TA values in one go. However, if this approach is used, we will introduce an unnecessary delay to the different times at which each individual TA value to be included in the message becomes available. For example, if a non-UE associated message is used to transfer 8 TA values, there is an additional delay introduced, from the time when the 1st TA value becomes available, until the time when the 8th TA value becomes available. With a non-UE associated procedure, the same message can contain multiple TA values, but not necessarily for the same UE, so the delay until the last TA value for a specific UE is transferred can increase significantly. If the TA values cannot be used because they are no longer valid, the advantage of Early UL synch is completely lost. If we use one UE-associated procedure instead, and we use it to transfer a single UE value, there is no extra delay, at the expense of an increased number of messages to be sent over Xn. As a compromise we could define a UE-associated procedure where multiple TA values for the same UE (and related to different cells) are grouped. So, even if the existing F1 messages (DU-CU/DU-CU TA Information Transfer) used to transfer TA values are non-UE associated, reusing the same approach for Xn can nullify the Early UL Sync advantage, and to avoid that, we prefer to use a UE-associated procedure to transfer TA values for multiple cells.

- The Xn procedure to transfer a TA information is UE-associated and is used to transfer a list of TA values, each TA value for a different LTM candidate cell.

Regarding whether the procedure should be a class 1 or a class 2, in this case we think the same approach used in F1 can be kept, i.e. use a class 2 procedure.

- The Xn procedure to transfer a TA information is a class 2 procedure.

## LTM cancel

At last meeting it was agreed to “Introduce a new procedure for candidate gNB-initiated LTM cancellation.”. We propose to confirm that the new procedure is named “LTM Handover Cancel” as reported in the current Baseline.

- The name of the new procedure for candidate gNB-initiated cancellation is LTM Handover Cancel.

## Subsequent LTM and Release of UE Context at source gNB

At last meeting the following WA was captured: “WA: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.”, which we propose to confirm.

- Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

## LTM configuration update

At the last RAN3 meeting it has been agreed that, to support subsequent LTM, UE association in-between candidate CUs is allowed when Xn connectivity exists. It is FFS when and how to establish the UE association:

Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. When and how to establish the UE association is FFS.

We think that after LTM execution, the LTM Configuration Update procedure can be used to support subsequent LTM in setting up the UE association over Xn (pair of XnAP UE IDs). We note that, during LTM preparation, even if the source gNB (e.g., gNB1) can contact via Xn each target candidate gNB (e.g., gNB2, gNB3, gNB4), i.e., even if Xn is available between the source gNB and each candidate gNBs, the source gNB cannot know whether Xn is available between each pair of candidates gNBs (e.g., gNB1 cannot know if Xn is available between gNB2 and gNB3). To support subsequent LTM at network side, setting up of the UE association between two nodes makes practical sense only when one of them serves the UE. For example, when gNB2 receives from gNB1 the CELL SWITCH NOTIFICATION message, it knows that from now on it serves the UE, and can initiate the setup of the UE association for the UE between itself and gNB3 (assuming Xn is available between gNB2 and gNB3). In case (for any reason) no Xn connectivity is available between these two nodes, the UE association should not be established, and subsequent LTM is not supported over Xn between gNB2 and gNB3. As just described, setting up the UE association over Xn is crucial to support subsequent Xn-based LTM inter-CU, so we think the use of the newly agreed procedure for this purpose is a natural choice. Using this procedure, the new source gNB chooses a UE XnAP ID and sends it to a candidate gNB in the initiating message, and the candidate gNB sends its assigned UE XnAP ID to the source gNB in the terminating message. When the procedure is executed successfully, the UE association is in place.

- To support subsequent LTM in the RAN, the LTM Configuration Update is used to allocate UE XnAP IDs between the new source gNB and the candidate gNBs.

Another aspect in which the LTM Configuration Update procedure can support subsequent LTM in the RAN is for retrieving the Preamble Index allocated for a UE. To do so, the source gNB can request the candidate gNBs to provide Preamble Index, which is returned in the response message.

- To support subsequent LTM in the RAN, the source gNB can use the LTM Configuration Update to request Preamble Index for a UE to the candidate gNBs.

- Agree on the TP for XnAP in R3-247484 that captures the above proposals.

## Considerations on data forwarding

LTM offers improvements in handover latency and interruption time compared to Layer 3 based mobility, something we need to ensure for inter-CU LTM.

Regarding the normal data forwarding we can make the following observations. In contrast to CHO, the LTM cell switch is initiated by the source gNB and not the UE, so the source gNB can initiate data forwarding as soon as it sends the LTM Cell Switch Command to the UE. Alternatively, the source gNB can wait until it receives the HANDOVER SUCCESS message. We believe that this should be left up to implementation. So, we propose that normal data forwarding may be initiated after the source gNB sends the LTM Cell Switch Command to the UE, but when exactly it is initiated, it is left to implementation.

- Normal data forwarding may be initiated after the source gNB sends the LTM Cell Switch Command to the UE, when exactly it is initiated is left to implementation.

## Security aspects for inter-CU LTM

We report below the text of “Reply LS on security handling for inter-CU LTM in non-DC cases” received by RAN2 in R2-2409534, where RAN3 is in copy.

SA WG3 has been considering the security mechanisms proposed for the inter-CU LTM feature. For this feature, SA WG3 want to preserve the principle that when there is a PDCP anchor change, then it is necessary to perform a key change (i.e. for solutions or handovers that do not require a PDCP anchor change, there is no need to provide rekeying). The rekeying method preferred by most companies in SA WG3 is that after every inter-CU LTM cell switch execution that changes the PDCP anchor, the UE is provided with the NCC value via RRC signalling to be used by the UE for key derivation at the next inter-CU LTM cell switch. This solution was preferred as it provides the NCC value using the protected RRC signalling and hence is considered to be secure. SA WG3 understands that, RAN WG2 is aiming to avoid RRC configuration/signalling between cell switches for subsequent LTM procedures. If it is necessary for the RAN WGs to avoid RRC signalling between cell switches, then SA WG3 can analyse further methods like, - Security threat of the unprotected MAC CE to deliver the NCC value. - Sending NCC value per candidate cell via RRC message during LTM preparation phase. SA WG3 will keep RAN WG2 and RAN WG3 informed as we make further progress on this topic.

According to the suggestion from SA3, Option 4 leverages the existing design of L3 handover. With each inter-CU LTM cell switch in the existing network architecture, the NCC value can be sent to the UE via RRC signaling for secure key derivation at the next switch, without any security concerns. Once the UE receives the NCC, it’s ready for another inter-CU LTM cell switch with security derivation.

Since the LTM candidate cell configurations are not modified, this approach also respects the principle of subsequent LTM and aligns with WID requirements. Providing the NCC after each cell switch keeps the LTM cell configuration stable and reuses the existing path switch procedure over the NG interface. When the NCC, NH pair is updated, the NCC is sent to the UE via RRC signaling, ensuring smooth preparation for subsequent cell switches.

Thus, providing the NCC after every inter-CU LTM cell switch is a strong approach for inter-CU LTM, and we recommend agreement on this preferred option from SA3.

Also, based on the above, it’s clear that discussions in SA3 are still ongoing. Consequently, RAN2’s assessment of security-related impacts is also not yet complete. For instance, Option 1 may require prefetching security information from the AMF, and there are still security concerns if the NCC is used in an unprotected MAC CE. Given the current uncertainties in RAN2 and SA3, RAN3 is not yet in a position to progress on evaluating the network signaling impact. We suggest holding off on further discussions in RAN3 until there is additional guidance from RAN2 and SA3.

- For inter-CU LTM, defer the discussion on network signaling impacts related to security until RAN2 makes progress.

## Inter-CU LTM with NR-DC

Based on the RAN2 agreements, we think RAN3 should now start to evaluate the overall signaling required to support inter-CU LTM for NR-DC. We propose to start discussing the scenarios below:

- Inter-MN LTM without SN Release

- Inter-MN LTM with SN Release

- SN LTM initiated SN Change

- RAN3 to discuss the following scenarios to support inter-CU LTM with NR-DC.

    - Inter-MN LTM without SN Release, with/without intra-SN LTM

    - Inter-MN LTM with SN Release

    - SN LTM initiated SN Change

The sections below illustrate a signaling overview for the proposed scenarios.

### Inter-MN LTM without SN Release

The Figure 1 below shows a signaling flow for Inter-MN LTM without SN Release. To avoid that inter-SN LTM is configured together with inter-MN LTM, the inter-SN LTM configuration is released. At the same time the intra-SN LTM configuration can be kept, or released, or modified.

Figure 1 - Inter-MN LTM without SN Release

The procedure for Inter-MN LTM without SN Release is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source MN containing measurements of neighbouring cells.

2.	The source MN decides to configure inter-gNB LTM.

3.	The source MN requests inter-gNB LTM for one or more candidate cells belonging to one or more candidate MN(s). The source MN initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and the CSI resource configuration for subsequent LTM.

3a/3b.	The candidate target MN sends an S-NODE ADDITION REQUEST to the source SN to request the release of the inter-SN LTM configuration (if any) and to keep, or release, or modify the intra-SN LTM configuration (if any). The source SN can send the modified intra-SN LTM configuration to the candidate target MN in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

4.	The candidate target MN prepares the LTM configuration(s) and sends inter-gNB response (HANDOVER REQUEST ACKNOWLEDGE) to the source MN including the generated RRC configurations for the accepted candidate cell.

5.	The source MN sends an LTM CONFIGURATION UPDATE message to the candidate MN(s) to update the LTM configurations of candidate cell(s).

6.	The candidate MN(s) sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source MN.

7.	The source MN sends an RRCReconfiguration message to the UE.

8.	The UE stores the LTM candidate configurations and sends an RRCReconfigurationComplete message to the source MN.

8a.	If early data forwarding is applied, the source MN sends the EARLY STATUS TRANSFER message.

9/10. Early synchronization to some LTM candidate cell(s) belonging to the candidate MN(s) may be performed. The candidate MN(s) sends the TA INFORMATION TRANSFER message to the source MN if early TA acquisition is performed to the candidate MN(s).

11.	 The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source MN. L1 measurement should be performed as long as RRC reconfiguration (step 7) is applicable.

12.	The source MN determines to initiate inter-gNB LTM.

13.	The source MN decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

14.	The source MN sends the CELL SWITCH NOTIFICATION message to the target MN to indicate the initiation of Cell Switch command to the UE.

15.	The target MN detects the UE access. The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

16.	The UE sends an RRCReconfigurationComplete message to the target MN.

17.	The target MN sends an S-NODE RECONFIGURATION COMPLETE to the source SN.

18/19. The target MN sends the HANDOVER SUCCESS message to the source MN to inform that the UE has successfully accessed the target cell. In return, the source MN sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE :	Late data forwarding may be initiated as soon as the source MN receives the HANDOVER SUCCESS message.

20. The target MN may send the UE CONTEXT RELEASE to inform the source MN to release radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

### Inter-MN LTM with SN Release

The Figure 2 below shows a signaling flow Inter-MN LTM with SN Release. In this example, the source SN rejects the addition from the target MN and the source SN is released.

Figure 2 - Inter-MN LTM with SN Release

The procedure for Inter-MN LTM with SN Release is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source MN containing measurements of neighbouring cells.

2.	The source MN decides to configure inter-gNB LTM.

3.	The source MN requests inter-gNB LTM for one or more candidate cells belonging to one or more candidate MN(s). The source MN initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and the CSI resource configuration for subsequent LTM.

3a/3b.	The candidate target MN sends an S-NODE ADDITION REQUEST to the source SN, which rejects the request and sends to the target MN the S-NODE ADDITION REQUEST REJECT message.

4.	The candidate target MN prepares the LTM configuration(s) and sends inter-gNB response (HANDOVER REQUEST ACKNOWLEDGE) to the source MN including the generated RRC configurations for the accepted candidate cell.

5.	The source MN sends an LTM CONFIGURATION UPDATE message to the candidate MN(s) to update the LTM configurations of candidate cell(s).

6.	The candidate MN(s) sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source MN.

7.	The source MN sends an RRCReconfiguration message to the UE.

8.	The UE stores the LTM candidate configurations and sends an RRCReconfigurationComplete message to the source MN.

8a.	If early data forwarding is applied, the source MN sends the EARLY STATUS TRANSFER message.

9/10. Early synchronization to some LTM candidate cell(s) belonging to the candidate MN(s) may be performed. The candidate MN(s) sends the TA INFORMATION TRANSFER message to the source MN if early TA acquisition is performed to the candidate MN(s).

11.	 The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source MN. L1 measurement should be performed as long as RRC reconfiguration (step 7) is applicable.

12.	The source MN determines to initiate inter-gNB LTM.

13.	The source MN decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

14.	The source MN sends the CELL SWITCH NOTIFICATION message to the target MN to indicate the initiation of Cell Switch command to the UE.

15.	The target MN detects the UE access. The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

16.	The UE sends an RRCReconfigurationComplete message to the target MN.

17/18. The target MN sends the HANDOVER SUCCESS message to the source MN to inform that the UE has successfully accessed the target cell. In return, the source MN sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE :	Late data forwarding may be initiated as soon as the source MN receives the HANDOVER SUCCESS message.

19/20. The target MN sends the S-NODE RELEASE REQUEST message to the source SN, which responds with an S-NODE RELEASE REQUEST ACKNOWLEDGE.

21. The target MN may send the UE CONTEXT RELEASE to inform the source MN to release radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

### SN LTM Initiated SN Change

The Figure 3 below shows a signaling flow for SN LTM initiated SN Change. To avoid that inter-SN LTM is configured together with inter-MN LTM, the inter-MN LTM configuration is released. At the same time the intra-SN LTM configuration can be kept, or released, or modified.

Figure 3 - SN LTM initiated SN Change

The procedure for SN LTM initiated SN Change is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source MN containing measurements of neighbouring cells.

2.	The source MN forwards the L3 measurement results to the source SN.

3.   The source SN decides to configure inter-gNB LTM.

4.	The source SN sends to the source MN a S-NODE CHANGE REQUIRED to request for inter-gNB LTM for one or more candidate cells belonging to one or more candidate target SN(s). The inter-MN LTM configuration is released (if any).

5.	The source MN sends an S-NODE ADDITION REQUEST with the request of the source SN for inter-gNB LTM for one or more candidate cells belonging to one or more candidate target SN(s).

6.	The candidate target SN prepares the LTM configuration(s) and sends inter-gNB response (S-NODE ADDITION REQUEST ACKNOWLEDGE) to the source MN including the generated RRC configurations for the accepted candidate cell.

7.	The source MN sends an S-NODE MODIFICATION REQUEST message to the source SN(s) to update the LTM configurations of candidate cell(s).

8.	The source SN(s) sends an S-NODE MODIFICATION REQUEST ACKNOWLEDGE message to the source MN.

9.	The source MN sends an RRCReconfiguration message to the UE.

10.	The UE stores the LTM candidate configurations and sends an RRCReconfigurationComplete message to the source MN.

11.	The source MN sends an S-NODE CHANGE CONFIRM to the source SN.

12-14. Early synchronization to some LTM candidate cell(s) belonging to the candidate SN(s) may be performed. The candidate SN(s) sends the TA INFORMATION TRANSFER message to the source MN if early TA acquisition is performed to the candidate SN(s). The source MN sends the TA INFORMATION TRANSFER message to the source SN.

15.	 The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source SN. L1 measurement should be performed as long as RRC reconfiguration (step 9) is applicable.

16.	The source SN determines to initiate inter-gNB LTM.

17.	The source SN decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

18.	The target SN detects the UE access. The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

19.	The UE sends an RRCReconfigurationComplete message to the source MN.

20.	The source MN sends an S-NODE RECONFIGURATION COMPLETE to the target SN.

- RAN3 to agree the above signaling flows and turn it into stage-2 TP for support of inter-CU LTM with NR-DC.

# Conclusions and Proposals

In the previous sections we made the following observations and proposals:

Observation 1	For inter-CU LTM, Xn has no notion of the logical nodes of a gNB (gNB-DU, gNB-CU).

In this paper we propose:

Proposal 1	Turn the WA into an agreement to use one Handover Preparation procedure to manage one LTM candidate cell, and resolve the error handling concerning multiple UE association in the stage 3.

Proposal 2	RAN3 to reuse a single reference configuration for the XnAP signaling.

Proposal 3	Keep the existing principle that Xn AP Identities and F1 AP Identities are independent.

Proposal 4	For inter-CU LTM, transfer via Xn a new identifier (e.g., an “Early UL Sync Configuration Provider ID”).

Proposal 5	For inter-CU LTM, unnecessary delay should be avoided over Xn when designing the signalling (e.g., to transfer TA values).

Proposal 6	Include the UE XnAP IDs of the source and target gNB in the message for TA information transfer.

Proposal 7	The Xn procedure to transfer a TA information is UE-associated and is used to transfer a list of TA values, each TA value for a different LTM candidate cell.

Proposal 8	The Xn procedure to transfer a TA information is a class 2 procedure.

Proposal 9	The name of the new procedure for candidate gNB-initiated cancellation is LTM Handover Cancel.

Proposal 10	Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

Proposal 11	To support subsequent LTM in the RAN, the LTM Configuration Update is used to allocate UE XnAP IDs between the new source gNB and the candidate gNBs.

Proposal 12	To support subsequent LTM in the RAN, the source gNB can use the LTM Configuration Update to request Preamble Index for a UE to the candidate gNBs.

Proposal 13	Agree on the TP for XnAP in R3-247484 that captures the above proposals.

Proposal 14	Normal data forwarding may be initiated after the source gNB sends the LTM Cell Switch Command to the UE, when exactly it is initiated is left to implementation.

Proposal 15	For inter-CU LTM, defer the discussion on network signaling impacts related to security until RAN2 makes progress.

Proposal 16	RAN3 to discuss the following scenarios to support inter-CU LTM with NR-DC.

Proposal 17	RAN3 to agree the above signaling flows and turn it into stage-2 TP for support of inter-CU LTM with NR-DC.