3GPP TSG-RAN WG3 #126	R3-247494

Orlando, FL, U.S.A., Nov 18-22, 2024

Agenda Item:	10.2

Source:	Ericsson

Title:	(TP for BL CR to 38.300 for SON) MRO enhancements for LTM

Document for:	Discussion and Decision

# 1	Introduction

One of the main objectives of the SONMDT Rel-19 WID [RP-234038] is to enhance the MRO features to optimize the LTM cell switch procedure.

MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

This contribution discusses considerations regarding MRO feature enhancements for LTM and for and CHO with candidate SCGs.

# 2	MRO enhancements for LTM

During RAN3#123-bis and RAN3#124, the following agreements were taken on MRO for LTM:

- Work on scenarios of near failure LTM

- Work on scenarios for the differentiation of too early LTM, too late LTM and LTM to wrong cell

- RAN3 takes RAN2#125-bis agreement on MRO for LTM scenarios as baseline for further study.

During RAN3#125, the following agreements were taken on MRO for LTM:

Reuse the existing connection failure definition of too late handover, too early handover, handover to wrong cell for LTM failure case. Add a sentence for LTM handover.

FFS to enhance failure scenario.

Take the existing detection mechanism descriptions as baseline.

For failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report to the DU responsible for the failure.

CU needs to forward the RLF related information to DU using existing Access and Mobility Indication procedure. FFS for additional information needed.

During RAN3#125bis, the following agreements were taken on MRO for LTM:

For TP to BL CR to TS 38.300:

- Remove editor’s note 1 and 2. No need to capture LTM recovery in the failure definitions.

- For detection mechanism, use same approach as for CHO i.e. reuse existing sub-bullets to add LTM.

- Change“successful handover" definition, as proposed in R3-245230.

For TP to BL CR to TS 38.401:

- Capture wrong LTM prepared cell selection.

Add failure type to F1AP ACCESS AND MOBILITY INDICATION message.

CU does not forward the RLF Report to DU if failure is due to wrong selection of prepared LTM cell.

What additional information need to be added to F1AP Access and Mobility Indication message?

- C-RNTI?

- ID of the source cell, target LTM cell and re-established cell have already been included in RLF report from UE

The next sections will discuss the scenarios to be discussed in RAN3.

## 2.1	LTM scenarios

During RAN3#126, the following text proposal to 38.300 was agreed:

##### 15.5.2.2.2 Connection failure due to intra-system mobility One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows: - Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell. - Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell. - Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell. In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure. In case of LTM cell switch, the "successful handover" in the definitions above refers to a successful completion of the RA procedure in case of RACH-based LTM or the successful reception of RRCReconfigurationComplete in case of RACH-less LTM. In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell. In case of LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell. Detection mechanism A failure indication may be initiated after a UE attempts to re-establish the radio link connection at NG-RAN node B after a failure at NG-RAN node A. NG-RAN node B may initiate the Failure Indication procedure towards multiple NG-RAN nodes if they control cells which use the PCI signalled by the UE during the re-establishment procedure. The NG-RAN node receiving this selects the UE context that matches the received Failure Cell ID and C-RNTI, and, if available, uses the shortMAC-I to confirm this identification, by calculating the shortMAC-I and comparing it to the received IE. A failure indication may also be sent to the node last serving the UE when the NG-RAN node fetches the RLF REPORT from UE by triggering: - The Failure Indication procedure over Xn; - The Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG. The detailed detection mechanisms for too late handover, too early handover and handover to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure: - Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO/LTM is configured but the CHO/LTM execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt). - Intra-system Too Early Handover: there is a recent handover/LTM cell switch for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell/the cell UE attempts LTM recovery is the cell that served the UE at the last handover/LTM initialisation or fall back to the source cell configuration in case of DAPS HO. - Intra-system Handover to Wrong Cell: there is a recent handover/LTM cell switch for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO recovery/the cell UE attempts LTM recovery is neither the cell that served the UE at the last handover/LTM initialisation nor the cell that served the UE where the RLF happened or the cell that the handover/LTM was initialized toward. The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO/LTM execution until connection failure.

As mentioned initially in R3-215230, the definition of “successful handover” in case LTM switch is different from the existing definition in TS 38.300, clause 9.2.3.5.2. This clause says: “For RACH-less LTM, the UE considers that LTM cell switch execution is successfully completed when the UE determines that the network has successfully received its first UL data.” Our definition should refer to this clause or copy the clause. This was almost agreed last meeting but finally left out. However, it is safer to not have two different definitions, even if the purpose is different.

Correct the definition of ”successful handover” in the LTM switch case. See TP in Annex 1.

## 2.2 	Successive failure cases.

During the RAN3#124 meeting, successive failure cases were discussed. The following figure was contributed to discuss further scenarios:

We analyse the figure as follows:

- Case a is already covered, as it is a failure with reestablishment.

- Case b is already covered, as it is a failure with a failed recovery.

- Case c is already covered, as it is a failure with a successful recovery.

- Case d has not been discussed in the context of LTM. It consists of a failure with a successful recovery, quickly followed by another failure and a reestablishment. This case has been discussed for CHO, and there is a solution defined for CHO. In this solution, the second RLF Report does not log the source cell ID, the time to connection failure and the last handover type. The reason for not logging those is that the report could then mistakenly be interpreted as describing a too early cell switch failure.

As case d has very similar properties to the same case for CHO, we propose to copy the solution defined for CHO (i.e. keep logging the source cell ID, time to connection failure and last handover type from the first failure).

For the case of LTM failure with a subsequent successful LTM recovery, do not log the source cell ID, the time to connection failure and the last handover type.

## 2.3 	Beam Failure Recovery after successful LTM cell switch

In LTM, the LTM Cell Switch command from the network to the UE includes a beam indication and an indication of an LTM candidate cell, based on which the UE accesses the indicated LTM candidate cell. That “beam indication” is specified as a Transmission Configuration Identifier (TCI) State Identity (ID) associated to the LTM candidate cell.

In a typical case, the network determines the beam to be included based on lower layer measurement reports from the UE (also called L1 measurement reports for LTM), which includes measurement per beam (SSB measurements, such as L1 RSRP for one or more SSBs). Since these measurements may not be layer 3 (L3) filtered, the radio conditions may change quite quickly, so it might not always be simple for the network to determine the optimal beam to indicate to the UE in an LTM cell switch. TCI states are sent from target DU to source DU, via the CU, at LTM preparation. There can be up to 128 TCI states signaled to the source DU. For DL pre-sync, the source DU can pick at most 16 of them. But in the LTM cell switch command, only one TCI state from the 16 can be chosen by the source DU

One problem that may occur is that the network may include a beam indication (e.g. TCI State ID=X) and, shortly after a successful LTM cell switch to an LTM candidate cell, the UE declares a Beam Failure Detection (BFD) for the beam indicated in the LTM cell switch for the indicated LTM candidate cell (let’s call it the target cell once the UE has performed the LTM cell switch there). Upon BFD in the target cell, the UE triggers a Beam Failure Recovery (BFR), selects another beam (e.g. corresponding to TCI State ID=Y) and continues the communication with the target cell without a Radio Link Failure (RLF).

RAN3 to support the optimization of BFR after successful LTM cell switch.

In order to avoid these failures, it would be beneficial for the source DU to understand that it selected the wrong TCI state, and to know the TCI state in which the UE successfully recovered. It is therefore proposed to signal this information from target DU to CU and to CU to source DU.

Target DU to signal BFD and BFR information (e.g. recovery TCI state) to the CU, which will forward this information to source DU.

## 2.4	Optimization of early UL synchronization

Early UL synchronization is an important part of LTM, since it is one of the mechanisms that allows the LTM cell switch execution to take place with really short interruption. To achieve this benefit, the source DU has to ensure that the early UL synchronization takes place with some margin before the LTM cell switch execution is actually triggered. On the other hand, if the source DU lets the TA obtained through early UL synchronization become too old, there is a risk that it is outdated and incorrect at the time of the LTM cell switch execution, potentially leading to an LTM failure. How fast a TA obtained through early UL synchronization becomes outdated is highly dependent on the circumstances, e.g. if UEs generally move fast or slow in the source cell or in certain beams in the source cell. This implies that it would be useful for the source DU to receive feedback on how well the early UL synchronization it triggers perform, e.g. how large are the UEs' transmission errors when they access the LTM target cell? Was it close to being so large that successful reception/decoding would have been precluded in the target DU? It is therefore proposed to acknowledge this scenario and to propose solutions to optimize early UL synchronization.

RAN3 to study mechanisms to optimize early UL synchronization e.g. target DU signals feedback on early UL synchronization performances to source DU.

## 2.5	Optimization triggered by RLF Report and its impact on F1

In case of LTM handover failure, there are at least two types of handover optimisation that the source gNB can perform:

Firstly, for failures due to the wrong selection of candidates, during RAN3#124 it was agreed that:

For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization.

During RAN3#125, it was agreed that:

For failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report to the DU responsible for the failure.

CU needs to forward the RLF related information to DU using existing Access and Mobility Indication procedure. FFS for additional information needed.

During RAN3#125bis, Hefei, it was agreed that:

CU does not forward the RLF Report to DU if failure is due to wrong selection of prepared LTM cell.

When the CU receives an RLF report indicating that the there was a LTM cell switch failure, and the failure was not due to the wrong selection of prepared LTM cell, the CU needs to determine whether to send the RLF Report to a DU, and if so, whether to send it to the source DU or the target DU or both. There are three cases here:

- The CU can determine that the RLF report indicates a too late LTM cell switch, which occurs when the UE stayed in the cell too long, i.e. the source DU set the triggering conditions for the cell switch to “too late”. In this case the RLF report should be sent to the source DU.

- The CU can determine that the RLF report indicates a too early LTM cell switch when the UE failed during cell switch or failed quickly in the target cell. The UE bounced back to the source cell, i.e. the source DU set the triggering conditions for the cell switch too “early”. In this case the RLF report should be sent to the source DU.

- The CU can determine that the RLF report indicates a LTM cell switch to wrong cell. In this case, there are two sub cases:

  - The re-establishment cell or recovery cell was in the candidate cell list. In this case, the list of candidates was right, but the triggering conditions for the cell switch was wrong. In this case the RLF report should be sent to the source DU.

  - The re-establishment cell was not in the candidate cell list. In this case, the list of candidates configured by the CU was wrong. In this case, the RLF report should not be sent to the DU but used for optimising in the CU.

When the CU analyses an RLF report for a failed LTM cell switch, the CU can determine whether the error lies within the CU or within the source DU.

When the CU analyses an RLF report for a failed LTM cell switch, the RLF reports is either kept within the CU or sent to the source DU. There is no need to send the RLF report to the target DU.

When the CU analyses an RLF report for a LTM cell switch failure and determines that the failure lies with a DU, the CU sends the RLF report to the source DU.

## 2.6	Optimization triggered by SHR and its impact on F1

When the CU receives an SHR indicating that the there was an almost-failure during the LTM cell switch, the CU needs to determine whether to send the SHR to a DU, and if so, whether to send it to the source DU or to the target DU or both. SHRs are triggered based on how close to expiry three timers, T304, T310 and T312 are. T304 is configured by the target cell, while T310 and T312 are configured by the source cell.

There are two cases here:

      - If SHR was triggered due to T304, which was configured by the target cell, then the target DU should be notified of the report.

      - If SHR was triggered by T310 or T312, who were configured by the source cell, then the source DU should be notified of the report.

If SHR was triggered due to T304, which was configured by the target cell, then the CU should send the SHR to the target DU.

If SHR was triggered due to T310 or T312, who were configured by the source cell, then the CU should send the SHR to the source DU.

## 2.7	Optimization of LTM cell switch ping-pong

Ping-pong LTM cell switch is a condition that needs to be avoided. Optimizing configuration to avoid ping-pong is done differently depending on where the ping-pong occurs.

- For intra-DU ping-pong, the DU is expected to optimize ping-pong by itself. Therefore, no communication between nodes needs to be specified.

- For Inter-DU ping pong, it should be noted that both the source and target cells are LTM candidate cells. Therefore, both the CU and the source and target DUs have copies of the UE context, and will keep it until after a successful LTM cell switch. Inter-DU ping pong has two sub cases:

  - For Inter-DU ping pong, as both the source and target DUs keep a copy of the UE context after cell switch, both source and target DUs knows the mobility history of the UE and therefore can detect ping-pong.

  - As the CU keeps a copy of the UE context, and as DU sends a F1AP ACCESS SUCCES message (including the new target cell ID) to the CU after a LTM cell switch, the CU can also detect inter-DU ping pong.

For these reasons, there is no need to send a list of recently visited cells, for example the UHI, between any nodes involved in LTM switch ping pong.

There is no need to send any list of recently visited cells, for example the UHI, between any nodes involved in LTM switch ping pong.

# 4	References

R2-125bis_Maint_eRedCap_SON-MDT_(Mattias)_20240419_Final.docx

3GPP TS 38.300, "NR and NG-RAN Overall Description; Stage 2"

R3-243846, Summary of Offline Discussion on MRO for LTM

# 5	Conclusion

- Correct the definition of ”successful handover” in the LTM switch case. See TP in Annex 1.

- For the case of LTM failure with a subsequent successful LTM recovery, do not log the source cell ID, the time to connection failure and the last handover type.

- RAN3 to support the optimization of BFR after successful LTM cell switch.

- Target DU to signal BFD and BFR information (e.g. recovery TCI state) to the CU, which will forward this information to source DU.

- RAN3 to study mechanisms to optimize early UL synchronization e.g. target DU signals feedback on early UL synchronization performances to source DU.

- When the CU analyses an RLF report for a failed LTM cell switch, the CU can determine whether the error lies within the CU or within the source DU.

- When the CU analyses an RLF report for a failed LTM cell switch, the RLF reports is either kept within the CU or sent to the source DU. There is no need to send the RLF report to the target DU.

- When the CU analyses an RLF report for a LTM cell switch failure and determines that the failure lies with a DU, the CU sends the RLF report to the source DU.

- If SHR was triggered due to T304, which was configured by the target cell, then the CU should send the SHR to the target DU.

- If SHR was triggered due to T310 or T312, who were configured by the source cell, then the CU should send the SHR to the source DU.

- There is no need to send any list of recently visited cells, for example the UHI, between any nodes involved in LTM switch ping pong.

# Annex 1:	TP to 38.300 on MRO Use Cases

First change

##### 15.5.2.2.2	 Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of LTM cell switch, the "successful handover" in the definitions above refers to a successful completion of the RA procedure in case of RACH-based LTM or the successful reception of the first UL data from the UE in case of RACH-less LTM.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell.

Detection mechanism

A failure indication may be initiated after a UE attempts to re-establish the radio link connection at NG-RAN node B after a failure at NG-RAN node A. NG-RAN node B may initiate the Failure Indication procedure towards multiple NG-RAN nodes if they control cells which use the PCI signalled by the UE during the re-establishment procedure. The NG-RAN node receiving this selects the UE context that matches the received Failure Cell ID and C-RNTI, and, if available, uses the shortMAC-I to confirm this identification, by calculating the shortMAC-I and comparing it to the received IE.

A failure indication may also be sent to the node last serving the UE when the NG-RAN node fetches the RLF REPORT from UE by triggering:

-	The Failure Indication procedure over Xn;

-	The Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

The detailed detection mechanisms for too late handover, too early handover and handover to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

-     Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO/LTM is configured but the CHO/LTM execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt).

-     Intra-system Too Early Handover: there is a recent handover/LTM cell switch for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell/the cell UE attempts LTM recovery is the cell that served the UE at the last handover/LTM initialisation or fall back to the source cell configuration in case of DAPS HO.

-     Intra-system Handover to Wrong Cell: there is a recent handover/LTM cell switch for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO recovery/the cell UE attempts LTM recovery is neither the cell that served the UE at the last handover/LTM initialisation nor the cell that served the UE where the RLF happened or the cell that the handover/LTM was initialized toward.

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO/LTM execution until connection failure.

End of changes